IEEE P802.11
Wireless LANs

| CID 186 and 211 Normative Text |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-01-14 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Jari Jokela | Nokia | Visiokatu 3, 33720 Tampere, Finland | +358 50 486 0445 | Jari.jokela@nokia.com |

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).   The material in this document is subject to change in form and content after further study.  The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart@ok-brit.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group.  If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Abstract

This document contains proposed resolution for CID 186 and CID 211. Text is aligned with 802.11v D0.07 and P802.11-REVma 9.0.

# Part 1 – CID 186

This part of the document includes normative text related to CID 186

Change the first sentence of 7.3.2.33 as follows:

      - TCLAS Processing element

The TCLAS Processing element is present in the ADDTS Request, ADDTS Response and FBMS Request frames if there are multiple TCLASs associated with the request.

Change 7.3.2.45 as follows:

      - FBMS Request element

The FBMS Request element defines information about the Broadcast/Multicast frames being requested by the non-AP STA. The format of FBMS Request element is shown in Figure v78.

|  | Element ID | Length | Multicast Element Count | FBMS Element 1 | FBMS Element n |
| --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | variable | variable |

- —FBMS Request element format

The Length field is set to 1+n, where n indicates the total length of all FBMS Elements contained in the element.

The Multicast Element Count indicates the number of the FBMS Elements present.

The format of the FBMS Element is shown in Figure v79.

|  | TCLAS Element | Optional TCLAS Element | Optional TCLAS Element | TCLAS Processing Element | Delivery Interval | Multicast Rate |
| --- | --- | --- | --- | --- | --- | --- |
| Octets: | variable | variable | variable | 3 | 1 | 1 |

- —FBMS Element format

The TCLAS Information Element defines the broadcast/multicast stream as defined in 7.3.2.31.

Optionally multiple TCLAS Elements are permitted to classify the Broadcast/Multicast stream.

The TCLAS Processing Element is optionally present and defines how multiple TCLAS Elements are processed as defined in 7.3.2.33.

The Delivery Interval defines the number of DTIMs that the stream will be transmitted at. The default value is 1.

The Multicast Rate specifies the highest data rate at which the STA can reliably receive multicast frames. If no value is provided by the STA, this field is set to 0.

# Part 2 – CID 211

This part of the document includes normative text related to CID 211

Change 7.3.2.41 as follows:

      - Multiple BSSID element

The format of the Multiple BSSID element is shown in Figure v73.

|  | Element ID | Length | Max BSSID indicator | Non-Transmitted BSSID Profile |
| --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | variable |

- —Multiple BSSID element format

The Element ID field is equal to the Multiple BSSID value in Table 26.

The value of Length field is the length of the Non-Transmitted BSSID profile (variable) + 1. More than one Multiple BSSID elements may be included in a Beacon frame. The AP determines the number of Multiple BSSID elements, and shall not fragment a Non-Transmitted BSSID Profile element across two Multiple BSSID elements.

The Max BSSID Indicator is n, where 2n is the maximum number of BSSIDs supported by the AP, including the transmitted BSSID. The actual number of SSIDs supported by the AP is not explicitly signalled. The non-transmitted BSSID(i) value corresponding to the ith  BSSID profile (indicated by the BSSID index)  is derived from the transmitting AP’s BSSID (AP_BSSID) as follows:

BSSID(i) = (AP_BSSID modified to set the  n LSBs to zero) | ((n LSBs of AP_BSSID) + i) mod 2n)

As shown in Table v45, the Non-Transmitted BSSID profile may include the Capability, Supported Rates, CF Parameter Set, Power Constraint, Quiet, TPC Report, Extended Supported Rates, RSN and Vendor Specific information element fields for each non-transmitted BSSID. The Timestamp, Beacon Interval, DS Parameter Set, FH Parameter Set, IBSS Parameter Set, Country, FH Parameters, FH Pattern Table, Channel Switch Assignment, IBSS DFS, and ERP Information element values for each non-transmitted BSSID is not included in the Multiple BSSID element, and is always the transmitted BSSID element values.  AID 0 Descriptor element shall be included if Multiple BSSID element is sent in Beacon frame and if TIM field indicates there are buffered broadcast and/or multicast frames for this non-transmitted BSSID.

- —Non-Transmitted-BSSID Profile

| Information | Notes |
| --- | --- |
| Timestamp | Not Included |
| Beacon Interval | Not Included |
| Capability | Optional |
| Service Set Identifier (SSID) | Included |
| Supported Rates | Optional |
| Frequency Hopping (FH Parameter Set) | Not Included |
| DS Parameter Set | Not Included |
| CF Parameter Set | Optional |
| IBSS Parameter Set | Not Included |
| Country | Not Included |
| FH Parameters | Not Included |
| FH Pattern Table | Not Included |
| Power Constraint | Optional |
| Channel Switch Announcement | Not Included |
| Quiet | Optional |
| IBSS DFS | Not Included |
| TPC Report | Optional |
| ERP Information | Not Included |
| Extended Supported Rates | Optional |
| RSN | Optional |
| QBSS Load | Optional |
| EDCA Parameter Set | Optional |
| QoS Capability | Optional |
| Multiple BSSID | Not Included |
| Multiple BSSID-Index | Included |
| AID 0 Descriptor | Included |
| Vendor Specific | Optional |

The Multiple BSSID-Index field includes the DTIM Count and DTIM Period for the non-transmitted BSSIDs. When the Multiple BSSID element is present in the probe request frame, one or more of the SSID elements may have a NULL SSID or zero element length to indicate a broadcast probe request.

Change the first sentence of 9.2.7.1.1  as follows:

        - FBMS operation

The non-AP STA that wishes to use the FBMS shall send a FBMS Request or (Re)association Request with all FBMS elements for which it wants to subscribe. This is a declaration of all streams in which the STA is interested. For each stream, the STA proposes a delivery interval for the requested FBMS element. The AP can adopt the proposed delivery interval or provide an alternate delivery interval for the stream. A status value of Accept is transmitted by the AP when the requested delivery interval is supported by the AP. A status value of Deny is transmitted by the AP when the AP denies the STA’s requested delivery interval and TCLAS completely. A status value of Override is transmitted by the AP when the AP denies the requested delivery interval but can support an alternate delivery interval for the requested TCLAS. The STA must comply with the AP’s override value. If the STA does not accept this overridden rate, then the STA must send a new request with the TCLAS Element removed.

The FBMS delivery interval is always an integer multiple of the DTIM period. If the AP denies the usage of FBMS for a particular traffic stream, normal broadcast and multicast transmission rules apply.

The AP shall support up to eight different delivery intervals. Corresponding to these eight delivery intervals are eight FBMS counters. Each counter decrements once per DTIM beacon and when the counter reaches zero, the delivery interval expires. Upon expiry, the AP schedules for transmission any frames present for broadcast/multicast streams assigned to that interval. Upon request from the STA, an AP assigns broadcast/multicast streams to a particular ID (the FBMSID), negotiates the Delivery Interval and assigns a counter (the FBMS Counter ID) using the FBMS Element.

An AP uses the AID 0 Descriptor element in Beacon frames to indicate to which broadcast or multicast addresses the buffered broadcast/multicast frames are targeted. This element is present only if the bit for AID 0 is set to 1. In case of non-transmitted BSSIDs AID 0 Descriptor element for non-transmitted BSSID is included in Multiple BSSID element sent in Beacons. AID 0 Descriptor element is present in Multiple BSSID element only if TIM field indicates there are buffered broadcast and/or multicast frames for the non-transmitted BSSID.

A non-AP STA may indicate that it is not using a particular FBMS Element anymore by transmitting a FBMS Request frame without that FBMS Element contained in it. The AP shall send a FBMS response with FBMS status value set to OK, upon receipt of the FBMS Request.

If an AP receives a FBMS Request for a FBMS stream which has already been assigned to a particular delivery interval (and FBMS Counter ID), the AP may adjust the corresponding FBMS current count so as to align the transmission time of said stream to the transmission time of other FBMS streams for which the STA is already receiving.  The AP accomplishes this by changing the current count. The Current Count shall be changed only by holding the value of the field same in two consecutive Beacons in which the Current Count field appears. The algorithm by which the AP chooses to align or offset the different FBMS counters is unspecified.

AP may update the Delivery Interval for a FBMSID by sending an unsolicited FBMS response to the appropriate address with updated Delivery Interval when the Current Count reaches zero.