IEEE P802.11
Wireless LANs

| Normative Text Proposal for Traffic Generation Information for Proactive Load Management |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-01-14 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Moo Ryong Jeong, Fujio Watanabe, Hiroshi Inamura | DoCoMo USA Labs. | 3240 Hillview Ave, Palo Alto, CA 94304-1201, USA | +1-650-496-4761 | jeong@docomolabs-usa.com |
| Akira Yamada | NTT DoCoMo Inc. | 3-5, Hikarinooka,Yokosuka-shi, Kanagawa 239-8536, Japan | +81-46-840-6527 | yamadaakir@nttdocomo.co.jp |
| Lei Du, Yong Bai, Lan Chen | DoCoMo Beijing Labs Inc. | 7/F Raycom Infotech Park Tower A, No.2 Kexueyuan South Road, Haidian District, Beijing, P.R.China | +86-10-82861501 | du@docomolabs-beijing.com.cn |

Abstract

This document contains a normative text proposal for traffic generation information for proactive load management to improve QoS effectiveness in accordance with Req 2030.

Choose one from the option 1 and the option 2.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart@ok-brit.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

I. Option 1

7. Frame formats

- 7.2 Format of individual frame types

- 7.2.3 Management frames

- 7.2.3.4 Association Request frame format

##### Insert a new row into table 10 as follows:

Table 10—Associstion Request frame body

| Order | Information | Notes |
| --- | --- | --- |
| 12 | Traffic Generation | The Traffic Generation element is present if dot11WirelessManagementImplemented is true. |

- 7.2.3.6 Reassociation Request frame format

##### Insert a new row into table 12 as follows:

Table 12—Reassociation Request frame body

| Order | Information | Notes |
| --- | --- | --- |
| 15 | Traffic Generation | The Traffic Generation element is present if dot11WirelessManagementImplemented is true. |

- 7.3 Management frame body components

### 7.3.2 Information Elements

### Change the table 26 as follows:

Table 26－Element IDs

| Information Element | Element ID | Length (in octets) |
| --- | --- | --- |
| Traffic Generation (see 7.3.2.49) | 71 | 2 |
| Reserved | 72-126 |  |

- Insert the following clause after 7.3.2.48:

##### 7.3.2.49 Traffic Generation Element

The Traffic Generation element contains a field for Traffic Generation Flags as shown in Figure X.

|  | Element ID | Length | Traffic Generation Flags |
| --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 |

Figure X---Traffic Generation Element format

The format of Traffic Generation Flag field is defined in Table 49a.

Table 49a－Traffic Generation Flags Definition

| Bit (s) | Description |
| --- | --- |
| 0 | UP 0 Traffic |
| 1 | UP 1 Traffic |
| 2 | UP 2 Traffic |
| 3 | UP 3 Traffic |
| 4 | UP 4 Traffic |
| 5 | UP 5 Traffic |
| 6 | UP 6 Traffic |
| 7 | UP 7 Traffic |

Each bit in the Traffic Generation Flag field serves as a flag for one of the eight user priorities. The bit is set to 1 to indicate the expectation of generating traffic belonging to the corresponding user priority (UP). The bit is set to 0 to indicate that such expectation does not exist.

Traffic Generation Flags are constructed at the SME. There are no normative requirements on how Traffic Generation Flags are to be constructed except that a Flag bit shall be set to 0 if there is no information available to SME enough to support the expectation of traffic generation. However, in the following two paragraphs, an informative description is given of how the Flags may be constructed.

The Traffic Generation Flags may be constructed from two aspects of traffic generation by applications: whether traffic generation is required for applications and whether a specific UP is required for the generated traffic. Note that such aspects may be known before the traffic is actually generated. For example, an application may be configured to generate traffic upon the initiation of a user session and to use a specific UP if traffic is generated. Such configuration can be done before the session initiation, and during the time between the configuration and the session initiation the application is in standby, waiting for the initiation of a user session. Phone application is a typical example of such applications.

The aspects of traffic generation by an application may be supplied to SME on initial configuration or on any changes of configuration by the application. The realization of traffic generation expectation, that is, actual traffic generation, is not considered as a change of configuration. When the traffic generation aspects are supplied, the Traffic Generation Flags are updated accordingly.

If there is any change in Traffic Generation Flags after most recent association or reassociation, a non-AP STA shall send to the serving AP a reassociation request message with updated Traffic Generation element.

It is up to vendor implementation how Traffic Generation elements are to be interpreted and used by APs. However, an informative description is given in this paragraph. An AP may maintain station counts for each user priority (UP); that is, the number of STAs that are associated with the AP and expected to generate traffic belonging to each UP is maintained. Similarly, an AP may maintain station counts for each access category (AC). Those station counts may be interpreted as the number of STAs that are expected to access channel to transmit MSDUs of  the corresponding UP or AC. Accordingly, AP and network may use those station counts for the purpose of BSS load management that is defined elsewhere in the standard. Note that if STAs are not generating traffic, actual load or channel access by the STAs does not exist yet. Because such potential load can be managed proactively before it becomes real load, load management based on Traffic Generation information can be effective to control blocking probability (probability that a successful user session will not be achieved), an aspect of quality a user experiences when initiating a session. Phone application is a typical example that benefits from proactive load management to control blocking probability.

- 10. Layer management

- 10.3 MLME SAP Interface

- 10.3.6 Associate

- 10.3.6.1 MLME-ASSOCIATE.request

- 10.3.6.1.2 Semantics of the service primitive

- Change the primitive parameter list as shown:

MLME-ASSOCIATE.request	(
PeerSTAAddress,
AssociateFailureTimeout,
CapabilityInformation,
ListenInterval,
Supported Channels,
RSN,
QoSCapability,
WirelessManagementCapabilities,
SupportedRegulatoryClasses, 
TrafficGeneration)

Insert the following row at the end of the parameter table:

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Traffic Generation | As defined in the Traffic Generation element | As defined in the Traffic Generation element | Specifies the Traffic Generation Flags of the non-AP STA. This parameter shall only be present if the MIB attribute dot11WirelessManagementImplemented is true. |

- 10.3.6.2 MLME-ASSOCIATE.confirm

- 10.3.6.2.2 Semantics of the service primitive

Change the primitive parameter list as shown:

MLME-ASSOCIATE.confirm                              (
ResultCode,
CapabilityInformation,
AssociationID,
SupportedRates,
EDCAParameterSet,
WirelessManagementCapabilities,
SupportedRegulatoryClasses
TrafficGeneration)

Insert the following row at the end of the parameter table:

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Traffic Generation | As defined in the Traffic Generation element | As defined in the Traffic Generation element | Specifies the Traffic Generation Flags of the non-AP STA. This parameter shall only be present if the MIB attribute dot11WirelessManagementImplemented is true. |

- 10.3.6.3 MLME-ASSOCIATE.indication

- 10.3.6.3.2 Semantics of the service primitive

Change the primitive parameter list as shown:

MLME-ASSOCIATE.indication	(
PeerSTAAddress,
CapabilityInformation,
ListenInterval,
SSID,
Supported Rates,
RSN,
QoSCapability,
WirelessManagementCapabilities,
SupportedRegulatoryClasses
TrafficGeneration)

Insert the following row at the end of the parameter table:

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Traffic Generation | As defined in the Traffic Generation element | As defined in the Traffic Generation element | Specifies the Traffic Generation Flags of the non-AP STA. This parameter shall only be present if the MIB attribute dot11WirelessManagementImplemented is true. |

- 10.3.6.4 MLME-ASSOCIATE.response

- 10.3.6.4.2 Semantics of the service primitive

Change the primitive parameter list as shown:

MLME-ASSOCIATE.response	(
PeerSTAAddress,
ResultCode,
CapabilityInformation,
AssociationID,
WirelessManagementCapabilities,
SupportedRegulatoryClasses
TrafficGeneration)

Insert the following row at the end of the parameter table:

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Traffic Generation | As defined in the Traffic Generation element | As defined in the Traffic Generation element | Specifies the Traffic Generation Flags of the non-AP STA. This parameter shall only be present if the MIB attribute dot11WirelessManagementImplemented is true. |

Change 10.3.7 as follows:

- 10.3.7 Reassociate

- 10.3.7.1 MLME-REASSOCIATE.request

- 10.3.7.1.2 Semantics of the service primitive

Change the primitive parameter list as shown:

MLME-REASSOCIATE.request	(
NewAPAddress,
ReassociateFailureTimeout,
CapabilityInformation,
ListenInterval,
Supported Channels,
RSN,
QoSCapability,
WirelessManagementCapabilities,
FBMSRequest,
SupportedRegulatoryClasses
TrafficGeneration)

Insert the following row at the end of the parameter table:

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Traffic Generation | As defined in the Traffic Generation element | As defined in the Traffic Generation element | Specifies the Traffic Generation Flags of the non-AP STA. This parameter shall only be present if the MIB attribute dot11WirelessManagementImplemented is true. |

- 10.3.7.2 MLME-REASSOCIATE.confirm

- 10.3.7.2.2 Semantics of the service primitive

Change the primitive parameters as shown:

MLME-REASSOCIATE.confirm	(
Resultcode,
CapabilityInformation,
AssoicationID,
SupportedRates,
EDCAParameterSet,
WirelessManagementCapabilities,
FBMSRequest,
SupportedRegulatoryClasses
TrafficGeneration)

Insert the following row at the end of the parameter table:

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Traffic Generation | As defined in the Traffic Generation element | As defined in the Traffic Generation element | Specifies the Traffic Generation Flags of the non-AP STA. This parameter shall only be present if the MIB attribute dot11WirelessManagementImplemented is true. |

- 10.3.7.3 MLME-REASSOCIATE.indication

- 10.3.7.3.2 Semantics of the service primitive

Change the primitive parameter list as shown:

MLME-REASSOCIATE.indication	(
PeerSTAAddress,
CurrentAPAddress,
CapabilityInformation,
ListenInterval,
SSID,
SupportedRates,
RSN,
QoSCapability,
WirelessManagementCapabilities,
FBMSRequest,
SupportedRegulatoryClasses
TrafficGeneration)

Insert the following row at the end of the parameter table:

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Traffic Generation | As defined in the Traffic Generation element | As defined in the Traffic Generation element | Specifies the Traffic Generation Flags of the non-AP STA. This parameter shall only be present if the MIB attribute dot11WirelessManagementImplemented is true. |

- 10.3.7.4 MLME-REASSOCIATE.response

- 10.3.7.4.2 Semantics of the service primitive

Change the primitive parameter list as shown:

MLME-REASSOCIATE.response	(
PeerSTAAddress,
ResultCode,
CapabilityInformation,
AssociationID,
WirelessManagementCapabilities,
FBMSRequest,
SupportedRegulatoryClasses
TrafficGeneration)

Insert the following row at the end of the parameter table:

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Traffic Generation | As defined in the Traffic Generation element | As defined in the Traffic Generation element | Specifies the Traffic Generation Flags of the non-AP STA. This parameter shall only be present if the MIB attribute dot11WirelessManagementImplemented is true. |

II. Option 2

Change 7.3.2.35 as follows:

- 7.3.2.35 Wireless Network Management Capability Information element

The Wireless Network Management Capability Information element contains information about the wireless network management capabilities of a STA as shown in Figure v1.

|  | Element ID | Length | Wireless Network Management Capabilities |
| --- | --- | --- | --- |
| Octets: | 1 | 1 | 2 |

- —Wireless Network Management Capability Information element format

The Element ID field is equal to the Wireless Network Management Capability value in Table 26.

The value of the length field is variable and depends on the length of the Wireless Network Management Capabilities field.  The minimum value of the Length field is 2.

The Wireless Network Management Capabilities field is a bit field indicating the advertised management capabilities of the STA. The Wireless Network Management Capabilities field is shown in Figure v2.

|  | B0 | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Event Log | Diagnostics | Multicast Alert | Presence | FBMS | Proxy ARP Service | Voice Service | Reserved |  |
| Bits: | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 9 |  |

- —Wireless Network Management Capabilities

- The Event Log bit set to 1 indicates the STA supports Event Log as described in Error: Reference source not found. The Event Log bit set to 0 indicates that the STA does not support this service.

- The Diagnostics bit set to 1 indicates the STA supports Diagnostics as described in Error: Reference source not found. The Diagnostics bit set to 0 indicates that the STA does not support this service.

- The Multicast Alert bit set to 1 indicates the STA supports Multicast diagnostics as described in Error: Reference source not found. The Multicast Alert bit set to 0 indicates that the STA does not support this service.

- The Presence bit set to 1 indicates that the STA supports Presence as described in clause Error: Reference source not found. The Presence bit set to 0 indicates that the STA does not support this service.

- The FBMS bit set to 1 indicates the STA supports FBMS as described in section Error: Reference source not found. The FBMS bit set to 0 indicates the STA does not support FBMS.

- The Proxy ARP Service bit set to 1 indicates the AP is providing proxy ARP service.  If Proxy ARP service is enabled, then the AP responds to broadcast ARP request on behalf of the STA. The Proxy ARP Service bit set to 0 indicates the AP is not providing proxy ARP service for any associated client.

- The Voice Service bit set to 1 indicates that the STA supports voice service. The Voice Service bit set to 0 indicates that the STA does not support voice service. The Voice Service bit is set from the configuration of a voice application, supplied via SME. A STA is capable of voice service if a voice application is configured to initiate voice service upon arrival or generation of voice session, and it is not capable of voice service otherwise. Voice service capability of a STA may change upon initial configuration or configuration change of the voice application. If voice service capability of a STA is changed after a most recent association or reassociation, the STA shall send to the serving AP a reassociation request frame with updated Voice Service bit. Initiation of a voice service is not considered as a change of configuration.

- All other bits are reserved, and are set to 0 on transmission and ignored on reception.

The lack of a Wireless Network Management Capability element is interpreted as the STA having no advertised Wireless Network Management Capabilities.