IEEE P802.11
Wireless LANs

| Resolution of TGT Internal Review CIDs 266 and 296 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-01-14 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Sangeetha Bangolae | Intel Corporation | 2111 NE 25th AVE, Hillsboro, OR 97124 | 503-264-6799 | sangeetha.l.bangolae@intel.com |
| Marc Emmelmann | Technical University of Berlin | Einsteinufer 25 10587 Berlin Germany | +49 (0)30/314-24580 | emmelmann@ieee.org |
| Fanny Mlinarsky | Octoscope | info@octoscope.com | 978-376-5841 | fm@octoscope.com |

Abstract

Resolution for comments 266 and 296

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart@ok-brit.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

# 6.7.1 Introduction and purpose

Change the last paragraph as follows:

These results may be used to produce an estimated cumulative distribution function (CDF) of the transition time. This metric is expected to be useful for latency-sensitive usage cases, such as packet voice over wireless networks, or for any other usage case where interruption of network service due to roaming potentially degrades the user experience.

# 6.7.4.3 Measurement procedure

Change the second paragraph as follows:

Additionally the following time components, if available, can be recorded as part of the measurement report

for different modifiers (where applicable):

  - Probe request/response exchange with the target AP after the last data packet send within the old BSS, Tprobe

  - Open System Authentication/Shared WEP authentication exchange, Tauth ,

  - (Re)Association time, T(re)assoc

  - 802.1x EAP authentication method and Key Exchange, T802.1x  , Tkey

If an IEEE Std 802.11e handshake is part of the Re-association process, then the time taken to exchange the

IEEE Std 802.11e messages should be included as part of this report as well. The IEEE Std 802.11e

exchange time is referred to as T802.11e.

Replace Figure 32, Figure 33, Figure 34 and Figure 35  with the following single figure:

Figure 32 Sample illustration of total BSS Transition Time

# 6.8.1 Introduction and purpose

Change the last paragraph as follows:

These results may be used to produce an estimated cumulative distribution function (CDF) of the transition time. This metric is expected to be useful for latency-sensitive usage cases, such as packet voice over wireless networks, or for any other usage case where interruption of network service due to roaming potentially degrades the user experience.

# 6.8.4.3 Measurement procedure

Change the second paragraph of the clause as follows:

- Additionally the following time components, if available, can be recorded as part of the measurement report for Fast BSS Transition Time (Over the Air or Over the DS):

  - Probe request/response exchange with the target AP after the last data packet send within the old BSS, Tprobe

  - Fast BSS Transition initiation exchange, TFTauth

  - (Re)Association time , T(re)assoc

If an IEEE Std 802.11e handshake is part of the Re-association process, then the time taken to exchange the

IEEE 802.11e messages should be included as part of this report as well. The IEEE Std 802.11e exchange

time is referred to as T802.11e.

Replace Figure 37, Figure 38, Figure 39 and Figure 40  with the following single figure:

Figure 34. Sample illustration of total Fast BSS Transition time.

References:

IEEE 802.11-05-0537r3, “Test Methodology Proposal for Measuring Fast BSS/BSS Transition Time”, S. Bangolae, et al.