IEEE P802.11
Wireless LANs

| Access Point Collaboration for enhancing QoS and Spectrum Efficiency |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-01-05 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Alex Ashley | NDS Ltd | NDS Ltd, One London Road, Staines, Middlesex, TW18 4EX, UK | +44 1784 848770 | aashley@nds.com |

Abstract

This document describes a mechanism to allow multiple APs on the same channel to cooperatively share time on the wireless medium. This enables enhanced QoS by allowing collaborating APs to avoid allocating overlapping CFPs.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart@ok-brit.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Background:

The spectrum assignments in which IEEE 802.11 can be used are a finite resource that is getting more congested as 802.11 becomes more popular. The use of this spectrum can be made more efficient by techniques such as intelligent channel selection, transmit power control and time division. It is the opinion of the author of this document that the first two techniques already have technologies within 802.11 drafts that could be used to enable them. This document describes a method to allow access points to collaborate by sharing time on the wireless medium. This method is compatible to legacy APs and STAs and is robust to abuse by a rogue AP.

The proposal is designed to fulfil item 2a “Access Point Coordination – Intra ESS” in the TGV accepted work list [1].

Modify Clause 7.2.3.1 as indicated below:

7.2.3.1 Beacon frame format

Insert new row in Table 8 as indicated below:

- Table 8—Beacon frame body

| Order | Information | Notes |
| --- | --- | --- |
| 30 | AP Collaboration | The AP Collaboration element is present only within Beacon frames generated by APs which support AP Collaboration and if dot11WirelessManagementImplemented is true. |

Modify Clause 7.2.3.9 as indicated below:

7.2.3.9 Probe Response frame format

Insert new row into table 15 as follows:

Table 15—Probe Response frame body

| Order | Information | Notes |
| --- | --- | --- |
| 26 | AP Collaboration | The AP Collaboration element is present only within probe response frames generated by APs which support AP Collaboration and if dot11WirelessManagementImplemented is true. |

Modify Clause 7.3.2 as indicated below:

7.3.2 Information Elements

Insert Element ID into Table 26 and change the Reserved row accordingly:

Table 26—Element IDs

| Information Element | Element ID |
| --- | --- |
| AP Collaboration Capability | x+14 |
| Reserved | x+15, 220 |

Modify Clause 7.3.2.23 as indicated below:

7.3.2.23 Quiet element

The Quiet element may be included in Beacon frames, as described in 7.2.3.1, and Probe Response frames, as described in 7.2.3.9, CFP Offer frames, as described in 7.4.2.5, and CFP Offer Response frames, as described in 7.4.2.6. The use of Quiet elements is described in 11.9.2 and 11.9.8.

Insert the following new clauses after 7.3.2.48:

7.3.2.49 AP Collaboration element

The AP Collaboration element defines information about the inter-AP collaboration features supported by the AP. The format of the AP Collaboration element is shown in figure v84.

|  | Element ID | Length | AP Collaboration Capabilities |
| --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 |

Figure v84—AP Collaboration information element

The length field shall be set to one.

The format of the AP Collaboration Capabilities field is defined in table v52.

Table v52—AP Collaboration Capabilities Bit description

| Bit | Capability | Description |
| --- | --- | --- |
| 0 | Time Collaboration | AP supports CFP Offer action |
| 1-7 | Reserved |  |

The Time Collaboration capability shall be set to true if dot11WirelessManagementImplemented is true and the AP supports sharing of time on the wireless medium and implements the CFP Offer and CFP Offer Response action frames. Otherwise this capability bit shall be set to false.

Modify Clause 7.4.2 as indicated below:

7.4.2 QoS Action frame details

Insert the following new rows into Table 45 and change the reserved value as shown:

Table 45—QoS Action field values

| Code | Meaning |
| --- | --- |
| 4 | CFP Offer |
| 5 | CFP Offer Response |
| 6-255 | Reserved |

Insert the following new clauses after 7.4.2.4:

7.4.2.5 CFP Offer frame format

The CFP Offer frame is transmitted by an AP to another AP to provide an offer of a period of time when the offering AP intends the BSS to be silent on the channel used to send this frame. The contents of this frame are shown in table v53.

Table v53—CFP Offer frame body

| Order | Information |
| --- | --- |
| 1 | Category |
| 2 | Action |
| 3 | Quiet |

The Category field is set to 1 (representing QoS).

The Action field is set to 4 (representing CFP Offer).

The Quiet field contains the time period that is being offered and is defined in sub-clause 7.3.2.23

The use of the CFP Offer frame is described in 11.9.8

7.4.2.6 CFP Offer Response frame format

The CFP Offer Response frame is transmitted in response to a CFP Offer frame. The frame body of the

CFP Offer Response frame contains the information shown in table v54.

Table v54—CFP Offer Response frame body

| Order | Information |
| --- | --- |
| 1 | Category |
| 2 | Action |
| 3 | Quiet |
| 4 | Status Code |

The Category field is set to 1 (representing QoS).

The Action field is set to 5 (representing CFP Offer Response).

The Quiet field contains the time period that was offered and is defined in sub-clause 7.3.2.23

The Status Code field contains the response to the time period that was offered and is defined in sub-clause 7.3.1.9

Insert the following new clauses after 10.3.40:

10.3.41 CFP offer request

This set of primitives supports the initiation of time-period based collaboration between peer SMEs.

10.3.41.1 MLME-CFPOFFER.request

10.3.41.1.1 Function

This primitive requests the transmission of a CFP Offer to a peer entity.

10.3.41.1.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-CFPOFFER.request	(
Peer MAC Address,
CFP Offer
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Peer MAC Address | MACAddress | Any valid individual MAC Address | The address of the peer MAC entity to which the CFP Offer shall be sent. |
| CFP Offer | Quiet element | Quiet element | A quiet element describing the offered silence period. See 7.3.2.23 |

10.3.41.1.3 When generated

This primitive is generated by the SME to request that a CFP Offer frame be sent to a peer entity to convey an offer of a period of silence on a channel.

10.3.41.1.4 Effect of receipt

On receipt of this primitive, the MLME constructs a CFP Offer frame containing the quiet element specified. This frame is then scheduled for transmission

10.3.41.2 MLME-CFPOFFER.confirm

10.3.41.2.1 Function

This primitive reports the result of a CFP Offer request.

10.3.41.2.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-CFPOFFER.confirm	(
CFP Offer,
ResultCode
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| CFP Offer | Quiet element | Quiet element | A quiet element describing the offered silence period. See 7.3.2.23 |
| ResultCode | Enumeration | SCCESS, INVALID PARAMETERS, or UNSPECIFIED FAILURE | Reports the outcome of a request to send a CFP Offer frame. |

10.3.41.2.3 When generated

This primitive is generated by the MLME when the request to transmit a CFP Offer frame completes.

10.3.41.2.4 Effect of receipt

On receipt of this primitive, the SME evaluates the result code.

10.3.41.3 MLME-CFPOFFER.indication

10.3.41.3.1 Function

This primitive indicates that a CFP Offer frame has been received requesting an AP collaboration transaction.

10.3.41.3.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-CFPOFFER.indication	(
Peer MAC Address,
CFP Offer
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Peer MAC Address | MACAddress | Any valid individual MAC Address | The address of the peer MAC entity from which the CFP Offer was sent. |
| CFP Offer | Quiet element | Quiet element | A quiet element describing the offered silence period. See 7.3.2.23 |

10.3.41.3.3 When generated

This primitive is generated by the MLME when a valid CFP Offer frame is received.

10.3.41.3.4 Effect of receipt

On receipt of this primitive, the SME either rejects the request or decides whether to accept or reject the collaboration offer.

10.3.42 CFP offer response

This set of primitives supports the response to a request for time-period based collaboration between peer SMEs.

10.3.42.1 MLME-CFPOFFERRESPONSE.request

10.3.42.1.1 Function

This primitive requests the transmission of a CFP Offer Response to a peer entity.

10.3.42.1.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-CFPOFFERRESPONSE.request	(
Peer MAC Address,
CFP Offer,
ResultCode
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Peer MAC Address | MACAddress | Any valid individual MAC Address | The address of the peer MAC entity to which the CFP Offer Response shall be sent. |
| CFP Offer | Quiet element | Quiet element | A quiet element describing the silence period that was offered. See 7.3.2.23 |
| ResultCode | Enumeration | SUCCESS, REQUEST DENIED, INVALID PARAMETERS, or UNSPECIFIED FAILURE | Indicates the result response to the CFP offer from the peer MAC entity. |

10.3.42.1.3 When generated

This primitive is generated by the SME to request that a CFP Offer Response frame be sent to a peer entity to convey the response to an offer of a period of silence on a channel.

10.3.42.1.4 Effect of receipt

On receipt of this primitive, the MLME constructs a CFP Offer Response frame containing the quiet element specified. This frame is then scheduled for transmission

10.3.42.2 MLME-CFPOFFERRESPONSE.confirm

10.3.42.2.1 Function

This primitive reports the result of a CFP Offer Response request.

10.3.42.2.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-CFPOFFERRESPONSE.confirm	(
CFP Offer,
ResultCode
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| CFP Offer | Quiet element | Quiet element | A quiet element describing the offered silence period. See 7.3.2.23 |
| ResultCode | Enumeration | SUCCESS, INVALID PARAMETERS, or UNSPECIFIED FAILURE | Reports the outcome of a request to send a CFP Offer Response frame. |

10.3.42.2.3 When generated

This primitive is generated by the MLME when the request to transmit a CFP Offer Response frame completes.

10.3.42.2.4 Effect of receipt

On receipt of this primitive, the SME evaluates the result code.

10.3.42.3 MLME-CFPOFFERRESPONSE.indication

10.3.42.3.1 Function

This primitive indicates that a CFP Offer Response frame has been received.

10.3.42.3.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-CFPOFFERRESPONSE.indication	(
Peer MAC Address,
CFP Offer,
ResultCode
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Peer MAC Address | MACAddress | Any valid individual MAC Address | The address of the peer MAC entity from which the CFP Offer Response was received. |
| CFP Offer | Quiet element | Quiet element | A quiet element describing the silence period that was offered. See 7.3.2.23 |
| ResultCode | Enumeration | SUCCESS, REQUEST DENIED, INVALID PARAMETERS, or UNSPECIFIED FAILURE | Indicates the result response to the CFP offer from the peer MAC entity. |

10.3.42.3.3 When generated

This primitive is generated by the MLME when a valid CFP Offer Response frame is received.

10.3.42.3.4 Effect of receipt

On receipt of this primitive, the SME either rejects the response or commences the collaboration transaction as described in 11.9.8.

11.9 DFS procedures

Insert the following new clause after 11.9.7:

11.9.8 Quieting channels for AP collaboration

An AP in a BSS may schedule quiet intervals by transmitting one or more Quiet elements in Beacon frames and Probe Response frames as described in 11.9.2. Typically these quiet intervals are used to provide time to perform tests on the channel. These quiet intervals can also be used to provide time-period based collaboration between APs operating on the same channel.

An AP that wishes to offer time to another AP that supports AP collaboration sends a CFP Offer frame to the recipient AP. An AP shall not send CFP Offer frames to an AP that does not advertise its ability to collaborate using the “AP Collaboration Capability” information element.

An AP that supports collaboration shall respond to a CFP Offer frame with a CFP Offer Response Frame. This response contains a copy of the offer and the decision by the recipient AP to accept or reject this offer. If the offer is accepted, the AP that made the offer should attempt to silence the BSS for the quiet interval specified in the offer. The method used to silence the BSS is not defined. Using silence elements in beacons and using CTS frames are examples of methods that could be used. It is permissible for an AP making a CFP offer to fail to honor an accepted offer.

The Quiet Count field, Quiet Duration field, and the Quiet Offset field in Quiet elements define the starting time and duration of the period of silence. The Quiet Period field in a CFP Offer shall be set to zero.

This specification does not mandate any rules for when an AP should make a CFP Offer or when it should accept or reject an offer. It is recommended to use an algorithm that rewards an AP for providing CFP Offers by reciprocation. An example of such an algorithm is “tit-for-tat” where an AP makes an initial CFP Offer and then only ever makes further offers if it receives a CFP Offer from another AP.

Insert the following new rows into A.4.14:

### A.4.14 Wireless Network Management extensions

| Item | Protocol Capability | References | Status | Support |
| --- | --- | --- | --- | --- |
| RME10 | AP Collaboration | 11.9.8 | (CFv&SM7):O | Yes, No, N/A |
| RME10.1 | CFP Offer | 7.4.2.5 | (CF1&RME10):M | Yes, No, N/A |
| RME10.2 | CFP Offer Response | 7.4.2.6 | (CF1&RME10):M | Yes, No, N/A |

References:

[1]	11-05-0796-03-000v-tgv-work-list.doc