IEEE P802.11
Wireless LANs

| A More Efficient KDF |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-01-15 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Dan Harkins | Tropos Networks | 555 Del Rey Ave | +1 408 470 7372 | dharkins@tropos.com |
| Jesse Walker | Intel Corporation | 211 NE 25th Ave | +1 503 712 1849 | jesse.walker@intel.com |

Abstract

This document proposes a more efficient KDF for use in 802.11r

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart@ok-brit.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

The current KDF derivation in the draft uses HMAC-SHA256. SHA256 by itself is a very computationally intensive hashing algorithm (twice as slow as SHA-1). Using it in HMAC form requires two SHA256 operations and is therefore even more computationally intensive.

The draft defines AES-CMAC for MAC computation and using AES-CMAC as a PRF would allow us to eliminate the number of cryptographic primitives necessary to implement 802.11r to one: AES. Can AES-CMAC be used as a PRF? A PRF is a function whose output is infeasible to distinguish from the equivalent-length output of a truly random function. There is a theorem from Goldreich, Goldwasser and Micali that says a cryptographically secure pseudorandom function can be constructed from cryptographically secure pseudorandom bit generators. Is AES a cryptographically secure pseudorandom bit generator? Well AES in CMAC mode is!

Furthermore, recent work by Phillip Rogaway and Thomas Shrimpton on the subject of cryptographic key-wrapping has illustrated the concept of a PRF that acts on a vector of strings (they call it a vPRF) instead of on a single string (which they denote an sPRF), a much more efficient construct. Efficiency is gained through the observation that constants over several iterations of the PRF need only be computed once. The constants that can take advantage of this are label, 0x00, context, and Length below.

This Task Group has agreed to submit our draft for outside cryptographic review. Phillip Rogaway and Thomas Shrimpton are well-known and well-respected cryptographers. In the referenced document they do comment on our draft. Their specific comment is this: “IEEE 802.11r does key derivation by applying a PRF to input that includes long constants (host and user names) that remain fixed across many derivations. We suggest that when a security protocoal wants to apply a PRF to what is logically a vector of strings the protocol should realize this with just such an abstraction. Concatenation should be avoided in achieving that abstraction (because it is, in general, both inefficient and wrong). The vPRF primitive should be realized in the protocol as a higher-level abstraction made from an sPRF.”

8.5.A.3 Key derivation function

Replace with:

The key derivation function for the FT key hierarchy, KDF, is defined as follows:

Output = KDF-Length(K, label, context) where

Input:	K, a 256 bit key derivation key

Label, a string identifying the purpose of the keys derived using this KDF

Context, a bit string that provides context to identify the derived key

Length, the length of the derived key in bits

Output: a Length-bit derived key

result = “”

iterations = (Length + 255) / 256

do i = 1 to iterations

result = result || vPRF-K(i, label, 0x00, context, Length)

od

return first Length bits of result and securely destroy the rest

where vPRF-K(X1, X2, … Xm) is a PRF of vectors using key K and is defined as follows:

if m = 0 then return AES-CMAC(K, 0x01)

S = AES-CMAC(K, 0x00)

do i = 1 to m – 1

S = S xor AES-CMAC(K, Xi)

od

if | Xm | >= 128 then

T = S xor AES-CMAC(K, Xm)

else

T = S xor AES-CMAC(K, Xm10*)

fi

return T

where Xi10* is as defined in RFC4493 and | Xi | indicates the length of Xi.

References:

- JH Song, et al, The AES-CMAC Algorithm, RFC4493.

- P. Rogaway and T. Shrimpton, Deterministic Authenticated-Encryption, Advances in Cryptology – EUROCRYPT ’06, St. Petersburg, Russia, 2006.

- O. Goldreich, S. Goldwasser, and S. Micali, How to construct random functions, Journal of the ACM, vol 33, no. 4, pp. 210-217, 1986.

- M. Bellare, J. Killian, and P. Rogaway, The Security of the Cipher Block Chaining Message Authentication Code, CRYPTO 2004