IEEE P802.11
Wireless LANs

| TGn LB84 Submission for Miscellaneous Frame CIDs |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-01-04 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Sanjiv Nanda | Qualcomm, Inc. | 5775 Morehouse Drive, San Diego, CA 92121, USA | +1 858 845 2375 | snanda@qualcomm.com |

Abstract

Many of the comments are resolved by existing modifications in D1.08. For a few CIDs, changes to the D1.08 are proposed.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart@ok-brit.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

# Introduction

## Interpretation of a Motion to Adopt

A motion to approve this submission means that the editing instructions and any changed or added material are actioned in the TGn Draft.  This introduction, is not part of the adopted material.

Editing instructions formatted like this are intended to be copied into the TGn Draft (i.e. they are instructions to the 802.11 editor on how to merge the TGn amendment with the baseline documents).

TGn Editor:  Editing instructions preceded by “TGn Editor” are instructions to the TGn editor to modify existing material in the TGn draft.   As a result of adopting the changes, the TGn editor will execute the instructions rather than copy them to the TGn Draft.

Summission Note: Notes to the reader of this submission are not part of the motion to adopt.  These notes are there to clarify or provide context.

# Comments

I have included proposed resolutions for most comments in “red” in column 7. These are submitted for review and discussion in the Frame or MAC adhoc groups.

| 1109 | 13 |  | 7 | what TID or AC does a TSPEC belong to? | clarify | Reject. There is no reerence to these terms on page 13 of the draft. The chair takes an action item to get further clarification from the commenter |
| --- | --- | --- | --- | --- | --- | --- |
| 1178 | 40 |  | 7.3.2.30 | What is the relationship between the MSDU size parameters and the size of an A-MPDU or A-MSDU? | Change the MSDU parameters to be MPDU parameters to accommodate aggregation schemes. | Reject. The A-MSDU and A-MPDU are two aggregation mechanisms. The specific maximum permitted values of an MSDU, A-MSDU and A-MPDU are defined in appropriate places in the draft. Requires a submission to further clarify the proposed relationship |
| 1553 | 22 | 9 | 7.2.1 | Isn't the HT control field included in HT Control Frames. If so, this this needs to be refelected in Clase 7.2.1 | See Coment | Countered. The HT control field can be included in Data and Management frames as well as in a new Wrapper Control frame. The Wrapper Control frame has been added to 7.2.1. Deferred until a decision is made on which frame types can carry HTC fields. |
| 1909 | 15 | 5 | 7.1.3.1.2 | QoS+Null is being re-defined here as a Management Action frame | Change entry in Table 1 in 7.1.3.1.2 row 11-Data-1100 to indicate "QoS Null (no data), or Management Action frame". Or, if my other comments on this subject are accepted, reject this one. | Accept. Editor, include instructions to modify Table 1 of Rev-ma to change the row for “type 10 subtype 1100 QoS Null (no data)” to “type 10 subtype 1100 QoS Null (no data) or Management Action” Chair to get more clarification from the commenter |
| 1911 | 15 | 8 | 7.1.3.1.5 | Changes to 7.1.3.1.5 are confusing. Are "frames exchanged between HT peers that are acknowledged using Block Ack" some of the ones that "have another fragment of the current MSDU or current MMPDU to follow."? If so, then the sentence should make that clear that the second sentence is overriding the statement in the first sentence. If not, then how is the second sentence different than the cases covered by the third ("all other frames")? | Please clarify the intent of this change | Counter. Frames transmitted under a Block Ack agreement between HT peers (i.e., using Compressed BlockAck) cannot be fragmented. Subclause 9.5 has been modified for clarification as requested. Will discuss this jointly with the MAC adhoc. This adhoc group believes that the second sentence beginning from "It is set to 0 …." should be removed. |
| 3753 | 43 |  | 7.3.2.47.2 | This field is clearly not necessary. If DS/CCK rates are not supported, do not include them in the supported rates (and extended supported rates) field. I do not see what extra information this field provides. BTW, that is how the beacon appears in the 5GHz (no DS/CCK rates in the supported rates:)) | Delete the field | Rejected. One bit in the HT Capabilities allows AP/STA to associate. Accepted. Bit has been removed and subclause 11.16.2 is deleted. The group had a straw poll and the result 3/3 |
| 3771 | 52 |  | 7.3.2.48 | Can a control channel's width can be anything other than 20 MHz? | Clarify (in the draft) or delete "20 MHz" from the description of "Control Channel" | Accept. Terminology has been clarified. When operating in 20 MHz, the primary channel is used, when operating in 40 MHz, the primary and secondary channels are used. Refer to Eldad's submission 1659 |
| 4349 | 42 | 3 | 7.3.2.47.2 | "Green Field preamble" is undefined Thus far | Define it in section 3 | Counter. HT Greenfield format is defined in clause 3. Editor: In D1.08 Table n23, modify “Indicates support for the reception of PPDUs with Greenfield preamble” to “Indicates support for the reception of PPDUs with HT Greenfield format.” Request a submission to define GF preamble, short GI, LDPC, MIMO power save, STBC etc.. |
| 6773 | 18 | 12 | 7.1.3.8 | Receiver shall honor TRQ, if it supports the Rx of Implicit TxBF, except when the MPDU containing TRQ does not trigger an immediate response, i.e. TRQ by itself shall not trigger a response frame. | Add; "Receiver shall honor TRQ, if it supports the Rx of Implicit TxBF, except when the MPDU containing TRQ does not trigger an immediate response, i.e. TRQ by itself shall not trigger a response frame." | Counter. Request more info from commenter |
| 6887 | 15 | 16 | 7.1.3.1.10 | Clarify that control and management frame types may be +HTC | "The HT Control Field may be included in any control, management or data frame type, except a non-QoS data frame." | Counter. QoS Data frames, Management frames and Control frames of subtype wrapper are permitted to be +HTC. |
| 6889 | 15 | 8 | 7.1.3.1.5 | Is Fragmentation between HT peers allowed? | Restate as "No fragmentation is allowed between HT Peers. Fragmentation can be used when an HT AP or STA is communicating with a non-HT STA or AP." | Counter. Frames transmitted under a Block Ack agreement between HT peers (i.e., using Compressed BlockAck) cannot be fragmented. Subclause 9.5 has been modified for clarification as requested. |
| 7223 | 15 | 17 | 7.1.3.1.10 | Although order bit signaling to indicate the presence of HT control field allows HT control field to be in All frames except the non-QoS data frames, but the use of +HTC frames should be limited to Unicast QoS data frames and BAR and BA control frames only. | Change the text to only allow HTC+ frame for all Unicast QoS data frames as well as BAR and BA control frames ONLY. | Counter. QoS Data frames, Management frames and Control frames of subtype wrapper are permitted to be +HTC.. |
| 7813 | 13 |  | 7.3.2.47.1 | Supported MCS table for STBC is not defined. | Add a separate MCS table for STBC | Reject. The MCS table entries for 1, 2 and 3 streams are applicable to STBC MCSs. John K. will take the action item to clarify with the commenter |
| 8065 | 30 |  | 7.2.2 | define HT PPDU for the multicast frames | Add after “in which case QSTAs use QoS data frames.” All HT STAs use non HT PPDU for broadcast data unless a transmitting HT STA knows that all STAs in BSS have HT capability, in which case the transmitting HT STAs uses HT PPDU to transmit data. All HT STAs use non HT PPDU for multicast data unless it is known to the transmitter that all STAs in the BSS that are members of the multicast group have HT capability, in which case HT STAs use HT PPDU | Reject. The transmitter of a multicast frame can determine when it is appropriate to use an HT PPDU. See submission 1557r4 |
| 8066 | 41 |  | 7.3.2.47.1 | Separated supported MCS set for 20Mhz and 40Mhz | New field | Counter. The Supported MCS Set field has been expanded to indicate the Maximum Data Rate that can be supported. This effectively allows the STA to indicate all the MCS that it can support for 20 MHz operation and the maximum data rate allows the STA to limit the MCS that it is unable to support with 40 MHz operation. Agreed with the suggestion. A submission is required to specify the structure. |
| 8075 | 51 |  | 7.3.2.48 | Separated basic MCS set for 20Mhz and 40Mhz | New field | Duplicate of CIS 8066. Submission required |
| 8108 | 15 | 8 | 7.1.3.1.5 | I believe this statement assumes the fact that clause 9 indicates that all Unicast frames transmitted by an HT to a peer HT using Block Ack are not fragmented. In this case this bit is set to zero anyway and therefore the inclusion of this statement is just confusing and redundant. | Delete the sentence that was added: "It is set to 0 in all frames exchanged between HT peers…..". | Counter. Frames transmitted under a Block Ack agreement between HT peers (i.e., using Compressed BlockAck) cannot be fragmented. Subclause 9.5 has been modified for clarification as requested. A vote was conducted to accept this comment. The result was 5 yes and 3 no - vote failed. |
| 9976 | 46 | 4 | 7.3.2.47.5 | How are the transition time fields defined? It is a mystery to me. |  | Rejected. The commenter has provided no resolution. The transition times are based on the best available information on implementations on switching between 20 MHz and 40 MHz operation. The specified values for the transition time were derived from numerous discussions within the Joint Proposal group in the year 2005 before the draft was constructed. Unfortunately some of the presentation materials are not available as references. The best way to move forward is to ask the commenter or anyone who has a problem with any of these values to come forward with a proposed change so the group can consider the change request. |
| 10070 | 20 | 18 | 7.1.3.8 | Bits themselves in HTC should not imply type of response, I think. Requesting immediate response in HTC overrifing frame exchange sequence definition based on frame types would introduce complexity. | How about this ? Replace " 10 – Immediate feedback" by " 10 - request a receiver to transmit immediate feedback only if the receiver has immediate transmission frame. If such kind of definition would be acceptable, there are a lots of required updates... | Counter. The field has been modified to indicate type of feedback requested, i.e., CSI, compressed etc., but does not now determine whether the response is immediate or delayed. see CID 1131 |
| 10297 | 30 |  | 7.2.2 | HT PPDU for the multicast frames is undefined | Add after “in which case QSTAs use QoS data frames.” All HT STAs use non HT PPDU for broadcast data unless a transmitting HT STA knows that all STAs in BSS have HT capability, in which case the transmitting HT STAs uses HT PPDU to transmit data. All HT STAs use non HT PPDU for multicast data unless it is known to the transmitter that all STAs in the BSS that are members of the multicast group have HT capability, in which case HT STAs use HT PPDU | Reject. The transmitter of a multicast frame can determine when it is appropriate to use an HT PPDU. Duplicate of CID 8065. See submission 1557r4 |
| 12003 | 90 | 8 | 9.9.1.2 | Zero TxOP extension for ZLF sounding | Extend "followed by ZLF" to "or A-MSDU or A-MPDU along with any ACK or Block Ack response frame" | Counter. Editor: In clause 9.9.1.2, change “A TXOP limit value of 0 indicates that a single MSDU or MMPDU or A-MSDU or A-MPDU along with any ACK or BlockAck response frame,” to “A TXOP limit value of 0 indicates that a single MSDU or MMPDU or A-MSDU or A-MPDU along with any ACK or BlockAck response frame as well as any NDP that are announced within the TXOP,” MAC: transfer to frame Defer - U - see CID 12001 |
| 12016 | 20 | 15 | 7.1.3.8 | No more different requests | Remove the sentence started by "The Feedback Request field contains position of the feedback:" remove table n7, | Counter. The field has been modified to indicate type of feedback requested, i.e., CSI, compressed etc., but does not now determine whether the response is immediate or delayed. see CID 1131 |
| 11948 | 24 | 23 | 7.2.1.7.3 | Add the following line for clarity. | Add: The ACK policy field in BlockAckReq and BlockAck is only defined for N-Delayed BlockAck. It is reserved under N-Immediate BlockAck. | Edit Notes (D1.05) EMR: <7.2.1.7 (to which this refers) already defines this. Adding as specified would created duplication. As a compromise, added the following note: "NOTE—The BAR Ack policy field in BlockAckReq and BlockAck is only defined for HT-delayed BlockAck. It is reserved under HT-immediate BlockAck." However, I do believe that this field is not used in the MTBA/PSMP case, in which case paragraph 2 of 7.2.1.7.3 should be replaced with "The BAR Ack Policy subfield of the Multiple TID BlockAckReq frame is reserved.">, to resolution (D1.05): accept |
| 11952 | 28 | 31 | 7.2.1.8.3 | Add the following line for clarity. | Add: The ACK policy field in BlockAckReq and BlockAck is only defined for N-Delayed BlockAck. It is reserved under N-Immediate BlockAck. | Edit Notes (D1.05) EMR: <7.2.1.7 (to which this refers) already defines this. Adding as specified would created duplication. As a compromise, added the following note: "NOTE—The BAR Ack policy field in BlockAckReq and BlockAck is only defined for HT-delayed BlockAck. It is reserved under HT-immediate BlockAck." However, I do believe that this field is not used in the MTBA/PSMP case, in which case paragraph 2 of 7.2.1.8.3 should be replaced with "The BA Ack Policy subfield of the Multiple TID BlockAckReq frame is reserved.">, to resolution (D1.05): accept |