IEEE P802.11
Wireless LANs

| Proposal for resolution of comments for P802.11.2-D0.11 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-01-17 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Neeraj K. Sharma | Intel | 2111 NE 25th Avenue, Hillsboro, OR 97124 | 503-712-7458 | neeraj.k.sharma@intel.com |
| Dennis Ward | Sirius Satellite Radio | 27200 Haggerty Rd, Suite B-3, Farmington Hills, MI 48331 | 248-699-0442 | dward@siriusradio.com |
| Uriel Lemberger | Intel | PO Box 1659, Matam Industrial Park, Haifa 31015 Israel | +972-4-865-5701 | uriel.lemberger@intel.com |
| Alexander Tolpin | Intel | PO Box 1659, Matam Industrial Park, Haifa 31015 Israel | +972-4-865-5430 | alexander.tolpin@intel.com |

Abstract

This document contains the proposal for resolution of some comments to draft P802.11.2-D0.11. The comments referenced to are listed in the document IEEE 802.11-06-0872r23 and the comments addressed are 72 and 239.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart@ok-brit.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

# Resolution for Comment [2] ID# 72

Clause 5.4.1.2.1

Replace the phrase

b) No obstacles in the vicinity that may cause signal reflections. “Vicinity” here refers to obstructions

or reflectors within 50 meters of the DUT-WLCP line of sight.

with

b) No obstacles in the vicinity that may cause signal reflections. “Vicinity” in this context refers to obstructions or reflectors within the first Fresnel zone except the test stand and test station cart on which the WLCP and DUT are placed, respectively.

Clause 5.6.2.1

Replace the phrase

a) An unoccupied floor of an office building with no barriers between the DUT and WLCP which may attenuate the signal. There should be no obstacles in the vicinity that may cause signal reflections. ‘Vicinity’ in this context refers to obstructions or reflectors within line of sight between the DUT and the WLCP.

with

a) An unoccupied floor of an office building with no barriers between the DUT and WLCP which may attenuate the signal. There should be no obstacles in the vicinity that may cause signal reflections. ‘Vicinity’ in this context refers to obstructions or reflectors within the first Fresnel zone excluding the ceiling, floor, or a wall on which the AP is mounted.

Add definition of Fresnel zone to section 3.2 Definitions

3.2.x Fresnel zone: Elliptical regions surrounding the LOS path between transmitting and receive antennas and depends on the frequency and distances between the two radios. The nth Fresnel zone is calculated as follows

Fn=nλd1d2d1+d2

where

Fn = The nth Fresnel Zone radius in meters

d1 = The distance of P from one end in meters

d2 = The distance of P from the other end in meters

 = The wavelength of the transmitted signal in meters

# Resolution for Comment [2] ID# 239

5.4.1.2.1 Resource requirements

Add item e) to the following list

The outdoor range should meet the following requirements:

a) No barriers between the DUT and WLCP which may attenuate the signal.

b) No obstacles in the vicinity that may cause signal reflections. “Vicinity” here refers to obstructions

or reflectors within 50 meters of the DUT-WLCP line of sight.

c) A flat test area that allows the DUT to be placed at a distance of at least 300 meters from the WLCP.

d) No extraneous signal in the test environment. A spectrum analyzer may be used to ensure that no

active interference is present.

e) The DUT or the WLCP shall not have any receptivity enhancing material surrounding either of the two devices.

5.5.1.2.1 Resource requirements

Add item e) to the following list

The indoor NLOS range should meet the following requirements:

a) For an enterprise situation, an unoccupied floor of an office building with a combination of wood

and metal framing, as well as window glass. For a home situation, an unoccupied home with a combination of wood framing and window glass.

b) There should be no active WLAN devices other than the DUT and the WLCP.

c) Test locations chosen should provide a wide range of signal quality and throughput capabilities,

based on signal strength and multipath delay characterization of the facility.

d) No extraneous signals should be present in the test environment. A spectrum analyzer may be used

to ensure that no active interference is present.

e) The DUT or the WLCP shall not have any receptivity enhancing material surrounding either of the two devices.

5.6.2.1 Resource requirements

Add item e) to the following list

The indoor LOS range should meet the following requirements:

a) An unoccupied floor of an office building with no barriers between the DUT and WLCP which may attenuate the signal. There should be no obstacles in the vicinity that may cause signal reflections. ‘Vicinity’ in this context refers to obstructions or reflectors within line of sight between the DUT and the WLCP.

b) There should be no active WLAN devices other than the DUT and the WLCP.

c) The test area should be flat, and should allow the DUT to be placed at a distance of at least 70 meters.

d) No extraneous signals should be present in the test environment. A spectrum analyzer may be used

to ensure that no active interference is present.

e) The DUT or the WLCP shall not have any receptivity enhancing material surrounding either of the two devices.

Table 1, Item 3, DUT X-Y placement accuracy

Replace the phrase

The DUT should be placed in test locations such that its positioning is repeatable run-to-run/day-to-day within an area represented by a circle centered at the intended test point, and with a radius of 1.5cm (for 2.4 GHz testing) and 0.75cm (for 5 GHz testing), as measured from a consistent reference point on the DUT base (e.g., back left hand corner of the DUT).

with

The DUT should be placed in test locations such that its positioning is repeatable run-to-run/day-to-day within an area represented by a circle centered at the intended test point, and with a radius of 1.5cm (for 2.4 GHz testing) and 0.75cm (for 5 GHz testing), as measured from a consistent reference point on the DUT base (e.g., back left hand corner of the DUT). The material type of the surface on which the DUT is placed must be recorded with the results and no metallic surfaces must be used within a wavelength from the antennas.

Table 1, Item 5 - AP test height and antenna orientation

Replace the phrase

APs used for performance testing should be placed as shown in Figure 11 with the base of the antennas at a fixed height h above the ground. The AP height is set according to the test purposes, and must be recorded with the results.

with

APs used for performance testing should be placed as shown in Figure 11 with the base of the antennas at a fixed height h above the ground. The AP height is set according to the test purposes, and must be recorded with the results. The material type surrounding the AP (e.g. ceiling and wall) must be recorded with the results.

Table 3, Item 3, DUT X-Y placement accuracy

Replace the phrase

The DUT should be placed in test locations such that its positioning is repeatable run-to-run/day-to-day within an area represented by a circle centered at the intended test point, and with a radius of 1.5cm (for 2.4 GHz testing) and 0.75cm (for 5 GHz testing), as measured from a consistent reference point on the DUT base (e.g., back left hand corner of the DUT).

with

The DUT should be placed in test locations such that its positioning is repeatable run-to-run/day-to-day within an area represented by a circle centered at the intended test point, and with a radius of 1.5cm (for 2.4 GHz testing) and 0.75cm (for 5 GHz testing), as measured from a consistent reference point on the DUT base (e.g., back left hand corner of the DUT). The material type of the surface on which the DUT is placed must be recorded with the results and no metallic surfaces must be used within a wavelength from the antennas.

Table 3, Item 5 - AP test height and antenna orientation

Replace the phrase

APs used for performance testing should be placed as shown in Figure 11 with the base of the antennas at afixed height h above the ground. The AP height is set according to the test purposes, and must be recorded with the results.

with

APs used for performance testing should be placed as shown in Figure 11 with the base of the antennas at afixed height h above the ground. The AP height is set according to the test purposes, and must be recorded with the results. The material type surrounding the AP (e.g. ceiling and wall) must be recorded with the results.

# References

- P802.11.2-D0.11. Draft Recommended Practice for the Evaluation of 802.11 Wireless Performance.

- IEEE 802.11-06-0872r23. Comments from TGT Internal Review.