IEEE P802.11
Wireless LANs

| Normative Text Changes for Network Advice on Preferred Channel |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-MAY-17 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Marian Rudolf | InterDigital Communications | Montreal, QC | 514-9046258 | Marian.Rudolf@InterDigital.com |
| Joe Kwak | InterDigital Communications | 482 Degas Bolingbrook, IL 60440 | 630-739-4159 | joekwak@sbcglobal.net |
| Jari Jokela | Nokia | Visiokatu 1, 33720 Tampere, Finland | +358504860445 | jari.jokela@nokia.com |

Abstract

This document contains normative text for the use of Net Advice transmissions on Preferred Channel for Power Savings.   As explained in the accompanying presentation 07/0120r_, the proposed normative text changes define a new Network Advice frame and a Preferred Channel procedure which insure activity on the designated Preferred Channel for the regulatory class in use.  This ensures that STAs attempting network discovery by scanning need only to scan the appropriate set of Preferred Channels to determine if a BSS is operating in any locale.  This saves 80%-85%  power while STA is doing network discovery prior to any association.  Based on comments received at the March meeting,  the AP capability to provide this function on channels where DFS is required is made an optional capability in the PICS.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart@ok-brit.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

[NOTE TO EDITOR:  Change bars are used to show changes and additions from draft 802.11vD0.05]

### 7.4 Action frame format details

### 7.4.6Wireless Network Management action details

### 7.4.6. Wireless Network Management action details

Insert new row into Table v47 as follows:

- Table v47—Wireless Network Management Action field values

| Action field value | Description |
| --- | --- |
| 15 | Network Advice |
| 16-255 | Reserved |

Insert and renumber new clause.7.4.6.x after 7.4.6.4 as follows:

### 7.4.6.X Network Advice frame format

The Network Advice frame is used by STAs in a BSS to advertise the presence of the BSS on the Preferred Channel of the Regulatory Class for that BSS.. The Network Advice  frame body format is shown in Figure x1 .

|  | Category | Action | Country String | Regulatory Class | BSS Channel Number |
| --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 3 | 1 | 1 |

Figure x1—Network Advice frame body format

The Category field is set to the value indicating the Wireless Network Management category, as specified in Table 24 in 7.3.1.11.

The Action field is set to the value indicating Event Log Request frame, as specified in  in .

Country String indicates the value contained in the dot11CountryString attribute of the STA transmitting the Network Advice frame.

Regulatory Class indicates the channel set for the advertised BSS.. Regulatory Class and Channel Number together specify the channel frequency for the advertised BSS.   Valid values of Regulatory Class are shown in Annex J.

BSS Channel Number indicates the channel number for the advertised BSS. Channel Number is defined within a Regulatory Class as shown in Annex J.

# 10. Layer management

## 10.3 MLME SAP Interface

Insert the following new clause after Clause 10.3.42 as indicated below:

### 10.3.x Network Advice

### 10.3.x Network Advice Request

This set of primitives supports the signaling of Network Advice Request frames between an AP and associated non-AP STAs.

#### 10.3.x.1 MLME-NETADVICE.request

##### 10.3.x.1.1 Function

This primitive requests transmission of a Network Advice frame on a Preferred Channel.

##### 10.3.x.1.2 Semantics of the Service Primitive

The primitive parameters are as follows:

MLME-NETADVICE.request	(
Country String,
Regulatory Class
BSS Channel Number
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Country String | Octet String, 3 octets | 0-255 | The Country String is the dot11CountryString attribute, a shortened representation of the Country IE. |
| Regulatory Class | Integer | 0 – 255 | For the BSS Channel Number, as defined in AnnexJ. |
| BSS Channel Number | Integer | 0 – 255 | Channel number for the advertised BSS, defined within the Regulatory Class in Annex J. |

##### 10.3.x.1.3 When Generated

This primitive is generated by the SME to request that a Network Advice frame be sent on the Preferred Channel to advertise the existence of an operating BSS on the channel indicated in the Network Advice frame.

##### 10.3.x.1.4 Effect of Receipt

On receipt of this primitive, the MLME constructs a Network Advice  frame of action type. The STA then attempts to transmit this frame on the Preferred Channel for the current Regulatory Class.

#### 10.3.x.2 MLME-NETADVICE.confirm

##### 10.3.x.2.1 Function

This primitive reports the result of a request to send a Network Advice frame on the Preferred Channel.

##### 10.3.x.2.2 Semantics of the Service Primitive

The primitive parameters are as follows:

MLME-NETADVICE.confirm	(
ResultCode,
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| ResultCode | Enumeration | SUCCESS, INVALID_ PARAMETERS, TIMEOUT, TRANSMISSION_FAILURE, UNSPECIFIED_ FAILURE | Indicates the result of the corresponding MLME-NETADVICE.request. |

##### 10.3.x.2.3 When Generated

This primitive is generated by the MLME when the request to transmit a Network Advice Request frame completes and indicates the results of the request.

##### 10.3.x.2.4 Effect of Receipt

On receipt of this primitive, the SME evaluates the ResultCode.

#### 10.3.x.3 MLME-NETADVICE.indication

##### 10.3.x.3.1 Function

This primitive indicates that a Network Advice Request frame was received from the AP with which the STA is associated.

##### 10.3.x.3.2 Semantics of the Service Primitive

The primitive parameters are as follows:

MLME-NETADVICE.indication	(
Country String,
Regulatory Class
BSS Channel Number
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Country String | Octet String, 3 octets | 0-255 | The Cuntry String is the dot11CountryString attribute, a shortened representation of the Country IE. |
| Regulatory Class | Integer | 0 – 255 | For the BSS Channel Number, as defined in AnnexJ. |
| BSS Channel Number | Integer | 0 – 255 | Channel number for the advertised BSS, defined within the Regulatory Class in Annex J. |

##### 10.3.x.3.3 When Generated

This primitive is generated by the MLME when a valid Network Advice frame is received.

##### 10.3.x.3.4 Effect of Receipt

On receipt of this primitive the SME shall operate according to the procedure in 11.15.x.

- 11  MAC sublayer management entity

- 11.15  Wireless Network Management Procedures

Insert and renumber new clause 11.15.x after 11.15.3 as follows:

- 11.15.X Peferred Channel Procedures

The Preferred Channel procedures permit a STA to save power while searching for suitable BSSs for WLAN service. Preferred Channel Procedures decrease the number of channels to be scanned while a STA is discovering available networks and BSSs.

Each defined Regulatory Class in Annex J includes a Channel Set which is an ordered list of channels in that regulatory class. The Preferred Channel is selected randomly from the Channel Set for each regulatory class.  The Preferred Channel is selected daily for each Regulatory Class. The Preferred Channel Procedures guarantee periodic transmissions on the Preferred Channel providing network access information.  In this way, only the Preferred Channel needs to be scanned within any regulatory class.

The Preferred Channel changes daily and is randomly selected from the Channel Set list of channels using the same RC4  PRNG used for WEP encryption.  The Preferred Channel Order is obtained as follows:

Preferred Channel Order = (call RC4(SEED,64))modQ

where a call to RC4() returns a random 64 bit number,

where Q is the number of channels in a defined channel set,

and where  SEED= DATE*10E12 +DATE*10E6 +DATE ,

where 	DATE= YYYY*365 + MM*31 +DD,

and where DDMMYYYY is the current date.

If the Preferred Channel Order is 0, the Preferred Channel for that  regulatory class is the first channel listed in the channel set.  If the Preferred Channel Order is 1, the Preferred Channel is the second channel listed, and so on.

- 11.15.X.1  AP Procedure for Preferred Channel without DFS

This clause defines procedures and requirements for an AP when operating on channels where DFS is not required.

If dot11SpectrumManagementRequired is false, an AP shall use an initial channel selection algorithm which favors selection of the Preferred Channel for beacon transmission when initiating operation of an infrastructure BSS.

If dot11SpectrumManagementRequired is false,, the following paragraph applies:

When an AP transmits beacons for an infrastructure BSS on a non-Preferred Channel, it shall periodically monitor the Preferred Channel for frame transmissions.  If no beacons and no network advice frames are received on the Preferred Channel, the AP shall set dot11NetworkAdviceEnabled to true in one or more STAs in the BSS in order to advertise the BSS on the Preferred Channel. Note that the AP may set its own dot11NetworkAdviceEnabled to true to satisfy the prior requirement. If the AP receives beacons on the Preferred Channel, the AP shall set dot11NetworkAdviceEnabled to false in all STAs in the BSS.

- 11.15.X.2  AP Procedure for Preferred Channel with DFS

This clause defines procedures and requirements for an AP when operating on channels where DFS is required.

If dot11SpectrumManagementRequired is true, the following paragraph applies:

When an AP transmits beacons for an infrastructure BSS on a non-Preferred Channel, it shall periodically monitor the Preferred Channel for frame transmissions.  If no beacons and no network advice frames are received on the Preferred Channel, the AP should perform a channel availability check on the Preferred Channel to ensure that there is no radar operating on the channel. If the Preferred Channel is available, the AP shall set dot11NetworkAdviceEnabled to true in one or more STAs in the BSS in order to advertise the BSS on the Preferred Channel. Note that the AP may set its own dot11NetworkAdviceEnabled to true. If an AP has set dot11NetworkAdviceEnabled to true at any STA, the AP shall ensure that a STA shall use radar interference detection to monitor the Preferred Channel for radar per the regulatory requirements.  If the AP receives notification of radar on the Preferred Channel by receiving a unicast Network Advice frame on the BSS operating channel, the AP shall set dot11NetworkAdviceEnabled to false in all STAs in the BSS.  If the AP receives beacons on the Preferred Channel, the AP shall set dot11NetworkAdviceEnabled to false in all STAs in the BSS.

- 11.15.X.3  STA Procedure for Preferred Channel

If dot11SpectrumManagementRequired is false, a STA which is initiating an IBSS shall use a channel selection algorithm which disfavors selection of the Preferred Channel for beacon transmission.

If dot11NetworkAdviceEnabled is true, the  STA shall send Network Advice frames to the broadcast address on the Preferred Channel at the maximum transmit power permitted by the regulations and at the beacon transmission data rate, otherwise the STA shall not transmit on the Preferred Channel,.  The STA shall schedule Network Advice frame transmissions once at each TBTT using the timing rules which apply for beacon transmissions.

If dot11SpectrumManagementRequired is true, any STA capable of radar interference detection which detects a radar on the Preferred Channel shall notify the AP that radar is present on the Preferred Channel by transmitting a unicast Network Advice frame to the AP on the BSS operating channel.

If dot11SpectrumManagementRequired is true, each STA shall set its dot11NetworkAdviceEndabled to false at 11:59:59 PM each day.

Insert the following new rows in clause A.4.14:

### A. 4.14 Wireless Network Management extensions

| Item | Protocol Capability | References | Status | Support |
| --- | --- | --- | --- | --- |
| RME5 | Preferred Channel Procedures | 11.15.X | CFv:M | Yes, No, N/A |
| RME5.1 | AP Procedure for Preferred Channel without DFS | 7.4.6.X, 11.15.X.1 | (CFv AND CF1):M | Yes, No, N/A |
| RME5.2 | AP Procedure for Preferred Channel with DFS | 7.4.6.X, 11.15.X.2 | (CFv AND CF1):O | Yes, No, N/A |
| RME5.3 | STA Procedure for Preferred Channel | 7.4.6.(X), 11.15.X.3 | CFv :M | Yes, No, N/A |

# Annex D

In the dot11StationConfig table of Annex D, insert the following entries at the end of the dot11StationConfigEntry sequence list:

Dot11StationConfigEntry ::=

SEQUENCE {

…

dot11NetworkAdviceImplemented TruthValue,

dot11NetworkAdviceEnabled TruthValue}

Insert the following elements at the end of the dot11StationConfigTable element definitions:

dot11NetworkAdviceImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This attribute, when TRUE, shall indicate that the Network Advice transmission on the Preferred Channel is implemented. The default value of this attribute shall be FALSE."

::= { dot11StationConfigEntry TBD }

dot11NetworkAdviceEnabled OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This attribute, when TRUE, indicates that the Network Advice transmission on the Preferred Channel is enabled. Otherwise, the capability is disabled. The default value of this attribute is FALSE."

::= { dot11StationConfigEntry TBD }