IEEE P802.11
Wireless LANs

| Comments 348 and 350 Resolutions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-01-15 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Dorothy Stanley | Aruba Networks | 1322 Crossman Ave Sunnyvale, CA 94089 | +1 630-363-1389 | dstanley@arubanetworks.com |

Abstract

This document contains proposed comment resolutions to TGv informal review comments 348 and 350. And proposed comment resolutions to TGv informal comments 130, 135, 136, 141.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart@ok-brit.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Comment 348: 7.3.2.39.1, p.40, line 10. Suggest getting rid of the client report types and just have additional types in Table v22, e.g. manufacturing info, operating parameters, etc. Don't see the value of the additional level of definition.

Proposed Resoolution:

Delete  7.3.2.38.5.5, shown below and modify Table v10 in  7.3.2.38 as shown.

          - Client Report Group Type

The format for the Client Report Group Type is shown in Figure v1.

|  | Group Type |
| --- | --- |
| Octets: | 1 |

- —Client Report Group Type format

The Client Report Group Type is a one byte field that indicates the type of information to be reported in the Client Report and is set to any of the allocated values in Table v1.

- —Client Report Group Type definitions

| Client Report Group Type | Value |
| --- | --- |
| Manufacturer Information | 0 |
| Operating Parameters | 1 |
| Capabilities | 2 |
| Configuration Profile | 3 |
| Reserved | 4 – 255 |

From 7.3.2.38: Change from:

The Diagnostic Request Type is set to a number that identifies a type of diagnostic request. The defined Diagnostic Request Types are shown in Table v10.

| Name | Report Type Value |
| --- | --- |
| Client Report | 0 |
| 802.11 Authentication | 1 |
| Association | 2 |
| 802.1X Authentication | 3 |
| Reserved | 4 – 255 |

- Table v10—Diagnostic Request Type definitions

The Diagnostic Request Type is set to a number that identifies a type of diagnostic request. The defined Diagnostic Request Types are shown in Table v10.

| Name | Request Type Value |
| --- | --- |
| Manufacturing Information Client Report | 0 |
| Operating Parameters Client Report | 1 |
| Capabilities Client Report | 2 |
| Configuration Profile | 3 |
| 802.11 Authentication | 4 |
| Association | 5 |
| 802.1X Authentication | 6 |
| Reserved | 7 – 255 |

Comment 350: 7.3.2.40.4: The values in this table are the same as those in v23. Suggest generalizing the ones in v23, and using the same table to avoid duplicate definition.

Change 7.3.2.39 as indicated below, and modify 7.3.2..40.4, as shown below:

        - Presence Status field

The Presence Status field provides the result of a Presence Request or Presence Configuration Request frame.  The format of the Presence Status field is shown in Figure v2.

|  | Element ID (4) | Length (1) | Status |
| --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 |

- Figure v64—Presence Status element

The Status field identifies the result of the presence request and is one of the values in Table 22 Table v3.

- Table v33—Status field values

| Status Value | Description |
| --- | --- |
| 0 | Success |
| 1 | Fail |
| 2 | Refuse |
| 3 | Incapable |
| 4-255 | Reserved |

      - Diagnostic Report element

The Diagnostic Report element contains a Diagnostic report. The format of the Diagnostic Report element is shown in Figure v3.

|  | Element ID | Length | Diagnostic Token | Diagnostic Report Type | Diagnostic Status | Diagnostic Report |
| --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 1 | 1 | variable |

- —Diagnostic Report element format

The Element ID field is equal to the Diagnostic Report value in Table 26.

<snip>

The Diagnostic Status field is set to a value in Table v4, indicating the client’s response to the Diagnostic Request indicated by the Dialog Token.

- —Diagnostic Status values

| Status Value | Description |
| --- | --- |
| 0 | Successful |
| 1 | Fail |
| 2 | Refused |
| 3 | Incapable |
| 4-255 | Reserved |

The Diagnostic Report field contains the results of the diagnostic request.

Comment 130: Definitions of “End of Motion” and "In motion" are ambiguous. If a STA is in "End of Motion", the STA is also in "Stationary". If a STA is in "Start of Motion", the STA is also in "In motion". In these cases, which state shall STA be in?

Proposed Resolution:

Table V-36 including these terms is shown below:

| Motion Indicator Value | Description |
| --- | --- |
| 0 | Stationary |
| 1 | Start of motion |
| 2 | In motion |
| 3 | End of motion |
| 4 | Unknown |
| 5-255 | Reserved |

Insert the following text below Table V-36:

A Motion Indicator Value of  “0” (Stationary) indicates that the device is stationary and not in motion.

A Motion Indicator Value of  “1” (Start of motion) indicates that the device was stationary and is now in motion.

A Motion Indicator Value of  “2” (In motion) indicates that the device is and has been in motion.

A Motion Indicator Value of  “3” (End of motion) indicates that the device was in motion and is now stationary.

A Motion Indicator Value of “4” (Unknown” indicates that information related to motion is unknown.

Comment 135:7.4.6.5  "Response Requested" field is missing. "Response Requested" filed is present in 10.3.32.1.2 and 11.15.4.2.

Proposed Resolution:

Change 7.4.6.5 as shown below:

The Presence Request frame uses the Action frame body format and is transmitted by a STA to deliver presence reporting information. The format of the Presence Request frame body is shown in Figure v4.

|  | Category | Action | Dialog Token | Response Requested | Presence Parameters Element |
| --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 1 | variable |

- —Presence Request frame body format

The Category field is set to the value indicating the Wireless Network Management category, as specified in Table 24 in Error: Reference source not found.

The Action field is set to the value indicating Presence Request, as specified in Error: Reference source not found in Error: Reference source not found.

The Dialog Token field is a nonzero value which identifies the Presence Request/Response transaction. The dialog token is unique for each presence request frame sent to a given destination MAC address.

The Response Requested value indicates if a Presence Response frame is requested by the transmitting STA. A value of 0 indicates that a Presence Response frame is not requested. A value of 1 indicates that a Presence Response frame is requested in response to the transmitted frame.

The Presence Parameters Element field contains the presence parameters element, described in Clause Error: Reference source not found. Error: Reference source not found defines the allowed Presence Parameters element fields for a Presence Parameters element that is included in the Presence Request frame.

In 10.3.32.3.2 and 10.3.32.3.2, Change from

The response requested value indicates if a presence response message is requested.

To

The response requested value indicates if a Presence Response frame is requested.

Comment 136: No description of usage of "management action pending".

The Management Action pending field appears in 7.4.6.6 as shown below:

- 7.4.6.6Presence Response frame format

The Presence Response frame uses the Action frame body format and is transmitted by a STA in response to the receipt of a Presence Request frame. The format of the Presence Response frame body is shown in Figure v5.

|  | Category | Action | Dialog Token | Management Action Pending | Presence Parameters Element |
| --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 1 | variable |

- —Presence Response frame body format

The Category field is set to the value indicating the Wireless Network Management category, as specified in Table 24 in Error: Reference source not found.

The Action field is set to the value indicating Presence Response, as specified in Error: Reference source not found in Error: Reference source not found.

The Dialog Token field is set to the nonzero value received in the Presence Request frame to identify the request/response transaction.

The Management Action Pending field is set to 0 if no management action is pending for the destination STA, and set to 1 if there is a management action pending for the destination STA. A value of 255 in the management action pending field indicates tthat the STA stop sending presence request frames.

The Presence Parameters Element field contains the presence parameters element, described in Clause Error: Reference source not found. Error: Reference source not found defines the allowed Presence Parameters element fields for a Presence Parameters element that is included in the Presence Response frame.

Proposed Resolution:

Insert the following text at the end of  11.15.4.2 Presence Response Procedures

If a STA received a Presence Response frame with the Management Action Pending field value set to “1”, the STA, if not already associated to a BSS, attempts to associate to the AP from which it received the Presence Response frame, or to another AP of its choosing to determine the management action that is pending.

Comment 141: "Radio Information" is present in both Presence Request and Presence Response frames. The corresponding usages or measurements of each field are unclear.

Background: 7.3.2.40.6 defines the Radio Information Field:

        - Radio Information field

The Radio Information field contains radio information. The format of the Radio Information field is shown in Figure v6.

|  | Element ID (6) | Length (5) | Transmit Power | Antenna ID | Antenna Gain | Received RSNI | RCPI |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 1 | 1 | 1 | 1 |

- —Radio Information Field

The transmit power is the transmit power of the radio transmitting the presence frame and is a signed integer, one octet in length, reported in dBm. A value of -127 indicates that the transmit power is unknown.

The Antenna ID is the identifying number for the antenna used to transmit the presence frame. The Antenna ID is defined in 7.3.2.29.

The antenna gain is the antenna gain of the antenna over which the presence frame will be transmitted and is a signed integer, one octet in length reported in dBi.  A value of -127 indicates that the antenna gain is unknown.

The received RSNI field contains the RSNI value (dBm) measured against the most recently received Presence Request frame.  A value of-127 indicates that the RSNI value is unknown.

The RCPI field contains the RCPI value measured against the most recently received Presence Request frame. A value of -127 indicates that the RCPI value is unknown.

Proposed Resolution:

Insert the following text at the end of  11.15.4.1Presence Request Procedures

A STA may include the Radio Information field values in the Presence Request frame, to provide radio related information to the receiving STA. Radio Information can be used to support location services.

Insert the following text at the end of  11.15.4.2 Presence Response Procedures

A STA may include the Radio Information field values in the Presence Response frame, to provide radio related information to the receiving STA. Radio Information can be used to support location services.

References: