IEEE P802.11
Wireless LANs

| Leader based Multicast Service Proposal |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-03-14 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | Email |
| Yongho Seok | LG Electronics | 16 Woomyeon-Dong, Seocho-Gu, Seoul 137-724, Korea | +8225264225 | yhseok@lge.com |
| Thierry Turletti Diego Dujovne | INRIA | 2004, route des Lucioles, Sophia Antipolis, France | +33492387879 | turletti@sophia.inria.fr ddujovne@sophia.inria.fr |
| Emily Qi | Intel Corporation | 2111 NE 25th Hillsboro OR 97124, USA | +15032647799 | Emily.h.qi@intel.com |
| Adrian Stephens | Intel Corporation | 15 JJ Thompson Avenue, Cambridge, CB 3 0FD, United Kingdom | +441223763457 | adrian.p.stephens@intel.com |
| Alex Ashley | NDS Ltd | NDS Ltd, One London Road, Staines, Middlesex, TW18 4EX, UK | +441784848770 | aashley@nds.com |
| Menzo Wentink | Conexant | Oudegracht 3a, Utrecht, Netherlands | +31651836231 | Menzo.wentink@conexant.com |
| Pedro CUENCA | Universidad de Castilla-La Mancha | EPSA, Campus Universitario s/n., Albacete, Spain | +34967599200 | puenca@dsi.uclm.es |

Abstract

This document contains proposed changes to the IEEE P802.11v Draft to address Req 2120 of the TGv Objectives. In particular, to enhance multicast data delivery, the proposed changes enable the utilization of leader-based transmission mechanisms. Such mechanisms aim to improve the throughput fairness between multicast connections and point-to-point connections and can optionally increase the reliability of multicast transmissions. Leader-based mechanisms consist of a leader election protocol and a multicast acknowledgement mechanism. The leader election protocol is responsible for electing the station, called leader, which generates an ACK frame for each successfully received multicast frame. If the AP does not receive an ACK frame, it carries out the binary exponential backoff mechanism as it is the case for point-to-point connections. Furthermore, the proposed leader-based mechanism can optionally retransmit multicast frames and use RTS/CTS exchange when multicast frames are exchanged.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart@ok-brit.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

- 3. Definitions

Insert the following new definitions:

3.v.5 Leader Based Multicast Service (LBMS): Leader Based Multicast Service aims to improve the throughput fairness between multicast connections and point-to-point connections and can optionally increase the reliability of multicast transmissions.

- Abbreviations and acronyms

Insert the following new acronyms in alphabetical order:

LBMS		Leader Based Multicast Service

Change 7.3.2.49 as shown:

- 7.3.2.49 Wireless Network Management Capability information element

The Wireless Network Management Capability Information element contains information about the wireless network management capabilities of a STA as shown in Table v15.

|  | Element ID | Length | Wireless Network Management Capabilities |
| --- | --- | --- | --- |
| Octets: | 1 | 1 | variable |

- Figure v15—Wireless Network Management Capability information element format

The Element ID field is equal to the Wireless Network Management Capability value in Table 26.

The value of the length field is variable and depends on the length of the Wireless Network Management Capabilities field.  The minimum value of the Length field is 2.

The Wireless Network Management Capabilities field is a bit-field indicating the advertised management capabilities of the STA. The Wireless Network Management Capabilities field is shown in Table v16.

|  | B0 | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Event Log | Diagnostics | Multicast Alert | Presence | FBMS | Proxy ARP Service | Co-located Interference Reporting | LBMS | Reserved |  |
| Bits: | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 8 |  |

- Figure v16—Wireless Network Management Capabilities

- The Event Log bit set to 1 indicates the STA supports Event Log as described in 11.15.2. The Event Log bit set to 0 indicates that the STA does not support this service.

- The Diagnostics bit set to 1 indicates the STA supports Diagnostics as described in 11.15.3. The Diagnostics bit set to 0 indicates that the STA does not support this service.

- The Multicast Alert bit set to 1 indicates the STA supports Multicast diagnostics as described in 11.15.11. The Multicast Alert bit set to 0 indicates that the STA does not support this service.

- The Presence bit set to 1 indicates that the STA supports Presence as described in 11.15.4. The Presence bit set to 0 indicates that the STA does not support this service.

- The FBMS bit set to 1 indicates the STA supports FBMS as described in 11.2.1.5. The FBMS bit set to 0 indicates the STA does not support FBMS.

- The Proxy ARP Service bit set to 1 indicates the AP is providing proxy ARP service.  If Proxy ARP service is enabled, then the AP responds to broadcast ARP request on behalf of the STA. The Proxy ARP Service bit set to 0 indicates the AP is not providing proxy ARP service for any associated STA.

- The Co-located Interference Reporting bit set to 1 indicates the STA supports Co-located Interference Reporting as described in 11.15.9. The Co-located Interference Reporting bit set to 0 indicates that the STA does not support this service.

- The LBMS bit set to 1 indicates the STA supports the LBMS as described in 9.2.7.2. The LBMS bit set to 0 indicates the STA does not support the LBMS.

- All other bits are reserved, and are set to 0 on transmission and ignored on reception.

The lack of a Wireless Network Management Capability element is interpreted as the STA having no advertised Wireless Network Management Capabilities.

Change 7.4.8 as shown:

- 7.4.8 Wireless Network Management action details

Several Action frame formats are defined for Wireless Network Management purposes. An Action field, in the octet field immediately after the Category field, differentiates the formats. The Action field values associated with each frame format are defined in Table v49.

- Table v49—Wireless Network Management Action field values

| Action field value | Description |
| --- | --- |
| 0 | Event Log Request |
| 1 | Event Log Report |
| 2 | Diagnostic Request |
| 3 | Diagnostic Report |
| 4 | Presence Request |
| 5 | Presence Response |
| 6 | Presence Configuration Request |
| 7 | Presence Configuration Response |
| 8 | BSS Transition Management Query |
| 9 | BSS Transition Management Request |
| 10 | BSS Transition Management Response |
| 11 | FBMS Request |
| 12 | FBMS Response |
| 13 | Co-located Interference Request |
| 14 | Co-located Interference Response |
| 15 | Leader Request |
| 16 | Leader Response |
| 17 | Leader Release |
| 18-255 | Reserved |

Insert the following after 7.4.8.15:

- 7.4.8.16 Leader Request

The Leader Request frame shall use the Action frame body format. The format of the Leader Request frame body is shown in Figure v105.

|  | Category | Action | Dialog Token | Length | Retransmission BSSID | Multicast Group Address #1 | Multicast Group Address #n |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 1 | 6 | 6 | 6 |

- Figure v105—Leader Request frame body format

The Category field is set to the value indicating the Wireless Network Management category, as specified in Table 24 in 7.3.1.11.

The Action field is set to the value indicating Leader Request frame, as specified in Table v49 in 7.4.8.

The Dialog Token field is set to a non-zero value chosen by the STA sending the Leader Request to identify the request/response transaction.

The Length field is set to 6n+1, where n indicates the total number of all multicast group addresses.

The Retransmission BSSID is set to a virtual BSSID. The BSSID of all retransmitted multicast frames shall be set to the Retransmission BSSID. Then, a non-AP STA that does not support the LBMS discards the retransmitted multicast frame because it does not associated with the corresponding BSSID.

The Multicast Group Address fields are set to the MAC addresses corresponding to the multicast groups to lead. The non-AP STA is requested to become the leader for each of these Multicast Group Addresses.

7.4.8.17 Leader Response

The Leader Response frame shall use the Action frame body format. The format of the Leader Response frame body is shown in Figure v106.

|  | Category | Action | Dialog Token | Length | Status Code #1 | Status Code #n |
| --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 1 | 1 | 1 |

- Figure v106—Leader Response frame body format

The Category field is set to the value indicating the Wireless Network Management category, as specified in Table 24 in 7.3.1.11.

The Action field is set to the value indicating Leader Response frame, as specified in Table v49 in 7.4.8.

The Dialog Token field is set to the value in any corresponding Leader Request frame. If the Leader Response corresponds to the unsolicited Leader Response, the Dialog Token field is set to zero.

The Length field is set to n+1, where n indicates the total number of all status codes.

The Status Code field (shown in Figure v107) is a bit field with the following bits defined:

|  | Decision Code | Multicast Option | ACK Policy | Retry Limit |
| --- | --- | --- | --- | --- |
| Bit: | 0 - 1 | 2 | 3 - 4 | 5 - 7 |

- Figure v107— Status Code field

- Table v50—Decision Code Definitions

| Value | Description |
| --- | --- |
| 0 | Accept |
| 1 | Reject – Unspecified reject reason |
| 2 | Reject – Withdrawn multicast group |
| 3 | Reject – Insufficient available resource |

- Decision code filed contains whether the leadership for the multicast group is accepted or not as defined in Table v50.

- Multicast Option is set to 1 to indicate that the multicast transmission policy is determined by the Status Code in Leader Response frame; else Multicast Option is set to 0 and the AP shall decide the multicast transmission policy.

- ACK Policy identifies the acknowledgement policy that is followed upon the delivery of the Multicast MPDU. The interpretation of these 2 bits is given in Table 6.

- Retry Limit is used to indicate the maximum retry limit of the multicast frames.

In order to inform the AP of its leadership acceptation (or rejection), the non-AP STA that receives the Leader Request frame shall send back to the AP a Leader Response frame with a status code for each multicast group address specified in the Leader Request frame received.

7.4.8.18 Leader Release

The Leader Release frame shall use the Action frame body format. The format of the Leader Release frame body is shown in Figure v108.

|  | Category | Action | Length | Multicast Group Address #1 | Multicast Group Address #n |
| --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 6 | 6 |

- Figure v108—Leader Release frame body format

The Category field is set to the value indicating the Wireless Network Management category, as specified in Table 24 in 7.3.1.11.

The Action field is set to the value indicating Leader Release frame, as specified in Table v49 in 7.4.8.

The Length field is set to 6n+1, where n indicates the total number of all multicast group addresses.

The Multicast Group Address fields are set to the MAC addresses of the multicast groups. Upon reception of the Leader Release frame, the non-AP STA shall cease sending acknowledment packets for the multicast groups specified in the Leader Release frame. Leader Release frames shall set the ACK policy sub-field in the frame to “normal ack” in order to remove the case of having multiple leaders for a multicast group because the release frame was not received by the previous leader.

Change 9.2.7 as shown:

- 9.2.7 Broadcast and multicast MPDU transfer procedure

In the absence of a PCF, when broadcast or multicast MPDUs are transferred from a STA with the ToDS bit clear, only the basic access procedure shall be used except when the LBMS  is used, see section 9.2.7.2. Regardless of the length of the frame, no RTS/CTS exchange shall be used. In addition, no ACK shall be transmitted by any of the recipients of the frame. However, when the LBMS is used, the ACK frame shall be transmitted by a selected receiver and RTS/CTS exchange can be used as an option.. Any broadcast or multicast MPDUs transferred from a STA with a ToDS bit set shall, in addition to conforming to the basic access procedure of CSMA/CA, obey the rules for RTS/CTS exchange, because the MPDU is directed to the AP. The broadcast/multicast message shall be distributed into the BSS. The STA originating the message shall receive the message as a broadcast/multicast message. Therefore, all STAs shall filter out broadcast/multicast messages that contain their address as the source address. Broadcast and multicast MSDUs shall be propagated throughout the ESS.

There is no MAC-level recovery on broadcast or multicast frames, except for those frames sent with the ToDS bit set and with the LBMS enabled. As a result, the reliability of this traffic is reduced, relative to the reliability of directed traffic, due to the increased probability of lost frames from interference, collisions, or time-varying channel properties.An AP that supports a leader-based multicast mechanism shall indicate this capability using the Wireless Network Management Capability information element. A non-AP STA shall inform the AP that it supports this functionality by using the Wireless Network Management Capability information element.

Insert the following after 9.2.7.1:

- 9.2.7.2 Leader Based Multicast Service (LBMS)

An AP that supports a LBMS shall indicate this capability using the Wireless Network Management Capability information element. A non-AP STA shall inform the AP that it supports this functionality by using the Wireless Network Management Capability information element.

- 9.2.7.2.1 Leader Election Protocol

The algorithm to select the non-AP STA to become a leader for a multicast group address is out of  scope of this specification. The leader may dynamically change according to varying channel conditions or group membership changes.  For example, the leader may be selected based on packet error rate statistics; the packet error rate of multicast receivers may be obtained through the Multicast Diagnostic Reports as specifed in 11.15.1.1. This specification describes the mechanism used by the AP to elect a non-AP STA (called selected leader) within the multicast group as a leader for this multicast group.

Three management action frames, specified in 7.4.8,  have been defined on this purpose: the Leader Request frame, the Leader Response frame and the Leader Release frame.

To elect a new leader for one or several multicast group(s), the AP shall send a LeaderRequest frame to the selected leader. Upon reception of the Leader Request frame, the selected leader shall inform the AP of its leadership acceptation (or rejection), by sending back to the AP a Leader Response frame indicating the status code for each multicast group address specified in the received Leader Request frame.

If there is already a leader for the multicast group(s) specified in this leader election, the AP shall indicate to the previous leader that it is no longer leader for these multicast groups. The AP shall send to the previous leader a Leader Release frame specifying the multicast group addresses to stop acknowledging frames under normal ack policy, before sending the Leader Request frame.

The selected leader may resign the leadership or update the multicast policy by sending an unsolicited Leader Response frame to the AP with the updated status code. The AP should ensure that the current leader is still in the BSS and trigger a new leader election in case for example some number of consecutive ACKs is missing.

- 9.2.7.2.2 Multicast Acknowledgement Mechanism

The leader is responsible of acknowledging each multicast frame successfully received for the multicast groups it has the leadership. The AP shall only elect one leader for a particular multicast group. The selected leader shall send an ACK frame after waiting SIFS upon successful reception of a data frame for this multicast group. If the AP does not receive the ACK frame from the leader during the ACK timeout, the AP shall increase the contention window by using the binary exponential backoff medium access mechanism defined in 9.1. The acknowledgements can also be used by the AP to adapt the PHY transmission rate of multicast frames and/or to improve the reliability of multicast transmission with retransmission, see section 9.2.7.2.3.

- 9.2.7.2.3 Multicast Retransmission Mechanism

When the LBMS is used, an optional retransmission mechanism is possible. The AP can disable multicast retransmission by forcing a retry limit of 0. If multicast retransmission is enabled, possibly duplicated multicast frames should be filtered out within the multicast receiver MAC as defined in 9.2.9.

When a STA that does not support the LBMS is associated in BSS, the retransmission BSSID that is assigned by the AP shall be used to retransmit the corresponding multicast frames in order to prevent the duplicated multicast frames. The BSSID of the received multicast frames is validated to ensure that the multicast originated from a STA in the BSS of which the receiving STA is a member.

- 9.2.7.2.4 Multicast RTS/CTS Mechanism

The standard RTS/CTS mechanism cannot be used for MPDUs sent in broadcast or multicast because it can cause collisions of multiple competing CTS frames sent in response. However, when the LBMS is used, the AP can optionally initiate the RTS/CTS exchange in order to either reduce the collision overhead of the multicast frame or solve the hidden terminal problem in the multiple overlapping BSSs.

The receiver address of the RTS frame shall be set to the MAC address of the selected leader. Then, the selected leader shall transmit a CTS frame after a SIFS period if the NAV after receiving the RTS frame indicates that the medium is idle.

Also, the CTS-to-self mechanism can be used for MPDUs sent in broadcast or multicast independent of  the LBMS.

Change 9.6 as shown:

- 9.6 Multirate support

Some PHYs have multiple data transfer rate capabilities that allow implementations to perform dynamic rate switching with the objective of improving performance. The algorithm for performing rate switching is beyond the scope of this standard, but in order to ensure coexistence and interoperability on multirate-capable PHYs, this standard defines a set of rules to be followed by all STAs.

Control frames that initiate a frame exchange shall be transmitted at one of the rates in the BSSBasicRateSet parameter except in the following cases:

—	When the transmitting STA’s protection mechanism is enabled and the control frame is a protection mechanism frame

—	When the control frame is a BlockAckReq or BlockAck frame

In the former case, the control frame shall be transmitted at a rate according to the separate rules for determining the rates of transmission of protection frames in 9.13. In the latter case, the control frame shall be transmitted in accordance with this subclause.

All frames with multicast and broadcast in the Address 1 field that have a UP of zero shall be transmitted at one of the rates included in the BSSBasicRateSet parameter, regardless of their type or subtype.

All frames with multicast and broadcast in the Address 1 field that have a UP of zero shall be transmitted at one of the rates included in the BSSBasicRateSet parameter except in the following case:

- When either multicast receivers or AP requests to use the higher multicast transmission rate than the rates in BSSBasicRateSet parameter

But, when a higher multicast transmission rate is used in the power saving mode, the last multicast MPDU after the DTIM shall be transmitted at one of the rates included in the BSSBasicRateSet parameter.

All data frames of subtype (QoS) (+)CF-Poll sent in the CP shall be transmitted at one of the rates in the BSSBasicRateSet parameter so that they will be understood by all STAs in the BSS, unless an RTS/CTS exchange has already been performed before the transmission of the data frame of subtype CF-Poll and the Duration field in the RTS frame covers the entire TXOP. All other data, BlockAckReq, and BlockAck frames and/or management MPDUs with unicast in the Address 1 field shall be sent using any data rate subject to the following constraints. No STA shall transmit a unicast frame at a rate that is not supported by the receiver STA, as reported in any Supported Rates and Extended Supported Rates element in the management frames transmitted by that STA. For frames of type (QoS) Data+CF-Ack, (QoS) Data+CF-Poll+CF Ack, and (QoS) CF-Poll+CF-Ack, the rate chosen to transmit the frame should be supported by both the addressed recipient STA and the STA to which the ACK frame is intended. The BlockAck control frame shall be sent at the same rate and modulation class as the BlockAckReq frame if it is sent in response to a BlockAckReq frame.

Under no circumstances shall a STA initiate transmission of a data or management frame at a data rate higher than the greatest rate in the OperationalRateSet, a parameter of the MLME-JOIN.request primitive. In the case where the supported rate set of the receiving STA is not known, the transmitting STA shall transmit at a rate contained in the BSSBasicRateSet parameter or a rate at which the transmitting STA has received a frame from the receiving STA.

To allow the transmitting STA to calculate the contents of the Duration/ID field, a STA responding to a received frame shall transmit its Control Response frame (either CTS or ACK), other than the BlockAck control frame, at the highest rate in the BSSBasicRateSet parameter that is less than or equal to the rate of the immediately previous frame in the frame exchange sequence (as defined in 9.12) and that is of the same modulation class (see 9.6.1) as the received frame. If no rate contained in the BSSBasicRateSet parameter meets these conditions, then the control frame sent in response to a received frame shall be transmitted at the highest mandatory rate of the PHY that is less than or equal to the rate of the received frame, and that is of the same modulation class as the received frame. In addition, the Control Response frame shall be sent using the same PHY options as the received frame, unless they conflict with the requirement to use the BSSBasicRateSet parameter.

An alternative rate for the control response frame may be used, provided that the duration of the control response frame at the alternative rate is the same as the duration of the control response frame at the originally chosen rate and the alternative rate is in either the BSSBasicRateSet parameter or the mandatory rate set of the PHY and the modulation of the control response frame at the alternative rate is the same type as that of the received frame.

For the Clause 17, Clause 18, and Clause 19 PHYs, the time required to transmit a frame for use in the Duration/ID field is determined using the PLME-TXTIME.request primitive (see 10.4.6) and the PLMETXTIME.confirm primitive (see 10.4.7), both defined in 17.4.3, 18.3.4, 19.8.3.1, 19.8.3.2, or 19.8.3.3 depending on the PHY options. In QoS STAs, the Duration/ID field may cover multiple frames and may involve using the PLME-TXTIME.request primitive several times.

Insert the following after 10.3.52

- 10.3.53 Leader Request

- This set of primitives supports the initialization of the leader election for the specific multicast groups between peer SMEs. Figure v113 depicts the Leader Management process using the Leader Request, Leader Response and Leader Release frames. The figure is only example and therefore is not meant to be exhautive of all possible protocol uses.

- Figure v113—Leader Management Process

- 10.3.53.1 MLME-LeaderRequest.request

- 10.3.53.1.1 Function

- This primitive requests the transmission of a Leader Request frame to a peer entity.

- 10.3.53.1.2 Semantics of the service primitive

- The primitive parameters are as follows:

MLME-LeaderRequest.request	(
Peer MAC Address,
Dialog Token,
Length,
Retransmission BSSID,
Multicast Group Address
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Peer MAC Address | MAC Address | Any valid individual MAC Address | The address of the peer MAC entity to which the Leader Request frame shall be sent. |
| Dialog Token | Integer | 0 – 255 | The dialog token to identify the Leader Request/Response transaction. |
| Length | Integer | 0 – 255 | The total length of the multicast group addresses. |
| Retransmission BSSID | MAC Address | Any valid individual MAC Address | The virtual BSSID for the retransmitted multicast frame |
| Multicast Group Address | MAC Address | Any valid individual MAC Address | The multicast group address for which the peer entity is requested to become the leader. |

- 10.3.53.1.3 When generated

- This primitive is generated by the SME to request that a Leader Request frame be sent to a peer entity to convey an indication that the peer entity is selected as the leader transmiting the ACK frame for the multicast frames.

- 10.3.53.1.4 Effect of receipt

On receipt of this primitive, the MLME constructs a Leader Request frame containing the specified multicast group address. This frame is then scheduled for transmission.

- 10.3.53.2 MLME-LeaderRequest.confirm

- 10.3.53.2.1 Function

- This primitive reports the request result of a Leader Request.

- 10.3.53.2.2 Semantics of the service primitive

- The primitive parameters are as follows:

MLME-LeaderRequest.confirm	(
Dialog Token,
Result Code 
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Dialog Token | Integer | 0 – 255 | The dialog token to identify the Leader Request/Response transaction. |
| Result Code | Enumeration | SUCCESS, INVALID PARAMETERS, or UNSPECIFIED FAILURE | Reports the outcome of a request to send a Leader Request frame. |

- 10.3.53.2.3 When generated

- This primitive is generated by the MLME when the request to transmit a Leader Request frame completes.

- 10.3.53.2.4 Effect of receipt

- On receipt of this primitive, the SME evaluates the result code.

- 10.3.53.3 MLME-LeaderRequest.indication

- 10.3.53.3.1 Function

- This primitive indicates that a Leader Request frame has been received requesting a Leader Request/Response transaction.

- 10.3.53.3.2 Semantics of the service primitive

- This primitive parameters are as follows:

MLME-LeaderRequest.indication	(
Peer MAC Address,
Dialog Token,
Length,
Retransmission BSSID,
Multicast Group Address
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Peer MAC Address | MAC Address | Any valid individual MAC Address | The address of the peer MAC entity from which the Leader Request was sent. |
| Dialog Token | Integer | 0 – 255 | The dialog token to identify the Leader Request/Response transaction. |
| Length | Integer | 0 – 255 | The total length of the multicast group addresses. |
| Retransmission BSSID | MAC Address | Any valid individual MAC Address | The virtual BSSID for the retransmitted multicast frame |
| Multicast Group Address | MAC Address | Any valid individual MAC Address | The multicast group address for which this station is requested to become the leader. |

- 10.3.53.3.3 When generated

- This primitive is generated by the MLME when a valid Leader Request frame is received.

- 10.3.53.3.4 Effect of receipt

- On receipt of this primitive, the SME either rejects the request or decides whether to accept or reject the Leader Request.

- 10.3.54 Leader Response

- This set of primitives supports the response to a request for the leader election between peer SMEs.

- 10.3.54.1 MLME-LeaderResponse.request

- 10.3.54.1.1 Function

- This primitive requests the transmission of a Leader Response frame to a peer entity.

- 10.3.54.1.2 Semantics of the service primitive

- The primitive parameters are as follows:

MLME-LeaderResponse.request	(
Peer MAC Address,
Dialog Token,
Length,
Status Code
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Peer MAC Address | MAC Address | Any valid individual MAC Address | The address of the peer MAC entity to which the Leader Response frame shall be sent. |
| Dialog Token | Integer | 0 – 255 | The dialog token to identify the Leader Request/Response transaction. |
| Length | Integer | 0 – 255 | The total length of the status codes. |
| Status Code | Integer | As specified in 7.4.8.17 | Indicates the result response to the Leader Request from the peer MAC entity. |

- 10.3.54.1.3 When generated

- This primitive is generated by the SME to request that a Leader Response frame be sent to a peer entity to convey a response to a Leader Request.

- 10.3.54.1.4 Effect of receipt

On receipt of this primitive, the MLME constructs a Leader Response frame containing the status code for the leadership acceptance of the speified multicast group address. This frame is then scheduled for tranmission.

- 10.3.54.2 MLME-LeaderResponse.confirm

- 10.3.54.2.1 Function

- This primitive reports the request result of a Leader Response.

- 10.3.54.2.2 Semantics of the service primitive

- The primitive parameters are as follows:

MLME-LeaderResponse.confirm	(
Dialog Token,
Result Code 
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Dialog Token | Integer | 0 – 255 | The dialog token to identify the Leader Request/Response transaction. |
| Result Code | Enumeration | SUCCESS, INVALID PARAMETERS, or UNSPECIFIED FAILURE | Reports the outcome of a request to send a Leader Response frame. |

- 10.3.54.2.3 When generated

- This primitive is generated by the MLME when the request to transmit a Leader Response frame completes.

- 10.3.54.2.4 Effect of receipt

- On receipt of this primitive, the SME evaluates the result code.

- 10.3.54.3 MLME-LeaderResponse.indication

- 10.3.54.3.1 Function

- This primitive indicates that a Leader Response frame has been received requesting an Leader Request/Response transaction.

- 10.3.54.3.2 Semantics of the service primitive

- The primitive parameters are as follows:

MLME-LeaderResponse.indication	(
Peer MAC Address,
Dialog Token,
Length,
Status Code
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Peer MAC Address | MAC Address | Any valid individual MAC Address | The address of the peer MAC entity from which the Leader Response was sent. |
| Dialog Token | Integer | 0 – 255 | The dialog token to identify the Leader Request/Response transaction. |
| Length | Integer | 0 – 255 | The total length of the status codes. |
| Status Code | Integer | As specified in 7.4.8.17 | Indicates the result response to the Leader Request from the peer MAC entity. |

- 10.3.54.3.3 When generated

- This primitive is generated by the MLME upon reception of  a valid Leader Response frame.

- 10.3.54.3.4 Effect of receipt

- On receipt of this primitive, the SME either rejects the response or commences the LBMS as described in 9.2.7.2 and 9.10.6.

- 10.3.55 Leader Release

- This set of primitives supports the indication of the leader release for the specific multicst groups between peer SMEs.

- 10.3.55.1 MLME-LeaderRelease.request

- 10.3.55.1.1 Function

- This primitive requests the transmission of a Leader Release frame to a peer entity.

- 10.3.55.1.2 Semantics of the service primitive

- The primitive parameters are as follows:

MLME-LeaderRelease.request	(
Peer MAC Address,
Length,
Multicast Group Address
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Peer MAC Address | MAC Address | Any valid individual MAC Address | The address of the peer MAC entity to which the Leader Release frame shall be sent. |
| Length | Integer | 0 – 255 | The total length of the multicast group addresses. |
| Multicast Group Address | MAC Address | Any valid individual MAC Address | The multicast group address for which this station is requested to release the leadership. |

- 10.3.55.1.3 When generated

- This primitive is generated by the SME to request that a Leader Release frame be sent to a peer entity to convey an indication that the peer entity shall release the leadership for the specified multicast group address.

- 10.3.55.1.4 Effect of receipt

On receipt of this primitive, the MLME constructs a Leader Release frame containing the specified multicast group address. This frame is then scheduled for tranmission.

- 10.3.55.2 MLME-LeaderRelease.confirm

- 10.3.55.2.1 Function

- This primitive reports the request result of a Leader Release.

- 10.3.55.2.2 Semantics of the service primitive

- The primitive parameter is as follows:

MLME-LeaderRelease.confirm	(
Result Code 
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Result Code | Enumeration | SUCCESS, INVALID PARAMETERS, or UNSPECIFIED FAILURE | Reports the outcome of a request to send a Leader Release frame. |

- 10.3.55.2.3 When generated

- This primitive is generated by the MLME when the request to transmit a Leader Release frame completes.

- 10.3.55.2.4 Effect of receipt

- On receipt of this primitive, the SME evaluates the result code.

- 10.3.55.3 MLME-LeaderRelease.indication

- 10.3.55.3.1 Function

- This primitive indicates that a Leader Release frame has been received.

- 10.3.55.3.2 Semantics of the service primitive

- This primitive parameters are as follows:

MLME-LeaderRelease.indication	(
Peer MAC Address,
Length,
Multicast Group Address
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Peer MAC Address | MAC Address | Any valid individual MAC Address | The address of the peer MAC entity from which the Leader Release was sent. |
| Length | Integer | 0 – 255 | The total length of the multicast group addresses. |
| Multicast Group Address | MAC Address | Any valid individual MAC Address | The multicast group address for which this station is requested to release the leadership. |

- 10.3.55.3.3 When generated

- This primitive is generateed by the MLME when a valid Leader Release frame is received.

- 10.3.55.3.4 Effect of receipt

On receipt of this primitive, the SME shall resign the leadership for the specified multicast group address as described in 9.2.7.2.

Insert the following new rows into A.4.15:

### A.4.16 Wireless Network Management extensions

| Item | Protocol Capability | References | Status | Support |
| --- | --- | --- | --- | --- |
| RME11 | Leader Based Multicast Service | 9.2.7.2 | CFv:O | Yes, No, N/A |
| RME11.1 | Leader Request | 7.4.6.16 | CFv:M | Yes, No, N/A |
| RME11.2 | Leader Response | 7.4.6.17 | CFv:M | Yes, No, N/A |
| RME11.3 | Leader Release | 7.4.6.18 | CFv:M | Yes, No, N/A |