|  |  |  |  | LB92 Comment Resolution |  |
| --- | --- | --- | --- | --- | --- |
| CID | Commenter: | Clause: | Addressed By: |  | Original Date Prepared |
| 436 | Engwer | 10.3.9.1.4 | Noens |  | 01/16/07 |

- COMMENT:  [From Spreadsheet]

What happens if the MAC cannot be reset within the requrie 2 TUs?  Can the SME expect an error condition to be returned?  If so, then RESET.confirm must be modified accordlingly.

- Commenter’s Suggested Remedy (If appropriate):  [From Spreadsheet]

Add text to describe what actiosn are taken and what conditions exist if the MAC cannot be reset within 2 TUs.

- Background, Explanation, Discussion, etc.:

2 TUs were selected after discussions with several silicon manufactures who assured representatives of TGp that meeting this requirement was not a problem.  The reason for selecting 2 TUs was to facilitate channel switching and knowing that operation will not be interrupted.  There will be no error condition returned as it is a requirement that the MAC be reset within the 2 TUs.

- Recommended Resolution of the Comment:

Reject but consider putting information in the informative introduction section that is to be added to the next draft.

- Motion (if technical and/or significant):

(And instructions to the editor.)

Move to Reject CID 436 and notify the editor with the information for possible inclusion of an explanation for this requirement within the introduction of the next draft.

Motion by: __Rick Noens________Date: _01/17/07________

Second:  ______________________

| Approve: | Disapprove: | Abstain: |
| --- | --- | --- |

References: