IEEE P802.11
Wireless LANs

| Normative Text for TIM Request |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-13-03 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Menzo Wentink | Conexant | Oudegracht 3a, Utrecht, Netherlands | +31 65 183 6231 | menzo.wentink@conexant.com |
| Jari Jokela | Nokia | Visiokatu 1, 33720 Tampere, Finland | +358504860445 | jari.jokela@nokia.com |
| Marc de Courville | Motorola | Parc les Algorithmes, 91193 Gif-sur-Yvette, France | +33169352518 | marc.de.courville@motorola.com |
| Yongho Seok | LG Electronics | 16 Woomyeon-Dong, Seocho-Gu, Seoul 137-724, Korea | +8225264225 | yhseok@tge.com |

Abstract

This document contains normative text for a TIM request proposal to meet the TGv Power Saving objective, REQ2010, improving mobile client power efficiency and battery life. The text is aligned with P802.11-REVma/D9.0 and P802.11v D0.05.

The TIM request is a mechanism which allows a client in standby mode to request for a TIM element from it´s AP in a power efficient way.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart@ok-brit.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

## 7. Frame formats

## 7.1 MAC frame formats

## 7.1.3 Frame fields

## 7.1.3.1 Frame control field

## 7.1.3.1.2 Type and Subtype fields

Change Table 1 as follows (only new and modified entries shown):

Table 1—Valid type and subtype combinations

| Type value b3 b2 | Type description | Subtype value b7 b6 b5 b4 | Subtype description |
| --- | --- | --- | --- |
| 01 | Control | 0000-0100 | Reserved |
| 01 | Control | 0101 | TIM Request |
| 01 | Control | 0110 | TIM Response |

Add two new clauses after Clause 7.2.1.9 as shown below:

#### 7.2.1.10 TIM Request frame format

The frame format for the TIM Request frame is defined in Figure p1.

|  | Frame Control | Duration/ID | BSSID | TA | FCS |
| --- | --- | --- | --- | --- | --- |
| Octets: | 2 | 2 | 6 | 6 | 4 |

- Figure p1 — TIM Request frame format

The Duration/ID field value may set to the time, in microseconds, required to transmit a solicited response frame plus a SIFS interval. If the calculated duration includes a fractional microsecond, that value is rounded up to the next higher integer.

The BSSID field shall contain the BSSID.

The TA field contains the address of the STA transmitting the TIM Request frame.

#### 7.2.1.11 TIM Response frame format

The frame format for the TIM Response frame is as defined in Figure p2.

|  | Frame Control | Duration/ID | RA | TIM | FCS |
| --- | --- | --- | --- | --- | --- |
| Octets: | 2 | 2 | 6 | 6-257 | 4 |

- Figure p2 — TIM Response frame format

The Duration/ID field value may be be set to 0.

The RA field contains the address of the recipient STA, which is copied from the TA field of the TIM Request.

The TIM field contains the TIM element, starting with the Element ID. AID 0 of the TIM shall be ignored by receving stations. The AP may generate a TIM which does not include AID 0.

## 11. MAC sublayer management entity

## 11.2 Power management

## 11.2.1 Power management in an infrastructure network

Add a new clause after 11.2.1.12:

- 11.2.1.12  STA operation for receiving a TIM

A station may request for a TIM element by sending a TIM Request frame. The TIM Response frame shall be sent after SIFS. The frames may be sent at any rate, i.e. there is no restriction to a basic rate.

# Annex A

## A.4 PICS proforma–IEEE Std. 802.11, 1999 Edition

### A.4.3 IUT configuration

### A.4.14 Wireless Network Management extensions

Insert the following entries into A.4.14 in Annex A as follows:

| Item | Protocol Capability | References | Status | Support |
| --- | --- | --- | --- | --- |
| RME12 | TIM Request/Response | 7.2.1.10, 7.2.1.11, 11.2.1.12 | CFv:M | Yes, No. N/A |