IEEE P802.11
Wireless LANs

| Comment 96 Resolution |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-02-06 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Dorothy Stanley | Aruba Networks | 1322 Crossman Ave Sunnyvale, CA 94089 | +1-630-363-1389 | dstanley@arubanetworks.com |

Abstract

This document contains the proposed text to resolve comment 96, from the informal review of the TGv draft.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart@ok-brit.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Comment 96:  “Table 26 needs to be consistent with Table 26 in P802.11ma D8.0”

Commenter’s suggested resolution: Subclause numbers shall be added in the field of "Information element". The column "Length (in octets)" shall be added.

Change Table 26 in TGv-D0.07, 7.3.2 as shown below:

Table 26—Element IDs

| Information Element | Element ID | Length (in octets) |
| --- | --- | --- |
| Wireless Network Management Capability (see 7.3.2.35) | x | 2 |
| Event Log Request (see 7.3.2.36) | x+1 | 2-255 |
| Event Log Report (see 7.3.2.37) | x+2 | 3-255 |
| Diagnostic Request (see 7.3.2.38) | x+3 | 2-255 |
| Diagnostic Report (see 7.3.2.39) | x+4 | 3-255 |
| Presence Parameters (see 7.2.3.40) | x+5 | 2-255 |
| Multiple BSSID (see 7.3.2.41) | x+6 | 1-255 |
| Multiple SSID (see 7.3.2.42) | x+7 | 1-255 |
| Multiple BSSID-Index (see 7.3.2.43) | x+8 | 1-3 |
| AID 0 Descriptor (see 7.3.2.44) | x+9 | 1-17 |
| FBMS Request (see 7.3.2.45) | x+10 | 1-255 |
| FBMS Response (see 7.3.2.46) | x+11 | 5-128 |
| Extended Channel Switch Announcement (see 7.3.2.47) | x+12 | 4 |
| Supported Regulatory Classes (see 7.3.2.48) | x+13 | 1-32 |
| Reserved | x+14, 220 |  |

- Change the length of the Wireless Network Management Capability Information element as shown below:

      - Wireless Network Management Capability information element

The Wireless Network Management Capability Information element contains information about the wireless network management capabilities of a STA as shown in Figure v1.

|  | Element ID | Length | Wireless Network Management Capabilities |
| --- | --- | --- | --- |
| Octets: | 1 | 1 | Variable 2 |

- —Wireless Network Management Capability information element format

The Element ID field is equal to the Wireless Network Management Capability value in Table 26.

The value of the length field is variable and depends on the length of the Wireless Network Management Capabilities field.  The minimum value of the Length field is 2.

Change the lengths in Figure v80 as shown below:

The FBMS Response defines information about the broadcast/multicast status. The format of the FBMS Response element is shown in .

|  | Element ID | Length | FBMS Status Element 1 | FBMS Status Element n |
| --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 5 5-16 | 5 5-16 |

- Figure v80—FBMS Response element format

The Length field is set to 1+n, where n indicates the total length of all FBMS Status Elements contained in the element.

References:

11-06-1615-09-000v-tgv-internal-review-comment-submissions