IEEE P802.11
Wireless LANs

| Text for Portal Announcement Frame Format |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-03-14 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Michael Bahr | Siemens AG, Corporate Technology | Otto-Hahn-Ring 6 81730 München, Germany | +49-636-49926 | bahr@siemens.com |

Abstract

The document provides a resolution to comments CIDs 403 and 5658 of LB 93.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart@ok-brit.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

# General

The document provides a resolution to comments CIDs 403 and 5658.

# Comments

## CID 403 (RFI 1)

There is no mesh management action frame format, that transports the PANN IE, defined in clause 7.4.6.

Add a clause for the "Portal Announcement frame format" in clause 7.4.6 and add it to table s25.

## CID 5658 (RFI 12)

There are no definitions on the PANN Management frames.

Add an item of PANN in Table s25: Mesh Management Action field values (3-addr action frames).

# Proposed Resolution by this document

Add to table s11 in clause 7.4.9 a row with Action field value of 14 and make changes to the row 14-220. Action field values might have to be adjusted due to other comment resolutions.

- Mesh management action frame details (3-addr action frames)

Action frame formats for mesh management are defined in this subclause. An Action field, in the octet field immediately after the Category field, differentiates the frame format. The Action field values associated with each frame format are defined in  Table s11.

| Table s11―Mesh Management Action field values (3-addr action frames) |  |  |
| --- | --- | --- |
| Action field value | Description | Application |
| 0 | Local Link state announcement | Neighbor discovery |
| 1 | Route Request | HWMP routing |
| 2 | Route Reply | HWMP routing |
| 3 | Route Error | HWMP routing |
| 4 | Route Reply Ack | HWMP routing |
| 5 | Congestion Control Request | Congestion Control |
| 6 | Congestion Control Response | Congestion Control |
| 7 | Neighborhood Congestion Announcement | Congestion Control |
| 8 | Mesh Deterministic Access (MDA) | MDA |
| 9 | Beacon Timing Request | Beaconing and Synchronization |
| 10 | Beacon Timing Response | Beaconing and Synchronization |
| 11 | Non-mesh Action Encapsulation | Action |
| 12 | Connectivity Report | Connectivity reporting |
| 13 | Radio Aware Optimized Link State Routing (RA-OLSR) | RA-OLSR |
| 14 | Portal Announcement | MPP Announcement Protocol |
| 15-220 | Reserved |  |
| 221 | Vendor Specific Mesh Management | Vendor Specific |
| 222-255 | Reserved |  |

Insert new clause 7.4.9.15

- 7.4.9.15 Portal Announcement Frame Format

The Portal Announcement frame format uses the Action frame body format and is transmitted by an MPP to announce its presence in the mesh network. This frame is transmitted in a broadcast manner. MPs rebroadcasts this frame after updating the contained Portal Announcement IE. The frame format is shown in Figure s-X1.

| Octets: 1 | 1 | Variable |
| --- | --- | --- |
| Category | Action | Portal Announcement Element |

Figure s-X1: Portal Announcement frame format

The Category field shall be set to 5 (representing mesh management).

The Action field shall be set to 14 (representing Portal Announcement)

The fields of the Portal Announcement element shall be set as described in 11A.4.2.