IEEE P802.11
Wireless LANs

| Protected Management Frame Pseudo-code |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-01-30 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Abhijit Choudhury | Cisco Systems | 3625 Cisco Way, San Jose CA 95134 | +1-408-525-8682 | abhijitc@cisco.com |
| Nancy Cam-Winget | Cisco Systems | 190 W Tasman, San Jose CA 95134 | +1-408-853-0532 | ncamwing@cisco.com |
| Jesse Walker | Intel | 2111 NE 25th Ave JF3-206 Hillsboro OR 97124 | +1-503-712-1849 | jesse.walker@intel.com |
| Dorothy Stanley | Aruba | 1322 Crossman Ave, Sunnyvale, CA 94089 | +1-630-363-1389 | dstanley@arubanetworks.com |
| Kapil Sood | Intel Corporation | 2111 NE 25th Ave JF3-206 Hillsboro OR 97124 | +1-503-264-3759 | kapil.sood@intel.com |

Abstract

This document contains proposed changes to the Per-frame Pseudo-code in Clause 8.7 in TGw draft D1.0 to address comments (#367, 370, 398, 391, 399, 400, 406, 407, 467, 895, 900 – in Group 14).

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

8.7.2.1 Per-MSDU Tx pseudo-code

8.7.2.1a Per-MMPDU Tx pseudo-code

if (dot11RSNAEnabled = TRUE) then

if (Robust Management Frame Protection bit of RSNA Capability Field is set for Tx) then

if (MMPDU has an individual RA and

((Protection for RA is off for Tx) or

((dot11RSNALegacyManagementFrames = TRUE) and

(Robust Management Frame Protection bit of RSNA Capability Field is not set for RA)))) then

Transmit the MMPDU without protection

else if ((MMPDU has individual RA) and (Pairwise key exists for the     MMPDU's RA)) then

// if we find a suitable Pairwise key

if (key is a null key) then

Discard the entire MMPDU

else

// Note that it is assumed that no entry will be in the key

// mapping table of a cipher type that is unsupported.

Set the Key ID subfield of the IV field to zero

if (cipher type of entry is AES-CCM) then

Transmit the MMPDU, to be protected after fragmentation using AES-CCM

else

// No other cipher type is supported for unicast

Discard the entire MMPDU

endif

endif

else if ((MMPDU has a multicast/broadcast RA) and (IGTK exists)) then

// if we find a suitable IGTK

if (key is a null key) then

Discard the entire MMPDU

else

Set the Key ID subfield of the MMIE to corresponding IGTK KeyID

if (cipher type of entry is AES-128-CMAC) then

Transmit the MMPDU with BIP

else

// No other cipher type is supported for

// multicast/broadcast

Discard the entire MMPDU

endif

endif

else // else we didn't find a key but we are protected,

// so discard

Discard the entire MMPDU

endif

else

// Management Frame Protection is not supported

Transmit the MMPDU without protection

endif

endif

8.7.2.2a Per-MPDU Tx pseudo-code for MMPDU

if ((dot11RSNAEnabled = TRUE) and (Robust Management Frame Protection bit of RSNA Capability Field is set for Tx)) then

if (MPDU is member of an MMPDU that is to be transmitted without   protection) then

Transmit the MPDU without protection

else if (MMPDU is to be protected using AES-CCM) then

Protect the MPDU using entry's PTK and AES-CCM

Transmit the MPDU

else if (MMPDU is to be protected using AES-128-CMAC) then

Protect the MPDU using IGTK and BIP

Transmit the MPDU

else

Discard the MPDU

endif

endif

8.7.2.3A Per-MPDU Rx pseudo-code for MMPDU

if (dot11RSNAEnabled = TRUE) then

if (Robust Management Frame Protection bit of RSNA Capability Field is set for Rx) then

if (Protected Frame subfield of the Frame Control Field is zero) then

if ((Protection for TA is off for Rx) or

((dot11RSNALegacyManagementFrames = TRUE for Rx) and

(Robust Management Frame Protection bit of RSNA Capability

Field is not set for TA))) then

Receive the unencrypted MPDU

else

Discard the frame without indication to LLC

Increment dot11WEPExcludedCount ???

endif

else if (Protection for TA is true for Rx) then

if ((MPDU has individual RA) and

(Pairwise key exists for the MPDU’s TA)) then

if (key is null) then

Discard the frame

Increment dot11WEPUndecryptableCount ???

elseif (entry has an AES-CCM key) then

Decrypt frame using AES-CCM key

if (the integrity check fails) then

Discard the frame

Increment dot11RSNAStatsCCMPDecryptErrors

endif

else

Discard the frame

Increment dot11WEPUndecryptableCount ???

else if ((MPDU has multicast/broadcast RA) and (IGTK exists)) then

if (key is null) then

Discard the frame

Increment dot11WEPUndecryptableCount ???

elseif entry has an IGTK then

Check integrity of the frame using IGTK for AES-128-CMAC

if the ICV fails then

Discard the frame

Increment dot11RSNAStatsCMACICVErrors

endif

else

Discard the frame

Increment dot11WEPUndecryptableCount ???

endif

else

Discard the frame

Increment dot11WEPUndecryptableCount ???

endif

else

Discard the frame

Increment dot11WEPUndecryptableCount ???

endif

elsif ((MPDU has multicast/broadcast RA) and

(Protected Frame subfield of the Frame Control Field is set)) then

Receive the MPDU and ignore the integrity check

else

Receive the unencrypted MPDU

endif

endif

8.7.2.4A Per-MMDPU Rx pseudo-code

if dot11RSNAEnable = TRUE and Robust Management Frame Protection bit of RSNA Capability Field is set then

if the frame was not protected then

Receive the MMPDU unprotected

else //Have a protected MMPDU

if (MMPDU has individual RA and the Pairwise key is an AES-CCM key) then

if its MPDUs had sequential PNs or it consists of only one MPDU then

Accept the MMPDU

else

Discard the MMPDU as a replay attack

Increment dot11RSNAStatsCCMPReplays

endif

elseif (MMPDU has multicast/broadcast RA) then

if its MPDUs had sequential PNs or it consists of only one MPDU then

Accept the MMPDU

else

Discard the MMPDU as a replay attack

Increment dot11RSNAStatsCMACReplays

endif

endif

endif

endif

References:

IEEE 802.11 WG LB88: IEEE P802.11w/1.0, October 2006-10-13