IEEE P802.11
Wireless LANs

| TGy LB94 Submission for clause 7, 10, 11 CIDs |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-02-12 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Peter Ecclesine | Cisco Systems | 170 W. Tasman Dr. MS SJ-10-5 San Jose, CA 95134-1706 | +1-408-527-0815 | petere@cisco.com |

Abstract

This document builds on 11-06/1775r2 E-CSA, and is aligned with P802.11 REV-ma D9.0, P802.11k/D7.0, P802.11r/D4.1 for baseline, P802.11n/D2.0 and P802.11y/D1.0 and addresses the following LB94 comments:

CIDs: 341, 345, 347, 348, 349, 350, 352, 353, 399, 540, 542, 543, 592, 623, 624

CIDs: 50, 71, 88, 158, 300, 351, 352, 353, 472, 544, 619, 625

CIDs: 6, 14, 15, 16, 28, 65, 102, 127, 128, 129, 130, 131, 132, 133, 145, 164, 165, 194, 195, 238, 253, 254, 255, 259, 260, 263, 264, 277, 288, 289, 290, 301, 354, 355, 356, 357, 358, 413, 435, 436, 437, 438, 457, 473, 508, 509, 545, 546, 547, 548, 549, 550, 593, 594, 626, 627, 628, 663, 664, 668, 683, 694

CIDs: 4, 17, 18, 51, 96, 134, 135, 136, 137, 138, 155, 172, 210, 211, 251, 359, 389, , 439, 448, 458, 459, 476, 477, 510, 511, 551, 552, 595, 596, 669, 670, 679

CIDs: 19, 63, 78, 139, 175, 212, 261, 278, 360, 391, 460, 478, 512, 513, 553, 554, 597, 598, 599, 629, 630

CIDs: 89, 156, 176, 252, 361, 362, 363, 364, 365, 400, 440, 449, 461, 474, 514, 555, 556, 557, 558, 620, 655, 673

CIDs: 21, 79, 103, 107, 114, 142, 197, 279, 368, 369, 370, 371, 414, 415, 464, 475, 560, 561, 562, 564, 631, 632, 633

CIDs: 2, 3, 7, 30, 66, 67, 68, 90, 104, 115, 116, 120, 121, 143, 144, 153, 154, 159, 160, 161, 180, 196, 198, 199, 200, 213, 247, 248, 249, 250, 262, 280, 291, 292, 372, 441, 442, 446, 447, 465, 466, 479, 565, 566, 634, 665, 671, 672, 676, 681

CIDs: 22, 87, 105, 111, 122, 146, 181, 182, 224, 302, 373, 375, 376, 467, 518, 519, 567, 568, 602

CIDs: 147, 166, 167, 201, 214, 406, 480, 520, 603

CIDs: 293, 377, 407, 408, 409, 468, 521, 604, 605

CIDs: 8, 9, 10, 23, 24, 31, 52, 55, 80, 91, 117, 118, 148, 149, 150, 157, 168, 183, 184, 202, 203, 208, 215, 216, 217, 218, 219, 256, 265, 281, 282, 283, 392, 393, 394, 395, 402, 403, 410, 411, 412, 416, 417, 418, 419, 469, 470, 471, 481, 482, 483, 484, 485, 522, 523, 524, 525, 526, 569, 570, 572, 606, 607, 609, 610, 611, 635, 636, 637, 638, 639, 641, 684, 696

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart@ok-brit.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

# Introduction

## Interpretation of a Motion to Adopt

A motion to approve this submission means that the editing instructions and any changed or added material are actioned in the TGy Draft.  This introduction, is not part of the adopted material.

Editing instructions formatted like this are intended to be copied into the TGy Draft (i.e. they are instructions to the 802.11 editor on how to merge the TGy amendment with the baseline documents).

TGy Editor:  Editing instructions preceded by “TGy Editor” are instructions to the TGy editor to modify existing material in the TGy draft.   As a result of adopting the changes, the TGy editor will execute the instructions rather than copy them to the TGy Draft.

Summission Note: Notes to the reader of this submission are not part of the motion to adopt.  These notes are there to clarify or provide context.

# Clauses 7.2.3

CIDs: 341, 345, 347, 348, 349, 350, 352, 353, 399, 540, 542, 543, 592, 623, 624

Discussion:

In 11y/D1.0:

- Comments 399, 540, 543, 624 suggest the E-CSA MIB variable have another name, and P802.11n/D2.0 uses dot11ExtendedChannelSwitchImplemented for that purpose. Propose 802.11y/D1.1 does also for Extended Channel Switch Announcement and Supported Regulatory Classes.

- Comments 341, 345, 347, 348, 349, 350 note that baseline includes P802.11k/D7.0 and P802.11r/D4.1, therefore the frame Orders should be changed to reflect that. Propose 802.11y/D1.1 does.

- Comments 542 and 632 note that Probe Response frame notes for ECSA say it ‘may be present if’, and suggest rewordinging ‘only’ if dot11…’ is true. Restrictions on information present in a Probe Response are in REV-ma text in paragraph two above Table 15, which says “A STA shall return only the information elements that it supports.” Most of the Notes in Probe Response are identical to Notes in Beacon frames. Propose P802.11y/D1.1 change 7.2.3.1 Beacon frame and 7.2.3.9 Probe Response ECSA Note to “may be present only if dot11ExtendedChannelSwitchImplemented is true.”

- Comments 352 and 353 request Table 15 Element IDs follow from ANA assignments, and correct references and lengths be provided. Propose 802.11y/D1.1 use the notation of 11n/D2.0 Table 15.

- Comment 592 asks why DSE LCI is in Beacon frames. The DSE LCI is required in Beacons to assist interference identification and resolution, which is required of all operation in the band.

Proposed Resolution:

Accept: 341, 345, 347, 348, 349, 350, 352, 353, 399, 540, 542, 543, 623, 624

Decline: Comment 592 to use/adopt P802.11k/D7.0 LCI in Beacon frame.

Editorial Instructions:

Tgy Editor: replace text in 7.2.3, pg 5-7, D1.0 as follows

Insert the following new elements in table 8 as shown:

Table 8: Beacon frame body

| Order | Information | Notes |
| --- | --- | --- |
| 2531 | DSE registered location | The DSE registered location information element shall be present if dot11DSERequired is true or dot11RegLocRequired is true. |
| 2632 | Extended Channel Switch Announcement | Extended Channel Switch Announcement element may be present only if dot11OFDMCCAEDRequireddot11ExtendedChannelSwitchImplemented is true. |

EDITORIAL NOTE— last entry in 11ma-D9.0 has order 24, 11k/D7.0 adds 5 and 11r/D4.1 adds 1.

- Association Request frame format

Insert new row into table 10 as shown below:

Table 10: Association Request frame format

| Order | Information | Notes |
| --- | --- | --- |
| 1011 | Supported Regulatory Classes | Supported Regulatory Classes is present if dot11OFDMCCAEDRequireddot11ExtendedChannelSwitchImplemented is true. |

EDITORIAL NOTE— last entry in 11ma-D9.0 has order 9 and 11r/D4.1 adds 1.

- Association Response frame format

Insert new rows into table 11 as shown below:

Table 11: Association Response frame body

| Order | Information | Notes |
| --- | --- | --- |
| 712 | Supported Regulatory Classes | Supported Regulatory Classes information element is present if dot11OFDMCCAEDRequireddot11ExtendedChannelSwitchImplemented is true. |
| 813 | DSE registered location | DSE registered location information element is present if dot11RegLocRequired is true or dot11DSERequired is true |

EDITORIAL NOTE— last entry in 11ma-D9.0 has order 6, 11k/D7.0 adds 2 and 11r/D4.1 adds 3.

- Reassociation Request frame format

Insert a new row into table 12 as shown below:

Table 12: Reassociation Request frame body

| Order | Information | Notes |
| --- | --- | --- |
| 1114 | Supported Regulatory Classes | Supported Regulatory Classes information element is present if dot11OFDMCCAEDRequireddot11ExtendedChannelSwitchImplemented is true. |

EDITORIAL NOTE— last entry in 11ma-D9.0 has order 10 and 11r/D4.1 adds 3.

- Reassociation Response frame format

Insert new rows into table 13 as shown below:

Table 13: Reassociation Response frame body

| Order | Information | Notes |
| --- | --- | --- |
| 713 | Supported Regulatory Classes | Supported Regulatory Classes information element is present if dot11OFDMCCAEDRequireddot11ExtendedChannelSwitchImplemented is true. |
| 814 | DSE registered location | DSE registered location information element is present if dot11RegLocRequired is true or dot11DSERequired is true |

EDITORIAL NOTE— last entry in 11ma-D9.0 has order 6, 11k/D7.0 adds 2 and 11r/D4.1 adds 4.

- Probe Response frame format

Insert the following changes to 7.2.3.9 –new rows into Table 15:

Table 15: Probe Response frame body

| Order | Information | Notes |
| --- | --- | --- |
| 2330 | DSE registered location | Is present only if dot11RegLocRequired is true or dot11DSERequired is true. |
| 2431 | Extended Channel Switch Announcement | Extended Channel Switch Announcement element may be present only if dot11OFDMCCAEDRequireddot11ExtendedChannelSwitchImplemented is true. |

- EDITORIAL NOTE— last entry in 11ma-D9.0 has order 22, 11k/D7.0 adds 6 and 11r/D4.1 adds 1.

Clause 7.3.2

CIDs: 50, 71, 88, 158, 300, 351, 352, 353, 472, 544, 619, 625

Discussion:

In 11y/D1.0:

- Comments 50, 71, 88, 158, 300, 351, 472, 544, 619, 625399, 540, 543, 624 suggest the 802.11 ANA has to assign Element IDs  Propose 802.11y/D1.1 use the editorial language of 11n/D2.0 to show this.

- Comments 352 and 353 want correct cross reference and length for inserted entries. Propose 802.11y/D1.1 do this.

Proposed Resolution:

Accept: 71, 351, 352, 353, 544, 625

Counter: based on discussion and editorial instructions in 07/0245; 50, 88, 158, 300, 472, 619.

Editorial Instructions:

Tgy Editor: replace text in 7.3.2, pg 7, lines 5-7  D1.0  as follows

    - Information elements

EDITORIAL NOTE—Numbers marked as <ANA> will be inserted once assigned by the 802.11 ANA.

Insert the following three rows (ignoring the header row) in Table 26—Element IDs in the correct position to preserve ordering by the “Element ID” column and update the “Reserved” range of codes row appropriately.

Table 26—Element IDs

| Information Element | Element ID | Length (in octets) |
| --- | --- | --- |
| DSE registered location (see 7.3.2.3649) | 51<ANA> | 1620 |
| Reserved | 521-126 |  |
| Extended Capabilities | 127 | 2 to 257 |
| Extended Channel Switch Announcement (see 7.3.2.50) | X<ANA> | 8 |
| Supported Regulatory Classes (see 7.3.2.51) | X+1<ANA> | 3 to 34 |
| Reserved | x+2128-220 |  |

# DSE registered location element

CIDs: 6, 14, 15, 16, 28, 65, 102, 127, 128, 129, 130, 131, 132, 133, 145, 164, 165, 194, 195, 238, 253, 254, 255, 259, 260, 263, 264, 277, 288, 289, 290, 301, 354, 355, 356, 357, 358, 413, 435, 436, 437, 438, 457, 473, 508, 509, 545, 546, 547, 548, 549, 550, 593, 594, 626, 627, 628, 663, 664, 668, 683, 694

Discussion:

A little history: IETF RFC 3825 LCI, July 2004, was modified and put in P802.11k/D1.0. 11-05/517r0 removed text that duplicated normative text in RFC 3825, and added a query/response mechanism to ask “where am I/where are you?”, which was subsequently modified adding an optional radio azimuth, with the resulting element specified in 11k/D7.0 7.3.2.22.9. From TGk LB73 on, I have processed all Location comments and made recommendations for their resolution. In developing TGy Dependent Station Enablement (06/864), the need to assist identifying interference sources became paramount, and we accepted making every Beacon frame contain the location of the registered STA. Every Beacon frame of every 802.11 STA operating in the band contains the DSE registered location element. For this reason, we sought to reduct the number of octets in the element, and found that 15 1/8 octets are required to convey the RFC-3825 fields, if you exclude unused Altitude Types and Datums. DSE adds information about Dependent operation, Enablement, and Operation by Agreement near national borders or within the protected distances of grandfathered FSS and Radiolocation stations, and that information is encoded within the unused bits of the Datum octet. DSE adds a Dependent Enablement Identifier field that other licensed operators can receive and take note of. A licensed operator receiving an interference complaint can identify the STA or STAs involved, can command individual STAs to change their operating parameters, like transmit power, data rate, channel bandwidth and channel frequency in an effort to resolve interference complaints. A licensed operator can disable any dependent STA, forcing it to seek other enablement.

TGk’s LCI measurement report element does not have reserved fields or spare bits defined that could be used for DSE information. TGk’s LCI has two lengths, depending on whether the optional Azimuth Report is present.

Combining TGk’s LCI with TGy’s DSE registered location element would increase the size by 1 octet in every Beacon frame of every 802.11 STA operating in the band. TGk’s measurement request/response does not appear in Beacon frames, so a few spare bits have little effect on other STAs operating in the band. It is painless to modify TGk’s LCI in Sponsor Ballot to be compatible with 11y/D1.1, disabling the Azimuth Report field in bands when dot11OFDMCCAEDRequired is true, and using Dependent Enablement Identifier instead. This resolution would not change the schedule of either 11k or 11y.

1. Comments

Proposed Resolution:

Accept: 28, 65, 127, 128, 129, 130, 131, 132, 133, 164, 165, 255, 277, 354, 355, 356, 358, 435, 437, 438, 456, 457, 509, 545, 546, 549, 550, 594, 627, 628, 668

Counter: based on discussion and editorial instructions in 07/0245; 6, 14, 15, 16, 102, 145, 238, 259, 260, 263, 413, 636, 663

Decline: based on discussion and editorial instructions in 07/0245; 194, 195, 253, 254, 264, 288, 289, 290, 301, 436, 473, 508, 547, 548, 593, 626, 664, 683, 694

Editorial Instructions:

Tgy Editor: replace text in 7.3.2.36, pg 9-11, D1.0  as follows

EDITORIAL NOTE—REV-ma/D9.0 ends at 35, 11k/D7.0 adds 9 and 11r/D4.1 adds 4.

Insert the following new clauses before 7.4 and renumber accordingly:

#### 7.3.2.3649 DSE registered location element

A DSE registered location element includes a DSE Location Configuration Information report, which contains Latitude, Longitude and Altitude information. The DSE registered location element format is shown in Figure y112z.

Figure y112z—DSE registered location element format

The Length field is set to 18.

The DSE LCI report format is shown in Figure y112Aaa.

Figure y112A—DSE Location Configuration Information report

EDITORIAL NOTE—Figure y112A has been deleted, and this cannot be shown by strikethroughs.

ThisThese structure and information fields are little-endian, per conventions defined in 7.1.1, and are based on the LCI format described in IETF RFC 3825, “Dynamic Host Configuration Protocol Option for Coordinate-based Location Configuration Information”.

The definition of elementfields within the DSE LCI reportregistered location element octets 2-17 shall be as defined in IETF RFC 3825 section 2.1 orexcept as defined herein.

Altitude Type (AT) codes are:

1: Meters - in 2s-complement fixed-point 22-bit integer part with 8-bit fraction

2: Floors - in 2s-complement fixed-point 22-bit integer part with 8-bit fraction

3: Height Above Ground, Mmeters - in 2s-complement fixed-point 22-bit integer part with 8-bit fraction

If AT = 3,an altitude resolution value 0.0 would indicate unknown altitude.  The most precise Altitude would have an altitude resolution value of 30.

Datum is a three-bit field, rather than the eight-bit field defined in RFC 3825, and the codes used are as defined in RFC 3825.

RegLoc Agreement is TRUE1 to report that the STA is operating within a national policy area or an international agreement area near a national border, otherwise it is FALSE0.

RegLoc DSE is TRUE1 to report that the enabling station is enabling the operation of STAs with DSE, otherwise it is FALSE0.

Dependent STA is TRUE1 to report that the STA is operating with the enablement of the enabling station whose LCI is being reported, otherwise it is FALSE0.

Res 2-bits - Reserved. The most-significant two bits of octet 17 are reserved.

Dependent Enablement Identifier is a 16-bit field with a dependent station identifier value set by the enabling station via the DSE registered location element in the (re) association response, or zero.

# Clause 7.3.2.37 Extended Channel Switch Announcement element

CIDs: 4, 17, 18, 51, 96, 134, 135, 136, 137, 138, 155, 172, 210, 211, 251, 359, 389, 390, 439, 448, 458, 459, 476, 477, 510, 511, 551, 552, 595, 596, 669, 670, 679

Discussion:

Similar text for Extended Channel Switch Announcement element is in 11n/D2.0, 11v/D0.08 and 11y/D1.0, with 11n/D2.0 being the newest version written (after 11y/D1.0), and 11y/D1.0 the only version with WG Letter Ballot comments. A strawpoll in TGn in London showed very strong preference in having one version of text that can be used in all amendments, and they voted unanimously to approve the text from 11-06/1775r2 to resolve all comments about ECSA. TGy should both update ECSA from D1.0 to use the dot11ExtendedChannelSwitchImplemented MIB variable, and the 11n/D2.0 changed text in 11.9.7, and then process LB94 comments against the harmonized text.

Proposed Resolution:

Accept: 51, 96, 136, 137, 210, 359, 476, 510, 551, 552, 595

Counter: based on discussion and editorial instructions in 07/0245; 138, 511, 596

Decline: based on discussion and editorial instructions in 07/0245; 4, 17, 18, 134, 135, 155, 172, 211, 251, 389, 439, 448, 458, 459, 477, 669, 670, 679

Deferred: 390

Editorial Instructions:

Tgy Editor: replace text in 7.3.2.37, pg 11, D1.0  as follows

- 7.3.2.3750 Extended Channel Switch Announcement element

The Extended Channel Switch Announcement element is used by an AP in a BSS or a STA in an IBSS to advertise when itthe BSS or IBSS is changing to a new channel or a new regulatory class. The announcement includes both the regulatory class and the channel number of the new channel. The element is only present when an extended channel switch is pending. The format of the Extended Channel Switch Announcement element is shown in Figure y112Baa.

|  | Element ID | Length | Channel Switch Mode | New Regulatory Class | New Channel Number | Channel Switch Count |
| --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 1 | 1 | 1 |

Figure y112Baa. —Extended Channel Switch Announcement information element. format

The Length field shall beis set to 4.

The Channel Switch Mode field indicates any restrictions on transmission until a channel switch. An AP in a BSS or a STA in an IBSS sets the Channel Switch Mode field to either 0 or 1 on transmission. A Channel Switch Mode set to 1 means that theany STA in a BSS to which the frame containing the element is addressed transmits no further frames within the BSS until the scheduled channel switch. A STA in an IBSS may treat a Channel Switch Mode field set to 1 as advisory. A Channel Switch Mode set to 0 does not impose any requirement on theeach receiving STA.

The New Regulatory Class field shall beis set to the number of the regulatory class to which the STA is moving, as defined in Annex J.

The New Channel Number field shall beis set to the number of the channel to which the STA is moving. The channel number shall beis a channel from the STA’s new Regulatory Class as defined in Annex J.

The Channel Switch Count field either shall be set to the number of TBTTs until the STA sending the Channel Switch Announcement element switches to the new channel or shall be set to 0. A value of 1 indicates that the switch shall occur immediately before the next TBTT. A value of 0 indicates that the switch shall occur at any time after the frame containing the element is transmitted.

The Extended Channel Switch Announcement element is included in Extended Channel Switch Announcement frames, as described in 7.4.1.6, and may be included in Beacon frames, as described in 7.2.3.1, and Probe Response frames, as described in 7.2.3.9. The use of Extended Channel Switch Announcement elements and frames is described in 11.9.7.

# Clause 7.3.2.38 Supported Regulatory Classes element

CIDs: 19, 63, 78, 139, 175, 212, 261, 278, 360, 391, 460, 478, 512, 513, 553, 554, 597, 598, 599, 629, 630

Discussion:

Similar text for Supported Regulatory Classes element is in 11n/D2.0, 11v/D0.08 and 11y/D1.0, with 11n/D2.0 being the newest version written (after 11y/D1.0), and 11y/D1.0 the only version with WG Letter Ballot comments. A strawpoll in TGn in London showed very strong preference in having one version of text that can be used in all amendments, and they voted unanimously to approve the text from 11-06/1775r2 to resolve all comments about ECSA and Supported Regulatory Classes. TGy should both update Supported Regulatory Classes from D1.0 to use the dot11ExtendedChannelSwitchImplemented MIB variable, and the 11n/D2.0 changed text in 11.9.7, and then process LB94 comments against the harmonized text.

Proposed Resolution:

Accept: 19, 63, 139, 175, 360, 512, 513, 553, 554, 597, 598, 629, 630

Counter: based on discussion and editorial instructions in 07/0245; 78, 261, 391, 599

Decline: based on discussion and editorial instructions in 07/0245; 212, 278, 460, 478

Editorial Instructions:

Tgy Editor: replace text in 7.3.2.38, pg 11-12, D1.0  as follows

- 7.3.2.3851 Supported Regulatory Classes element

The Supported Regulatory Classes element is used by a STA to advertise the Regulatory Classes that it supports. The format of the Supported Regulatory Classes element is shown in Figure y112Cab.

|  | Element ID | Length | Current Regulatory Class | Alternate Regulatory Class(es) |
| --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | Length-1 |

Figure y112Cab. —Supported Regulatory Classes information element. format

The length of the Supported Regulatory Classes element is between 1 and 32 octets. The Current Regulatory Class octet shall indicates the Regulatory Class in use for transmission and reception. The aAlternate Regulatory Class(es) field lists  numbers shall beare in increasingascending order all other Regulatory Classes that the STA is capable of operating with in this Country.

The Supported Regulatory Classes element may be included in Association Request frames, as described in 7.2.3.4, Association Response frames, as described in 7.2.3.5, in Reassociation Request frames, as described in 7.2.3.6, and Reassociation Response frames, as described in 7.2.3.7.

# Clause 7.4.1 Spectrum management action details

CIDs: 89, 156, 176, 252, 361, 362, 363, 364, 365, 400, 440, 449, 461, 474, 514, 555, 556, 557, 558, 620, 655, 673

Discussion:

Similar text for ECSA element is in 11n/D2.0, 11v/D0.08 and 11y/D1.0, with 11n/D2.0 being the newest version written (after 11y/D1.0), and 11y/D1.0 the only version with WG Letter Ballot comments. A strawpoll in TGn in London showed very strong preference in having one version of text that can be used in all amendments, and they voted unanimously to approve the text from 11-06/1775r2 to resolve all comments about ECSA and Supported Regulatory Classes. TGy should both update ECSA from D1.0 to use the dot11ExtendedChannelSwitchImplemented MIB variable, and the 11n/D2.0 changed text in 7.4.1, and then process LB94 comments against the harmonized text.

Proposed Resolution:

Accept: 176, 363, 364, 365, 555, 556, 558

Counter: based on discussion and editorial instructions in 07/0245; 361, 362, 400, 474, 557

Decline: based on discussion and editorial instructions in 07/0245; 89, 156, 252, 440, 449, 461, 514, 620, 655, 673

Editorial Instructions:

Tgy Editor: replace text in 7.4.1, pg 12, D1.0  as follows

    - Spectrum management action details

TGn Editor: Change clause 7.4.1’s text as shown below:

- 7.4.1 Spectrum management action details

Five Several Action frame formats are defined for spectrum management. An Action Value field, in the octet field immediately after the Category field, differentiates the five formats. The Action Value field values associated with each frame format are defined in Table 44.

EDITORIAL NOTE—Numbers marked as <ANA> will be inserted once assigned by the 802.11 ANA.

Insert the following two rows (ignoring the header row) in Table 44—Spectrum management Action Value field values in the correct position to preserve ordering by the “Action Value field value” column and update the “Reserved” range of codes row appropriately.

Insert a penultimate row and change the last for of 7.4.1.1 – Table 44 as shown:

- Table 44—sSpectrum management aAction vValue field values

| Action Value field value | Description |
| --- | --- |
| 0 | Measurement Request |
| 1 | Measurement Response |
| 2 | TPC Request |
| 3 | TPC Report |
| 4 | Channel Switch Announcement |
| 5<ANA> | Extended Channel Switch Announcement |
| <ANA> | DSE Registered Location Announcement |
| 56-255 | Reserved |

      - Measurement Request frame format

      - Measurement Report frame format

      - TPC Request frame format

      - TPC Report frame format

      - Channel Switch Announcement frame format

Insert the following subclause after 7.4.1.5 and renumber accordingly:

##### 7.4.1.6 Extended Channel Switch Announcement frame format

The Extended Channel Switch Announcement frame uses theis an Action frame. body format andIt is transmitted by an AP in an infrastructure BSS or a STA in an IBSS to advertise a channel switch. The format of the Extended Channel Switch Announcement frame body is shown in Figure y117Aa.

|  | Category | Action | Extended Channel Switch Announcement Element |
| --- | --- | --- | --- |
| Octets: | 1 | 1 | 6 |

Figure y117Aa. —Extended Channel Switch Announcement frame body format.

The Category field shall beis set to 0 (representing spectrum management).

The Action Value field shall beis set to 5 (representingindicate an Extended Channel Switch Announcement frame).

The Extended Channel Switch Announcement element shall beis set as described 7.3.2.37.

##### 7.4.1.7 DSE Registered Location Announcement frame format

The DSE Registered Location Announcement frame is an Action frame. It is transmitted by a dependent STA to advertise the registered location of it’s enabling STA. The format of the DSE Registered Location Announcement frame body is shown in Figure y117b.

|  | Category | Action | DSE registered location Element |
| --- | --- | --- | --- |
| Octets: | 1 | 1 | 20 |

Figure y117b—DSE Registered Location Announcement frame body format

The Category field is set to 0 (representing spectrum management).

The Action Value field is set to indicate a DSE Registered Location Announcement frame.

The DSE Registered Location element is set as described 7.3.2.49.

# Clause 10 Layer management

CIDs: 21, 79, 103, 107, 114, 142, 197, 279, 368, 369, 370, 371, 414, 415, 464, 475, 560, 561, 562, 564, 631, 632, 633

Discussion:

Similar text for Extended Channel Switch Announcement element is in 11n/D2.0, 11v/D0.08 and 11y/D1.0, with 11n/D2.0 being the newest version written (after 11y/D1.0), and 11y/D1.0 the only version with WG Letter Ballot comments. A strawpoll in TGn in London showed very strong preference in having one version of text that can be used in all amendments, and they voted unanimously to approve the text from 11-06/1775r2 to resolve all comments about ECSA. TGy should both update ECSA from D1.0 to use the dot11ExtendedChannelSwitchImplemented MIB variable, and the 11n/D2.0 changed text in 11.9.7, and then process LB94 comments against the harmonized text.

Proposed Resolution:

Accept: 21, 79, 103, 107, 114, 279, 368, 369, 370, 371, 414, 415, 475, 560, 561, 564, 631, 632, 633

Counter: based on discussion and editorial instructions in 07/0245; 562

Decline: based on discussion and editorial instructions in 07/0245; 142, 197

Editorial Instructions:

Tgy Editor: replace text in 10, pg 15-22, D1.0  as follows

      - MLME-SCAN.confirm

        - Semantics of the service primitive

Insert the following row at the end of the BSSDescription table:

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| DSE registered location | As defined in information element | As defined in frame format | The information from the DSE registered location information element, if such a field is present in Probe Response or Beacon, else null. Present only when DSE functionality is required, as specified in 11.14, or when dot11DSERequired is true. |

Change 10.3.6 as follows:

    - Associate

      - MLME-ASSOCIATE.request

        - Semantics of the service primitive

Change the primitive parameter list as shown:

MLME-ASSOCIATE.request	(
PeerSTAAddress,
AssociateFailureTimeout,
CapabilityInformation,
ListenInterval,
Supported Channels,
RSN,
QoSCapability,

RCPI,

RSNI,
SupportedRegulatoryClasses, 
VendorSpecificInfo
)

Insert the following row at the end of the parameter table, before VendorSpecificInfo:

| Name | Type | Valid Rrange | Description |
| --- | --- | --- | --- |
| SupportedRegulatoryClasses | As defined in the Supported Regulatory Classes element | As defined in the Supported Regulatory Classes element | Specifies the Supported Regulatory Classes capabilities of the non-AP STA. This parameter shall only be present if the MIB attribute dot11OFDMCCAEDRequireddot11ExtendedChannelSwitchImplemented is true. |

      - MLME-ASSOCIATE.confirm

        - Semantics of the service primitive

Change the primitive parameter list as shown:

MLME-ASSOCIATE.confirm	(
ResultCode,
CapabilityInformation,
AssociationID,
EDCAParameterSet, 
SupportedRegulatoryClasses, 
VendorSpecificInfo
)

Insert the following row at the end of the parameter table, before VendorSpecificInfo:

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| SupportedRegulatoryClasses | As defined in the Supported Regulatory Classes element | As defined in the Supported Regulatory Classes element | Specifies the Supported Regulatory Classes capabilities of the non-AP STA. This parameter shall only be present if the MIB attribute dot11ExtendedChannelSwitchImplemented is true. |

      - MLME-ASSOCIATE.indication

        - Semantics of the service primitive

Change the primitive parameter list as shown:

MLME-ASSOCIATE.indication	(
PeerSTAAddress,
CapabilityInformation,
ListenInterval,

SSID,
Supported Rates,
RSN,
QoSCapability,

RCPI,

RSNI,
SupportedRegulatoryClasses,

DSERregisteredLlocation, 
VendorSpecificInfo
)

Insert the following rows at the end of the parameter table, before VendorSpecificInfo:

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| SupportedRegulatoryClasses | As defined in the Supported Regulatory Classes element | As defined in the Supported Regulatory Classes element | Indicates the Supported Regulatory Classes capabilities of the AP. This parameter shall only be present if the MIB attribute dot11ExtendedChannelSwitchImplemented is true. |
| DSERregisteredLlocation | As defined in the DSE registered location element | As defined in the DSE registered location element | Indicates the DSE registered location including the Dependent Enablement Identifier assigned by the enabling station. This parameter shall only be present if dot11DSERequired is true or dot11RegLocRequired is true. |

      - MLME-ASSOCIATE.response

        - Semantics of the service primitive

Change the primitive parameter list as shown:

MLME-ASSOCIATE.response	(
PeerSTAAddress,
ResultCode,
CapabilityInformation,
AssociationID,
EDCAParameterSet,

RCPI,

RSNI,
SupportedRegulatoryClasses, 
DSERregisteredLlocation, 
VendorSpecificInfo
)

Insert the following rows at the end of the parameter table, before VendorSpecificInfo:

| Name | Type | Valid Rrange | Description |
| --- | --- | --- | --- |
| SupportedRegulatoryClasses | As defined in the Supported Regulatory Classes element | As defined in the Supported Regulatory Classes element | Indicates the Supported Regulatory Classes capabilities of the AP. This parameter shall only be present if the MIB attribute dot11OFDMCCAEDRequireddot11ExtendedChannelSwitchImplemented is true. |
| DSERregisteredLlocation | As defined in the DSE registered location element | As defined in the DSE registered location element | Indicates the DSE registered location including the Dependent Enablement Identifier assigned by the enabling station. This parameter shall only be present if dot11DSERequired is true or dot11RegLocRequired is true. |

Change 10.3.7 as follows:

    - Reassociate

      - MLME-REASSOCIATE.request

        - Semantics of the service primitive

Change the primitive parameter list as shown:

MLME-REASSOCIATE.request	(
NewAPAddress,
ReassociateFailureTimeout,
CapabilityInformation,
ListenInterval,
Supported Channels,
RSN,
QoSCapability,

Content of FT Authentication Information elements,
SupportedRegulatoryClasses, 
VendorSpecificInfo
)

Insert the following row at the end of the parameter table, before VendorSpecificInfo:

| Name | Type | Valid Rrange | Description |
| --- | --- | --- | --- |
| SupportedRegulatoryClasses | As defined in the Supported Regulatory Classes element | As defined in the Supported Regulatory Classes element | Specifies the Supported Regulatory Classes of the non-AP STA. This parameter shall only be present if the MIB attribute dot11OFDMCCAEDRequireddot11ExtendedChannelSwitchImplemented is true. |

      - MLME-REASSOCIATE.confirm

        - Semantics of the service primitive

Change the primitive parameter list as shown:

MLME-REASSOCIATE.confirm	(
ResultCode,
CapabilityInformation,
AssociationID,
SupportedRates,
EDCAParameterSet,

Content of FT Authentication Information elements,
SupportedRegulatoryClasses, 
VendorSpecificInfo
)

Insert the following row at the end of the parameter table, before VendorSpecificInfo:

| Name | Type | Valid Rrange | Description |
| --- | --- | --- | --- |
| SupportedRegulatoryClasses | As defined in the Supported Regulatory Classes element | As defined in the Supported Regulatory Classes element | Specifies the Supported Regulatory Classes of the non-AP STA. This parameter shall only be present if the MIB attribute dot11ExtendedChannelSwitchImplemented is true. |

      - MLME-REASSOCIATE.indication

        - Semantics of the service primitive

Change the primitive parameter list as shown:

MLME-REASSOCIATE.indication	(
PeerSTAAddress,
CurrentAPAdress,
CapabilityInformation,
ListenInterval,
SSID,

SupportedRates

RSN,

QoSCapability,

RCPI,

RSNI,

Content of FT Authentication Information elements,
SupportedRegulatoryClasses, 
DSERregisteredLlocation, 
VendorSpecificInfo
)

Insert the following rows at the end of the parameter table, before VendorSpecificInfo:

| Name | Type | Valid Rrange | Description |
| --- | --- | --- | --- |
| SupportedRegulatoryClasses | As defined in the Supported Regulatory Classes element | As defined in the Supported Regulatory Classes element | Specifies the Supported Regulatory Classes of the non-AP STA. This parameter shall only be present if the MIB attribute dot11ExtendedChannelSwitchImplemented is true. |
| DSERregisteredLlocation | As defined in the DSE registered location element | As defined in the DSE registered location element | Indicates the DSE registered location including the Dependent Enablement Identifier assigned by the enabling station. This parameter shall only be present if dot11DSERequired is true or dot11RegLocRequired is true. |

      - MLME-REASSOCIATE.response

        - Semantics of the service primitive

Change the primitive parameter list as shown:

MLME-REASSOCIATE.response	(
PeerSTAAddress,
ResultCode,
CapabilityInformation,
AssociationID,
EDCAParameterSet,

RCPI,

RSNI,

Content of FT Authentication Information elements,
SupportedRegulatoryClasses, 
DSERregisteredLlocation, 
VendorSpecificInfo
)

Insert the following rows at the end of the parameter table, before VendorSpecificInfo:

| Name | Type | Valid Rrange | Description |
| --- | --- | --- | --- |
| SupportedRegulatoryClasses | As defined in the Supported Regulatory Classes element | As defined in the Supported Regulatory Classes element | Specifies the Supported Regulatory Classes of the non-AP STA. This parameter shall only be present if the MIB attribute dot11OFDMCCAEDRequireddot11ExtendedChannelSwitchImplemented is true. |
| DSERregisteredLlocation | As defined in the DSE registered location element | As defined in the DSE registered location element | Indicates the DSE registered location including the Dependent Enablement Identifier assigned by the enabling station. This parameter shall only be present if dot11DSERequired is true or dot11RegLocRequired is true. |

    - Start

- 10.3.10.1 MLME-START.request

#### 10.3.10.1.2 Semantics of the service primitive

Change the primitive parameters as shown:

MLME-START.request	(

SSID,

BSSType,

BeaconPeriod,

DTIMPeriod,

CF parameter set,

PHY parameter set,

IBSS parameter set,

ProbeDelay.

CapabilityInformation,

BSSBasicRateSet,

OperationalRateSet,

Country,

IBSS DFS Recovery Interval,

EDCAParameterSet,

DSE registered location,

VendorSpecificInfo

)

Insert the following entry to the end of the table in the subclause, before VendorSpecificInfo:

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| DSE registered location | As defined in information element7.3.2.49 | As defined in frame format7.3.2.49 | The information from the DSE registered location information element. Present only if dot11RegLocRequired is true or dot11DSERequired is true. |

Insert the following subclauses after 10.3.2935.3.4  as follows:

- 10.3.36 Extended Channel Switch Announcement

The following MLME primitives support the signaling of Extended Channel Switch Announcement.

10.3.3036.1 MLME-EXTCHANNELSWITCH.request

10.3.3036.1.1 Function

This primitive requests that an Extended Channel Switch Announcement frame be sent by an AP.

10.3.3036.1.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-EXTCHANNELSWITCH.request	(

Mode,

Regulatory Class,

Channel Number,

Channel Switch Count,

VendorSpecificInfo

)

| Name | Type | Valid Rrange | Description |
| --- | --- | --- | --- |
| Mode | Integer | 0,1 | Channel switch mode, as defined for the Extended Channel Switch Announcement element. |
| Regulatory Class | Integer | As defined in Annex J | Specifies the new regulatory class. |
| Channel Number | Integer | As defined in Annex J | Specifies the new channel number. |
| Channel Switch Count | Integer | 0 – 2550-255 | Specifies the number of TBTTs until the channel switch event, as described for the Extended Channel Switch Announcement element. |
| VendorSpecificInfo | A set of information elements | As defined in 7.3.2.26 | Zero or more information elements. |

10.3.3036.1.3 When generated

This primitive is generated by the SME to request that an Extended Channel Switch Announcement frame be sent to a non-AP STA that is associated to the AP.

10.3.3036.1.4 Effect of receipt

On receipt of this primitive, the MLME constructs an Extended Channel Switch Announcement frame. The AP then attempts to transmit this to other STAs that are associated.

10.3.3036.2 MLME-EXTCHANNELSWITCH.confirm

10.3.3036.2.1 Function

This primitive reports the result of a request to switch channel.

10.3.3036.2.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-EXTCHANNELSWITCH.confirm	(

ResultCode,

VendorSpecificInfo

)

| Name | Type | Valid Rrange | Description |
| --- | --- | --- | --- |
| ResultCode | Enumeration | SUCCESS, INVALID PARAMETERS, STA_IS_NOT_AN_AP or UNSPECIFIED FAILURE | Reports the result of an extended channel switch request. |
| VendorSpecificInfo | A set of information elements | As defined in 7.3.2.26 | Zero or more information elements. |

10.3.3036.2.3 When generated

This primitive is generated by the MLME when an extended channel switch request completes. Possible unspecified failure causes include an inability to schedule an extended channel announcement.

10.3.3036.2.4 Effect of receipt

The SME is notified of the results of the extended channel switch procedure.

10.3.3036.3 MLME-EXTCHANNELSWITCH.indication

10.3.3036.3.1 Function

This primitive indicates that an Extended Channel Switch Announcement frame was received from an AP.

10.3.3036.3.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-EXTCHANNELSWITCH.indication	(

Peer MAC Address,

Mode,

Regulatory Class,

Channel Number,

Channel Switch Count,

VendorSpecificInfo

)

| Name | Type | Valid Rrange | Description |
| --- | --- | --- | --- |
| PeerMAC Address | MACAddress | Any valid individual MAC Address | The address of the peer MAC entity from which the Measurement Report frame was received. |
| Mode | Integer | 0,1 | Channel switch mode, as defined for the Channel Switch Announcement element. |
| Regulatory Class | Integer | As defined in Annex J | Specifies the new regulatory class. |
| Channel Number | Integer | As defined in Annex J | Specifies the new channel number. |
| Channel Switch Count | Integer | 0 – 2550-255 | Specifies the number of TBTTs until the channel switch event, as described for the Extended Channel Switch Announcement element. |
| VendorSpecificInfo | A set of information elements | As defined in 7.3.2.26 | Zero or more information elements. |

10.3.3036.3.3 When generated

This primitive is generated by the MLME when a valid Extended Channel Switch Announcement frame is received.

10.3.3036.3.4 Effect of receipt

On receipt of this primitive, the SME decides whether to accept the switch request.

10.3.3036.4 MLME-EXTCHANNELSWITCH.response

10.3.3036.4.1 Function

This primitive is used to schedule an accepted extended channel switch.

10.3.3036.4.2 Semantics of the service primitive

The primitive parameters are as follows:

MLME-EXTCHANNELSWITCH.response	(

Mode,

Regulatory Class,

Channel Number,

Channel Switch Count,

VendorSpecificInfo

)

| Name | Type | Valid Rrange | Description |
| --- | --- | --- | --- |
| Mode | Integer | 0,1 | Channel switch mode, as defined for the Channel Switch Announcement element. |
| Regulatory Class | Integer | As defined in Annex J | Specifies the new regulatory class. |
| Channel Number | Integer | As defined in Annex J | Specifies the new channel number. |
| Channel Switch Count | Integer | 0 – 2550-255 | Specifies the number of TBTTs until the channel switch event, as described for the Extended Channel Switch Announcement element. |
| VendorSpecificInfo | A set of information elements | As defined in 7.3.2.26 | Zero or more information elements. |

10.3.3036.4.3 When generated

This primitive is generated by the SME to schedule an accepted extended channel switch request.

10.3.3036.4.4 Effect of receipt

On receipt of this primitive, the MLME schedules the extended channel switch.  The actual channel switch is at the appropriate time through the MLME-PLME interface using the PLME-SET primitive of the dot11CurrentFrequency MIB attribute. If Mode = 0, STA shall refrain from transmitting until the time of the extended channel switch.

# Clause 11.9.7 CSA procedures

CIDs: 2, 3, 7, 30, 66, 67, 68, 90, 104, 115, 116, 120, 121, 143, 144, 153, 154, 159, 160, 161, 180, 196, 198, 199, 200, 213, 247, 248, 249, 250, 262, 280, 291, 292, 372, 441, 442, 446, 447, 465, 466, 479, 565, 566, 634, 665, 671, 672, 676, 681

Discussion:

Similar text for Extended Channel Switch Announcement element is in 11n/D2.0, 11v/D0.08 and 11y/D1.0, with 11n/D2.0 being the newest version written (after 11y/D1.0), and 11y/D1.0 the only version with WG Letter Ballot comments. A strawpoll in TGn in London showed very strong preference in having one version of text that can be used in all amendments, and they voted unanimously to approve the text from 11-06/1775r2 to resolve all comments about ECSA. TGy should both update ECSA from D1.0 to use the dot11ExtendedChannelSwitchImplemented MIB variable, and the 11n/D2.0 changed text in 11.9.7, and then process LB94 comments against the harmonized text.

Proposed Resolution:

Accept: 67, 90, 120, 121, 143, 159, 160, 161, 180, 198, 199, 372, 465, 676

Counter: based on discussion and editorial instructions in 07/0245; 7, 66, 68, 144, 200, 213, 262, 280, 479, 565, 681

Decline: based on discussion and editorial instructions in 07/0245; 2, 3, 30, 104, 115, 116, 153, 154, 196, 247, 248, 249, 250, 291, 292, 441, 442, 446, 447, 466, 566, 634, 665, 671, 672

Editorial Instructions:

Tgy Editor: replace text in 11.9.7, pg 24-25, D1.0  as follows

Change clause 11.9.7.1 as follows:

      - Selecting and advertising a new channel in an infrastructure BSS

The decision to switch to a new operating channel in an infrastructure BSS shall be made only by the AP.

Change second sentence of clause 11.9.7.1 as follows:

An AP may make use of the information in the Supported Channel, and if present, Supported Regulatory Classes elements, and the results of measurements undertaken by the AP and other STAs in the BSS to assist the selection of the new channel. The algorithm to choose a new channel is beyond the scope of this standard, but shall satisfy applicable regulatory requirements, including uniform spreading rules and channel testing rules. The AP shall attempt to select a new channel that is supported by all associated STAs, although it should be noted that this might not always be possible.

Insert new second paragraph and change third paragraph of clause 11.9.7.1 as follows:

In the following text wherever Channel Switch Announcement is referred to, the following rules apply:

- If an AP is switching to a new channel in a different regulatory class, then the AP shall use either the Extended Channel Switch Announcement elements and frames instead of the Channel Switch Announcement elements and frames, or both the Extended Channel Switch Announcement and the Channel Switch Announcement elements and frames.

- If an AP is switching to a channel within the same regulatory class, and dot11OFDMCCAEDRequireddot11ExtendedChannelSwitchImplemented is true, then the AP shall send the Extended Channel Switch Announcement elements and frames, else if dot11ExtendedChannelSwitchImplemented is false, the AP shall send the Channel Switch Announcement elements and frames. Optionally if dot11ExtendedChannelSwitchImplemented is false, the Extended Channel Switch Announcement elements and frames may also be usedsent.

An AP shall inform associated STAs that the AP is moving to a new channel and maintain the association by advertising the switch using Channel Switch Announcement elements in Beacon frames, Probe Response frames, either of theand Channel Switch Announcement frames until the intended channel switch time. The AP may force STAs in the BSS to stop transmissions until the channel switch takes place usingby setting the Channel Switch Mode field in the Channel Switch Announcement element to 1. If possible, the channel switch should be scheduled so that all STAs in the BSS, including STAs in power save mode, have the opportunity to receive at least one Channel Switch Announcement element before the switch. The AP may send theany Channel Switch Announcement frame in a BSS without performing a backoff, after determining that the WM is idle for one PIFS period.

A STA that receives a Channel Switch Announcement element may choose not to perform the specified switch, but to take alternative action. For example, it may choose to move to a different BSS.

A STA in a BSS that is not the AP shall not transmit the Channel Switch Announcement element.

Insert new first paragraph in clause 11.9.7.2 as follows:

- 11.9.7.2 Selecting and advertising a new channel in an IBSS

In the following text wherever Channel Switch Announcement is referred to, the following rules apply:

- If a DFS owner is switching to a new channel in a different regulatory class, then the DFS owner shall use the Extended Channel Switch Announcement elements and frames instead of the Channel Switch Announcement elements and frames or both the Extended Channel Switch Announcement and the Channel Switch Announcement elements and frames.

- If a DFS owner is switching to a channel within the same regulatory class, and dot11OFDMCCAEDRequireddot11ExtendedChannelSwitchImplemented is true, then the DFS owner shall send the Extended Channel Switch Announcement elements and frames, else if dot11ExtendedChannelSwitchImplemented is false, the DFS owner shall send the Channel Switch Announcement elements and frames. Optionally if dot11ExtendedChannelSwitchImplemented is false, the Extended Channel Switch Announcement elements and frames may also be usedsent.

# Clause 11.10 DSE procedures

CIDs: 22, 87, 105, 111, 122, 146, 181, 182, 224, 302, 373, 375, 376, 467, 518, 519, 567, 568, 602

Discussion:

In 11y/D1.0:

Proposed Resolution:

Accept: 22, 87, 181, 182, 302, 373, 375, 376, 519, 567, 568

Counter: based on discussion and editorial instructions in 07/0245; 105, 111, 122, 146, 518

Decline: based on discussion and editorial instructions in 07/0245; 224, 467, 602

Editorial Instructions:

Tgy Editor: replace text in 11.10, pg 25,  D1.0  as follows

EDITORIAL NOTE—11 ma D9.0 last subclause is 11.9.7.2, 11k/D7.0 adds 4.

Insert the following new subclauses after clause 11.9.137.2 and renumber accordingly:

  - DSE procedures

Regulations that apply to the US 3650 MHz band require enabling stations to implement a mechanism to enable mobile station operation. Similar regulations exist in other regulatory domains. This standard describes such a mechanism, referred to as dependent station enablement (DSE).

This subclause describes DSE procedures that can be used to satisfy these and similar future regulatory requirements. Regulations that apply to the US 3650 MHz band require fixed stations and enabling stations to have their operating locations registered. Licensees ofwith stations suffering or causing harmful interference are expected to cooperate and resolve problems by mutually satisfactory arrangements. The DSE procedures may also satisfy needs in other frequency bands and be useful for other purposes.

STAs shall use the DSE procedures defined in this subclause if dot11DSERequired is true.  dot11DSERequired shall be TRUEtrue when regulatory authorities require DSE. It may also be set to TRUEtrue in other circumstances. If licensees in more than one frequency band of a country use DSE procedures, then aeach dot11RegLocRequired, and aeach dot11DSERequired, and each dot11ExtendedChannelSwitchImplemented shall be specific to eacha frequency band using the DSE procedures of that country. For each frequency band using the DSE procedures, the corresponding dot11ExtendedChannelSwitchImplemented shall be true.

The DSE procedures provide for the following:

--—Fixed station and enabling station operation

--—Creation of a Basic Service Area for dependent station operation

--—Dependent STA operation with DSE

For the purpose of DSE, a STA with dot11DSERequired set to FALSEfalse shall not supportrespond to the DSE registered location orderrequest unless dot11RegLocRequired is set to TRUEtrue.

# Clause 11.10.1 DSE Fixed station and enabling station operation

CIDs: 147, 166, 167, 201, 214, 406, 480, 520, 603

Discussion:

In 11y/D1.0:

Proposed Resolution:

Accept: 166, 201, 214, 480

Counter: based on discussion and editorial instructions in 07/0245; 167, 520, 603

Decline: based on discussion and editorial instructions in 07/0245; 147, 406

Editorial Instructions:

Tgy Editor: replace text in 11.10.1, pg 25,  D1.0  as follows

    - Fixed station and enabling station operation

Fixed stations and enabling stations shall have dot11RegLocRequired set to TRUEtrue, and shall transmit the DSE registered location element in every beacon, and shall set the Dependent STA bit in the DSE registered location element to FALSE0. If the registered location of the fixed or enabling station is within a national policy area, such as within a Fixed Satellite Service exclusion zone, or the registered location is within an international agreement area near a national border, the RegLoc Agreement bit in the DSE registered location element shall be set to TRUE1, otherwise it shall be set to FALSE0.

The DSE registered location element Latitude, Longitude and Altitude fields shall be reported toat their fullbest known resolutions, which may exceed the resolutions required by regulatory authorities. The Altitude Type shall be 3, Height Above Ground Meters (altitude in meters above adjacent terrain), unless another altitude type is required for operation in the regulatory domain. The Datum value shall be 1 (World Geodetic System 1984), unless another datum is required for operation in the regulatory domain.

# Clause 11.10.2 Creation of a BSA for dependent station operation

CIDs: 293, 377, 407, 408, 409, 468, 521, 604, 605

Discussion:

In 11y/D1.0:

Proposed Resolution:

Accept: 293, 377, 408, 409

Counter: based on discussion and editorial instructions in 07/0245; 407, 468, 521, 604

Decline: based on discussion and editorial instructions in 07/0245; 605

Editorial Instructions:

Tgy Editor: replace text in 11.10.2, pg 26,D1.0  as follows

    - Creation of a BDSE-SA for dependent station operation

An enabling station may allow the creation of a BSADSE-SA for dependent STA operation where regulatory requirements permit, and signifies the creation of a BSADSE-SA by setting the RegLoc DSE bit in the DSE registered location element to TRUE1 in the DSE registered location element. Dependent STA operation is conditional on receiving and decoding a DSE registered location element with RegLoc DSE bit set to true1 directly from an enabling station. Before association to any one enabling station, a dependent STA may have detected several enabling stations, and may attempt association with any one and fail, then attempt association to another and another. An enabling station in an DSE-SA BSA may schedule quiet intervals using DFS procedures, so that all dependent stations can listen for others in the frequency band.

# Clause 11.10.3 Dependent STA operation with DSE

CIDs: 8, 9, 10, 23, 24, 31, 52, 55, 80, 91, 117, 118, 148, 149, 150, 157, 168, 183, 184, 202, 203, 208, 215, 216, 217, 218, 219, 256, 265, 281, 282, 283, 392, 393, 394, 395, 402, 403, 410, 411, 412, 416, 417, 418, 419, 469, 470, 471, 481, 482, 483, 484, 485, 522, 523, 524, 525, 526, 569, 570, 572, 606, 607, 609, 610, 611, 635, 636, 637, 638, 639, 641, 684, 696

Discussion:

In 11y/D1.0:

Proposed Resolution:

Accept: 23, 31, 91, 117, 183, 215, 217, 256, 281, 392, 393, 410, 412, 416, 417, 418, 470, 471, 483, 522, 523, 524, 525, 569, 570, 572, 607, 609, 611, 637, 638, 639, 641

Counter: based on discussion and editorial instructions in 07/0245; 8, 9, 10, 52, 80, 118, 148, 149, 150, 157, 168, 184, 202, 203, 208, 216, 218, 219, 265, 282, 283, 402, 403, 481, 482, 484, 526, 606, 635, 636, 684

Decline: based on discussion and editorial instructions in 07/0245; 24, 55, 394, 395, 411, 419, 469, 485, 610, 696

Editorial Instructions:

Tgy Editor: replace text in 11.10.2, pg 26,D1.0  as follows

    - Dependent STA operation with DSE

Dependent STAs shall have dot11RegLocRequired and dot11DSERequired set to TRUEtrue.

For the purpose of DSE, the following statements apply:

—A STA with dot11DSERequired set to TRUE shall not operate in an infrastructure BSS or IBSS unless it has received a bBeacon frame or pProbe rResponse frame from an enabling station with the Spectrum Management bit set to 1 in the Capability Information field, and with the DSE registered location element RegLoc DSE bit set to 1;

—EveryA dependent STA that is not associated with an enabling station shall not transmit, except to authenticate and associate with an enabling station, unless such action is as mandated in the regulatory domain (e.g. Emergency Services);

—EveryA dependent STA shall associateonly attempt association with an enabling stationAP if this AP that is sending beaconstransmitting Beacon frames with DSE registered location element RegLoc DSE bit set to TRUE1;

—An associated dependent STA shall operate under the control of the enabling stationAP to which the dependent stationit is associated;

—EveryA dependent STA shall create a dependent DSE registered location element with the enabling station’s DSE registered location element and having the RegLoc DSE bit set to 0 and the Dependent STA bit set to TRUE1. Before association, the Dependent Enablement Identifier (DEI) field shall be set to zero. Upon association or Reassociation, the DEI field shall be set to the DEI value received by the dependent STA in the MLME-ASSOCIATE.response or MLME-REASSOCIATE.response, and this DSE registered location element shall be used in constructing Probe Response frames;

—EveryA dependent STA that is not associated with an enabling station shall only transmit for up to [8] dot11DSEAssociateTimeLimit seconds, measured from the time of first transmission, inwhile attempting to associate,

then if it is not associated with an enabling station, it shall not transmit for [512] dot11DSEAssociateFailHoldTime seconds,

before it can again attempt to authenticate and associate;

—EveryAn associated dependent STA shall cease transmission within [60]dot11DSERenewalTime seconds if it doeshas not received either a bBeacon frame or a Probe Response frame with the RegLoc DSE bit set to TRUE1 in the DSE registered location element, and set to zero all fields in octets 2-19 of the dependentit’s DSE registered location element to zero;

—Every dependent STA shall respond to a Request information Probe Request having the DSE registered location element (asking “what enabling station’s signal did you receive and decode that enables you to transmit?”) with a Probe Response DSE the dependent DSE registered location element;

—EveryA dependent STA shall create a Probe ResponseSpectrum management Action frame containing order 23 (DSE), with the dependent DSE Registered Location Announcement element. The dependent STA shall count the sum of dot11TransmittedFragmentCount, dot11MulticastTransmittedFrameCount and dot11ReceivedFragmentCount, and schedule this Probe ResponseAction frame to be sent to selfthe broadcast address using normal frame transmission rules, whenever the sum modulo [256] dot11DSETransmitDivisor changes has a remainder of zero.