IEEE P802.11
Wireless LANs

| TGu MLME Correction Suggestions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-02-20 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Eleanor Hepworth | Siemens Roke Manor | Roke Manor, Romsey, UK, SO51 0ZN | +44 1794 833146 | eleanor.hepworth@roke.co.uk |
| Angelo Centonza | Siemens Roke Manor | Roke Manor, Romsey, UK, SO51 0ZN | +44 1794 833000 | angelo.centonza@roke.co.uk |

Abstract

This document describes proposed changes to the TGu draft to address comments 138, 140, 142 and 143 (11-06-1857r6).

This document is based on P802.11u-d0.03 and P802.11ma-d9.

In section 3.2.2.2, it is proposed to remove “No Available AP” as it is not clear what this adds beyond what is currently provided in the standard (i.e. why is it needed?).

MLME–ASSOCIATE and MLME-REASSOCIATE are basically unchanged from the version in P802.11u-d0.03, but are invluded here because they have not been formally voted into the draft yet.

Comment 143 seems to point to a section number that is inconsistent with P802.11ma-d9 – have renumbered 10.3.25.1 MLME-ADDTS.request to 10.3.24.1.

Would suggest changing “REJECTED_HOME_WITH_SUGGESTED_CHANGES” to “REJECTED_REMOTE_WITH_SUGGESTED_CHANGES”, although this hasn’t been changed in the proposed text below.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@ok-brit.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

- Layer management

  - MLME SAP Interface

    - Scan

      - MLME-SCAN.request

      - MLME-SCAN.confirm

        - Function

        - Semantics of the service primitive

Change the second row as shown below:

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Result Code | Enumeration | SUCCESS, INVALID_ PARAMETERS, NOT_SUPPORTED, No Available AP | Indicates the result of the MLME-SCAN.confirm |

Add the following to the BSSDescription elements:

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| InterworkingCapability | As defined in Interworking Capability element | As defined in frame format | The values from the Interworking Capability information element if such an element was present in the probe response or beacon. |
| GASCapability | As defined in GAS Capability element | As defined in frame format | The values from the GAS Capability information element if such an element was present in the probe response or beacon. |
| GASTIM | As defined in GAS TIM element | As defined in frame format | The values from the GAS TIM information element if such an element was present in the probe response or beacon. |
| ESSID | As defined in ESSID element | As defined in frame format | The values from the ESSID information element if such an element was present in the probe response or beacon. |
| DefaultESRealm | As defined in Default ES Realm element | As defined in frame format | The values from the Default ES Realm information element if such an element was present in the probe response or beacon. |

    - Associate

      - MLME-ASSOCIATE.request

        - Semantics of the service primitive

Change the primitive parameter list as shown:

MLME-ASSOCIATE.request (

PeerSTAAddress,

AssociateFailureTimeout,

Capability Information,

ListenInterval,

Supported Channels,

RSN,

QoSCapability,

InterworkingCapability,

VendorSpecificInfo

)

Insert the following row in the parameters table:

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| InterworkingCapability | As defined in Interworking Capability element | As defined in frame format | Specifies the Interworking capabilities of the MAC entity. This parameter shall only be present if the MIB attribute dot11InterworkingEnabled is true. |

      - MLME-ASSOCIATE.confirm

        - Semantics of the service primitive

Change the primitive parameter list as shown:

MLME-ASSOCIATE.confirm (

PeerSTAAddress,

AssociateFailureTimeout,

Capability Information,

ListenInterval,

Supported Channels,

RSN,

QoSCapability,

InterworkingCapability,

VendorSpecificInfo

)

Insert the following row in the parameters table:

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| InterworkingCapability | As defined in Interworking Capability element | As defined in frame format | Specifies the Interworking capabilities of the MAC entity. This parameter shall only be present if the MIB attribute dot11InterworkingEnabled is true. |

      - MLME-ASSOCIATE.indication

        - Semantics of the service primitive

Change the primitive parameter list as shown:

MLME-ASSOCIATE.indication (

PeerSTAAddress,

AssociateFailureTimeout,

Capability Information,

ListenInterval,

Supported Channels,

RSN,

QoSCapability,

InterworkingCapability,

VendorSpecificInfo

)

Insert the following row in the parameters table:

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| InterworkingCapability | As defined in Interworking Capability element | As defined in frame format | Specifies the Interworking capabilities of the MAC entity. This parameter shall only be present if the MIB attribute dot11InterworkingEnabled is true. |

      - MLME-ASSOCIATE.response

        - Semantics of the service primitive

Change the primitive parameter list as shown:

MLME-ASSOCIATE.indication (

PeerSTAAddress,

AssociateFailureTimeout,

Capability Information,

ListenInterval,

Supported Channels,

RSN,

QoSCapability,

InterworkingCapability,

VendorSpecificInfo

)

Insert the following row in the parameters table:

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| InterworkingCapability | As defined in Interworking Capability element | As defined in frame format | Specifies the Interworking capabilities of the MAC entity. This parameter shall only be present if the MIB attribute dot11InterworkingEnabled is true. |

    - Reassociate

      - MLME-REASSOCIATE.request

        - Semantics of the service primitive

Change the primitive parameter list as shown:

MLME-REASSOCIATE.request (

PeerSTAAddress,

AssociateFailureTimeout,

Capability Information,

ListenInterval,

Supported Channels,

RSN,

QoSCapability,

InterworkingCapability,

VendorSpecificInfo

)

Insert the following row in the parameters table:

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| InterworkingCapability | As defined in Interworking Capability element | As defined in frame format | Specifies the Interworking capabilities of the MAC entity. This parameter shall only be present if the MIB attribute dot11InterworkingEnabled is true. |

      - MLME-REASSOCIATE.confirm

        - Semantics of the service primitive

Change the primitive parameter list as shown:

MLME-REASSOCIATE.confirm (

PeerSTAAddress,

AssociateFailureTimeout,

Capability Information,

ListenInterval,

Supported Channels,

RSN,

QoSCapability,

InterworkingCapability,

VendorSpecificInfo

)

Insert the following row in the parameters table:

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| InterworkingCapability | As defined in Interworking Capability element | As defined in frame format | Specifies the Interworking capabilities of the MAC entity. This parameter shall only be present if the MIB attribute dot11InterworkingEnabled is true. |

      - MLME-REASSOCIATE.indication

        - Semantics of the service primitive

Change the primitive parameter list as shown:

MLME-REASSOCIATE.indication (

PeerSTAAddress,

AssociateFailureTimeout,

Capability Information,

ListenInterval,

Supported Channels,

RSN,

QoSCapability,

InterworkingCapability,

VendorSpecificInfo

)

Insert the following row in the parameters table:

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| InterworkingCapability | As defined in Interworking Capability element | As defined in frame format | Specifies the Interworking capabilities of the MAC entity. This parameter shall only be present if the MIB attribute dot11InterworkingEnabled is true. |

      - MLME-REASSOCIATE.response

        - Semantics of the service primitive

Change the primitive parameter list as shown:

MLME-REASSOCIATE.indication (

PeerSTAAddress,

AssociateFailureTimeout,

Capability Information,

ListenInterval,

Supported Channels,

RSN,

QoSCapability,

InterworkingCapability,

VendorSpecificInfo

)

Insert the following row in the parameters table:

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| InterworkingCapability | As defined in Interworking Capability element | As defined in frame format | Specifies the Interworking capabilities of the MAC entity. This parameter shall only be present if the MIB attribute dot11InterworkingEnabled is true. |

    - TS management interface

      - MLME-ADDTS.confirm

        - Semantics of the service primitive

Add the following changes to the ResultCode row in the table.

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| ResultCode | Enumeration | SUCCESS, INVALID_ PARAMETERS, REJECTED_ WITH_ SUGGESTED_ CHANGES, REJECTED_ LOCAL_ WITH_ SUGGESTED_ CHANGES, REJECTED_ HOME_ WITH_ SUGGESTED_ CHANGES, REJECTED_ FOR_DELAY_ PERIOD, TIMEOUT, TRANSMISSION_ FAILURE | Indicates the results of the corresponding MLMEADDTS.request primitive. |

Make the following changes to the text under the table.

For other values of ResultCode, no new TS has been created. In the case of REJECTED_LOCAL_WITH_SUGGESTED_CHANGES, the TSPEC represents an alternative proposal by the HC based on information about the current status of the MAC entity. In the case of REJECTED_HOME_WITH_SUGGESTED_CHANGES, the TSPEC represents an alternative proposal by the HC based on information received across the SSPN interface.  A TS is not created with this definition. If the suggested changes are acceptable to the non-AP STA, it is the responsibility of the non-AP STA to set up the TS with the suggested changes.

      - MLME-ADDTS.response

        - Semantics of the service primitive

Add the following changes to the ResultCode row in the table.

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| ResultCode | Enumeration | SUCCESS, INVALID_ PARAMETERS, REJECTED_ WITH_ SUGGESTED_ CHANGES, REJECTED_ LOCAL_ WITH_ SUGGESTED_ CHANGES, REJECTED_ HOME_ WITH_ SUGGESTED_ CHANGES, REJECTED_ FOR_DELAY_ PERIOD, TIMEOUT, TRANSMISSION_ FAILURE | Indicates the results of the corresponding MLMEADDTS.request primitive. |

Make the following changes to the text under the table.

If the result code is REJECTED_LOCAL_WITH_SUGGESTED_CHANGES or REJECTED_HOME_WITH_SUGGESTED_CHANGES, the TSPEC and TCLAS parameters represent an alternative proposed TS either based on information local to the MAC entity, or using additional information received across the SSPN interface. The TS, however, is not created. The TSID and direction values within the TSPEC are as in the matching MLME-ADDTS.indication primitive. The difference may lie in the QoS (e.g., minimum data rate, mean data rate, and delay bound) values, as a result of admission control performed at the SME of the HC on the TS requested to be added (or modified) by the non-AP STA. If sufficient bandwidth is not available, the QoS values may be reduced. In one extreme, the minimum data rate, mean data rate, and delay bound may be all set to 0, indicating that no QoS is to be provided to this TS.