IEEE P802.11
Wireless LANs

| Mesh Configuration Element |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-02-22 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Guenael Strutt | Motorola | 1064 Greenwood Blvd Suite 400 — Lake Mary, FL 32746-5418 | +1 407-562-4050 | Guenael.Strutt@motorola.com |

Abstract

Resolution of LB93 comments related to WLAN Mesh Capability element

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Affected CIDs

G14 category (“peer capacity”):

  - CIDs 7, 337, 1282, 1283, 2130, 2299, 4311, (5260) 5260 (S4) is not part of G14 but it should be

Rearranging bits in the Mesh Configuration element:

  - CIDs 2302, 469

Rearranging clauses (Active Protocol, Active Metric):

  - CIDs 350, 793, 355, 794, 2643, 3539, 3790, 4495, 356, 357, 360, 732, 1353, 4496, 4481, 5637

Remove Active Profile

  - CID 2316

General cleanup

  - CID 2941

The editor should perform the following four tasks:

1) Remove WLAN from “WLAN Mesh Capability” (30 occurrences in D1.00)

2) Replace all occurrences of “Mesh Capability” with “Mesh Configuration”

3) Add italicized text to 11A.1.5.1 Overview:

An MP must be able to establish at least one mesh link with a peer MP, and may be able to establish many such links simultaneously. It is possible that there are more candidate peer MPs than the device is capable of maintaining peer links with simultaneously. In this case, the MP must select which MPs to establish peer links with based on some measure of signal quality or other information received from candidate neighbor MPs, such as a Peer Link Capacity value.  However, the MP shall not establish a peer link with another MP that has advertised a Peer Link Capacity of 0 [0000] in the last received Mesh Configuration element.  Peer Link Capacity values between 1 [0001] and 15 [1111] may be interpreted as a willingness to establish peer links, based on connectivity, available memory or remaining battery life; the higher the value, the higher the willingness.

4) Replace 7.3.2.36-39 with the following

- 7.3.2.36 Mesh Configuration element

The WLAN Mesh Configuration element is used to advertise WLAN Mesh services. It is contained in beacons transmitted by MPs, in probe request/response messages and Peer Link Management messages.

| Octets: 1 | 1 | 1 | 4 | 4 | 4 | 2 |
| --- | --- | --- | --- | --- | --- | --- |
| ID | Length | Version | Active Protocol ID | Active Metric ID | Channel Precedence | Mesh Capability |

Figure s1: Mesh Configuration element

The fields contained in the element are as shown in Table s1.

Table s1: Mesh Configuration element fields

| Field | Value/description |
| --- | --- |
| ID |  |
| Length |  |
| Version | 1 |
| Active Path Selection Protocol ID | Path selection protocol in use |
| Active Path Selection Metric ID | Path selection metric in use |
| Channel precedence | Channel precedence value |
| Mesh Capability | Peer settings |

Active Path Selection Protocol ID

The path selection protocol identifier field identifies the path selection protocol for unicast, multicast and broadcast transmission.  The format of the protocol identifier is shown in Figure s2.

| Octets: 3 | 1 |
| --- | --- |
| OUI | Path selection protocol identifier |

Figure s2: Path selection protocol identifier field format

The path selection protocol identifier specifies the protocol which is currently used to generate routing information in this network, as defined in Clause Error: Reference source not found.  Values for the protocol identifier are given in Table s3.

Table s2: Path selection protocol identifier field values

| OUI | Value | Meaning |
| --- | --- | --- |
| 00-0F-AC | 0 | Hybrid Wireless Mesh Protocol (default path selection protocol) |
| 00-0F-AC | 1 | Radio Aware OLSR (optional path selection protocol) |
| 00-0F-AC | 2-254 | Reserved for future use |
| 00-0F-AC | 255 | Null protocol |
| Vendor OUI | Other | Vender specific |

A Null protocol indicates the MP has no active layer 2 path selection and forwarding. An MP with Null protocol will not send or respond to path selection protocol messages.

Active Path Selection Metric ID

The path selection metric identifier identifies the path metric which is currently used by the active path selection protocol in the mesh network. The format of the path selection metric identifier is shown in Figure s3.

| Octets: 3 | 1 |
| --- | --- |
| OUI | Path selection metric identifier |

Figure s3: Path selection metric identifier format

The path selection metric identifier specifies the metric that is used when selecting routes in this network, as defined in Table s3.

Table s3: Path selection metric identifier Values

| OUI | Value | Meaning |
| --- | --- | --- |
| 00-0F-AC | 0 | Airtime path metric (default path metric) |
| 00-0F-AC | 1-254 | Reserved for future use |
| 00-0F-AC | 255 | Null metric |
| Vendor OUI | Other | Vender specific |

Null metric is used in conjunction with Null protocol setting.

Channel precedence

The channel precedence field is set to the value of channel precedence of the unified channel graph to which the MP interface belongs.  The simple unification mode is defined in Clause Error: Reference source not found).  A value of 0 identifies that the logical interface used by the MP is not currently operating in simple unification mode.

Mesh capability

The mesh capability field comprises a set of values indicating whether an MP is a possible candidate for Peer Link establishment.  The details of the mesh capability field are shown in Figure s14 and Table s2.

| Bits: 4 | 1 | 1 | 1 | 1 | 1 |
| --- | --- | --- | --- | --- | --- |
| Peer Link Capacity | Connected to AS | Power Save Supported | Power Save Active | Power Save Required from Peer | Synchronization Supported |

| 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| --- | --- | --- | --- | --- | --- | --- |
| Synchronization Active | Synchronization Required from Peer | MDA Supported | MDA Active | MDA Required from Peer | MDA not allowed | MDA Mixed Mode |

Figure s4: Mesh Capability field format

Table s4: Mesh Capability field details

| Capability | Field detail |
| --- | --- |
| Peer candidacy | Peer Link Capacity |
|  | Connected to AS |
| Power Save | Power Save Supported |
|  | Power Save Active |
|  | Power Save Required from Peer |
| Synchronization | Synchronization Supported |
|  | Synchronization Active |
|  | Synchronization Required from Peer |
| MDA | MDA Supported |
|  | MDA Active |
|  | MDA Required from Peer |
|  | MDA not allowed |
|  | MDA Mixed Mode |

The Peer candidacy capability includes 2 sub-fields.

- Peer Link Capacity is a measure of the willingness of a MP to establish peer links with other MPs.  A Peer Link Capacity value of 0 [0000] indicates that the MP is unwilling or incapable of establishing new peer links (this could be, for example, if there isn’t any memory left to accommodate a new peer link entry, or if the battery level of the device is critically low).  Inversely, a Peer Link Capacity value of 15 [1111] indicates that the MP is eager to establish new peer links.  See Clause 11A.1.5.1.

- The “connected to AS” subfield specifies whether the mesh point is connected to an AS (Authentication Server), enabling the MP to support authentication and key management with IEEE 802.1X.

The Power Save capability includes 5 sub-fields.

- The “Power Save Supported” subfield indicates if the Mesh Point supports power save mode.

- The “Power Save Active” sub filed indicates the current Power Save Management Mode of the mesh point. A set bit indicates the mesh point is operating in Power Save mode. A cleared bit indicates it is not operating in power save mode.

- The “Power Save Required from Peer” subfield indicates if this mesh point requires peers attempting to associate or directly communicate with it to support Power Save mode.

The Synchronization capability includes 3 sub-fields.

- The “Synchronization Supported” subfield is set to 1 if the Mesh Point supports timing synchronization with peer MPs, and 0 otherwise.

- The “Synchronization Active” subfield is set to 1 if the MP is currently a synchronizing MP, and 0 otherwise.

- The “Synchronization Required from Peer” subfield is set to 1 if the mesh point requests MP peers attempting to communicate with it to synchronize with it, and 0 otherwise.

The MDA capability includes 5 sub-fields:

- The “MDA Supported” subfield is set to 1 if the Mesh Point supports MDA services, and 0 otherwise.

- The “MDA Active” subfield is set to 1 if the mesh point has MDA services active, and 0 otherwise. When set to 1, the MP provides full MDA services as described in Error: Reference source not found. When set to 0, the MP does not interpret any frames described for MDA operation and does not provides any services described in Error: Reference source not found. This field is ignored and interpreted as 0, if “MDA capable” bit is set to 0.

- The “MDA Required from Peer” field is set to 1 if the mesh requires that all MPs that are capable of MDA have MDA active, and to 0 otherwise. This is an informative flag and the participating MPs are not required to act on it. This field is ignored and interpreted as 0 if either of “MDA Capable” or “MDA Active” bits is set to 0. This field is a mesh wide field, and common for all MPs that belong to a single mesh.

- The “MDA not allowed” subfield is set to 1 if the mesh requires that all MPs participating in the mesh have MDA not activated, and 0 otherwise. If this field is set to 1, the “MDA Active” and “MDA Active Required in Mesh” fields are ignored and interpreted as 0. This field is a mesh wide field, and common for all MPs that belong to a single mesh. If this field is set to 1 in a mesh, any new MP that wishes to participate in the mesh is required to not use/invoke MDA services.  When “MDA not allowed” is set to 0, MPs that are capable of providing MDA services may use MDA in the Mesh for portions of their traffic.

- The “MDA Mixed Mode” field is set to 1, if MDA traffic may be transmitted using EDCA access along with MDA access, and 0 otherwise. If this filed is set to 0, any flow that is set to access channel using MDA may only transmit during MDAOPs of the transmitter MP.

Previous text (for reference):

- Mesh Capability element

The “Mesh Capability” element shown in  is used to advertise Mesh services. It is contained in Beacon frames transmitted by MPs, and is also contained in probe request/response messages and (re)association request/response messages.

| Octets:1 | 1 | 1 | 4 | 4 | 2 | 1 | 1 | 1 | 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ID | Length | Version | Active Protocol ID | Active Metric ID | Peer Capacity | Power Save capability | Synch- ronization Capability | MDA Capability | Channel Precedence |
| - Mesh Capability element |  |  |  |  |  |  |  |  |  |

The Element ID is set to the value given in Error: Reference source not found for this information element. The Length field is set to 18. The version is set to 1.

MPs may support one or more path selection protocols and path metrics. However, only one path selection protocol and one path metric may be active in a particular mesh network at a time.

The Active Protocol ID field indicates the path selection protocol in use and is described in Error: Reference source not found. The Active Metric ID field indicates the path selection metric in use and is described in Active.

The Peer capacity field indicates the MP’s capacity for establishing additional peer links and is described in Peer Capacity field.

The Power Save capability field indicates support for power save mode and current power save mode and is described in Power Save Capability field.

The Synchronization Capability field indicates support for synchronization services and current synchronization status and is described in Synchronization Capability field.

The MDA Capability field indicates support for MDA services and current status and is described in MDA Capability field.

The channel precedence field is set to the value of the channel precedence of the unified channel graph to which the MP interface belongs.

- Path selection protocol identifier field

The path selection protocol identifier indicates the path selection protocol for individually addressed and group addressed transmission. It has format shown in Error: Reference source not found.

| Octets: 3 | 1 |
| --- | --- |
| OUI | Path selection protocol identifier |
| - Path selection protocol identifier field |  |

The path selection protocol identifier specifies the protocol which is currently used to generate routing information in this network, as defined in 11A.3.

Identifier values are listed in Path.

| - Path selection protocol identifier values |  |  |
| --- | --- | --- |
| OUI | Value | Meaning |
| 00-0F-AC | 0 | Hybrid Wireless Mesh Protocol (default path selection protocol) |
| 00-0F-AC | 1 | Radio Aware OLSR (optional path selection protocol) |
| 00-0F-AC | 2-254 | Reserved for future use |
| 00-0F-AC | 255 | Null protocol |
| Vendor OUI | Other | Vendor specific |

A Null protocol indicates the MP has no active layer 2 path selection and forwarding. An MP with Null protocol does not send or respond to path selection protocol messages.

- Path selection metric identifier

- field

The path selection metric identifier is contained in the Active Metric field. This information identifies the path metric which is currently used by the active path selection protocol in the mesh network. The path selection metric identifier has the format shown in Error: Reference source not found.

| Octets: 3 | 1 |
| --- | --- |
| OUI | Path selection metric identifier |
| - Path selection metric identifier field |  |

The path selection metric identifier specifies the metric that is used when selecting routes in this network, as defined in Path selection metric identifier values.

| - Path selection metric identifier values |  |  |
| --- | --- | --- |
| OUI | Value | Meaning |
| 00-0F-AC | 0 | Airtime path metric (default path metric) |
| 00-0F-AC | 1-254 | Reserved for future use |
| 00-0F-AC | 255 | Null metric |
| Vendor OUI | Other | Vendor specific |

The null metric is used in conjunction with null path selection protocol setting.

- Peer Capacity field

The peer capacity value is treated as a single field, with the least significant octet transmitted first. It contains four sub-fields as shown in Sync.

| B0 | B1 | B2 | B15 |
| --- | --- | --- | --- |
| Operating in simple unification mode | Connected to AS | Peer capacity |  |
| 1 | 1 | 14 |  |
| - Peer Capacity field |  |  |  |

The “operating in simple unification mode (11A.1.7.4)” bit is set to one if the PHY is currently operating in the simple unification mode, and zero otherwise. The “connected to AS” bit specifies whether the MP is connected to an AS (Authentication Server), enabling the MP to support authentication and key management with IEEE 802.1X.  The “peer capacity” subfield is set to the number of additional MP peers that the device can accommodate.

- Power Save Capability field

The Power Save capability field is shown in Sync.

| B0 | B1 | B2 | B3 B7 |
| --- | --- | --- | --- |
| Supporting Power Save Mode | Require Power Save Mode from Peer | Power Save Mode Enabled | Reserved |
| Bits: 1 | 1 | 1 | 5 |
| - Power Save Capability field |  |  |  |

The “Supporting Power Save Mode” bit indicates if the MP supports power save mode.

The “Require Power Save Mode from peer” bit indicates if this MP requires peers attempting to associate or directly communicate with it to support Power Save mode.

The “Power Save Mode Enabled” bit, when set to 1, indicates the MP is operating in Power Save Mode. A bit value of 0 indicates it is not operating in Power Save Mode.

- Synchronization Capability field

The Synchronization Capability field includes 3 sub-fields as shown in Sync.

| B0 | B1 | B2 | B3 B7 |
| --- | --- | --- | --- |
| Supporting Synchronization | Requests Synchronization from Peer | Synchronizing with Peer MP | Reserved |
| Bits: 1 | 1 | 2 | 5 |
| - Synchronization Capability field |  |  |  |

The “Supporting Synchronization” subfield is set to 1 if the MP supports timing synchronization with peer MPs, and 0 otherwise.

The “Requests Synchronization from Peer” subfield is set to 1 if the MP requests MP peers attempting to communicate with it to synchronize with it, and 0 otherwise.

The “Synchronizing with Peer MP” subfield is set to 1 if the non-AP MP is currently a synchronizing MP, and 0 otherwise.

- MDA Capability field

The MDA Capability field includes 5 sub-fields as shown in MDA Capabilit.

| B0 | B1 | B2 | B3 | B4 | B5 B7 |
| --- | --- | --- | --- | --- | --- |
| MDA Capable | MDA Active | MDA Active Requested in Mesh | MDA Not Allowed in Mesh | MDA EDCA Mixed Mode Enabled | Reserved |
| Bits: 1 | 1 | 1 | 1 | 1 | 3 |
| - MDA Capability field |  |  |  |  |  |

The “MDA Capable” subfield is set to 1 if the MP supports MDA, and 0 otherwise.

The “MDA active” subfield is set to 1 if MDA is active, and 0 otherwise. When set to 1, the MP provides full MDA services as described in 9.14. When set to 0, the MP does not interpret any frames described for MDA operation and does not provides any services described in 9.14. This field is ignored and interpreted as 0, if “MDA capable” bit is set to 0.

The “MDA Active Requested in Mesh” field is set to 1 if the mesh requires that all MPs that are capable of MDA have MDA active, and to 0 otherwise. This is an informative flag and the participating MPs are not required to act on it. This field is ignored and interpreted as 0 if either of “MDA Capable” or “MDA Active” bits are set to 0. This field is a mesh wide field, and common for all MPs that belong to a single mesh.

The “MDA Not Allowed in Mesh” subfield is set to 1 if the mesh requires that all MPs participating in the mesh have MDA not activated, and 0 otherwise. If this field is set to 1, the “MDA Active” and “MDA Active Required in Mesh” fields are ignored and interpreted as 0. This field is a mesh wide field, and common for all MPs that belong to a single mesh. If this field is set to 1 in a mesh, any MP that wishes to participate in the mesh is required to not use/invoke MDA services.

When “MDA not Allowed in Mesh” is set to 0, MPs that are capable of providing MDA services may use MDA in the Mesh for portions of their traffic.

The “MDA EDCA Mixed Mode Enabled” field is set to 1, if MDA traffic may be transmitted using EDCA access along with MDA access, and 0 otherwise. If this field is set to 0, any flow that is set to access the channel using MDA may only transmit during MDAOPs of the transmitter MP.

- Active Mesh Profile Announcement element

The “Active Mesh Profile Announcement” element is used to notify the profile pair of the active path selection protocol and the active path metric to a peer MP. It is contained in association request messages transmitted by the association requesting MP. The profile pair is selected by the association requesting MP. The format of the Active Mesh Profile Announcement element is shown in Error: Reference source not found.

| Octets: 1 | 1 | 1 | 4 | 4 |
| --- | --- | --- | --- | --- |
| ID | Length | Version | Active Protocol ID | Active Metric ID |
| - Active Mesh Profile Announcement element |  |  |  |  |

The Element ID is set to the value given in Error: Reference source not found for this information element. The Length field is set to 9. The version field is set to 1.

The Active Protocol ID indicates the path selection protocol in use and is formatted as described in Error: Reference source not found.

The Active Metric ID indicates the path selection metric in use and is formatted as described in Active.