IEEE P802.11
Wireless LANs

| Mesh Configuration Element |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-04-13 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Guenael Strutt | Motorola | 1064 Greenwood Blvd Suite 400 — Lake Mary, FL 32746-5418 | +1 407-562-4050 | Guenael.Strutt@motorola.com |
| Juan Carlos Zuniga | InterDigital Communications | 1000, rue Sherebrook, Ouest, 10e étage, Montreal, QC, H3A 3G4 | +1514-904-6251 | JuanCarlos.Zuniga@InterDigital.com |
| Guido R. Hiertz | Philips | ComNets, RWTH Aachen University, Kopernikusstr. 16, 52074 Aachen, Germany | +49-241-802-5829 | hiertz@ieee.org |
| Dee Denteneer | Philips | Philips Research, HTC 27 (WL 1.132), 5656 AE Eindhoven, The Netherlands | +31-402-746-937 | dee.denteneer@philips.com |
| Kazuyuki Sakoda | Sony Corporation | 5-1-12 Kita-Shinagawa, Shinagawa-ku, Tokyo, 141-0001 Japan | +81-3-5448-4017 | sako@wcs.sony.co.jp |
| Jarkko Kneckt | Nokia | Itämerenkatu 11-13 00180 Helsinki, Finland | +358504821550 | Jarkko.Kneckt@nokia.com |

Abstract

Resolution of LB93 comments related to WLAN Mesh Capability element

- Changes from r0 to r1

  - Removed suggested changes to Peer Link Capacity until consensus is built.  Also, the base reference text is D1.03.  Clarified the word “required” by stating “support required” in the capability fields.

- Changes from r1 to r2

  - Resolution to MDA Capability comments: CID 4313, 4314, 5234, 526, 1806 by revising the number of capability bits required.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Affected CIDs

G14 category (“peer capacity”):

  - CIDs 7, 337, 1282, 1283, 2130, 2299, 4311, (5260) 5260 (S4) is not part of G14 but it should be

Rearranging bits in the Mesh Configuration element:

  - CIDs 2302, 469

Rearranging clauses (Active Protocol, Active Metric):

  - CIDs 350, 793, 355, 794, 2643, 3539, 3790, 4495, 356, 357, 360, 732, 1353, 4496, 4481, 5637

Remove Active Profile

  - CID 2316

General cleanup

  - CID 2941

MDA Capability

  - CID 4313, 4314, 5234, 526, 1806

The editor should perform the following four tasks:

1) Replace all occurrences of “Mesh Capability” with “Mesh Configuration”

2) Replace 7.3.2.36-39 with the following

- 7.3.2.36 Mesh Configuration element

The Mesh Configuration element shown in [Figure sXX] is used to advertise Mesh services. It is contained in beacon frames transmitted by MPs, and is also contained in probe request/response messages and Peer Link Management messages.

| Octets: 1 | 1 | 1 | 4 | 4 | 4 | 2 |
| --- | --- | --- | --- | --- | --- | --- |
| ID | Length | Version | Active Protocol ID | Active Metric ID | Channel Precedence | Mesh Capability |

Figure s1: Mesh Configuration element

The fields contained in the element are as shown in Table s1.

Table s1: Mesh Configuration element fields

| Field | Value/description |
| --- | --- |
| ID |  |
| Length |  |
| Version | 1 |
| Active Path Selection Protocol ID | Path selection protocol in use |
| Active Path Selection Metric ID | Path selection metric in use |
| Channel precedence | Channel precedence value |
| Mesh Capability | Peer settings |

Active Path Selection Protocol ID

The path selection protocol identifier field identifies the path selection protocol for individually addressed and group addressed transmission.  The format of the protocol identifier is shown in Figure s2.

| Octets: 3 | 1 |
| --- | --- |
| OUI | Path selection protocol identifier |

Figure s2: Path selection protocol identifier field

The path selection protocol identifier specifies the protocol that is currently used to generate path selection information in this network, as defined in Clause Error: Reference source not found.  Identifier values are listed in Table s3.

Table s2: Path selection protocol identifier field values

| OUI | Value | Meaning |
| --- | --- | --- |
| 00-0F-AC | 0 | Hybrid Wireless Mesh Protocol (default path selection protocol) |
| 00-0F-AC | 1 | Radio Aware OLSR (optional path selection protocol) |
| 00-0F-AC | 2-254 | Reserved for future use |
| 00-0F-AC | 255 | Null protocol |
| Vendor OUI | Other | Vendor specific |

A Null protocol indicates the MP has no active layer 2 path selection and forwarding. An MP with Null protocol does not send or respond to path selection protocol messages.

Active Path Selection Metric ID

The path selection metric identifier identifies the path metric that is currently used by the active path selection protocol in the mesh network. The format of the path selection metric identifier is shown in Figure s3.

| Octets: 3 | 1 |
| --- | --- |
| OUI | Path selection metric identifier |

Figure s3: Path selection metric identifier field

The path selection metric identifier specifies the metric that is currently used when selecting paths in this network, as defined in Table s3.

Table s3: Path selection metric identifier field values

| OUI | Value | Meaning |
| --- | --- | --- |
| 00-0F-AC | 0 | Airtime path metric (default path metric) |
| 00-0F-AC | 1-254 | Reserved for future use |
| 00-0F-AC | 255 | Null metric |
| Vendor OUI | Other | Vendor specific |

Null metric is used in conjunction with the null path selection protocol setting.

Channel precedence

The channel precedence field is set to the value of channel precedence of the unified channel graph to which the MP interface belongs.  Usage of channel precedence is described in Clause Error: Reference source not found).  A value of 0 identifies that the logical interface used by the MP is not currently operating in simple unification mode.

Mesh capability

The mesh capability field comprises a set of values indicating whether an MP is a possible candidate for Peer Link establishment.  The details of the mesh capability field are shown in Figure s14 and Table s2.

| Bits: 3 | 1 | 1 | 1 | 1 | 1 |
| --- | --- | --- | --- | --- | --- |
| Peer Link Capacity | Connected to AS | Power Save Supported | Power Save Active | Power Save Support Required from Peer | Synchronization Enabled |

| 1 | 1 | 1 | 5 |
| --- | --- | --- | --- |
| Synchronization Active | Synchronization Support Required from Peer | MDA Enabled | Reserved |

Figure s4: Mesh Capability field format

Table s4: Mesh Capability field details

| Capability | Field detail |
| --- | --- |
| Peer candidacy | Peer Link Capacity |
|  | Connected to AS |
| Power Save | Power Save Supported |
|  | Power Save Active |
|  | Power Save Support Required from Peer |
| Synchronization | Synchronization Enabled |
|  | Synchronization Active |
|  | Synchronization Support Required from Peer |
| MDA | MDA Enabled |
| Reserved | Reserved for future use |

The Peer candidacy capability includes 2 sub-fields.

- Peer Link Capacity is the capacity of an MP to establish peer links with other MPs.  See Clause 11A.1.5.1.

- The “connected to AS” subfield specifies whether the mesh point is connected to an AS (Authentication Server), enabling the MP to support authentication and key management with IEEE 802.1X.

The Power Save capability includes 5 sub-fields.

- The “Power Save Supported” subfield indicates if the Mesh Point supports power save mode.

- The “Power Save Active” sub filed indicates the current Power Save Management Mode of the mesh point. A set bit indicates the mesh point is operating in Power Save mode. A cleared bit indicates it is not operating in power save mode.

- The “Power Save Support Required from Peer” subfield indicates if this mesh point requires peers attempting to associate or directly communicate with it to support Power Save mode.

The Synchronization capability includes 3 sub-fields.

- The “Synchronization Enabled” subfield is set to 1 if the Mesh Point supports timing synchronization with peer MPs, and 0 otherwise.

- The “Synchronization Active” subfield is set to 1 if the MP is currently a synchronizing MP, and 0 otherwise.

- The “Synchronization Support Required from Peer” subfield is set to 1 if the mesh point requests MP peers attempting to communicate with it to synchronize with it, and 0 otherwise.

The MDA capability includes 1 sub-fields:

- The “MDA Enabled” subfield is set to 1 if the Mesh Point supports MDA services, and 0 otherwise.

Previous text in D1.03 (for reference)

- Mesh Capability element

The “Mesh Capability” element shown in  is used to advertise Mesh services. It is contained in Beacon frames transmitted by MPs, and is also contained in probe request/response messages and (re)association request/response messages.

| Octets:1 | 1 | 1 | 4 | 4 | 2 | 1 | 1 | 1 | 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ID | Length | Version | Active Protocol ID | Active Metric ID | Peer Capacity | Power Save capability | Synch- ronization Capability | MDA Capability | Channel Precedence |
| - Mesh Capability element |  |  |  |  |  |  |  |  |  |

The Element ID is set to the value given in Error: Reference source not found for this information element. The Length field is set to 18. The version is set to 1.

MPs support one or more path selection protocols and one or more path metrics. However, only one path selection protocol and one path metric may be active in a particular mesh network at a time.

The Active Protocol ID field indicates the path selection protocol in use and is described in Path selection protocol identifier field. The Active Metric ID field indicates the path selection metric in use and is described in Error: Reference source not found.

The Peer capacity field indicates the MP’s capacity for establishing additional mesh links and is described in Peer Capacity field.

The Power Save capability field indicates support for power save mode and current power save mode and is described in Power Save Capability field.

The Synchronization Capability field indicates support for synchronization services and current synchronization status and is described in Synchronization Capability field.

The MDA Capability field indicates support for MDA services and current status and is described in MDA Capability field.

The channel precedence field is set to the value of the channel precedence of the unified channel graph to which the MP interface belongs.

It is described in Channel precedence field

- Path selection protocol identifier field

The path selection protocol identifier indicates the path selection protocol for individually addressed and group addressed transmission. The format of the path selection protocol identifier is shown in Error: Reference source not found.

| Octets: 3 | 1 |
| --- | --- |
| OUI | Path selection protocol identifier |
| - Path selection protocol identifier field |  |

The path selection protocol identifier specifies the protocol that is currently used to generate path selection information in this network, as defined in 11A.3.

Identifier values are listed in Path.

| - Path selection protocol identifier values |  |  |
| --- | --- | --- |
| OUI | Value | Meaning |
| 00-0F-AC | 0 | Hybrid Wireless Mesh Protocol (default path selection protocol) |
| 00-0F-AC | 1 | Radio Aware OLSR (optional path selection protocol) |
| 00-0F-AC | 2-254 | Reserved for future use |
| 00-0F-AC | 255 | Null protocol |
| Vendor OUI | 0-255 | Vendor specific |

A Null protocol indicates the MP has no active layer 2 path selection and forwarding. An MP with Null protocol does not send or respond to path selection protocol messages.

- Path selection metric identifier

- field

The path selection metric identifier is contained in the Active Metric field. This information identifies the path metric that is currently used by the active path selection protocol in the mesh network. The path selection metric identifier has the format shown in Error: Reference source not found.

| Octets: 3 | 1 |
| --- | --- |
| OUI | Path selection metric identifier |
| - Path selection metric identifier field |  |

The path selection metric identifier specifies the metric that is used when selecting paths in this network, as defined in Path selection metric identifier values.

| - Path selection metric identifier values |  |  |
| --- | --- | --- |
| OUI | Value | Meaning |
| 00-0F-AC | 0 | Airtime link metric defined in 11A.5 (default path selection metric) |
| 00-0F-AC | 1-254 | Reserved for future use |
| 00-0F-AC | 255 | Null metric |
| Vendor OUI | 0-255 | Vendor specific |

The null metric is used in conjunction with null path selection protocol setting.

- Peer Capacity field

The peer capacity value is treated as a single field, following the conventions of 7.1.1. It contains four sub-fields as shown in Sync.

| B0 | B1 | B2 | B15 |
| --- | --- | --- | --- |
| Reserved |  | Peer capacity |  |
| Bits: 2 |  | 14 |  |
| - Peer Capacity field |  |  |  |

The “peer capacity” subfield is set to the number of additional MP peers that the device can accommodate.

- Power Save Capability field

The Power Save capability field is shown in Sync.

| B0 | B1 | B2 | B3 B7 |
| --- | --- | --- | --- |
| Supporting Power Save Mode | Require Power Save Mode from Peer | Power Save Mode Enabled | Reserved |
| Bits: 1 | 1 | 1 | 5 |
| - Power Save Capability field |  |  |  |

The “Supporting Power Save Mode” bit is set to 1 if the MP supports power save mode and set to 0 if it does not support power save mode.

The “Require Power Save Mode from peer” bit is set to 1 if the MP requires MPs attempting to establish a peer link with it to support Power Save mode and set to 0 otherwise.

The “Power Save Mode Enabled” bit, when set to 1, indicates the MP is operating in Power Save Mode. A bit value of 0 indicates it is not operating in Power Save Mode.

- Synchronization Capability field

The Synchronization Capability field includes 3 sub-fields as shown in Sync.

| B0 | B1 | B2 | B3 B7 |
| --- | --- | --- | --- |
| Supporting Synchronization | Requests Synchronization from Peer | Synchronizing with Peer MP | Reserved |
| Bits: 1 | 1 | 2 | 5 |
| - Synchronization Capability field |  |  |  |

The “Supporting Synchronization” subfield is set to 1 if the MP supports timing synchronization with peer MPs, and 0 otherwise.

The “Requests Synchronization from Peer” subfield is set to 1 if the MP requests MP peers attempting to communicate with it to synchronize with it, and 0 otherwise.

The “Synchronizing with Peer MP” subfield is set to 1 if the non-AP MP is currently a synchronizing MP, and 0 otherwise.

- MDA Capability field

The MDA Capability field includes 5 sub-fields as shown in MDA Capabilit.

| B0 | B1 | B2 | B3 | B4 | B5 B7 |
| --- | --- | --- | --- | --- | --- |
| MDA Capable | MDA Active | MDA Active Requested in Mesh | MDA Not Allowed in Mesh | MDA EDCA Mixed Mode Enabled | Reserved |
| Bits: 1 | 1 | 1 | 1 | 1 | 3 |
| - MDA Capability field |  |  |  |  |  |

The “MDA Capable” subfield is set to 1 if the MP supports MDA, and 0 otherwise. When the “MDA Capable” subfield is set to 0, all other MDA capability fields are also set to 0.

The “MDA active” subfield is set to 1 if MDA is active, and 0 otherwise. When set to 1, the MP provides full MDA services as described in 9.21. When set to 0, the MP does not interpret frames described for MDA operation and does not provide services described in 9.21. This field is ignored, if “MDA capable” bit is set to 0.

The “MDA Active Requested in Mesh” subfield is set to 1 if the the MP requests that all MPs that are capable of MDA have MDA active, and to 0 otherwise. This is an informative flag and the participating MPs are not required to act on it. This field is ignored if either of “MDA Capable” or “MDA Active” bits are set to 0. This field is a mesh wide field, and common for all MPs that belong to a single mesh.

The “MDA Not Allowed in Mesh” subfield is set to 1 if the mesh requires that all MPs participating in the mesh have MDA disabled, and 0 otherwise. If this field is set to 1, the “MDA Active” and “MDA Active Requested in Mesh” fields are ignored. This field is a mesh wide field, and common for all MPs that belong to a single mesh. If this field is set to 1 in a mesh, an MP that wishes to participate in the mesh is required to not use/invoke MDA services.

When the “MDA not Allowed in Mesh” subfield is set to 0, MPs that are capable of providing MDA services may use MDA in the Mesh for portions of their traffic.

The “MDA EDCA Mixed Mode Enabled” subfield is set to 1, if MDA traffic may be transmitted using EDCA access and MDA access, and 0 otherwise. If this subfield is set to 0, a flow accessing the channel using MDA may only transmit during MDAOPs of the transmitter MP.

- Channel precedence field

The channel precedence field is shown in .

| B0 |  | B31 |
| --- | --- | --- |
|  | Channel precedence value |  |
|  | Bits: 32 |  |
| - Channel precedence field |  |  |

Usage of the channel precedence field is described in 11A.1.7.

Previous text in D1.00 (for reference):

      - WLAN Mesh Capability element

The “WLAN Mesh Capability” element is used to advertise WLAN Mesh services. It is contained in beacons transmitted by MPs, and is also contained in probe request/response messages and (re)association request/response messages.

| Octets: 1 | 1 | 1 | 4 | 4 | 2 | 1 | 1 | 2 | 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ID | Length | Version | Active Protocol ID | Active Metric ID | Peer Capacity | Power Save capability | Synch-ronization Capability | MDA Capability | Channel Precedence |

Figure s5: WLAN Mesh Capability Element

The fields contained in the element are as shown in Table s1.

Table s5: WLAN Mesh Capability Element Fields

| Field | Value/description |
| --- | --- |
| ID | T.B.D |
| Length | Variable |
| Version | 1 |
| Active Protocol ID | Path selection protocol in use |
| Active Metric ID | Path selection metric in use |
| Peer capacity | Peer capacity value |
| Power Save capability | Support for power save mode and current power management mode |
| Synchronization Capability | Support for synchronization services and current synchronization status |
| MDA Capability | Support for MDA services and current status |
| Channel precedence | Channel precedence value |

MPs may support one or more path selection protocols and path metrics. However, only one path selection protocol and one path metric may be active in a particular WLAN mesh network at any point in time.

The WLAN Mesh Capability Element indicates an active path selection protocol and an active path metric.

The peer capacity value is treated as a single field, with the least significant octet transmitted first. It contains four sub-fields as shown in Error: Reference source not found.

| 15 | 14 | 13 | 12 | 0 |
| --- | --- | --- | --- | --- |
| Operating as MP | Operating in simple unification mode | Connected to AS | Peer capacity |  |

Figure s6: Peer Capacity Field

The “operating as MP” subfield is set to one if the device is currently operating as a MP, and zero otherwise. The “operating in simple unification mode (Clause Error: Reference source not found)” subfield is set to one if the logical radio interface is currently operating in the simple unification mode, and zero otherwise. The “connected to AS” subfield specifies whether the mesh point is connected to an AS (Authentication Server), enabling the MP to support authentication and key management with IEEE 802.1X.  The “peer capacity” subfield is set to the number of additional MP peers that the device can accommodate.

The Power Save capability field includes 5 sub-fields.

| 7 | 6 | 5 | 4-0 |
| --- | --- | --- | --- |
| Supporting Power Save Mode | Require Power Save Mode from Peer | Current Power Management Mode | Reserved |

Figure s7: Power Save Capability field

The “Supporting Power Save Mode” subfield indicates if the Mesh Point supports power save mode.

The “Require Power Save Mode from peer” subfield indicates if this mesh point requires peers attempting to associate or directly communicate with it to support Power Save mode.

The Current Power Management Mode” sub filed indicates the current Power Save Management Mode of the mesh point. A set bit indicates the mesh point is operating in Power Save mode. A cleared bit indicates it is in active mode.

The Synchronization Capability field includes 3 sub-fields.

| Bits: 0 | 1 | 2 | 3-7 |
| --- | --- | --- | --- |
| Supporting Synchronization | Requests Synchronization from Peer | Synchronizing with peer MP | Reserved |

Figure s8: Synchronization Capability Field

The “Supporting Synchronization” subfield is set to 1 if the Mesh Point supports timing synchronization with peer MPs, and 0 otherwise.

The “Requests Synchronization from Peer” subfield is set to 1 if the mesh point requests MP peers attempting to communicate with it to synchronize with it, and 0 otherwise.

The “Synchronizing with Peer MP” subfield is set to 1 if the non-AP MP is currently a synchronizing MP, and 0 otherwise.

The MDA Capability field includes 5 sub-fields:

| Bits: 0 | 1 | 2 | 3 | 4 | 5-7 |
| --- | --- | --- | --- | --- | --- |
| MDA Capable | MDA Active | MDA Active Requested in Mesh | MDA Not Allowed in Mesh | MDA EDCA Mixed Mode Enabled | Reserved |

Figure s9: MDA Capability Field

The “MDA Capable” subfield is set to 1 if the Mesh Point supports MDA services, and 0 otherwise.

The “MDA active” subfield is set to 1 if the mesh point has MDA services active, and 0 otherwise. When set to 1, the MP provides full MDA services as described in Error: Reference source not found. When set to 0, the MP does not interpret any frames described for MDA operation and does not provides any services described in Error: Reference source not found. This field is ignored and interpreted as 0, if “MDA capable” bit is set to 0.

The “MDA Active Requested in Mesh” field is set to 1 if the mesh requires that all MPs that are capable of MDA have MDA active, and to 0 otherwise. This is an informative flag and the participating MPs are not required to act on it. This field is ignored and interpreted as 0 if either of “MDA Capable” or “MDA Active” bits are set to 0. This field is a mesh wide field, and common for all MPs that belong to a single mesh.

The “MDA Not Allowed in Mesh” subfield is set to 1 if the mesh requires that all MPs participating in the mesh have MDA not activated, and 0 otherwise. If this field is set to 1, the “MDA Active” and “MDA Active Required in Mesh” fields are ignored and interpreted as 0. This field is a mesh wide field, and common for all MPs that belong to a single mesh. If this field is set to 1 in a mesh, any new MP that wishes to participate in the mesh is required to not use/invoke MDA services.

When “MDA not Allowed in Mesh” is set to 0, MPs that are capable of providing MDA services may use MDA in the Mesh for portions of their traffic.

The “MDA EDCA Mixed Mode Enabled” field is set to 1, if MDA traffic may be transmitted using EDCA access along with MDA access, and 0 otherwise. If this filed is set to 0, any flow that is set to access channel using MDA may only transmit during MDAOPs of the transmitter MP.

The channel precedence field is set to the value of channel precedence of the unified channel graph to which the MP interface belongs.

      - Path selection protocol identifier element

The path selection protocol identifier element is contained in the Active Protocol field. This information identifies the path selection protocol for unicast, multicast and broadcast transmission. Protocol profile has format shown in Figure s2.

| Octets: 3 | 1 |
| --- | --- |
| OUI | Path selection protocol identifier |

Figure s10: Path selection protocol identifier element format

The path selection protocol identifier specifies the protocol which is currently used to generate routing information in this network, as defined in Clause Error: Reference source not found.

Table s6: Path selection protocol identifier Values

| OUI | Value | Meaning |
| --- | --- | --- |
| 00-0F-AC | 0 | Hybrid Wireless Mesh Protocol (default path selection protocol) |
| 00-0F-AC | 1 | Radio Aware OLSR (optional path selection protocol) |
| 00-0F-AC | 2-254 | Reserved for future use |
| 00-0F-AC | 255 | Null protocol |
| Vendor OUI | Other | Vender specific |

A Null protocol indicates the MP has no active layer 2 path selection and forwarding. An MP with Null protocol will not send or respond to path selection protocol messages.

      - Path selection metric identifier element

The path selection metric identifier is contained in the Active Metric field. This information identifies the path metric which is currently used by the active path selection protocol in the WLAN mesh network. Path selection metric identifier has format shown in Figure s3.

| Octets: 3 | 1 |
| --- | --- |
| OUI | Path selection metric identifier |

Figure s11: Path selection metric identifier format

The path selection metric identifier specifies the metric that is used when selecting routes in this network, as defined in Table s3.

Table s7: Path selection metric identifier Values

| OUI | Value | Meaning |
| --- | --- | --- |
| 00-0F-AC | 0 | Airtime path metric (default path metric) |
| 00-0F-AC | 1-254 | Reserved for future use |
| 00-0F-AC | 255 | Null metric |
| Vendor OUI | Other | Vender specific |

Null metric is used in conjunction with Null protocol setting.

      - Active Profile Announcement element

The “Active Profile Announcement” element is used to notify the profile pair of the active path selection protocol and the active path metric to a peer MP. It is contained in association request message transmitted by the association requesting MP. The profile pair is selected by the association requesting MPs local mechanism.

| Octets: 1 | 1 | 1 | 4 | 4 |
| --- | --- | --- | --- | --- |
| ID | Length | Version | Active Protocol ID | Active Metric ID |

Figure s12: Active Profile Announcement Element

The fields contained in the element are as shown in Error: Reference source not found.

Table s8: Active Profile Announcement Element Values

| Field | Value/description |
| --- | --- |
| ID | T.B.D |
| Length | Variable |
| Version | 1 |
| Active Protocol ID | Path Selection Protocol in use |
| Active Metric ID | Path Selection Metric in use |

References:

- IEEE P802.11s/D1.00, November 2006

- doc.: IEEE 802.11-07/0250r1 Summary of Unresolved General Comments for 2/14 TGs Telecon

- doc.: IEEE 802.11-07/0023r13 Resolution of comments received during IEEE 802.11 Letter Ballot 93