IEEE P802.11
Wireless LANs

| GAS Comment Resolution to 802.11u-D0.03 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-02-20 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Dave Stephenson | Cisco Systems, Inc. | 170 W. Tasman Dr. San Jose, CA 95134 | +1 408 527 7991 | daves@cisco.com |
| Eleanor Hepworth | Siemens Roke Manor | Roke Manor, Romsey, UK, SO51 0ZN | +44 1794 833146 | eleanor.hepworth@roke.co.uk |

Abstract

This document is a work in progress and was prepared for the TGu Ad Hoc Meeting on February 20-22, 2007.  A final version of this document is being prepared which will be suitable for presentation to TGu and/or the WG.

This document describes proposed chnages to the GAS Protocol to resolve the following internal review comments (cf. 11-06-1857r6): CID # 147, 149, 150, 151, 153, 154, 156, 158-161, 163, 164, 166-169, 172, 173, 175, 176 and 253.

This document is based on P802.11u-d0.03 and P802.11ma-d9.

- Ele: build the entire GAS MIB: suggest to add GAS MIB to STAConfigTableEntry.

- Daves: change names of primitives so that there are just MLME-GAS.

- Daves: delete preferreddeliverymethod in gas primitive

- Daves: change advertising protocol IE so that there is only 1 bit for delivery method.

- daves: we need new status code stating advertising server is not reachable.

- Daves: describe in section 7.4.6.4 which IEs in GAS comeback response frame are included on fragmented response.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@ok-brit.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

  - Management frame body components

    - Fields that are not information elements

      - Status Code field

Insert the following to the contents of Table 23 as shown:

Table 23—Status Codes

| Status Code | Meaning |
| --- | --- |
| 52 | No outstanding GAS request |
| 53 | GAS Query Protocol(s) not supported |
| 54 | GAS Response not received from backend network |
| 55 | Requested delivery method not supported for this GAS Query protocol |
| 56 | GAS Query Response frame received from DS is larger than 802.11 MSDU size limit |
| 56 | GAS Query Response larger than permitted per configured AP policy |
| 57-65535 | Reserved |

    - Information elements

      - GAS Request Information element

The GAS Request Information element contains information about the details of the GAS request. The format of the GAS Request element is shown in Figure u1.

|  | Element ID | Length | Advertisement Protocol IE | Query |
| --- | --- | --- | --- | --- |
| Octets: | 1 | 2 | variable | variable |

Figure u1—GAS Request Information element format

The Element ID field is equal to the GAS Request value in Table 26.

The Length field is 2 octets long. The value of the Length field is variable and dependent on the contents of the Advertisment Protocol IE and Query.

The Advertisement Protocol IE is used by a STA to request a query using a specific advertising protocol.  This field is defined in clause Error: Reference source not found.

The actual format of the Query depends on the Advertisement Protocol ID value. For example, if the Advertisement Protocol ID value is 0 (see Error: Reference source not found), then the format is as per the IEEE 802.21 specification.

The GAS Request element is included in a GAS Initial Request action frame or a GAS Comeback Response frame..

      - GAS Response Information element

The GAS Response Information element contains information about the details of the advertisement response. The format of the GAS Response element is shown in Figure u2.

|  | Element ID | Length | Advertisement Protocol IE | Response Info |
| --- | --- | --- | --- | --- |
| Octets: | 1 | 2 | variable | Variable |

Figure u2—GAS Response Information element format

The Element ID field is equal to the GAS Response value in Table 26.

The Length field is 2 octets long. The value of the Length field is variable and dependent on the contents of the Advertisment Protocol IE and Response Info.

The Advertisement Protocol IE field is equal to Advertisement Protocol IE which was specified in the preceding request.

The Response info contains the result of the preceding query request. The actual format of the Response Info depends on the Advertisement Protocol ID value. For example, if the Advertisement Protocol ID value is 0 (see Error: Reference source not found), then the format is as per the IEEE 802.21 specification.

GAS Response Element is sent by the AP to the non-AP STA in the GAS Initial Response or GAS Comeback Response frame to provide the information about one or more requested network services.

- 10	Layer management

- 10.3	MLME SAP Interface

    - Network Selection Support

This set of primitives support the process of Generic Advertisement Services for network selection.

      - MLME-GAS-Request.request

        - Function

This primitive requests the information of a specific advertisement service from the network. It is valid at the non-AP STA and requests the AP to provide generic advertisement service.

        - Semantics of the service primitive

The primitive parameters are as follows:

MLME-GAS-Request.request (

AdvertisementProtocolID,

Query

QueryFailureTimeout

PreferredDeliveryMethod

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Dialog Token | Integer | 0 – 255 | The dialog token to identify the GAS request. |
| AdvertisementProtocolID | Integer | N/A | Identifies which protocol is used to format Query. This is either an 802.11 assigned Advertisement Protocol ID or a vendor-specific ID. |
| Query | String | N/A | Query string formatted using protocol identified in AdvertisementProtocolID. E.g., if the AdvertisementProtocolID value is 0, then Query is formatted as per the IEEE 802.21 specification. |
| QueryFailureTimeout | Integer | > 1 | The time limit, in units of beacon intervals, after which the GAS query procedure will be terminated. |
| PreferredDeliveryMethod | Enumeration | UNICAST, MULTICAST | The preferred GAS delivery method to use if both are supported by the AP. If only one method is supported by the AP, this input is ignored. |

        - When generated

This primitive is generated by the SME at a non-AP STA to request a specific advertisement service from the DS.

        - Effect of receipt

The non-AP STA operates according to the procedures defined in 11.10.1

      - MLME-GAS-Response.confirm

        - Function

This primitive reports the status code and query response to a GAS query of a specific advertisement service from the DS. It is valid at a non-AP STA

        - Semantics of the service primitive

The primitive parameters are as follows:

MLME-GAS-Response.confirm (

StatusCode,

ResponseInfo

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| StatusCode | Enumeration | GAS_Not_Supported, Requested_Info_Not_Available, Response_too_large | Indicates the result response to the GAS-request from the peer MAC entity. |
| DialogToken | Integer | 0 – 255 | The dialog token to identify the GAS request. |
| GASQueryID | Integer | As defined in frame format | Used to match the query and the response information. |
| GASComebackDelay | Integer | As defined in frame format | specifies the delay time to retrieve the response information |
| GASMulticastAddress | MACAddress | A multicast MAC address | Specifies the multicast MAC address to which the response info frame is addressed. A response info frame is an action frame with a query response. |
| AdvertisementProtocolID | Integer | N/A | Protocol ID identifying which protocol is used to format Query |
| ResponseInfo | String | N/A | Query response string formatted using protocol identified in AdvertisementProtocolID. E.g., if the AdvertisementProtocolID value is 0, then Query is formatted as per the IEEE 802.21 specification. |

        - When generated

This primitive is generated by the MLME as a response to the MLME-GASRequest.request primitive indicating the result of that request.

The primitive is generated when the non-AP STA receives a GAS Initial Response action frame or GAS Comeback Response action frame from the AP.

        - Effect of receipt

The non-AP STA operates according to the procedures defined in Error: Reference source not found.

      - MLME-GAS-Request.indication

        - Function

This primitive reports to the AP SME about the GAS Request.

        - Semantics of the service primitive

The primitive parameters are as follows:

MLME-GAS-Request.indication (

AdvertisementProtocolID,

Query,

Non-APSTAAddress

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| AdvertisementProtocolID | Integer | N/A | Identifies which protocol is used to format the Query. This is either a 802.11 assigned Advertisement Protocol ID or a vendor-specific ID. |
| Query | String | N/A | Query string formatted using protocol identified in AdvertisementProtocolID. E.g., if the AdvertisementProtocolID value is 0, then Query is formatted as per the IEEE 802.21 specification. |
| Non-APSTAAddress | MACAddress | Any valid individual MAC address | Specifies the address of the peer MAC entity from which query message is received. |

        - When generated

This primitive is generated by the MLME as a result of receipt of a request for the GAS advertisement service from a non-AP STA.

        - Effect of receipt

The AP is notified of the request from the non-AP STA.

The SME shall operate according to the procedures defined in Error: Reference source not found.

The SME generates an MLME-GAS-Response.response primitive within a dot11GASResponseTimeout.

      - MLME-GAS-Response.response

        - Function

This primitive responds to the request for an advertisement service by a specified non-AP STA MAC entity.

        - Semantics of the service primitive

The primitive parameters are as follows:

MLME-GAS-Response.response (

StatusCode,

NonAPSTAAddress,

ResponseInfo

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| StatusCode | Enumeration | GAS_Not_Supported Requested_Info_Not_Available Response_too_large | Indicates the result response to the GAS-request from the peer MAC entity. |
| Non-APSTAAddress | MACAddress | Any valid individual MAC address | Specifies the address of the peer MAC entity to which query response information is transmitted. |
| GASQueryID | Integer | As defined in frame format | Used to match the query and the response information. |
| GASComebackDelay | Integer | As defined in frame format | specifies the delay time to retrieve the response information |
| GASMulticastAddress | MACAddress | A multicast MAC address | Specifies the multicast MAC address to which the response info frame is addressed. A response info frame is a data frame with a query response. |
| AdvertisementProtocolID | Integer | N/A | Identifies which protocol is used to format the Query. This is either an 802.11 assigned Advertisement Protocol ID or a vendor-specific ID. |
| ResponseInfo | String | N/A | Query response string formatted using protocol identified in AdvertisementProtocolID. E.g., if the AdvertisementProtocolID value is 0, then Query is formatted as per the IEEE 802.21 specification. |

        - When generated

This primitive is generated by the SME at the AP as a result of an MLME-GAS-Request.indication primitive.

        - Effect of receipt

The primitive causes the MAC entity at the AP to send a GAS Initial Response frame in the corresponding GAS response action management frame to the non-AP STA.

      - MLME-GAS-Comeback-Request.request

        - Function

This primitive requests the information of a specific advertisement service from the network in a comeback request. It is valid at the non-AP STA and requests the AP.

        - Semantics of the service primitive

The primitive parameters are as follows:

MLME-GAS-Comeback-Request.request (

GASQueryID

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| GASQueryID | Integer | As defined in frame format | Used to match the query and the response information. |

        - When generated

This primitive is generated by the SME at a non-AP STA to request a specific GAS advertisement service from the network side using a comeback request.

        - Effect of receipt

The non-AP STA operates according to the procedures defined in Error: Reference source not found

      - MLME-GAS-Comeback-Response.confirm

        - Function

This primitive reports the status code of the GAS comeback request with an optional query ID, comeback delay information and the response elements, if available at that point.

        - Semantics of the service primitive

The primitive parameters are as follows:

MLME-GAS-Comeback-Request.confirm (

StatusCode,

GASQueryID,

GASComebackDelay,

AdvertisementProtocolID,

ResponseInfo

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| StatusCode | Enumeration | NO_Outstanding_GAS_Request_Available GAS_Not_Supported GAS_Response_Not_Received Requested_Delivery_Method_Not_Supported Requested_Info_Not_Available GAS Query Response frame received from DS is larger than 802.11 MSDU size limit GAS Query Response larger than permitted per configured AP policy | Indicates the result response to the GAS-Comeback-request from the peer MAC entity. |
| GASQueryID | Integer | As defined in frame format | Used to match the query and the response information. |
| GASComebackDelay | Integer | As defined in frame format | specifies the delay time to retrieve the response information |
| AdvertisementProtocolID | Integer | N/A | Identifies which protocol is used to format the Query. This is either a 802.11 assigned Advertisement Protocol ID or a vendor-specific ID. |
| ResponseInfo | String | N/A | Query response string formatted using protocol identified in AdvertisementProtocolID. E.g., if the AdvertisementProtocolID value is 0, then Query is formatted as per the IEEE 802.21 specification. |

        - When generated

This primitive is generated by the MLME as a result of an MLME-GAS-Comeback-Request.request primitive indicating the result of that request.

The primitive is generated when the non-AP STA receives a response in the form of a GAS Comeback Request frame in the corresponding action frame from the AP.

        - Effect of receipt

The non-AP STA operates according to the procedures defined in Error: Reference source not found

      - MLME-GAS-Comeback-Request.indication

        - Function

This primitive reports to the AP SME the request GAS Comeback-Request.

        - Semantics of the service primitive

The primitive parameters are as follows:

MLME-GAS-Comeback-Request.indication (

GASQueryID,

Non-APSTAAddress

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| GASQueryID | Integer | As defined in frame format | Used to match the query and the response information. |

        - When generated

This primitive is generated by the MLME as a result of receipt of a request for the advertisement service from a non-AP STA.

        - Effect of receipt

The AP is notified of the request from the non-AP STA.

The SME should operate according to the procedures defined in Error: Reference source not found.

The SME generates an MLME-GAS-Comeback-Response.response primitive within a dot11GASResponseTimeout.

      - MLME-GAS-Comeback-Response.response

        - Function

This primitive responds to the request for an advertisement service by a specified non-AP STA MAC entity.

        - Semantics of the service primitive

The primitive parameters are as follows:

MLME-GAS-Comeback-Request.response (

StatusCode,

Non-APSTA Address,

GASQueryID,

GASComebackDelay,

AdvertisementProtocolID,

ResponseInfo

)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| StatusCode | Enumeration | NO_Outstanding_GAS_Request_Available GAS_Not_Supported GAS_Response_Not_Received Requested_Delivery_Method_Not_Supported Requested_Info_Not_Available GAS Query Response frame received from DS is larger than 802.11 MSDU size limit GAS Query Response larger than permitted per configured AP policy | Indicates the result response to the GAS-Comeback-request from the peer MAC entity. |
| Non-APSTAAddress | MACAddress | Any valid individual MAC address | Specifies the address of the peer MAC entity to which query response information is transmitted. |
| GASQueryID | Integer | As defined in frame format | Used to match the query and the response information. |
| GASComebackDelay | Integer | As defined in frame format | specifies the delay time to retrieve the response information |
| AdvertisementProtocolID | Integer | N/A | identifies which protocol is used to format Query |
| ResponseInfo | String | N/A | Query response string formatted using protocol identified in AdvertisementProtocolID. E.g., if the AdvertisementProtocolID value is 0, then Query is formatted as per the IEEE 802.21 specification. |

        - When generated

This primitive is generated by the SME at the AP as a result of an MLME-GAS-Comeback-Request.indication primitive.

        - Effect of receipt

The primitive causes the MAC entity at the AP to send a GAS Comeback-response frame in the corresponding GAS Comeback-request action management frame to the non-AP