IEEE P802.11
Wireless LANs

| Normative Text for Presence Advertisement for e911 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-05-14 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Allan Thomson | Cisco Systems | 170 W. Tasman Drive, San Jose, CA 95134 | +1-408-853-5570 | allant@cisco.com |
| Dave Stephenson | Cisco Systems | 170 W. Tasman Drive, San Jose, CA 95134 | +1-408-527-7991 | daves@cisco.com |
| Hong Cheng | Panasonic Singapore Laboratories | BLK1022 TaiSeng Ave #06-3530 TaiSeng Ind. Est. Singapore 534415 | +65 65505477 | hong.cheng@sg.panasonic.com |
| Amy Zhang | Huawei Technologies | Bantian, Longgang District Shenzhen 518129,P.R.China | +86 755 896 50361 | amyzhang@huawei.com |
| Necati Canpolat | Intel Corporation | 2111 NE 25th Ave. Hillsboro, OR 97124 | +1 503 264 8014 | necati.canpolat@intel.com |
| Angelo Centonza | Siemens Roke Manor | Roke Manor, Romsey, UK, SO51 0ZN | +44 1794 833000 | angelo.centonza@roke.co.uk |

Abstract

Currently, both TGv (and TGk) can advertise location capability, but a non-AP STA cannot count on this advertisement as an indication that the AP will provide a location service.  In both TGv and TGk, the STA would have to associate, request location services for local and/or remote and could then be told the AP is incapable. This is a problem for e911 because of the time involved to go through all these steps.  A better way is if STA can determine whether location service will be provided before associating to the AP. If it can't get this service, then STA could look for this service on another AP.

This submission contains the normative text changes necessary to clarify the rules when an AP advertises presence capability to provide for the TGu emergency-services requirement that an AP that advertises location presence capability can provide location of itself to associated non-AP STAs..

The text is aligned with P802.11v-D0.11.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart@ok-brit.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

[Make the following changes to 7.3.2.49]

Insert the following new rows in Table 4:

- Table 4—The Wireless Network Management Capabilities field

| Bit | Information | Notes |
| --- | --- | --- |
| 7 | E911 CIVIC Location | - The E911 CIVIC bit is set to 0 and ignored when the Presence bit is set to 0. The E911 CIVIC bit set to 1 indicates that the AP has its location defined in CIVIC format, as described by the Location Descriptor sub-element in 7.3.2.54.9, and will provide it to the non-AP STA upon request. The E911 CIVIC bit set to 0 indicates that the AP does not have its location defined in CIVIC format or is not capable to provide CIVIC format location to the non-AP upon request. |
| 8 | E911 Geo Location | - The E911 Geo bit is set to 0 and ignored when the Presence bit is set to 0. The E911 Geo bit set to 1 indicates that the AP has its location defined in Geo format, as described by the Location Descriptor sub-element in 7.3.2.54.9 and will provide it to the non-AP STA upon request. The E911 Geo bit set to 0 indicates the AP does not have its location defined in Geo format or is not capable to provide Geo format location to the non-AP upon request |
| 9-n*8 | Reserved | - All other bits are reserved, and are set to 0 on transmission and ignored on reception. |

The lack of a Wireless Network Management Capability element is interpreted as the STA having no advertised Wireless Network Management Capabilities.

- 11.15.3 Presence advertising procedures

Insert the following text in Section 11.15.3before the first paragraph.

An AP that advertises presence capabilities shall set the Presence Bit to 1 in the Wireless Network Management Capabilities IE. If the Presence bit is set to 0 then no location services are provided by the AP. Whenever the presence bit is set to 1 an AP may also follow these procedures:

- If the AP has its location configured in CIVIC format, as described by the Location Descriptor sub-element in section 7.3.2.54.9 then the AP shall set to the E911 CIVIC bit to 1.

- If the AP has its location configured in Geo format,as described by the Location Descriptor sub-element in section 7.3.2.54.9 then the AP shall set to the E911 Geo bit to 1.

A STA may advertise its location information to a peer STA. Advertisement is accomplished by including a Presence Parameters information element in either a Beacon, Association Request, Reassociation Request, Presence Request, Presence Configuration Request, Probe Response or Presence Response frame. For the case of a Presence Response frame the STA shall set the dialog token field to zero to indicate this frame is not in response to a corresponding Presence Request frame. To advertise, the STA shall include both the Location Descriptor and Location Data sub-elements in the Presence Parameters information element. The STA shall define the Location Descriptor fields as follows. The Location Descriptor shall indicate Local. The Location Format Descriptor, Resolution Descriptor and Encoding Descriptor shall all be set to match the data contained in the Location Data sub-element. Depending on the options defined in the Location Descriptor, the STA may also include the Location Source Identifier sub-element.