IEEE P802.11
Wireless LANs

| FBMS Counter ID |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-03-12 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Jiyoung Huh | LG Electronics | 16 Woomyeon-Dong, Seocho-Gu, Seoul 137-724, Korea | +82-2-526-4151 | jyhuh@lge.com |

Abstract

This submission contains the normative text related to broadcast and multicast enhancements (Req. #2120).  Specifically the intention is to improve terminals power efficiency (Req. #2010). Furthermore it contains aspects related to advertisements (Req. #1300).

The text is aligned with P802.11v-D0.08.

This document contains normative text to make the FBMS Status Sub-element more efficient by using the FBMS counter field instead of FBMS ID.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart@ok-brit.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Change 7.3.2 60  as shown:

- 7.3.2.60   FBMS Response element

The FBMS Response defines information about the broadcast/multicast status. The format of the FBMS Response element is shown in .

|  |  |  | One or more FBMS Status Sub-elements |
| --- | --- | --- | --- |
|  | Element ID | Length | FBMS Status Sub-elements |
| Octets: | 1 | 1 | variable |

- Figure v81—FBMS Response element format

The Length field is set to the total length of all FBMS Status Sub-elements contained in the element.

The FBMS Status Sub-elements field contains one ore more FBMS Status sub-elements. The format of the FBMS Status sub-element is shown in .

|  | Element Status | Delivery Interval | FBMSID | FBMS Counter ID | Multicast Rate | Multicast Address | Multicast Diagnostic Interval |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 1 | 1 | 6 | 4 |

- Figure v82—FBMS Status Sub-element format

The Element Status field indicates the status of the AP responding to the STA's requested delivery interval, as inidcated in Error: Reference source not found.

- Table v47—Element Status Definition

| Value | Description |
| --- | --- |
| 1 | Accept |
| 1 | Deny, due to malformed request or ambiguous classifier. |
| 2 | Deny, due to lack of resources on AP. |
| 3 | Deny, due to requested classifier(s) matching 2 or more existing streams on different intervals |
| 4 | Deny, by policy, requested stream is not permitted to participate in FBMS |
| 5 | Override, due to existing stream with different delivery interval |
| 6 | Override, due to policy limits on AP. |
| 7 | Override, due to AP changed the delivery interval. |
| 8 | Override, due to AP multicast rate policy |
| 9-255 | Reserved |

The Delivery Interval field defines the number of DTIMs that the stream is transmitted as defined by the AP.

The FBMSID field is assigned by the AP and provides a unique identifier for this stream within the BSS.

The FBMSID Counter ID field provides a unique identifier for this stream counter within the BSS. The format of the FBMS Counter is shown in Figure v78.

The Multicast Rate field specifies the data rate to be used for the multicast service. If the value of the Multicast Rate field is set to 0 then the data rate is undefined.

The Multicast Address field specifies the multicast MAC address for the multicast service.

The Multicast Diagnostic Interval field specifies the number of beacon intervals for which the AP keeps multicast service traffic counts. The STA uses the Multicast Diagnostic Interval to determine how frequently to send Multicast Diagnostic Reports.