IEEE P802.11
Wireless LANs

| Emergency Alert System (EAS) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-03-13 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Eleanor Hepworth | Siemens Roke Manor | Roke Manor, Romsey, UK, SO51 0ZN | +44 1794 833146 | eleanor.hepworth@roke.co.uk |
| Angelo Centonza | Siemens Roke Manor | Roke Manor, Romsey, UK, SO51 0ZN | +44 1794 833000 | angelo.centonza@roke.co.uk |

Abstract

This document describes the addition of an Emergency Alert System (EAS) notification functionality to the TGu draft.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart@ok-brit.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

# Background

“One of the most fundamental and significant statutory mandates of the Federal Communications Commission (Commission) is the promotion of safety of life and property through the use of wire and radio communication.

For over forty years, the Commission has sought to satisfy this mandate in large part by requiring that the American public be provided with an effective and robust national alert and warning system.  Since 1994, this function has been performed by the Emergency Alert System (EAS), which is jointly administered by the Commission, the Federal Emergency Management Agency (FEMA), one of the component agencies of the Department of Homeland Security (DHS), and the Department of Commerce and its component, the National Oceanic and Atmospheric Administration's National Weather Service (NWS)”

“Review of the Emergency Alert System”, EB Docket No. 04-296, Federal Communications Commission, Washington D.C. 20554, November 10, 2005

# Introduction

This document suggests an EAS notification functionality for IEEE 802.11 and recommends its placement within the Emergency Services section (clause 7.3.2.36) of the TGu draft specification document.

- Abbreviations and acronyms

Add the following acronyms:

EAS	Emergency Alert System

EASN	Emergency Alert System Notification

Change the following clauses as shown:

  - Management frame body components

    - Information elements

      - Interworking Capability Information Element

The Interworking Capability Information element contains information about the interworking service capabilities of a STA as shown in Figure u1.

|  | Element ID | Length | Interworking Capabilities |
| --- | --- | --- | --- |
| Octets: | 1 | 1 | 2 |

Figure u1—Interworking Capability Information element format

The Element ID field is equal to the Interworking Capability value in Table 26.

The Length field is the length of the Interworking Capabilities field. The value of the Length field is 2.

The Interworking Capabilities field is a bit field indicating the advertised interworking capabilities of the STA. The Interworking Capabilities field is shown in Figure u2.

|  | B0 | B1 | B2 | B3 |  |  | B4 B15 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | QoS Map | Expedited Bandwidth Request | Emergency Services Only | Emergency Alert System Notification |  |  | Reserved |
| Bits: | 1 | 1 | 1 | 1 |  |  | 13 |

Figure u2—Interworking Capabilities field

- The QoS Map Service capability bit set to 1 indicates the AP supports QoS Map Service.  The QoS Map Service capability bit set to 0 indicates the STA does not support QoS Map Service.

- The Expedited Bandwidth Request Service capability bit set to 1 indicates that the STA supports Expedited Bandwidth Request Service.  The Expedited Bandwidth Request Service capability bit set to 0 indicates the STA does not support Expedited Bandwidth Request Service.

- The ESO (Emergency Services Only on this SSID) capability set to 1 indicates higher layer emergency call services are reachable via this SSID.  The ESO capability set to 0 indicates other means are necessary to determine if higher layer emergency call services are reachable via this SSID.  This bit shall only be set to 1 if all the following conditions are true (cf. Annex P.1 for further information):

- SSID is configured for open authentication and used exclusively for providing emergency services

- dot11QosOptionImplemented is true

- Expedited Bandwidth Request Service capability bit is set to 1

- Location information capability is enabled on AP if AP is located in a regulatory domain that requires this.

- DS supports end-to-end QoS from STA to all APs in ESS to PSAP

The Emergency Alert System Notification (EASN) capability bit set to 1 indicates that higher layer EAS is in operation.  The EASN capability bit set to 0 indicates that no higher layer EAS is in operation.  The setting of this bit may then require an STA to request further information form the higher layers to received the full EAS information.

- All other bits are reserved and shall be set to 0 on transmission and ignored on reception

The lack of an Interworking Capability element shall be interpreted as the STA having no advertised Interworking Capabilities.

#### 7.3.2.37 Advertising Protocol Information element

The Advertising Protocol Information element contains information which identifies a particular advertisement protocol and its delivery method(s) as shown in Figure u4a.

|  | Element ID | Length | Delivery Method | Advertisement Protocol ID |
| --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | variable |

Figure u4a— Advertisement Protocol Information element format

The Element ID field is equal to the Advertisement Protocol value in Table 26 or the Vendor Specific Element ID.

The Length field is the length of the Advertisement Protocol Information element. The Length field is determined by the Advertisement Protocol ID.

The Delivery Method field is a 1-octet field which specifies multicast or unicast delivery method.  The format of this field is shown in Figure u4b.

The Advertisement Protocol ID is a variable length field. If the first octet of this field is the vendor specific Advertising Protocol ID as provided in Table u1a, then this field contains the Vendor Specific Information Element per clause 7.3.2.26.  If this first octet of this field is not the vendor specific Advertising Protocol ID, then its length is 1 octet and its value is per Table u1a.

|  | B0 | B1 | B2 | B7 |
| --- | --- | --- | --- | --- |
|  | Multicast Delivery | Unicast Delivery | Reserved |  |
| Bits: | 1 | 1 | 6 |  |

Figure u4b— Delivery Method field

A STA using the Advertisement Protocol Information element requests the desired delivery method of advertisement protocol by setting either Bit0 or Bit1 as appropriate.

- Bit 0 is set to 1 if the STA supports multicast delivery as described in clause11.11.1.  Bit 0 is set to 0 if the STA does not support multicast delivery for this Advertisement Protocol IE.

- Bit 1 is set to 1 if the STA supports unicast delivery as described in clause 11.11.2.  Bit 1 is set to 1 if the STA does not support unicast delivery for this Advertisement Protocol IE.

The STA shall either support multicast delivery or unicast delivery.  Optionally, an STA may support both delivery methods.

Note : Also require some MIB entries??

Table u1—Advertisement Protocol ID definitions

| Name | Value |
| --- | --- |
| MIH Information Service | 0 |
| MIH Command and Event Services Capability Discovery | 1 |
| Emergency Alert System (EAS) Service | 2 |
| Reserved | 3-220 |
| Vendor Specific | 221 |
| Reserved | 222 – 255 |

  - MIH Information Service in Table u1 is a mechanism defined in IEEE 802.21 to support information retrieval from an information repository in the DS.

  - MIH Command and Event Services capability discovery is a mechanism defined in IEEE 802.21 to support discovering capabilities of command service and event service entities in the DS.

  - Emergency Alert System (EAS) service is a mechanism which allows a network to disseminate EAS notifications to unauthenticated and unassciated IEEE 802.11 STAs.

  - Advertisement Protocol ID is reserved for a Vendor Specific protocol which shall have the format defined in clause 7.3.2.26.

  - Note: after approval of this amendment, Advertisement Protocol IDs will be managed by 802.11 Assigned Numbers Authority.