IEEE P802.11
Wireless LANs

| Proposal for resolving CIDs 238 and 242 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-03-9 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Neeraj K. Sharma | Intel | 2111 NE 25th Avenue, Hillsboro, OR 97124 | 503-712-7458 | neeraj.k.sharma@intel.com |

Abstract

This document contains the proposal for resolution of some comments to draft P802.11.2-D0.12. The comments referenced to are listed in the document IEEE 802.11-06-0872r24 and the comments addressed are 238 and 242.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart@ok-brit.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

# Resolution for Comments [2] ID# 238 and 242

5.1 Overview

Add the following text and Table 1 at the end of this clause:

For over the air test methodologies c) to f), Table 1 describes the test parameters that are common to all these environments, as well as the corresponding requirements for their values. Other test parameters specific to a test methodology or deviation from Table 1 will be discussed in the relevant clauses.

Table 1—Test parameters for over the air test environments

| Item | Test parameter | Requirement | Applicable figure |
| --- | --- | --- | --- |
| 1 | endstation test height | The endstation should be positioned parallel to and 80 cm ± 1.5 cm from the ground/floor, as measured from the ground/floor surface to the base of the device. This requirement is intended to approximate a ’typical’ use height. | Figure 8 |
| 2 | endstation lid/LCD angle | The endstation’s lid/LCD should be positioned such that it is at 110 degrees ± 1 degree relative to the endstation’s base which is placed on a level ground/floor. This requirement is intended to approximate a nominal LCD viewing angle. An inclinometer, other angle measurement device, and/or a test jig should be utilized to ensure adherence to this requirement. This requirement is applicable only to devices that have an integral display whose viewing angle is adjustable. | Figure 9 |
| 3 | DUT X-Y placement accuracy | The DUT should be placed in test locations such that its positioning is repeatable run-to-run/day-to-day within an area represented by a circle centered at the intended test point, and with a radius of 1.5cm (for 2.4 GHz testing) and 0.75cm (for 5 GHz testing), as measured from a consistent reference point on the DUT base (e.g., back left hand corner of the DUT). This requirement is intended to keep the DUT positioned within less than ¼ wave length run-to-run/day-to-day as one of the factors necessary in presenting a consistent multipath condition to the device. Location marking, fixturing, or some other robust method should be utilized to ensure adherence to this requirement. | Figure 10 |
| 4 | endstation spinning | The endstation should be spun at all test locations. The rate of rotation should be 1 RPM clockwise, and each trial should include a number of complete turns of the endstation. Large devices may be spun around their geometric center point, as long as the antenna is not at the center point. Small devices should be placed on the edge of a turntable. The radius of the arc of travel of the antenna of the endstation while on the turntable should be at least 15 cm. |  |
| 5 | AP test height and antenna orientation | APs used for performance testing should be placed as shown in Figure 11 with the base of the antennas at a fixed height h above the ground/floor. The AP height is set according to the test purposes, and must be recorded with the results. This requirement is intended to define a consistent antenna orientation and location. Fixturing is recommended to provide consistent AP placement. | Figure 11 |
| 6 | Distance between DUT and WLCP | The DUT should be placed at a maximal distance of 300 meters from the WLCP, with the test area marked at x meter increments (0m, 25m, 50m, etc.). This requirement is intended to enable throughput measurements to be easily made at repeatable locations. | Figure 12 |
| 7 6 | Co-channel noise | Except for specific cases of noise rejection and adjacent channel interference testing, all performance testing should be performed with co-channel noise less than or equal to -100 dBm from any 802.11 or non-802.11 source, over the occupied 3-dB bandwidth of the modulation of interest (16.6 MHz for OFDM, 22 MHz for DSSS-CCK). This requirement is intended to provide an co-channel quiet RF environment for the DUT. It should be noted that non-co-channel RF sources are acceptable, to allow for activity such as simultaneous 802.11a and 802.11b/g testing. Active monitoring using a spectrum analyzer or other device should be employed prior to testing and, to the extent possible, during testing, to ensure that this requirement is met. |  |
| 8 7 | Consistent test equipment environment | All test equipment (i.e., WLCPs, DUT platforms, etc.) should remain identical run-to-run for a given test, in terms of the actual physical devices as well as their ‘soft’ attributes, including but not limited to: firmware, drivers, O/S images, configuration settings, etc. The intent of this requirement is to provide consistent test conditions run-to-run/day-to-day for a given test, to ensure that test data can be compared. |  |
| 9 8 | Climate | Outdoor: There are no climate criteria established for outdoor testing, although air temperature and relative humidity should be recorded. The condition of the ground between the DUT and the WLCP should be recorded. Indoor: The climate of the test environment should be set as follows: fixed air temperature (e.g. 22° C ± 2° C), monitored or assured with adequate environmental control. The relative humidity should be recorded. The intent of this requirement is to establish a baseline criterion for the purpose of consistency. |  |
| 10 | Test range lock-down | During testing, people on the test range should be kept to a minimum, and no person should be located within 50 meters of the line of sight between the DUT and the WLCP. The intent of this requirement is to maintain a consistent multipath environment for the DUT and WLCP at any given test location, by fixing the position of any RF reflecting or absorbing objects. |  |

5.4.1.2.3 Test parameters

Replace:

Table 1 describes the test parameters relevant to this environment, as well as the corresponding requirements for their values.

With

Table 2 describes additional test parameters relevant to this environment and the corresponding requirements for their values.

Replace Table 1 with Table 2 below:

Table 2-Additional outdoor LOS test parameters for PC external antenna model

| Item | Test parameter | Requirement | Applicable figure |
| --- | --- | --- | --- |
| 9 | Distance between DUT and WLCP | The DUT should be placed at a maximal distance of 300 meters from the WLCP, with the test area marked at x meter increments (0m, 25m, 50m, etc.). This requirement is intended to enable throughput measurements to be easily made at repeatable locations. | 12 |
| 10 | Test range lock-down | During testing, people on the test range should be kept to a minimum, and no person should be located within 50 meters of the line of sight between the DUT and the WLCP. The intent of this requirement is to maintain a consistent multipath environment for the DUT and WLCP at any given test location, by fixing the position of any RF reflecting or absorbing objects. |  |

5.4.2 PDA and handheld model

5.4.2.1 Introduction and purpose

Change:

The procedures for configuring, setting up and preparing the environment for testing PDAs and handheld

devices according to this model should generally follow those described in 5.4.1, with modifications to test items the exception of specific test parameters described in items 1, 2 and 5 2 of 5.4.1.2.3 and a new test parameter (item 11), as noted below in 5.4.2.2.3.

Replace Table 2 with Table below.

Table 3-Additional outdoor LOS test parameters for PDA and handheld device model

| Item | Test parameter | Requirement | Applicable figure |
| --- | --- | --- | --- |
| 1 | endstation test height | The endstation should be positioned parallel to and 125 cm ± 1.5 cm from the ground, as measured from the ground surface to the base of the device. This requirement is intended to approximate a ’typical’ use height. | Figure 8 |
| 2 | endstation lid/LCD angle | The endstation’s lid/LCD should be positioned such that it is at 60 110 degrees ± 1 degree relative to a level ground. This requirement is intended to approximate a nominal LCD viewing angle. An inclinometer, other angle measurement device, and/or a test jig should be utilized to ensure adherence to this requirement. This requirement is applicable only to devices that have an integral display whose viewing angle is adjustable. |  |
| 11 | DUT mode of operation | The two modes of operation of the DUT are portrait and landscape. These two modes are to be tested across the 300 meter test site. Testing should be carried out using both the modes and results reported. |  |

5.5.1.2.3 Test parameters

Replace:

Table 3 describes the test parameters relevant to this environment, as well as the corresponding requirements for their values.

With

Table 4 describes additional test parameters relevant to this environment and the corresponding requirements for their values.

Replace Table 3 with Table 4 below:

Table 4—Additional Indoor NLOS test parameters for PC external antenna model

| Item | Test parameter | Requirement | Applicable figure |
| --- | --- | --- | --- |
| 9 | Location of DUT and WLCP | The DUT should be placed at locations that are chosen to gradually increase in distance from the WLCP, with varying number of obstructions. The signal strength reported by the DUT can be used to identify suitable locations for testing. This requirement is intended to enable performance measurements to be easily made at repeatable locations. |  |
| 10 | Test range lock-down | Elements of the test range, including items such as furniture, hard or soft walls, doors, and window coverings should be held in fixed static positions during testing. In cases where items such as furniture may need to be moved from time to time, their ‘test position’ should be marked, and they should be moved to these marked positions prior to the start of any performance testing. During testing, people on the test range should be kept to a minimum, and no person should be within 50 meters of the DUT or WLCP. The intent of these two requirements is to maintain a consistent multipath environment for the DUT and WLCP at any given test location, by fixing the position of any RF reflecting or absorbing objects. |  |

5.5.2 PDA and handheld model

5.5.2.1 Introduction and purpose

Change:

The procedures for configuring, setting up and preparing the environment for testing PDAs and handheld

devices according to this model should generally follow those described in 5.5.1, with modifications to test the exception of specific test parameters described in items 1, 2 and 5 2 of 5.5.1.2.3, and a new test parameter (item 11), as noted below in 5.5.2.2.3.

Replace Table 4 with Table 5 below:

Table 5—Indoor NLOS test parameters for PDA and handheld model

| Item | Test parameter | Requirement | Applicable figure |
| --- | --- | --- | --- |
| 1 | endstation test height | The endstation should be positioned parallel to and 125 cm ± 1.5 cm from the ground, as measured from the ground surface to the base of the device. This requirement is intended to approximate a ’typical’ use height. | Figure 8 |
| 2 | endstation lid/LCD angle | The endstation’s lid/LCD should be positioned such that it is at 60 110 degrees ± 1 degree relative to a level ground. This requirement is intended to approximate a nominal LCD viewing angle. An inclinometer, other angle measurement device, and/or a test jig should be utilized to ensure adherence to this requirement. This requirement is applicable only to devices that have an integral display whose viewing angle is adjustable. |  |
| 11 | DUT mode of operation | The two modes of operation of the DUT are portrait and landscape. These two modes are to be tested across the 300 meter test site. Testing should be carried out using both the modes and results reported. |  |

5.6.2.3 Test parameters

Change:

The test parameters for this setup are the same as those of the OTA indoor NLOS environment, as already

described in Table 3 of 5.5, with the following change to item 6, as shown in Table 5. Tables 4 and 5 for PC external antenna and PDA and handheld models, respectively, with the following change to item 6 9, as shown in Table 5 6. The figure reference within the table is intended to illustrate the geometry corresponding to the specification of the test parameter.

Replace Table 5 with Table 6 below:

Table 6—Indoor LOS test parameters

| Item | Test parameter | Requirement | Applicable figure |
| --- | --- | --- | --- |
| 9 | Location of DUT and WLCP | The range should allow placing the DUT at a distance of at least 70 meters from the WLCP, with the test area marked at x meter increments. This requirement is intended to enable throughput measurements to be easily made at repeatable locations. | Figure 14 |

5.7.2.3 Test parameters

Change:

Table 6 describes the test parameters relevant to this environment, Table 7 describes the modifications to test parameters (items 1 and 5) in Table 1 in clause 5.1 and an additional test parameter  relevant to this environment (item 9), as well as the corresponding requirements for their values. Figure references within the table, when provided, are intended to illustrate the geometries corresponding to the specification of individual test parameters.

Table 6 7—OTA shielded enclosure test parameters

| Item | Test parameter | Requirement | Applicable figure |
| --- | --- | --- | --- |
| 1 | endstation test height | The endstation should be positioned parallel to and 80 cm ± 1 cm from the floor, as measured from the floor surface to the base of the device. This requirement is intended to approximate a ‘typical’ use height. This height may vary depending on the construction of the shielded enclosure. It must be ensured that the DUT lies in the quiet zone of the shielded enclosure. | Figure 8 |
| 5 | External antenna height and orientation | The external antenna in the shielded enclosure used for performance testing should be placed such that the antenna is pointing upwards and perpendicular to the floor with the base of the antenna at a fixed height (e.g. 275 cm ± 1.5 cm) above the floor. This requirement is intended to define a consistent antenna orientation. Fixtures are recommended to provide consistent antenna placement. If it is necessary to simulate the placement of an AP on a ceiling, this requirement should be changed to position the antenna pointing downwards rather than upwards. |  |
| 9 | Location of DUT and external antenna | The location of the DUT with respect to the external antenna should be carefully calibrated and noted. This distance should not change while the level of attenuation is varied. | Figure 14 |

# References

- P802.11.2-D0.12. Draft Recommended Practice for the Evaluation of 802.11 Wireless Performance.

- IEEE 802.11-06-0872r24. Comments from TGT Internal Review.