IEEE P802.11
Wireless LANs

| 802.11n LB97 Submission: Table to Clarify the Usage of the “Not Sounding” bit, “Smoothing”bit, and “HT-Length” field of the TXVECTOR and HT SIGNAL field |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-03-08 |  |  |  |  |
| Author(s): George Vlantis |  |  |  |  |
| Name | Company | Address | Phone | email |
| George Vlantis | STMicroelectronics, Inc. | 1060 East Brokaw Road, San Jose, CA 95131 USA | +1.408.451.8109 | George.Vlantis@st.com |
| Pratima Pai | STMicroelectronics, Ltd. | 1000 Aztec West, Almondsbury, Bristol BS32 4SQ – U.K. | +44.145.446.2412 | Pratima.Pai@st.com |

Abstract

This submission has been uploaded as a contribution to facilitate discussion of a particular comment made by the author during Letter Ballot 97 (LB97) on IEEE Draft P802.11n_D2.0.pdf.  At this writing (Revision 0), the LB97 voting deadline has not arrived, and therefore a Comment ID for the comment is not available.

In Draft 2.0, the “Not Sounding” and “Smoothing” bits plus either the zero value or non-zero value of the “HT-Length” field indicate different behaviors both at the transmitter (through the TXVECTOR mechanism) in Subclause 20.3.2; Table n56, page 224, lines 10 to 48 and at the receiver via the HT-SIGNAL field of the preamble, specified in Subclause 20.3.9.4.3, Table n63, page 250, lines 12 to 21).

The author’s comment asks whether the interpretation expressed in the “Applicable” field in the Table provided below is correct for all 8 combinations of the 2 bits and the zero or non-zero state of the “HT-Length” field.  Once the correct interpretation has been ascertained, the author suggests that the table could be incorporated into the draft as an ancillary table to clarify the interpretation of these 8 combinations for the reader.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart@ok-brit.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Please confirm the following use cases

Table nXX – Applicability of TXVECTOR, RXVECTOR, and HT-SIGNAL Fields

| Not Sounding | HT-Length | Smoothing | Description | Applicable |
| --- | --- | --- | --- | --- |
| 1 | 0 | 0 | Not-sounding, Null-Data Packet where Smoothing is not recommended | No, undefined |
| 1 | 0 | 1 | Not-sounding, Null-Data Packet where smoothing is recommended | No, undefined |
| 1 | Nonzero | 0 | Not-sounding, Normal Data Packet where Smoothing is not recommended | Yes, defined |
| 1 | Nonzero | 1 | Not-sounding, Normal Data Packet where Smoothing is recommended | Yes, defined |
| 0 | 0 | 0 | Sounding, Null-Data Packet where Smoothing is not recommended | Yes, optional |
| 0 | 0 | 1 | Sounding, Null-Data Packet where smoothing is recommended | Yes, optional |
| 0 | Nonzero | 0 | Sounding + Normal Data Packet where Smoothing is not recommended | Yes, optional |
| 0 | Nonzero | 1 | Sounding + Normal Data Packet where Smoothing is recommended | Yes, optional |

- The author asks to confim the correctness of the table.  The rows that are in bold text should be more closely scrutinized.

- Once the correct table has been ascertained, the author suggests that the table be incorporated into the draft to provide clarity to the reader.

References: