IEEE P802.11
Wireless LANs

| Suggested comment resolution on ATIM window parameter |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-03-09 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | Email |
| Kazuyuki Sakoda | Sony Corporation | 2-17-1 Higashi-gotanda, Shinagawa-ku, Tokyo, 141-0022 Japan | +81-3-6409-3201 | sako@wcs.sony.co.jp |

Abstract

This document provides the suggested updates to the current draft spec, which potentially resolve CID 1486, 1997, 3772, 3777, and 3781.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Summary of CIDs labelled as ATIM issues

Following table is captured from [2].

| CID | Resolution Status | Comment | Proposed Change by Commenter | Resolution | Resolution Notes |
| --- | --- | --- | --- | --- | --- |
| 1486 | Open | I find the use of ATIM in the draft confusing. In REVma, the ATIM window is only for use with IBSS networks. However, in some cases, the draft refer to ATIM windows without it being clear that an MP is necessarily in IBSS mode. For example, a peer link between an MP and an MAP is operating in infrastructure mode, and would not have the ability to use any ATIM-based operations. | Please clarify if ATIM is being used in a new way, and if so, make that explicit. | Defer-submission-needed | The usage of ATIM is the same as defined for IBSS |
| 1997 | Open | It is not clear how ATIM period is indicated in infrastructure beacon frame,when infrastructure beacons are used for power save as described in 11A.9.4. | Add information how the ATIM period is sigmalled in infrastructure beacon frame. | Defer-submission-needed | Jarkko will submit a resolution (Include IBSS parameter set in the infrastructure beacon) |
| 3772 | Open | Omission. In table8, explaination of beacon frame contents, "IBSS parameter set element" and "TIM element" have to be updated as well, since these fields are reused for mesh operation. | Change the contents of table8 regarding "IBSS parameter set element" and "TIM element" as following. IBSS parameter set element : The IBSS Parameter Set information element is present only within Beacon frames generated by STAs in an IBSS or MP in a mesh. Traffic indication map (TIM) : The TIM information element is present only within Beacon frames generated by APs or MPs supporitng power management. | Defer | related to power save |
| 3777 | Open | Omission. In table15, explaination of probe response frame contents, "IBSS parameter set element" have to be updated as well, since this field is reused for mesh operation. | Change the contents of table15 regarding "IBSS parameter set element"as following. IBSS parameter set element : The IBSS Parameter Set information element is present only within Beacon frames generated by STAs in an IBSS or MP in a mesh. | Defer | related to power save |
| 3781 | Open | Omission. The description on IBSS parameter set element needs to be updated, since this information element is reused in mesh. | Change the description on subclause 7.3.2.7 of 802.11ma-D9.0 as following. "The IBSS Parameter Set element contains the set of parameters necessary to support an IBSS and mesh. The information field contains the ATIM Window parameter. See Figure 61. The ATIM Window field is 2 octets in length and contains the ATIM Window length in TU." | Defer | related to powersave |

Suggested resolution [3]

- Define a new “Mesh ATIM parameter element”.

- MP includes “Mesh ATIM parameter element” within beacon frame and probe resonse frame, if it performs power management.

Corresponding chnages in the text should be made in the following subclauses:

7.2.3.1 Beacon frame format

7.2.3.9 Probe Response frame format

7.3.2. Information elements

Suggested updates to the draft spec

The following text was taken from the Draft Spec D1.00[1], and has been modified with “Track Changes” on:

7.2.3.1 Beacon frame format

Change the contents of Table 8 as shown:

Table 8: Beacon frame body

| Order | Information | Notes |
| --- | --- | --- |
| 4 | Service Set Identifier (SSID) | When dot11WLANMeshService is true but the interface on which the beacon is being sent is not configured as an Access Point, the SSID IE shall be set to the wildcard value. [Note: the SSID is a required IE in beacon frames. To avoid having non-mesh STAs send association requests to non-MAP Mesh Points, a valid SSID should not be included in beacons sent by non-MAP Mesh Points. To avoid backward compatibility issues, rather than removing the SSID IE from MP (non-MAP) beacons the wildcard value is used.] |
| (Ed: insert unchanged table entries for completeness) |  |  |
| 26 | OFDM Parameter Set | The OFDM Parameter Set information element is present within Beacon frames generated by STAs using Clause 17 PHYs. |
| 27 | Mesh ID | The Mesh ID information element shall be present within Beacon frames only when dot11WLANMeshService is true. |
| 28 | WLAN Mesh Capability | The WLAN Mesh Capability information element shall be present within Beacon frames only when dot11WLANMeshService is true. |
| 29 | Neighbor List | The Neighbor List information element shall be present within DTIM Beacon frames generated when dot11WLANMesh Service is true and MP is supporting Transmission to MP in power save mode. |
| 30 | DTIM | The DTIM IE shall be present in beacon frames generated by when dot11WLANMesh Service is true and MP is supporting Transmission to MP in power save mode. |
| 31 | Mesh ATIM window parameter | The Mesh ATIM window parameter element is present only when dot11WLANMesh Service is true and the MP is operating in power save mode. |
| 32 | Mesh Portal Reachability | The Mesh Portal Reachability information element shall be present within Beacon frames only when dot11WLANMeshService is true. |
| 33 | Beacon Timing | The Beacon Timing information element shall be present within Beacon frames only when dot11WLANMeshService is true. |
| 34 | MDAOP Advertisements | The MDAOP Advertisements information element shall be present within Beacon frames only when dot11WLANMeshService is true. |
| 35 | MDAOP Set Teardown | The MDAOP Set Teardown information element shall be present within Beacon frames only when dot11WLANMeshService is true. |
| 36 | MKDDIE | The MKDDIE element shall be present only when dot11WLANMeshService is true |

7.2.3.9 Probe Response frame format

Add the following to the contents of Table 15 as shown:

Table 15: Probe Response frame body

| Order | Information | Notes |
| --- | --- | --- |
| 25 | OFDM Parameter Set | The OFDM Parameter Set information element is present within Probe Response frames generated by STAs using Clause 17 PHYs. |
| 26 | Mesh ID | The Mesh ID information element shall be present within Probe Response frames only when dot11WLANMeshService is true. |
| 27 | WLAN Mesh Capability | The WLAN Mesh Capability information element shall be present within Probe Response frames only when dot11WLANMeshService is true. |
| 28 | DTIM | The DTIM information element is only present in probes generated when dot11WLANMeshService is true and MP supports Power Save mode. |
| 29 | Mesh ATIM window parameter | The Mesh ATIM window parameter element is present only when dot11WLANMesh Service is true and the MP is operating in power save mode. |
| 30 | Mesh Portal Reachability | The Mesh Portal Reachability information element shall be present within Probe Response frames only when dot11WLANMeshService is true. |
| 31 | Beacon Timing | The Beacon Timing information element shall be present within Probe Response frames only when dot11WLANMeshService is true. |
| 32 | MKDDIE | The MKDDIE element shall be present only when dot11WLANMeshService is true |

Insert the following new subclause:

7.3.2.54 Mesh ATIM window parameter element

The Mesh ATIM window parameter element contains a Mesh ATIM window parameter which is necessary to support mesh power management. See figure sXX.

| Octets: 1 | 1 | 2 |
| --- | --- | --- |
| ID | Length | ATIM Window parameter |

Figure sXX: Mesh ATIM window parameter element

The Mesh ATIM window parameter field is 2 octets in length and contains the ATIM window length in TU.

References:

[1] Draft Amendment: ESS Mesh Networking.  doc.: IEEE P802.11s/D1.00, November 2006.

[2] 11-07-0023-19-000s-lb93-tgs-draft-1-0-comment-spreadsheet.xls

[3] 11-07-0319-00-000s-suggested-comment-resolution-on-atim-window-parameter.ppt