IEEE P802.11
Wireless LANs

| Suggested comment resolution on ATIM window parameter |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-05-11 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | Email |
| Kazuyuki Sakoda | Sony Corporation | 2-17-1 Higashi-gotanda, Shinagawa-ku, Tokyo, 141-0022 Japan | +81-3-6409-3201 | sako@wcs.sony.co.jp |

Abstract

This document provides the suggested updates to the current draft spec, which potentially resolve CID 1476, 1486, 1997, 3772, 3777, 3781, and 5638.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Summary of CIDs labelled as ATIM issues

Following table is captured from [2].

| CID | Resolution Status | Comment | Proposed Change by Commenter | Resolution | Resolution Notes |
| --- | --- | --- | --- | --- | --- |
| 1476 | Open | The four points presented in these lines all involve use of the ATIM window. However, the ATIM window is only available for IBSS networks, and an MP may choose to be in either IBSS or infrastructure mode as per 11A.8.3 (page 191). | Please clarify this. | Defer |  |
| 1486 | Open | I find the use of ATIM in the draft confusing. In REVma, the ATIM window is only for use with IBSS networks. However, in some cases, the draft refer to ATIM windows without it being clear that an MP is necessarily in IBSS mode. For example, a peer link between an MP and an MAP is operating in infrastructure mode, and would not have the ability to use any ATIM-based operations. | Please clarify if ATIM is being used in a new way, and if so, make that explicit. | Defer-submission-needed | The usage of ATIM is the same as defined for IBSS |
| 1997 | Open | It is not clear how ATIM period is indicated in infrastructure beacon frame,when infrastructure beacons are used for power save as described in 11A.9.4. | Add information how the ATIM period is sigmalled in infrastructure beacon frame. | Defer-submission-needed | Jarkko will submit a resolution (Include IBSS parameter set in the infrastructure beacon) |
| 3772 | Open | Omission. In table8, explaination of beacon frame contents, "IBSS parameter set element" and "TIM element" have to be updated as well, since these fields are reused for mesh operation. | Change the contents of table8 regarding "IBSS parameter set element" and "TIM element" as following. IBSS parameter set element : The IBSS Parameter Set information element is present only within Beacon frames generated by STAs in an IBSS or MP in a mesh. Traffic indication map (TIM) : The TIM information element is present only within Beacon frames generated by APs or MPs supporitng power management. | Defer | related to power save |
| 3777 | Open | Omission. In table15, explaination of probe response frame contents, "IBSS parameter set element" have to be updated as well, since this field is reused for mesh operation. | Change the contents of table15 regarding "IBSS parameter set element"as following. IBSS parameter set element : The IBSS Parameter Set information element is present only within Beacon frames generated by STAs in an IBSS or MP in a mesh. | Defer | related to power save |
| 3781 | Open | Omission. The description on IBSS parameter set element needs to be updated, since this information element is reused in mesh. | Change the description on subclause 7.3.2.7 of 802.11ma-D9.0 as following. "The IBSS Parameter Set element contains the set of parameters necessary to support an IBSS and mesh. The information field contains the ATIM Window parameter. See Figure 61. The ATIM Window field is 2 octets in length and contains the ATIM Window length in TU." | Defer | related to powersave |
| 5628 | Open | MPs in Power Save mode use the IBSS parameter to indicate traffic, however the description of the IBSS parameter Set in 802.11 draft9.0 is not available for MPs. | Recommended to add a description of the IBSS parameter Set in Beacon frame body, shown in Table 8: Table 8: Beacon frame body | Defer-submission-needed | Hui Ma will provide a document |

Suggested resolution [3]

- Define a new “Mesh ATIM parameter element”.

- MP includes “Mesh ATIM parameter element” within beacon frame and probe resonse frame, if it performs power management.

Corresponding chnages in the text should be made in the following subclauses:

7.2.3.1 Beacon frame format

7.2.3.9 Probe Response frame format

7.3.2. Information elements

Suggested updates to the draft spec

The following text was taken from the Draft Spec D1.03[1], and has been modified with “Track Changes” on:

7.2.3.1 Beacon frame format

Change the contents of the of Table 8-Beacon frame body as follows::

| - Beacon frame body |  |  |
| --- | --- | --- |
| Order | Information | Notes |
| 4 | Service Set Identifier (SSID) | When dot11WLANMeshService is true and the interface on which the beacon is being sent is not configured as an Access Point, the SSID information element is set to the wildcard value. |

Insert the following additional rows (preserving their order) inTable 8-Beacon frame body just before the Vendor Specific information element and insert contiguous numbering in the “Order” column:

| Order | Information | Notes |
| --- | --- | --- |
|  | Mesh ID | The Mesh ID information element is present within Beacon frames when dot11WLANMeshService is true. |
|  | Mesh Capability | The Mesh Capability information element is present within Beacon frames when dot11WLANMeshService is true. |
|  | Mesh Neighbor List | The Mesh Neighbor List information element is present within frames with the DTIM bit set when dot11WLANMesh Service is true and the MP transmits to other MPs in power save mode. |
|  | Mesh TIM | The Mesh TIM element is present in Beacon frames generated by the MP when dot11WLANMesh Service is true and MP is supporting Transmission to MP in power save mode. |
| - 41 | Mesh ATIM Window | The Mesh ATIM window parameter element is present only when dot11WLANMesh Service is true, and the MP intends to operate in power save mode. |
| - 42 | Mesh Portal Reachability | The Mesh Portal Reachability information element is present within Beacon frames when dot11WLANMeshService is true. |
| - 43 | Beacon Timing | The Beacon Timing information element is present within Beacon frames when dot11WLANMeshService is true. |
| - 44 | MDAOP Advertisements | The MDAOP Advertisements information element is present within Beacon frames when dot11WLANMeshService is true. |
| - 45 | MSCIE | The MSCIE element is present when dot11WLANMeshService is true. |

7.2.3.9 Probe Response frame format

Insert the following additional rows (preserving their order) in before the last row of Table 15-Probe Response frame body just before the Vendor Specific information element and insert contiguous numbering in the “Order” column:

| - Probe Response frame body |  |  |
| --- | --- | --- |
| Order | Information | Notes |
|  | Mesh ID | The Mesh ID information element is present within Probe Response frames when dot11WLANMeshService is true. |
|  | Mesh Capability | The Mesh Capability information element is present within Probe Response frames when dot11WLANMeshService is true. |
|  | Mesh TIM | The Mesh TIM element is present within Probe Response frames only when dot11WLANMeshService is true and MP supports Power Save mode. |
| - 40 | Mesh ATIM Window | The Mesh ATIM window parameter element is present only when dot11WLANMesh Service is true, and the MP intends to operate in power save mode. |
| - 41 | Mesh Portal Reachability | The Mesh Portal Reachability information element is present within Probe Response frames when dot11WLANMeshService is true. |
| - 42 | Beacon Timing | The Beacon Timing information element is present within Probe Response frames when dot11WLANMeshService is true. |
| - 43 | MSCIE | The MSCIE element is present when dot11WLANMeshService is true. |

Insert the following new subclause:

7.3.2.54 Mesh ATIM window parameter element

The Mesh ATIM window parameter element contains the ATIM Window parameter, and is present when the MP is operating in power save mode or intends to operate in power save mode.

| Octets: 1 | 1 | 2 |
| --- | --- | --- |
| ID | Length | ATIM Window parameter |

Figure sXX: Mesh ATIM window parameter element

The Mesh ATIM window parameter field is 2 octets in length and contains the ATIM window length in TU.

References:

[1] Draft Amendment: ESS Mesh Networking.  doc.: IEEE P802.11s/D1.03, April 2007.

[2] 11-07-0023-29-000s-lb93-tgs-draft-1-0-comment-spreadsheet.xls

[3] 11-07-0319-02-000s-suggested-comment-resolution-on-atim-window-parameter.ppt