IEEE P802.11
Wireless LANs

| Generic Advertisment Service (GAS) MIB |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-03-13 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Eleanor Hepworth | Siemens Roke Manor | Roke Manor, Romsey, UK, SO51 0ZN | +44 1794 833146 | eleanor.hepworth@roke.co.uk |
| Angelo Centonza Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s). The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein. Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11. Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard." Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication. Please notify the Chair <stuart@ok-brit.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>. Abstract This document describes the addition of a Generic Advertisment Service (GAS) MIB to the TGu draft. | Siemens Roke Manor | Roke Manor, Romsey, UK, SO51 0ZN | +44 1794 833000 | angelo.centonza@roke.co.uk |

# Annex D

Insert the following elements just before PHY attributes on 802.11ma-d9, page nn  line nn:

-- Generic Advertising Service (GAS) Attributes

-- DEFINED AS "The Generic Advertising Service management object class provides the necessary support for an advertising service to interwork with external systems."

dot11gas OBJECT IDENTIFIER ::= { ieee802dot11 5 }

-- GAS GROUPS

-- dot11GasCountersTable			::= { dot11gas 1 }

-- dot11GasAdvertisementTable		::= { dot11gas 2 }

Insert the following elements just before end of the 802.11 MiB in 802.11ma-d9, page nn  line nn:

-- **********************************************************************

-- * dot11GasCounters TABLE

-- **********************************************************************

dot11GasCountersTable OBJECT-TYPE

SYNTAX         SEQUENCE OF dot11GasCountersEntry

(Note: Surely this should be “dot” with a small d here??)

MAX-ACCESS     not-accessible

STATUS         current

DESCRIPTION

"This object is a table of GAS counters implemented as a

table to allow for multiple instantiations on an STA."

::= { dot11gas 1 }

dot11GasCountersEntry OBJECT-TYPE

SYNTAX     Dot11GasCountersEntry

MAX-ACCESS not-accessible

STATUS     current

DESCRIPTION

"This object provides the attributes identifying

a GAS counter within an STA.”

INDEX { dot11GasCountersIndex }

::= { dot11GasCountersTable 1 }

Dot11GasCountersEntry ::=

SEQUENCE 	{

dot11GasTransmittedFragmentCount	Counter32,

dot11GasFailedCount			Counter32,

dot11GasRetryCount			Counter32,

.............	dot11GasMultipleRetryCount		Counter32,

.............	dot11GasFrameDuplicateCount		Counter32,

.............	dot11GasACKFailureCount			Counter32,

.............	dot11GasReceivedFragmentCount		Counter32,

.............	dot11GasTransmittedFrameCount		Counter32,

.............	dot11GasDiscardedFrameCount		Counter32,

.............	dot11GasRetriesReceivedCount		Counter32,

.............	dot11GasResponseTimeout			Integer

}

dot11GasCountersIndex OBJECT-TYPE

SYNTAX INTEGER (1..12)

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"The auxiliary variable used to identify instances of the

columnar objects in the GasCounters Table."

::= { dot11GasCountersEntry 1 }

dot11GasTransmittedFragmentCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter shall be incremented for an acknowledged MPDU, for

a particular UP, with an individual address in the address 1

field or an MPDU with a multicast address in the address 1

field, either belonging to a particular TID. This counter has

relevance only for TIDs between 0 and 7."

::= { dot11GasCountersEntry 2 }

dot11GasFailedCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter shall increment when an MSDU, for a particular UP,

is not transmitted successfully due to the number of transmit

attempts exceeding either the dot11ShortRetryLimit or

dot11LongRetryLimit. This counter has relevance only for TIDs

between 0 and 7."

::= { dot11GasCountersEntry 3 }

dot11GasRetryCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter shall increment when an MSDU, of a particular UP,

is successfully transmitted after one or more retransmissions.

This counter has relevance only for TIDs between 0 and 7."

::= { dot11GasCountersEntry 4 }

dot11GasMultipleRetryCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter shall increment when an MSDU, of a particular UP,

is successfully transmitted after more than one retransmissions.

This counter has relevance only for TIDs between 0 and 7."

::= { dot11GasCountersEntry 5 }

dot11GasFrameDuplicateCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter shall increment when a frame, of a particular UP,

is received that the Sequence Control field indicates is a

duplicate.  This counter has relevance only for TIDs between 0

and 7."

::= { dot11GasCountersEntry 6 }

dot11GasACKFailureCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter shall increment when an ACK is not received in

response to an MPDU of a particular UP. This counter has

relevance only for TIDs between 0 and 7."

::= { dot11GasCountersEntry 7 }

dot11GasReceivedFragmentCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter shall be incremented for each successfully

Received MPDU of type Data of a particular UP. This counter has

Relevance only for TIDs between 0 and 7."

::= { dot11GasCountersEntry 8 }

dot11GasTransmittedFrameCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter shall increment for each successfully transmitted

MSDU of a particular UP. This counter has relevance only for

TIDs between 0 and 7."

::= { dot11GasCountersEntry 9 }

dot11GasDiscardedFrameCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter shall increment for each Discarded MSDU of a

particular UP. This counter has relevance only for TIDs between

0 and 7."

::= { dot11GasCountersEntry 10 }

dot11GasRetriesReceivedCount OBJECT-TYPE

SYNTAX Counter32

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This counter shall increment for each received MPDU of a

particular TID with the retry bit set to 1."

::= { dot11GasCountersEntry 11 }

dot11GasResponseTimeout OBJECT-TYPE

SYNTAX Integer

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This parameter shall indicate the GAS response timeout value in

ms."

::= { dot11GasCountersEntry 12 }

-- **********************************************************************

-- * End of dot11GasCounters TABLE

-- **********************************************************************

-- **********************************************************************

-- * dot11GasAdvertisement TABLE

-- **********************************************************************

dot11GasAdvertisementTable OBJECT-TYPE

SYNTAX         SEQUENCE OF dot11GasAdvertisementEntry

MAX-ACCESS     not-accessible

STATUS         current

DESCRIPTION

"This object is a table of Advertisment Protocols

contained within a STA."

::= { dot11gas 2 }

dot11GasAdvertismentEntry OBJECT-TYPE

SYNTAX     Dot11GasAdvertismentEntry

MAX-ACCESS not-accessible

STATUS     current

DESCRIPTION

"This object provides the attributes identifying

a particular advertisement procotol within an STA.”

INDEX { dot11GasAdvertisementIndex }

::= { dot11GasAdvertisment Table 1 }

Dot11GasAdvertismentEntry ::=

SEQUENCE 	{

dot11GasAdvertismentID	INTEGER,

}

dot11GasAdvertisementIndex OBJECT-TYPE

SYNTAX INTEGER (1..16)

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"The auxiliary variable used to identify instances of the

columnar objects in the GasAdvertisment Table."

::= { dot11GasAdvertisementEntry 1 }

dot11GasAdvertismentID OBJECT-TYPE

........SYNTAX INTEGER {

........		MIH Information Service(1),

........		MIH Command and Event Services Capability Discovery(2)

Emergency Alert System (EAS) Service(3)}

........MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This object identifies the GAS Advertisment Protocol."

::= { dot11GasAdvertismentEntry 2 }

dot11GasDeliveryMethod OBJECT-TYPE

........SYNTAX INTEGER {

........		Unicast(1),

........		Multicast(2)}

........MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This object identifies the delivery method of each GAS

Advertisment Protocol in this row of the table."

::= { dot11GasAdvertismentEntry 3 }

-- **********************************************************************

-- * End of dot11GasAdvertisment TABLE

-- **********************************************************************