IEEE P802.11
Wireless LANs

| Interworking rewrite |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-03-14 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Jan Kruys | Cisco Systems | Cisco Way Bld 14 San Jose, CA | + 31 348 453719 | jkruys@cisco.com |
| Guenael Strutt | Motorola | 1064 Greenwood Blvd Lake Mary, FL 32746 | +1 407 562 4050 | Guenael.strutt@motorola.com |
| Gabriel Montenegro | Microsoft Corporation | One Microsoft Way Redmond, WA 98052 | +1 425 706 9988 | gmonte@microsoft.com |
| Michael Bahr | Siemens AG, Corporate Technology | Otto-Hahn-Ring 6, 81730 München, Germany | +49-89-636-49926 | bahr@siemens.com |
| Hrishikesh Gossain | Motorola | 1064 Greenwood Blvd Lake Mary, FL 32746 | +1-407-562-4093 | hrishikesh.gossain@motorola.com |
| Jorjeta Jetcheva | Firetide | 16795 Lark Ave., Los Gatos, CA 95032 | +1-408-355-7215 | jjetcheva@firetide.com |
| Malik Audeh | Tropos | 555 Del Rey Ave, Sunnyvale, CA 94085 | +1-408-331-6834 | audeh@tropos.com |
| Hang Liu | Thomson | Two Independence Way – Suite 300 Princeton, NJ 08540 | +1 609 987-7335 | hang.liu@thomson.net |
| Susan Hares | NextHop | 825 Victors Way, Suite 100 – Ann Arbor, MI 48108 | +1 734 222 1610 | skh@nexthop.com |

Abstract

Resolution of CIDs 173, 559, 1135, 1230, 1252, 1325, 1831, 1988, 2468, 3635, 3741, 3869, 3873, 4460, 28, 4318, 5658 and 5690

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

- Interworking

- Overview of interworking in a WLAN mesh

A WLAN Mesh functions like an IEEE 802 LAN segment that is compatible with IEEE 802.1D.

The combination of MP functionality and the IEEE 802.1D bridging functionality is called an MPP.

The WLAN Mesh MAC entity appears as a single port to an IEEE 802.1D MAC Relay entity in IEEE 802.1D (to which it presents the Internal Sublayer Service interface) or any other higher-layer entity, such as a Spanning Tree Protocol entity, or a layer 3 router (to which it provides the MAC service to LLC, e.g., LLC type 1 to support the Spanning Tree Protocol entity).

A mesh network may have zero or more MPPs which may be connected to one or more LAN segments. In case two MPPs connect the WLAN mesh to one external LAN segment, broadcast loops may occur and the IEEE 802.1D bridging protocol may cause the LAN ports of one of the MPPs to be closed. These cases can be prevented by proper configuration measures.

The default routing protocol supports the transfer of “proxy information” towards the MPP.  The MPP shall support the Internal Sublayer Service interface to transfer this information to the MAC Relay Entity as defined in IEEE 802.1D.  As a result, the bridge, or, more specifically, the Learning Process within the MAC Relay Entity will learn the addresses of all the MPs and their attached stations.

- Portal announcement protocol

This subclause describes the function, generation and processing of the Portal Announcement.

Enabling the Announcement in a given MPP is outside the scope of this standard.

- Function

The Portal Announcement (PANN) is used to announce the presence of an MP configured as an MPP  in the mesh.  Portal  Announcements allow MPs to select the appropriate MPP and build a route towards it.

An MP which receives a Portal Announcement message shall propagate the Portal Announcement as described below but may ignore its content.

The MPP sequence number of the Portal Announcement is incremented by the MPP before each transmission.

- Conditions for generating and sending a PANN

An MP shall transmit a PANN in the following cases:

Case A: Original transmission

All of the following applies:

- The MP is configured as an MPP

- At every PORTAL_ANNOUNCEMENT_INTERVAL

Content:

| Field | Value |
| --- | --- |
| ID | PANN information element ID |
| Length | As required |
| Flags | Not used |
| Hop Count | 0 |
| Time to Live | Maximum number of hops allowed for the Portal Announcement |
| Address | MPP MAC address |
| Sequence Number | A sequence number specific for the MPP |
| Metric | 0 |

Case B:	Forwarding

All of the following applies:

- The MP has received a Portal Announcement

- PORTAL_PROPAGATION_DELAY has expired

- The decremented TTL of the Portal Announcement is equal to or greater than 1

Content:

| Field | Value |
| --- | --- |
| ID | As received |
| Length | As received |
| PANN Flags | As received |
| Hop Count | As received + 1 |
| Time to Live | As received – 1 |
| Originator Address | As received |
| Sequence Number | As received |
| Metric | As received  own metric towards the transmitting MP |

- PANN processing

A received Portal Announcement is subject to certain acceptance criteria. Processing depends on the contents of the Portal Announcement and the information available at the receiving MP.

- Acceptance criteria

The Portal Announcement will be discarded if

- the Sequence Number of the Portal Announcement is lower than the Sequence Number of the previously received Portal Announcement from this MPP, or

- the Sequence Number of the Portal Announcement is the same as the Sequence Number of the previously received Portal Announcement from this MPP, and the metric is worse that the metric of the previously received Portal Announcement from this MPP

- Effect of receipt

The following applies only to Portal Announcements that were not discarded during the check described in Acceptance criteria above.

- The receiving MP shall initiate a PANN_PROPAGATION_DELAY

- The receiving MP shall transmit a Portal Announcement as defined in Conditions for generating and sending a PANN, Case B

- MP behavior

When an MP receives Portal Announcements sent by MPPs in the mesh, it records the MAC address and path metric to all active MPPs in the mesh.

When an MP has a data message to send, it first follows the data forwarding procedures defined in Error: Reference source not found.  If the MP is not able to determine an intra-mesh route to the destination MAC address, the MP shall assume that the destination is outside the mesh and shall forward the message to all active MPPs in the mesh (see Egress message handling).  If the destination appears to be outside the mesh but there is no MPP available, the MP has a problem that is efficiently solved by putting the frame in the bit bucket.

- MPP data forwarding behavior

Forwarding of frames by the MPP into the WLAN Mesh follows the procedures given in clause 11A.3.4

MPPs learn the addresses of the MPs and of devices attached to these MPs through the receipt of messages carrying proxy information (see clause 11A.6.8). The bridge learns via the Learning Process in the MAC Relay Entity per usual IEEE 802.1D procedures.

- Egress message handling

A frame sent by an MP in the WLAN Mesh has the following final destinations:

- An MP address or a proxied address that the MPP knows is inside the Mesh

The MPP forwards the frame to the destination MP.

- An address that the MPP knows is outside the Mesh

The MPP forwards the frame on the external network.

- An address unknown to the MPP

The MPP forwards the frame on the external network and on the mesh. For the latter it has two options:

  - To attempt to establish a path to the destination

  - To broadcast the frame within the mesh using the Mesh Broadcast procedure (see clause 11A.3.4.4)

The criteria for making this choice are outside the scope of this standard.

- Ingress message handling

A frame received by an MPP in an MA_UNITDATA.request at the MAC service has two possible destinations:

- An MP address or proxied address that the MPP knows is inside the Mesh

The MPP forwards the frame to the destination MP.

- An address unknown to the MPP

The MPP has two options:

- To attempt to establish a route to the destination

- To broadcast the frame within the mesh using the Mesh Broadcast procedure (see clause 11A.3.4.4)

The criteria for making this choice are outside the scope of this standard.

- Operational considerations for interworking moved to Annex T per CID 3218