IEEE P802.11
Wireless LANs

| Proxy Frame Forwarding |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-03-15 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | Email |
| Hrishikesh Gossain | Motorola | 1064 Greenwood Blvd., Suite -400, Lake Mary, FL: 32746 | +1-407-562-4093 | Hrishikesh.Gossain@motorola.com |
| Meiyuan Zhao | Intel Corporation | RNB-6-61, 2200 Mission College Blvd. Santa Clara, CA 95052 | +1-408-653-5517 | meiyuan.zhao@intel.com |
| James Murphy | Trapeze Networks | 5753 W. Las Positas Blvd. Pleasanton, CA 94588 | +1-925-474-2233 | jmurphy@trapezenetworks.com |
| Michael Bahr | Siemens AG, Corporate Technology | Otto-Hahn-Ring 6 81730 Munchen Germany | +49-89-636-49926 | bahr@siemens.com |
| Guenael Strutt | Motorola | 1064 Greenwood Blvd, Lake Mary, FL 32746 | +1-407-562-4050 | Guenael.Strutt@motorola.com |
| Kyeong Soo (Joseph) Kim | STMicroelectronics | 1060 East Brokaw Road, MS 212, San Jose, Ca 95131 | +1-408-451-8137 | Kyeongsoo.kim@st.com / kimks@ieee.org |
| Susan Hares | NextHop Technologies | 825 Victors Way Ann Arbor, MI 48108 | +1-734-222-1610 | skh@nexthop.com |
| Jan Kruys | Cisco Systems | 10 Haarlerbergweg 110-1CH Amsterdam, Netherlands | +31 203572447 | jkruys@cisco.com |
| Jorjeta Jetcheva | Firetide |  | 1 (408) 355-7215 | jjetcheva@firetide.com |
| Casey Barker | Motorola | 1064 Greenwood Blvd., Suite -400, Lake Mary, FL: 32746 | +1-407-562-4016 | Casey.Barker@motorola.com |

Abstract

This document provides comment resolution for following CIDs:

4372, 4374, 4859, 489, 4385, 4386, 461, 489, 756, 787, 2254, 2255, 2256, 2259, 2258, 1327, 1990, 3986, 4033

This text is created to address the comments relevant to the RFI2 issue identifier. It adds two new frame formats (Proxy Update and Proxy Update Confirmation) to update an MP’s proxy entry to a destination MP. More details in destination sequence number handling is included. It provides update on RREP processing section on how to create and maintain precursor list. It updates RREQ/RREP IE to include a proxied address. A new section to generate gratuitous RREP is added.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Insert the following new clause after  11:

- Mesh networking

Mesh discovery and peer link establishment

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

- Information elements

Insert the following entries in Table 26 and change the Reserved row as shown.

| - Element IDs |  |  |
| --- | --- | --- |
| Information element | Element ID | Length (in octets) |
| Mesh Capability | ABSB (see note below) | 19 |
| Active Mesh Profile Announcement | ABSB (see note below) | 9 |
| Mesh ID | ABSB (see note below) | 0 to 255 |
| Local Link state announcement | ABSB (see note below) | 4 |
| Target Transmission Rate | ABSB (see note below) | 18 |
| Offered Traffic Load | ABSB (see note below) | 16 |
| Neighborhood Congestion | ABSB (see note below) | 3 |
| Peer Link Close | ABSB (see note below) | 9 |
| Peer Link Open | ABSB (see note below) | 4 |
| Peer Link Confirm | ABSB (see note below) | 8 |
| Mesh Portal Reachability | ABSB (see note below) | 1 to 255 |
| Mesh Portal/Root Announcement | ABSB (see note below) | 28 to 255 |
| UCG Switch Announcement | ABSB (see note below) | 13 |
| Mesh Neighbor List | ABSB (see note below) | 1 to 255 |
| Mesh DTIM | ABSB (see note below) | 2 |
| Beacon Timing | ABSB (see note below) | 5 to 255 |
| MDAOP Setup Request | ABSB (see note below) | 8 to 255 |
| MDAOP Setup Reply | ABSB (see note below) | 3 |
| MDAOP Advertisements Request | ABSB (see note below) | 0 |
| MDAOP Advertisements | ABSB (see note below) | 1 to 255 |
| MDAOP Set Teardown | ABSB (see note below) | 7 |
| Connectivity Report | ABSB (see note below) | 14 to 255 |
| PANN | ABSB (see note below) | 17 |
| RANN | ABSB (see note below) | 21 |
| RREQ | ABSB (see note below) | 36 to 255 |
| RREP | ABSB (see note below) | 32 to 255 |
| RERR | ABSB (see note below) | 13 |
| RA-OLSR HELLO | ABSB (see note below) | 22 to 255 |
| RA-OLSR Topology Control (TC) | ABSB (see note below) | 23 to 255 |
| RA-OLSR Multiple Interface Declaration (MID) | ABSB (see note below) | 23 to 255 |
| RA-OLSR Local Association Base Advertisement (LABA) | ABSB (see note below) | 20 to 255 |
| RA-OLSR Local Association Base Checksum Advertisement (LABCA) | ABSB (see note below) | 11 to 255 |
| RA-OLSR Association Base Block Request (ABBR) | ABSB (see note below) | 11 to 255 |
| MKD domain | ABSB (see note below) | 7 |
| MSA Handshake | ABSB (see note below) | 88 to 255 |
| Mesh Key Holder Security | ABSB (see note below) | 98 |
| Mesh Encrypted Key | ABSB (see note below) | 82 to 255 |
| EAP Authentication | ABSB (see note below) | 42 |
| EAP Message | ABSB (see note below) | 2 to 255 |
| Proxy Update | ABSB (see note below) | 12 to 256 |
| Proxy Update Confirmation | ABSB (see note below | 10 |
| Reserved | ABSB (see note below) |  |

- Assignment of values for these information elements needs to be approved by IEEE 802.11 ANA. Until that time, these values are marked as Allocate By Sponsor Ballot (ABSB). Final values will be requested from IEEE 802.11 ANA once this amendment reaches the 75% approval threshold in Sponsor Ballot.

- RREQ information element

- RREQ information element relocated from 11A.5.5.2

The RREQ element is used for discovering a route to one or more destinations, building a proactive (reverse) routing tree to the root MP, and confirming a route to a destination (optional).

The format of the RREQ element is shown in RREQ element.

| Octets: 1 | 1 | 1 | 1 | 1 | 4 | 6 | 4 | 0 or 6 | 4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Element ID | Length | Flags | Hopcount | Time to Live | RREQ ID | Originator Address | Originator Sequence Number | Proxied Address | Lifetime |  |  |  |  |  |  |  |  |
| 4 | 1 | 1 | 6 | 4 | ... | 1 | 6 | 4 |  |  |  |  |  |  |  |  |  |
| Metric | Destination Count | Per Destination Flags #1 | Destination Address #1 | Destination Seq. Num. #1 | ... | Per Destination Flags #N | Destination Address #N | Destination Seq. Num. #N |  |  |  |  |  |  |  |  |  |
| - RREQ element |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

The Element ID is set to the value given in Element IDs for this information element. The length is set to 37 to 255 octets.

The Flags field is set as follows. Bit 0: Portal Role (0 = non-portal, 1 = portal), Bit 1: (0 = group addressed, 1 = individually addressed) (see 11A.6.2), Bit 2: Proactive RREP (0 = off, 1 = on), Bit 6: Address Extension (AE) (1= if (destination count ==1 && proxied device address present), 0 = otherwise), Bit 3-5, – 7: Reserved.

The Hop Count field is set to the number of hops from the originator to the MP transmitting the request.

The Time to Live field is set to the maximum number of hops allowed for this element.

The RREQ ID field is set to some unique ID for this RREQ.

The Originator Address is set to the originator MAC address.

The Originator Sequence Number is set to a sequence number specific to the originator.

Proxied Address field is the MAC address of proxied entity (e.g. STA) in case the RREQ is generated because of a frame received from outside the mesh (e.g. BSS) and proxied entity is the source of the frame. This field is only present if the AE flag is set to 1.

The Lifetime field is set to the time for which MPs receiving the RREQ consider the forwarding information to be valid.

The Metric is set to the cumulative metric from the originator to the MP transmitting the RREQ.

The Destination Count N gives the number of Destinations (N) contained in this RREQ.

The format of the Per Destination Flags field is shown in RREQ Per-Destination Flags field format.

| B0 | B1 | B2 B7 |
| --- | --- | --- |
| DO | RF | Reserved |
| Bits: 1 | 1 | 6 |
| - RREQ Per-Destination Flags field format |  |  |

Per Destination Flags are set as follows.

Bit 0: DO (Destination Only): If DO=0, an intermediate MP with active forwarding information to the corresponding destination responds to the RREQ with a unicast RREP; if DO=1, only the destination can respond with a unicast RREP. The default value is 1.

Bit 1: RF (Reply-and-Forward): The RF flag controls the forwarding of RREQ at intermediate MPs. When DO=0 and the intermediate MP has active forwarding information to the corresponding destination, the RREQ is not forwarded if RF=0 and forwarded if RF=1. The default value is 1. When DO=1, the RF flag has no effect.

Bit 2-7: Reserved

The Destination Address is the MAC address of the destination MP.

The Destination Sequence Number is the latest sequence number received in the past by the originator for any route towards the destination.

Detailed usage of the RREQ element is described in 11A.6.5.

- RREP information element

- RREP information element relocated from 11A.5.6.2

The RREP element is used to establish a forward route to a destination and to confirm that a destination is reachable.

The format of the RREP element is shown in Route Reply element.

| Octets: 1 | 1 | 1 | 1 | 1 | 6 | 4 | 0 or 6 | 4 |  | 4 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ID | Length | Flags | Hopcount | Time to Live | Destination MP Address | Destination Seq.Num. | Destination Proxied Address | Lifetime |  | Metric |  |  |  |  |  |  |  |
| 6 | 4 | 1 | 6 | 4 |  | 6 |  | 4 |  |  |  |  |  |  |  |  |  |
| Originator Address #1 | Originator Seq. Num. | Dependent MP Count N | Dependent MP MAC Address #1 | Dependent MP DSN #1 | ... | Dependent MP MAC Address #N |  | Dependent MP DSN #N |  |  |  |  |  |  |  |  |  |
| - Route Reply element |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

The Element ID is set to the value given in Element IDs for this information element. The length is set to 32 to 255 octets.

The Flags field is set as follows: Bit 6: Address Extension (AE) (1= proxied address present, 0 = otherwise) 0 – 5, 7: Reserved .

The Hop Count field is set to the number of hops from the route destination to the local MP.

The Time to Live field is set to the maximum number of hops allowed for this element.

The Destination MP Address is the MAC address [of the destination for which a route is supplied].

The Destination Sequence Number field is set to the DSN of the target of the RREQ.

Destination Proxied Address field is set to Proxied address on behalf of which RREP is sent.  This field is present only if Bit 6 (AE) in Flags = 1.

The Lifetime field, if applicable, reflects the Lifetime of the RREQ this RREP responds to.

The Metric field indicates the cumulative metric from the route destination to the local MP.

The Originator Address Field is set to the MAC address of the originator of the RREQ.

The Originator Sequence Number field is set to the sequence number of the originator of the RREQ.

The Dependent MP Count N field indicates the number of dependent MPs (N).

The Dependent MP MAC Address # indicates the MAC address of dependent MP.

The Dependent MP DSN # indicates the Destination Sequence Number associated with MAC address of dependent MP.

The detailed usage of the RREP element is described in 11A.6.6.

      - Proxy Update (PU) Information Element

The PU information element is transmitted by a source MP to a destination MP to update its proxy information. This frame is typically transmitted in a unicast manner. The frame format is shown in Figure xxx.

| Octets: 1 | 1 | 1 | 1 | 6 |
| --- | --- | --- | --- | --- |
| ID | Length | Flags | Sequence number | Proxy Address |

| 2 | 6 |  | 6 |
| --- | --- | --- | --- |
| Number of proxied addresses (N) | Proxied MAC Address #1 | ... | Proxied MAC Address #N |

Figure sxx: PU Element

The Element ID is set to the value given in Element IDs for this information element.

The length is set to 12 + N*6.

The Flags field is set as follows. Bit 0: (0 = add proxy information, 1 = delete proxy information). Bit 1 – 7: Reserved

The sequence number field is set to the sequence number of PU. The Source MP shall set the Sequence Number field in the PU Element to a value from a single modulo-256 counter that is incrementing by 1 for each new PU

The Proxy Address is set to the MAC address of proxy MP.

Number of Proxied Address is set to the number of proxied addresses reported to the destination MP.

Proxied MAC Address is the MAC address of the proxied entities.

      - Proxy Update Confirmation (PUC) Information Element

The PUC information element is transmitted by an MP in response to a PU. This frame is typically transmitted in a unicast manner. The frame format is shown in Figure sxx.

| Octets: 1 | 1 | 1 | 1 | 6 |
| --- | --- | --- | --- | --- |
| ID | Length | Flags | Sequence number | Destination MP Address |

Figure sxx: PUC Element

The Element ID is set to the value given in Element IDs for this information element.

The length is set to 10.

The Flags field is set as follows. Bit 0 – 7: Reserved

The sequence number field is the sequence number of received PU which is being confirmed.

The Destination MP Address is set to the MAC address of the recipient of the PU.

11A.4.3 Proxy Update (PU)

This clause describes the function, generation and processing of the PU IE.

- 11A.4.3.1 Function

A PU IE is generated by an MP to inform a destination MP of its proxied addresses.

- 11A.4.3.2 Conditions for generating and sending a PU

An MP will send out a PU in the following cases:

- The MP needs to inform a destination MP of its proxy information

- A change is made in the local proxy information of the MP due to addition or deletion of proxy entries.

- If a root receives a PU IE with a add proxy information or if it detects a new proxied address in its DS (e.g. through bridge learning) and the root already has proxy information for one or more of these proxied address, it shall send a PU IE with Bit 0 set to 1 (delete proxy information) to each of the old proxy. PU IE in this case includes the list of corresponding proxied addresses which should be removed by the proxy.

- On a periodic basis to refresh the proxy information at the destination MP

The PU IE is a unicast sent from the MP to a destination MP. This request may be repeated after every PU_ TIMEOUT for MAX_PU times until the MP receives a PUC IE.

The content of the PU shall be:

| Field | Value/description |
| --- | --- |
| ID | PUDT request IE ID |
| Length | Length of the IE |
| Flags | Bit 0: 0: add proxy information; 1: delete proxy information 1 – 7: Reserved |
| Sequence number | Sequence number of the PU |
| Proxy Address | MAC address of the proxy MP |
| Number of Proxied Device Address (N) | Number of proxied addresses reported to the destination MP |
| Proxied Device MAC Address | MAC address of proxied entities to which the MP is providing mesh services |

      - PU processing

PU is a unicast message sent from an MP to a destination MP.  The destination MP shall update the proxy address in its proxy information with the list of proxied addresses reported in the PU.

    - Proxy Update confirmation (PUC)

This clause describes the function, generation and processing of the PUC IE.

      - Function

A PUC IE is generated by a destination MP in response to a PU IE.

      - Conditions for generating and sending a PUC

The PUC is unicast from the destination MP to the MP which sent the PU. The content of the PUC shall be:

| Field | Value/description |
| --- | --- |
| ID | PUC ID |
| Length | Length of the IE |
| Flags | Bit 0- 7: Reserved |
| Sequence number | Sequence number of the PU which is being confirmed |
| Destination MP Address | MAC address of the recipient of the PU |

      - PUC processing

On receiving a PUC IE, the MP shall no longer send any PU with the same sequence number.

- Hybrid Wireless Mesh Protocol (HWMP): default path selection protocol for interoperability

- Overview

- General

The Hybrid Wireless Mesh Protocol (HWMP) is a mesh routing protocol that combines the flexibility of on-demand routing with proactive topology tree extensions. The combination of reactive and proactive elements of HWMP enables optimal and efficient path selection in a wide variety of mesh networks (with or without infrastructure).

HWMP uses a common set of protocol primitives, generation and processing rules inspired by Ad Hoc On Demand Distance Vector (AODV) routing protocol [IETF RFC 3561] adapted for Layer-2 address-based routing and link metric awareness. HWMP is completely specified herein and does not require reference to AODV.

HWMP supports two modes of operation depending on the configuration.  These modes are:

On demand mode: this mode allows MPs to communicate using peer-to-peer routes.  The mode is used in situations where there is no root MP configured.  It is also used in certain circumstances if there is a root MPconfigured and an on demand route can provide a better route to a given destination in mesh.

Proactive tree building mode: this can be performed by using either the RREQ or RANN mechanism.

These modes are not exclusive: on demand and proactive modes may be used concurrently.

All HWMP modes of operation utilize common processing rules and primitives.  HWMP information elements are the Route Request (RREQ), Route Reply (RREP), Route Error (RERR) and Root Announcement (RANN).  The metric cost of the links determines which routes HWMP builds. In order to propagate the metric information between MPs, a metric field is used in the RREQ, RREP and RANN messages.

Routing in HWMP uses a sequence number mechanism to maintain loop-free connectivity at all times.  Each MP maintains its own sequence number, which is propagated to other MPs in the HWMP information elements.

- On demand routing mode

If a source MP needs to find a route to a destination MP using the on demand routing mode, it broadcasts a RREQ with the destination MP specified in the destination list and the metric field initialized to 0.

When an MP receives a RREQ it creates a route to the source or updates its current route if the RREQ contains a greater sequence number, or the sequence number is the same as the current route and the RREQ offers a better metric than the current route.  If a new route is created or an existing route modified, the RREQ is also forwarded (re-broadcast).  Each MP may receive multiple copies of the same RREQ that originated in the source, each RREQ traversing a unique path from the source to the MP.

Whenever an MP forwards a RREQ, the metric field in the RREQ is updated to reflect the cumulative metric of the route to the RREQ’s source.  After creating or updating a route to the source, the destination MP sends a unicast RREP back to the source.

Intermediate MPs generate RREPs only if the “Destination Only (DO)” flag is not set, e.g. DO = 0, for corresponding destinations, provided that they have routes to these destinations. If the DO flag is set to 1, which is default, only the destination MP can generate a RREP. If an intermediate MP receives a RREQ with the DO flag set to 0 for a destination and this intermediate MP already has a valid route to the destination, it issues a unicast RREP to the source. Furthermore, if the “Reply and Forward (RF)” flag is set to 1 for the destination, this intermediate MP forwards the RREQ with the DO flag for the destination set to 1 (the reason to set the DO flag to 1 is to suppress any RREP messages from the subsequent intermediate MPs). Otherwise, there is no RREQ forwarding at intermediate MP. The purpose of the “Destination Only” and “Reply and Forward” mechanisms is to enable an MP to quickly establish a route using the RREP generated by the intermediate MP and send data frames with a low route discovery delay and buffer requirement, while allowing that the route with the best path metric is chosen (or validated) after the route establishment procedure has been completed. The source sets the DO flag to 0 and RF flag to 1 for a destination in the RREQ only when it does not have a valid route and wants to discover a new route to this destination. As described below, the DO flag in the maintenance RREQ is always set to 1.

Intermediate MPs create a route to the destination on receiving the RREP, and also forward the RREP toward the source. When the source receives the RREP, it creates a route to the destination. If the destination receives further RREQs with a better metric, then the destination updates its route to the source to the new route and also sends a fresh RREP to the source along the updated route. A bidirectional, best metric end-to-end route is established between the source and destination.

In HWMP the RREQ processing at intermediate MPs is controlled per destination.

- Proactive tree building mode

There are two mechanisms for proactively disseminating routing information for reaching the root MP.  The first method uses a proactive Route Request (RREQ) message and is intended to create routes between the root MP and all MPs in the network proactively.   The second method uses a Root Announcement (RANN) message and is intended to distribute route information for reaching the root MP but the actual routes to the root MP can be built on-demand.

An MP configured as root MP would send either proactive RREQ or RANN messages periodically.

- Proactive RREQ mechanism

The RREQ tree building process begins with a proactive Route Request message sent by the root MP, with the destination address set to all ones (broadcast address), the DO flag set to 1 and the RF flag set to 1. The RREQ contains the distance metric (set to 0 by the root MP) and a sequence number.  The proactive RREQ is sent periodically by the root MP, with increasing sequence numbers.

Any MP hearing a proactive RREQ creates or updates its forwarding information to the root MP, updates the metric and hop count of the RREQ, records the metric and hop count to the root MP, and then transmits the updated RREQ. Information about the presence of and distance to available root MP(s) is disseminated to all MPs in the network.

Each MP may receive multiple copies of a proactive RREQ, each traversing a unique path from the root MP to the MP. An MP updates its current route to the root MP if and only if the RREQ contains a greater sequence number, or the sequence number is the same as the current route and the RREQ offers a better metric than the current route to the root MP.  The processing of the proactive RREQ is the same as in the on-demand mode described in On demand routing mode.

If the proactive RREQ is sent with the “Proactive RREP” bit set to 0, the recipient MP may send a gratuitous RREP if required (for example, if the MP has data to send to the root MP and requires establishing a bidirectional route with the root MP). If the RREQ is sent with a “Proactive RREP” bit set to 1, the recipient MP shall send a gratuitous RREP. The gratuitous RREP establishes the route from the root MP to the MP.  When the route from an MP to a root MP changes, and the root MP RREQ was sent with a “Proactive RREP” bit set to 1, the recipient MP shall send a gratuitous RREP to the root MP containing the addresses of the MPs which have established a route to the root MP through the current MP.

- Proactive RANN mechanism

The root MP periodically floods a RANN message into the network.  The information contained in the RANN is used to disseminate path metrics to the root MP.

Upon reception of a RANN, each MP that has to create or refresh a route to the root MP sends a unicast RREQ to the root MP via the MP from which it received the RANN.

The unicast RREQ follows the same processing rules defined in the on demand mode.

The root MP sends a RREP in response to each RREQ.  The unicast RREQ creates the reverse route from the root MP to the originating MP, while the RREP creates the forward route from the MP to the root MP.

When the route from an MP to a root MP changes, it may send a RREP with the addresses of the MPs which have established a route to the root MP through the current MP.

- Definitions

- Illustration of definitions

Originator: The originator is the MP that initiates an HWMP routing message that is flooded through the mesh network (e.g. RREQ). The transmission of the corresponding information element by the originator is called “original transmission”.

Forwarding Information: The forwarding information maintained by an MP consists of at least a destination, the Destination Sequence Number (DSN), the next hop to the destination, the path metric and the lifetime of this forwarding information. Stored forwarding information can be active or inactive (invalidated). The latter means that the forwarding information is still known for future reference but not used for forwarding.

Forward Route: The stored forwarding information of the path from the originator to the destination of a route discovery. Set up by route replies.

Reverse Route: The stored forwarding information of the path from the destination to the originator of a route discovery. Set up by route requests.

Unreachable destination: A destination is considered unreachable in two cases:

- An MP does not have active forwarding information for the destination and the MP did not receive a RREP in response to its route request for this destination within the set waiting time.

- The destination cannot be reached over the established path, because it contains a broken link.

Destination Sequence Number: An integer sequence number associated with an MP, which is used to distinguish newer from older routing information. See also Destination Sequence Number (DSN) (Destination Sequence Number).

Time-to-Live: An integer number that is used to limit the number of hops of frames in the mesh network. See also Time-to-Live (TTL).

Unicast/Broadcast RREQ: A “unicast RREQ” is a RREQ information element contained in a management frame that is transmitted in an individually addressed frame to the next hop toward the destination of the RREQ.  A “broadcast RREQ” is a RREQ information element contained in a management frame that is broadcast.

Local MP: The mesh point which is the reference MP for the corresponding descriptions.

Dependent MP: A neighboring MP which has established a route to the root MP through the local MP.

- General rules for processing HWMP information elements

This subclause describes the rules for the processing of the following components of the HWMP messages and IEs:

- Destination Sequence Number

- TTL

- Metric

Note: It is assumed that the receiving MP only receives HWMP messages from MPs with which it has established a secure link. Therefore, all HWMP messages received are presumed to have originated in the same mesh network that the receiving MP belongs to.

- Forwarding

Many HWMP IEs are intended to be flooded across a mesh network and therefore are forwarded by MPs that receive them. Each forwarding is subject to certain rules or limitations as explained in the following subclauses.  Some parameters in the HWMP IEs is updated before forwarding, e.g. path metric.

- Destination Sequence Number (DSN)

The DSN is included in the RANN, RREQ or RREP information elements. The DSN is updated whenever an MP receives new (i.e. not stale) information about the sequence number from a RREQ, RREP or RERR that may be received related to that destination MP.  HWMP depends on each MP in the network to own and maintain its destination sequence number to guarantee the loop-freedom of all routes towards that MP. A destination MP increments its own sequence number in two circumstances:

- Immediately before an MP originates a route discovery, it shall increment its own sequence number. This prevents conflicts with previously established reverse routes towards the originator of a RREQ.

- Immediately before a destination MP originates a RREP in response to a RREQ, it shall update its own sequence number to the maximum of its current sequence number and the destination sequence number in the RREQ packet.

In general, when an MP receives an information element with a DSN that is less than the last received DSN for that originator, it discards the received information element. If they are the same, the outcome (IE discarded or not) depends on the type of the information element and some additional conditions. These cases are noted in the applicable information element descriptions.

A destination MP shall increment its sequence number by treating the sequence number value as if it were an unsigned number. To accomplish sequence number rollover, if the sequence number has already been assigned to be the largest possible number representable as a 32-bit unsigned integer (i.e., 4294967295), then when it is incremented it then has a value of zero (0). On the other hand, if the sequence number currently has the value 2147483647, which is the largest possible positive integer if 2's complement arithmetic is in use with 32-bit integers, the next value is 2147483648, which is the most negative possible integer in the same numbering system. The representation of negative numbers is not relevant to the increment of HWMP sequence numbers. This is in contrast to the manner in which the result of comparing two HWMP sequence numbers is to be treated.

In order to ascertain that information about a destination is not stale, the MP compares its current numerical value for the sequence number with that obtained from the incoming HWMP message. This comparison shall be done using signed 32-bit arithmetic, this is necessary to accomplish sequence number rollover. If the result of subtracting the currently stored sequence number from the value of the incoming sequence number is less than zero, then the information related to that destination in the HWMP message shall be discarded, since that information is stale compared to the MP's currently stored information.

The only other circumstance in which an MP may change the destination sequence number in one of its forwarding information entries is in response to a lost or expired link to the next hop towards that destination MP.  The node determines which destinations use a particular next hop by consulting its forwarding information.  In this case, for each destination that uses the next hop, the MP increments the sequence number and marks the route as invalid (see also Error: Reference source not found). Whenever any forwarding information containing a sequence number at least equal to the recorded sequence number for an affected destination is received by an MP that has marked that forwarding information as invalid, the node will update its forwarding information according to the information contained in the update.

An MP may change the sequence number in the forwarding information of a destination MP if the link to the Next Hop MP towards the destination MP breaks.

- Time-to-Live (TTL)

The TTL is set by the originator of an HWMP information element. It gives the maximum number of hops that the information element is allowed to be forwarded.

A receiving MP decrements the value of the TTL field before forwarding the information element. If the decremented value is zero, the information element is discarded.

- Forwarding delay

In general, forwarding of an HWMP information element is not subject to a delay.  Exceptions exist for the RANN information element as described below.

- Forwarding information

The forwarding information maintained by an MP consists at least of a destination, the DSN of the destination, the next hop to the destination, the path metric to the destination and the lifetime of this forwarding information.

Stored forwarding information can be active or inactive (invalidated). The latter means that the forwarding information is still known for future references but not used for forwarding.

- Creation and update of forwarding information

HWMP path selection information elements create or update the forwarding information in the MPs that process these information elements. The creation and update of forwarding information follows the same rules for RREQ, RREP, and RANN. These rules are given below. “HWMP_IE” stands for the information element under consideration (RREQ, RREP, or RANN). Exceptions are stated in the corresponding subclauses of the descriptions of the information elements. In the following text, the “” indicates an entry into the local forwarding data base, i.e., the routing table.

- If the MP does not have any valid forwarding information to the originator of the HWMP_IE (hwmp_ie.originator_address), the value of the destination sequence number is set to hwmp_ie->destination_sequence_number ( destination sequence number), it creates this information from the field hwmp_ie.originator_address ( destination), the transmitter address of the management frame containing the HWMP_IE ( next hop), the accumulation of the value of field hwmp_ie.metric with the metric of the last link ( path metric), and the value of field hwmp_ie.lifetime ( lifetime).

- If the MP does not have stored any valid forwarding information to the transmitter of the HWMP_IE, it creates this information from the transmitter address of the management frame containing the HWMP_IE ( destination and next hop), the destination sequence number is set to invalid, the metric of the last link ( path metric), and the value of field hwmp_ie.lifetime ( lifetime).

- If the MP has stored any forwarding information to the originator of the HWMP_IE (hwmp_ie.originator_address), valid and the acceptance criteria of the corresponding IE as described in sections (Error: Reference source not found, Error: Reference source not found, Error: Reference source not found) are satisfied,  then the MP updates this forwarding information with the transmitter address of the management frame containing the HWMP_IE ( next hop), the value of the destination sequence number is set to hwmp_ie->destination _sequence_number ( destination sequence number), the accumulation of the value of field hwmp_ie.metric with the metric of the last link ( path metric), and the larger one of the lifetime of the stored forwarding information and the value of field hwmp_ie.lifetime (lifetime).

- If the MP has stored any valid forwarding information to the transmitter of the HWMP_IE and if the path metric of this information is larger than the metric of the last link, then the MP updates this forwarding information with the transmitter address of the management frame containing the HWMP_IE ( next hop), the destination sequence number is set to invalid, the metric of the last link ( path metric), and the larger one of the lifetime of the stored forwarding information and the value of field hwmp_ie.lifetime ( lifetime).

- Metric of last link

The term metric of last link specifies the current link metric between the transmitter of the information element under consideration and the MP that received this information element. The latter is the MP under consideration.

- Root Announcement (RANN)

This subclause describes the function, generation and processing of the Root Announcement information element.

- Function

The RANN element, described in 7.3.2.70, is used for announcing the presence of an MP configured as Root MP. RANN IEs are sent out periodically by the root MP.

The RANN information element propagates path metric information across the network so that each MP can select a best metric path to the announced root MP. This mechanism allows bidirectional trees to be built, using a robust procedure based on individually addressed frames initiated by the MPs.  This ensures that the root MP is aware of all MPs in the mesh.

Receiving MPs shall propagate the RANN as described below.

- Conditions for generating and sending a RANN

An MP sends out a RANN in the following cases:

Case A: Original transmission

All of the following applies:

- The MP is configured as a Root MP

- at every ROOT_ANNOUNCEMENT_INTERVAL

Content:

| Field | Value |
| --- | --- |
| ID | RANN information element ID |
| Length |  |
| Flags | Bit 0: Portal Role (0 = non-portal, 1 = portal) Bit 1 – 7: Reserved |
| Hop Count | 0 |
| Time to Live | Maximum number of hops allowed for this IE |
| Originator Address | Own MAC address |
| Destination Sequence Number | Last used DSN + 1 |
| Lifetime | HWMP_PATH2ROOT_TIMEOUT (shall be greater than ROOT_ANNOUNCEMENT_INTERVAL) |
| Metric | initial metric value (0 for airtime metric) |

Case B:	Forwarding

All of the following applies:

- the MP has received and accepted a RANN – See RANN Reception

- RANN_PROPAGATION_DELAY has expired – See Effect of receipt

Content:

| Field | Value |
| --- | --- |
| ID | As received |
| Length | As received |
| Flags | As received |
| Hop Count | As received + 1 |
| TTL | As received – 1 |
| Originator Address | As received |
| Destination Sequence Number | As received |
| Lifetime | As received |
| Metric | As received + link metric to the transmitting MP |

- RANN Reception

Received RANN IEs are subject to certain acceptance criteria. Processing and actions taken depend on the content of the RANN and the forwarding information maintained by the receiving MP.

See also General rules for processing HWMP information elements: General rules for message processing

- Acceptance criteria

The RANN is discarded if any of the following is true:

- The DSN < previous DSN from this originator

- (DSN = previous DSN) AND (updated path metric >= previous path metric)

- Effect of receipt

The following applies only to RANN that has not been discarded.

- The receiving MP shall initiate a timer for RANN_PROPAGATION_DELAY

- The receiving MP may

- initiate a RREQ/RREP exchange with the root MP to set up or update a route to the root MP.

See RREQ, when generated, case C

- The receiving MP shall record the Originator Address, together with the DSN, hopcount, metric.

- The receiving MP shall transmit a RANN as defined in Conditions for generating and sending a RANN, Case B

- Route Request  (RREQ)

This subclause describes the function, generation and processing of the Route Request information element.

- Function

The Route Request (RREQ) element, described in 7.3.2.71, is used for three purposes:

- Discovering a route to one or more destinations

- Building a proactive (reverse) routing tree to the root MP

- Confirming a route to a destination (optional)

- Conditions for generating and sending a RREQ

An MP shall send a RREQ element in a RREQ frame in the following cases:

Case A: Original Transmission (Route Discovery)

All of the following applies:

- The MP needs to establish an on-demand route to one or more destinations MPs or the proxy of a given destination for which there is no ongoing route discovery initiated by this MP.

- The MP has not sent more than (HWMP_RREQ_RATELIMIT – 1) route request messages during the last second. If this is the case, the transmission of the RREQ has to be postponed until this condition becomes true.

- The MP has not made more than (HWMP_MAX_RREQ_RETRIES – 1) repeated attempts at route discovery towards the destination of the RREQ.

Content:

| Field | Value |  |
| --- | --- | --- |
| ID | TBD |  |
| Length | 27 + N*11 |  |
| Flags | Bit 0: 0 (no portal role) Bit 1: 0 (broadcast) Bit 2: 0 (no proactive RREP applicable) Bit 6: Address Extension (AE) (1= if (destination count ==1 && proxied address present), 0 = otherwise) Bit 3 – 5, 7: Reserved |  |
| Hop Count | 0 |  |
| Time to Live | Maximum number of hops allowed for this information element, e.g., HWMP_NET_DIAMETER. |  |
| RREQ ID | Previous RREQ ID + 1 |  |
| Originator Address | Own MAC address |  |
| Originator’s Destination Sequence Number |  | Previous Originator DSN + 1. See Note 2 |
| Proxied Address |  | Present only if Bit 6 in Flags = 1. This value is set to the proxied address which is the source of the frame. |
| Lifetime |  | The time for which MPs receiving the RREQ consider the forwarding information to be valid, e.g. HWMP_ACTIVE_ROOT_TIMEOUT. |
| Metric |  | 0 |
| Destination Count |  | (N) |
| Per Destination Flags |  | DO flag, RF flag, as required |
| Destination Address |  | MAC address of requested destination |
| Destination Sequence Number |  | The latest sequence number received in the past by the originator for any route towards the destination. |

Note 1: Repeated attempts by an MP at route discovery towards a given (set of) destination(s) shall be limited to HWMP_MAX_RREQ_RETRIES and utilize a binary exponential backoff between transmissions. The minimum waiting time for the RREP corresponding to a RREQ is 2 * HWMP_RT_NETDIAMETER_TRAVERSAL_TIME.

Note 2: In order to improve route stability (and further reduce overhead), an MP may use the same Originator Destination Sequence Number (Originator DSN) for a certain time interval.  The Originator DSN is incremented only after at least HWMP_RT_NETDIAMETER_TRAVERSAL_TIME  has elapsed since the previous increment. This mechanism prevents MPs from changing the route frequently to the source every time the source sends a burst of RREQs within a very short time.  This element of the protocol allows a source MP to immediately initiate on-demand route discovery to a new destination without affecting recently refreshed routes to the source in other MPs.

Case B:	Original Transmission (Route Maintenance) (optional implementation enhancement)

All of the following applies:

- the MP has a route to a given destination that is not a Root MP

- the HWMP_ROUTE_MAINTENANCE_INTERVAL has expired

Content:

| Field | Value |  |  |
| --- | --- | --- | --- |
| ID | TBD |  |  |
| Length | As required |  |  |
| Flags | Bit 0: 0 (no portal role) Bit 1: 0 (broadcast) Bit 2: 0 (no proactive RREP applicable) Bit 6: 0 (no address extension) Bit 3 – 5, 7: Reserved |  |  |
| Hop Count | 0 |  |  |
| Time to Live | Maximum number of hops allowed for this information element = HWMP_NET_DIAMETER. |  |  |
| RREQ ID | Previous RREQ ID +1 |  |  |
| Originator Address | Own MAC address |  |  |
| Originator’s Destination Sequence Number |  | Originator DSN + 1. See Note 2 under Case A. |  |
| Lifetime |  | The time for which MPs receiving the RREQ consider the forwarding information to be valid, e.g. HWMP_ACTIVE_ROOT_TIMEOUT. |  |
| Metric |  | 0 |  |
| Destination Count |  | N |  |
| Per Destination |  | Flags = DO flag = 1, RF flag = 0 |  |
|  |  | Address = Destination MAC Address |  |
|  |  | Destination Sequence Number (for this destination) |  |

Case C: Root Path Confirmation.

All of the following applies:

- the MP has a route to a Root MP which broadcasts RANNs

- the HWMP_ROUTE_CONFIRMATION_INTERVAL has expired

Content:

| Field | Value |  |  |
| --- | --- | --- | --- |
| ID | TBD |  |  |
| Length | As required |  |  |
| Flags | Bit 0: 0 (no portal role) Bit 1: 1 (individually addressed) Bit 2: 0 (no proactive RREP applicable) Bit 6: 0 (no address extension) Bit 3 – 5, 7: Reserved |  |  |
| Hop Count | 0 |  |  |
| Time to Live | 1 |  |  |
| RREQ ID | Not used |  |  |
| Originator Address | Own MAC address |  |  |
| Originator’s Destination Sequence Number |  | Originator DSN + 1. See Note 2 under Case A. |  |
| Lifetime |  | The time for which MPs receiving the RREQ consider the forwarding information to be valid, e.g. HWMP_ACTIVE_ROOT_TIMEOUT. |  |
| Metric |  | 0 |  |
| Destination Count |  | 1 |  |
| Per Destination |  | Flags: DO flag = 1, RF flag = 0 |  |
|  |  | Address = Root MP MAC Address |  |
|  |  | Destination Sequence Number (for this destination) |  |
| Etc |  |  |  |

Case D:	RREQ Forwarding

Case D1 (destination count = 1, no RREP generation):

All of the following applies:

- the MP has received and accepted a RREQ – See Acceptance criteria

- Destination_count = 1

- the MP is not the destination of the RREQ OR the destination of the RREQ is the MAC broadcast address (all 1’s).

- the MP is not the proxy of the destination address.

- the MP has no active forwarding information for the requested destination

- [Destination Only flag of the destination in the RREQ is ON (DO = 1)] 
OR
[{Destination Only flag of the destination in the RREQ is OFF (DO = 0)} AND {MP has no active forwarding information for the requested destination rreq.destination_address}]

Content for D1:

| Field | Value |  |  |
| --- | --- | --- | --- |
| ID | TBD |  |  |
| Length | 37 |  |  |
| Flags | as received |  |  |
| Hop Count | As received + 1 |  |  |
| Time to Live | As received – 1 |  |  |
| RREQ ID | As received |  |  |
| Originator Address | As received |  |  |
| Originator’s Sequence Number |  | As received |  |
| Proxied Address |  | As received. This field is only present if bit 6 of the Flags field (AE flag) is set to 1. |  |
| Lifetime |  | As received |  |
| Metric |  | As receiver + own metric towards transmitter of received RREQ |  |
| Destination Count |  | 1 |  |
| Per Destination flags #1 |  | as received |  |
| Destination MAC address #1 |  | as received |  |
| Destination Sequence Number #1 |  | as received |  |

Case D2 (destination count = 1, RREP generation as intermediate MP):

All of the following applies:

- the MP has received and accepted a RREQ – See Acceptance criteria

- rreq.destination_count = 1

- the MP is not the destination of the RREQ

- the MP has active forwarding information for the requested destination rreq.destination_address

- Destination Only flag of the destination in the RREQ is OFF (DO = 0)

- Reply and Forward flag of the destination in the RREQ is ON (RF = 1)

Content for D2:

| Field | Value |  |  |
| --- | --- | --- | --- |
| ID | TBD |  |  |
| Length | 37 |  |  |
| Flags | As received |  |  |
| Hop Count | As received + 1 |  |  |
| Time to Live | As received – 1 |  |  |
| RREQ ID | As received |  |  |
| Originator Address | As received |  |  |
| Originator’s Sequence Number |  | As received |  |
| Proxied Address |  | As received. This field is only present if Bit 6 of the Flags field (AE flag) is set to 1. |  |
| Lifetime |  | As received |  |
| Metric |  | As receiver + own metric towards transmitter of received RREQ |  |
| Destination Count |  | 1 |  |
| Per Destination flags #1 |  | Bit 0 (DO): 1 (set to 1 before forwarding because MP sent a RREP) Bit 1 (RF): as received |  |
| Destination MAC address #1 |  | As received |  |
| Destination Sequence Number #1 |  | As received |  |

Case D3 (destination count > 1): All of the following applies

- the MP has received and accepted a RREQ – See Acceptance criteria

- there is at least one requested destination left after processing the RREQ according to RREQ processing.

Content for D3:

| Field | Value |  |  |
| --- | --- | --- | --- |
| ID | TBD |  |  |
| Length | 26 + N * 11 |  |  |
| Flags | As received |  |  |
| Hop Count | As received + 1 |  |  |
| Time to Live | As received – 1 |  |  |
| RREQ ID | As received |  |  |
| Originator Address | As received |  |  |
| Originator’s Sequence Number |  | As received |  |
| Lifetime |  | As received |  |
| Metric |  | As receiver + own metric towards the transmitter |  |
| Destination Count |  | 1  destination count  received destination count received destination count less the number of requested destinations, for which the processing MP - is the destination or - has active forwarding information for the requested destination and the corresponding Destination Only flag is off (DO=0) and Reply and Forward flag is on (RF = 1) |  |
| For the per destination fields (per destination flags, destination MAC address, destination sequence number) assume the following: - destination #A: If destination A would have been the only requested destination, it would generate a RREQ for forwarding according to case D1 - destination #B: If destination B would have been the only requested destination, it would generate a RREQ for forwarding according to case D2 |  |  |  |
| Per Destination Flags #A |  | As received |  |
| Destination MAC address #A |  | As received |  |
| Destination Sequence Number #A |  | As received |  |
| Per Destination Flags #B |  | Bit 0 (DO): 1 (set to 1 because MP sent RREP) Bit 1 (RF): as received |  |
| Destination MAC address #B |  | As received |  |
| Destination Sequence Number #B |  | As received |  |

Case E:	Proactive RREQ (original transmission)

All of the following applies:

- The Root MP is configured to send proactive root RREQs

- The Root Announcement interval has expired

Content:

| Field | Value |  |  |
| --- | --- | --- | --- |
| ID | TBD |  |  |
| Length | 37 |  |  |
| Flags | Bit 0: As needed (portal role) Bit 1: 0 (broadcast) Bit 2: As needed (proactive RREP) Bit 6: 0 (no address extension) Bit 3 – 5, 7: Reserved |  |  |
| Hop Count | 0 |  |  |
| Time to Live | Maximum number of hops allowed for this information element, e.g. HWMP_NET_DIAMETER. |  |  |
| RREQ ID | Previous RREQ ID + 1 |  |  |
| Originator Address | root MP MAC address |  |  |
| Originator’s Destination Sequence Number |  | Previous DSN of root MP + 1 |  |
| Lifetime |  | HWMP_PATH2ROOT_TIMEOUT (shall be greater than ROOT_ANNOUNCEMENT_INTERVAL) |  |
| Metric |  | 0 |  |
| Destination Count |  | 1 |  |
| Per Destination |  | DO = 1, RF = 1 |  |
|  |  | Address = All ones (broadcast address) |  |
|  |  | Destination Sequence Number (none) |  |

- RREQ processing

Received RREQ IEs are subject to certain acceptance criteria. Processing and actions taken depend on the contents of the RREQ and the information available to the receiving MP.

See also General rules for processing HWMP information elements: General rules for message processing

- Acceptance criteria

The RREQ is discarded if any of the following is true:

- The Originator DSN < previous Originator DSN

- (DSN = previous DSN) AND (updated path metric >= previous path metric)

Otherwise, the RREQ information element is accepted.

See also General rules for processing HWMP information elements: General rules for message processing

- Effect of receipt

The following applies only to a RREQ that has not been discarded:

- The receiving MP shall record the RREQ ID, the Originator Address, and entries for each destination MAC Address and DSN

- The receiving MP shall update the active forwarding information it maintains for the originator and previous hop MPs of the RREQ (see Creation and update of forwarding information)

- If the MP is addressed by the RREQ or is the proxy of the destination MAC address it will initiate the transmission of an RREP to the originator (11A.6.6.2 Case A). If the RREQ carries a proxied address (indicated by the AE flag), the MP updates its proxy information with the proxied address and sets the RREQ originator address as the corresponding proxy.

- If step 3 has not been applicable for the MP and the AE flag in the RREQ is set to 1, the MP may update its proxy information with proxied address and setting the RREQ originator address as corresponding proxy.

- If the MP has active forwarding information to any of the requested destinations and the DO flag for such a destination is OFF (DO=0), it initiates the transmission of a RREP to each of these destinations (see Conditions for generating and sending a RREP Case A)

- If there are destinations in the RREQ that have been not processed in steps 3 or 4 or that have been processed in step 4 but the corresponding Reply and Forward Flag is ON (RF = 1),  the receiving MP shall forward the RREQ as defined in Conditions for generating and sending a RREQ, Case D.

- If the MP is initiating a RREP transmission on behalf of another destination (intermediate reply), it should update its forwarding information by placing the last hop MP (from which it received the RREQ) into the precursor list for the forward route entry for the destination. In addition, this intermediate MP also updates its forwarding information for the MP originating the RREQ by placing the next hop towards the destination in the precursor list for the reverse route entry

- Route Reply (RREP)

This subclause describes the function, generation and processing of the Route Reply information element.

- Function

The purpose of the Route Reply (RREP) element, described in 7.3.2.72, is

- to establish a forward route to a destination and

- to confirm that a destination is reachable.

- Conditions for generating and sending a RREP

An MP sends out a RREP element in a RREP frame in the following cases:

Case A: Original transmission

A RREP is transmitted if the MP has received a RREQ fulfilling all of the following conditions:

- One of the following applies:

- The Destination Address of the RREQ is the same as MAC address of the receiving MP

- The destination address of the RREQ = all 1’s (broadcast) and the RREP flag is set to 1 (”Proactive RREP”)

- The destination address of the RREQ is currently proxied by the MP

- One of the following applies:

- The Originator DSN of the RREQ (rreq.orig_dsn) is greater than the DSN of the last RREQ received from the same originator address (which includes the case that there is no route to the originating MP)

- The Metric is better than the path selection metric currently associated with the Originator Address and the Originator DSN of the RREQ (rreq.orig_dsn) is equal to the DSN of the last RREQ received from the same originator address

The content of the generated RREP shall be:

| Field | Value |  |
| --- | --- | --- |
| ID | RREP information element ID |  |
| Length | As required |  |
| Flags | Bit 6: Address Extension (AE) (1= proxied device address present, 0 = otherwise) 0 – 5, 7: Reserved |  |
| Hop Count | 0 |  |
| Time to Live | Maximum number of hops allowed for this IE |  |
| Destination MP Address | Own MAC address |  |
| Destination Sequence Number | Sequence number of the target of the RREQ |  |
| Destination Proxied Address | Proxied address on behalf of which RREP is sent. Present only if Bit 6 (AE) in Flags = 1. |  |
| Lifetime | As per the RREQ that triggered the transmission of this RREP |  |
| Metric | 0 |  |
| Originator Address | MAC address of the originator (of the RREQ) |  |
| Originator Sequence Number | Sequence number of the originator (of the RREQ) |  |
| Dependent MP Count N | N |  |
| Dependent MP MAC Address # | MAC address of dependent MP |  |
| Dependent MP DSN # | Destination Sequence Number associated with MAC address of dependent MP |  |

Note: the destination address of the action frame carrying the RREP information element is the Originator Address of the RREQ that triggered the RREP.

Case B:	RREP Propagation

A RREP is propagated if all of the following applies:

- the MP has received and accepted the RREP – See Acceptance criteria

- the MP is not the destination of the RREP

Content:

| Field | Value |
| --- | --- |
| ID | As received |
| Length | As received |
| Flags | As received |
| Hop Count | As received + 1 |
| Time to Live | As received – 1 |
| Destination MP Address | As received |
| Destination Sequence Number | As received |
| Destination Proxied Address | As received |
| Lifetime | As received |
| Metric | As received own metric towards the transmitting MP |
| Originator Address | MAC address of the originator (of the RREQ) |
| Originator Sequence Number | Sequence number of the originator (of the RREQ) |
| Dependent MP Count N | N |
| Dependent MP MAC Address # | MAC address of dependent MP |
| Dependent MP DSN # | Destination Sequence Number associated with MAC address of dependent MPAs received |

Note: the destination address of the action frame carrying the RREP information element is the next hop to the Originator Address of the RREQ or RANN that triggered the RREP.

Case C:	Intermediate reply

A RREP is transmitted if the MP has received a RREQ fulfilling all of the following conditions:

- The RREQ Destination Only flag is set to 0

- The receiving MP has  active forwarding information with:

- A destination which is the same as the Destination Address of the RREQ

- A DSN which is greater than or equal to the DSN of the RREQ (rreq.dest_dsn)

- A path metric which is greater than or equal to the Metric or the RREQ

- A non-zero lifetime

The content of the generated RREP shall be:

| Field | Value |
| --- | --- |
| ID | RREP information element ID |
| Length | As required |
| Flags | Bit 0: 0 1– 7: Reserved |
| Hop Count | 0 |
| Time to Live | Maximum number of hops allowed for this IE |
| Destination MP Address | Destination MAC address from the RREQ |
| Destination Sequence Number | DSN of the stored forwarding information of the Destination MAC address of the RREQ |
| Destination Proxied Address | As received |
| Metric | 0 |
| Originator Address | MAC address of the originator (of the RREQ) |
| Originator Sequence Number | Sequence number of the originator (of the RREQ) |
| Dependent MP Count N | N |
| Dependent MP MAC Address # | MAC address of dependent MP |
| Dependent MP DSN # | Destination Sequence Number associated with MAC address of dependent MP |

Case D: Gratuitous RREP

All of the following applies

- The MP has a received RREQs broadcast with the "Proactive RREP" flag set to 0

- The MP needs the root MP to establish a path to itself

The content of the generated RREP shall be:

| Field | Value |
| --- | --- |
| ID | RREP IE ID |
| Length | As required |
| Flags | Bit 0: 0 1– 7: Reserved |
| Hop Count | 0 |
| Time to Live | Maximum number of hops allowed for this IE |
| Destination MP Address | Own MAC address |
| Destination Sequence Number | DSN of the originator of the RREP |
| Destination Proxied Address | 0 |
| Metric | 0 |
| Originator Address | MAC address of the originator (of the RREQ) |
| Originator Sequence Number | Sequence number of the originator (of the RREQ) |
| Dependent MP Count N | N |
| Dependent MP MAC Address # | MAC address of dependent MP |
| Dependent MP DSN # | Destination Sequence Number associated with MAC address of dependent MP |

- RREP processing

Received RREP IEs are subject to certain acceptance criteria. Processing and actions taken depend on the contents of the RREP and the information available to the receiving MP.

- Acceptance criteria

The RREP is discarded if any of the following is true:

- The DSN < previous DSN from this originator

- The Time to Live is 1 or less

- Effect of receipt

The following applies only to a RREP that has not been discarded

- The receiving MP shall record the Originator Address, together with the DSN, hopcount and metric according to the rules defined in Creation and update of forwarding information.

- The receiving MP may record the list of dependent MPs if present in the RREP.

- If the receiving MP is not the final destination of the RREP, the RREP is forwarded as per Case B above.

- If the receiving MP is the final destination of the RREP and the AE-flag is set, the MP shall store the proxy information with the destination proxied address and sets the corresponding proxy as the destination MP address.

- If the receiving MP is not the final destination of the RREP and the AE-flag is set, the MP may store the proxy information with the destination proxied address and sets the corresponding proxy as the destination MP address.

- If the MP propagates the RREP, the precursor list for the destination address is updated by adding the next hop MP to which the RREP is propagated. In addition, at the MP the precursor list for the originator address is updated by adding the next hop MP towards the destination address.

- Route Error information element (RERR)

This subclause describes the function, generation and processing of the route error information element.

- Function

The RERR information element is used for announcing a broken link to all traffic sources that have an active path over this broken link.  The active forwarding information associated with the unreachable destinations should no longer be used for forwarding.

A RERR IE may be either broadcast (if there are many precursors), unicast (if there is only one precursor), or iteratively unicast to all precursors depending on the size of precursor list for the destinations. The RERR IE should contain those destinations that are part of the created list of unreachable destinations and have a non-empty precursor list. The neighboring node(s) that should receive the RERR are all those that belong to a precursor list of at least one of the unreachable destination(s) in the newly created RERR.

A RERR information element is propagated by MPs receiving a RERR if certain conditions are fulfilled.

An MP generating or receiving a RERR may decide to establish routes to unreachable destinations using any of the available HWMP mechanisms.

- Conditions for generating and sending a RERR

An MP sends out a RERR in the following cases:

Case A: Original transmission

All of the following applies:

- The MP detects a link break to the next hop of an active path in its stored forwarding information while transmitting frames to it.  Note: the detection may be triggered by the fact that an MP is unable to forward a data frame to a next hop MP.

- The MP did not send more than HWMP_RERR_RATELIMIT – 1 RERR messages during the last second.

Actions before sending the RERR:

- The MP first makes a list of unreachable destinations consisting of the unreachable MP and any additional destinations in the local forwarding table that uses the unreachable MP as the next hop.

- The destination sequence numbers in all valid stored forwarding information of unreachable destinations announced in this RERR is incremented.

- The stored forwarding information for each unreachable destination announced in this RERR is invalidated.

Content:

| Field | Value |
| --- | --- |
| ID | RERR information element ID |
| Length | 2 + N * 10 |
| Mode Flags | Bit 0 – 7: Reserved |
| Number of Destinations | Number of announced unreachable destinations in RERR. A destination is unreachable if its next hop in the stored forwarding information is an unreachable neighbor. |
| Destination Address | MAC address of detected unreachable destination #1 |
| Destination Sequence Number | Last used DSN for Destination Address #1 + 1 |

Case B:	RERR Propagation

All of the following applies:

- The MP creates a list of unreachable destinations which consists of those destinations in the RERR for which there exists valid forwarding information that has the transmitter of the RERR as the next hop.

- The MP received a RERR from a neighbor for one or more of its active paths in its stored forwarding information.

- The MP has not sent or forwarded more than HWMP_RERR_RATELIMIT – 1 RERR messages during the last second.

Content:

| Field | Value |
| --- | --- |
| ID | RERR information element ID |
| Length | 2 + N * 10 |
| Mode Flags | Bit 0 – 7: Reserved |
| Number of Destinations | Number of announced unreachable destinations in RERR ( received value) A destination is unreachable if its next hop in the corresponding stored forwarding information is the transmitter of the received RERR. |
| Destination Address | MAC address of detected unreachable destination #1 (as received, but maybe at different position in destination list) |
| Destination Sequence Number | as received (but maybe at different position in destination list) |

- RERR Reception

Received RERR IEs are subject to certain acceptance criteria. Processing and actions taken depend on the contents of the RERR and the information available to the receiving MP.

See also General rules for processing HWMP information elements: General rules for message processing

- Acceptance criteria

The RERR is not discarded if the following applies:

- The MP that receives the RERR has forwarding information stored where

- the destination is contained in the list of unreachable destinations of the RERR and

- the next hop is the transmitter of the received RERR

- Effect of receipt

The following applies only to RERR that has not been discarded.

- The destination sequence numbers in all valid stored forwarding information of unreachable destinations announced in this RERR is set to the values taken from the corresponding fields of the RERR (rerr.dsnX) if the latter is greater than the destination sequence number currently stored in the forwarding information ( destination sequence number).

- The stored forwarding information of unreachable destinations announced in this RERR is invalidated.

- The receiving MP shall transmit a RERR as defined in Conditions for generating and sending a RANN, Case B

- HWMP parameters

See T.3 for recommended parameter values.

- to Annex T per CID 3447