| A-MSDU Protection |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-03-09 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Nancy Cam-Winget | Cisco | 190 W Tasman Dr, San Jose, CA 95134 | 408 853 0532 | ncamwing@cisco.com |
| Doug Smith | Cisco Systems | 500 Alden Road, Ste 207 Markham, Ontario L3R 5H5 Canada | +1-905-470-4803 | dsmit@cisco.com |
| Brett Douglas | Cisco Systems | 301 S. Prospect Rd, Ste. 4 Bloomington, Il 61704-4908 | +1 309 661 7656 | brettld@cisco.com |

This submission defines a means to allow the protection of A-MSDUs and coexistence with STAs that do not support A-MSDU protection.

TGn Editor: Include into the TGn draft with the following text:

### 7.3.2.25.3 RSN capabilities

Replace Figure 91 with the following figure that usurps 2 bits to signal the A-MSDU protection capability and HT security capability bits, changing "Reserved" as appropriate:

Editorial note: the bits are currently positioned in B10 and B11 so as not to collide with other current drafts, but the fields are left as <ANA1> and <ANA2>  as they are to be assigned by ANA.

| B0 | B1 | B2-3 | B4-5 | B6 | B7 | B8 | B9 | <ANA1> | <ANA2> | B12-15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pre-Auth | No Pairwise | PTKSA Replay Counter | GTKSA Replay Counter | AES-128-CMAC | Robust Management Frame protection | Reserved | Peerkey Enabled | A-MSDU protection | HT security | Reserved |

Figure 91  RSN Capabilities field format

Insert the following text in the appropriate Bit assignment order in 7.3.2.25.3:

<ANA1> : A-MSDU protection.  A STA sets the A-MSDU protection subfield of the RSN Capabilities field to 1 to signal it supports A-MSDU protection (see 11.18).

<ANA2>: HT security.  A STA sets the HT security subfield of the RSN Capabilities field to 1 when it discards unprotected A-MSDUs (see 11.18).

### 8.3.3.3.2 Construct AAD

Change item " g)"  of the last paragraph in 8.3.3.3.2 Construct AAD as follows:

g) QC – QoS Control field, if present, a 2-octet field that includes the MSDU priority. The QC TID and, when dot11AMSDUProtected is TRUE, bit 7 (A-MSDU presence) are is used in the construction of the AAD, and the .  The remaining QC fields are set to 0 for the AAD calculation (bits 4 to 6 and bits 8 to 15 are set to 0).

Add new subclause “11.18 A-MSDU protection” after “11.17 STA-STA HT Information Exchange”

### 11.18 A-MSDU Protection procedures

A-MSDU protection is valid only if RSNA is selected to protect data messages and dot11AMSDUProtected is set to TRUE.  A STA set the A-MSDU Protection capability field in the RSNIE to 1 when dot11AMSDUProtected is TRUE;  the HT Security capability field in the RSNIE shall be set to 1 when dot11HTSecurity is TRUE.

An RSNA and HT-capable STA may choose to associate with, RSNA STAs with or without the capability for A-MSDU protection set in the RSNIE and with or without the HT security set in the RSNIE.  A STA may choose not to associate with a STA that does not advertise A-MSDU protection or HT Security in the RSN capabilities.  The STA procedures for an ESS and IBSS are defined in Figure n61-A.

| STA State | STA Action |
| --- | --- |
| A-MSDU Protection = 0, HT Security = 0 | May transmit unprotected A-MSDU Shall receive unprotected A-MSDU Shall discard protected A-MSDU |
| A-MSDU Protection = 0, HT Security = 1 | Shall not transmit protected or unprotected A-MSDU Shall discard unprotected A-MSDU Shall discard protected A-MSDU |
| A-MSDU Protection = 1, HT Security = 0 | Shall not transmit unprotected A-MSDU If peer STA is A-MSDU Protection capable then - May transmit protected A-MSDU - Shall receive protected A-MSDU - Shall discard unprotected A-MSDU If peer STA is not A-MSDU Protection capable then - Shall receive unprotected A-MSDU - Shall discard protected A-MSDU |
| A-MSDU Protection = 1, HT Security = 1 | Shall not transmit unprotected A-MSDU Shall discard unprotected A-MSDU If peer STA is A-MSDU Protection capable then - May transmit protected A-MSDU - Shall receive protected A-MSDU If peer STA is not A-MSDU Protection capable then - Shall discard protected A-MSDU |

Figure n61-A   A-MSDU Protection Procedures

### Annex D

Add the following text in Annex D at the end of the "dot11HTStationConfigEntry" SEQUENCE, as new entries:

dot11AMSDUProtected	TruthValue,

dot11UnprotectedAMSDUAllowed	TruthValue

Add the following text in Annex D after the last "dot11RDResponderOptionImplemented" OBJECT-TYPE:

dot11AMSDUProtected	OBJECT-TYPE

MAX-ACCESS	read-write

Status			current

DESCRIPTION

"This attribute, when TRUE, indicates that the STA implementation is capable of protecting A-MSDU.  The default value of this attribute is FALSE."

::= { dot11HTStationConfigEntry 14 }

dot11HTSecurity	OBJECT-TYPE

MAX-ACCESS	read-write

Status			current

DESCRIPTION

"This attribute, when TRUE, indicates that the STA is configured to disallow the reception of unprotected A-MSDUs.  The default value of this attribute is FALSE."

::= { dot11HTStationConfigEntry 15 }

References:

IEEE P802.11n/ D2.0