| A-MSDU Protection |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-03-09 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | Email |
| Nancy Cam-Winget | Cisco | 190 W Tasman Dr, San Jose, CA 95134 | 1 408 853 0532 | ncamwing@cisco.com |
| Doug Smith | Cisco Systems | 500 Alden Road, Ste 207 Markham, Ontario L3R 5H5 Canada | 1 905 470 4803 | dsmit@cisco.com |
| Brett Douglas | Cisco Systems | 301 S. Prospect Rd, Ste. 4 Bloomington, Il 61704-4908 | 1 309 661 7656 | brettld@cisco.com |

This submission defines a means to allow the protection of A-MSDUs and coexistence with STAs that do not support A-MSDU protection.

TGn Editor: Include into the TGn draft with the following text:

### 7.3.2.25.3 RSN capabilities

Replace Figure 91 with the following figure that usurps 2 bits to signal the A-MSDU bolster capability and A-MSDU authentication required bits, changing "Reserved" as appropriate:

Editorial note: the bits are currently positioned in B10 and B11 so as not to collide with other current drafts, but the fields are left as <ANA1> and <ANA2>  as they are to be assigned by ANA.

| B0 | B1 | B2-3 | B4-5 | B6 | B7 | B8 | B9 | <ANA1> | <ANA2> | B12-15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pre-Auth | No Pairwise | PTKSA Replay Counter | GTKSA Replay Counter | AES-128-CMAC | Robust Management Frame protection | Reserved | Peerkey Enabled | A-MSDU Bolster | A-MSDU Authentication Required | Reserved |

Figure 91  RSN Capabilities field format

Insert the following text in the appropriate Bit assignment order in 7.3.2.25.3:

<ANA1> : A-MSDU Bolster.  A STA sets the A-MSDU Bolster subfield of the RSN Capabilities field to 1 to signal it supports A-MSDU bolster (see 11.18).

<ANA2>: A-MSDU Authentication Required.  A STA sets the A-MSDU Authentication Required subfield of the RSN Capabilities field to 1 when it disallows Protected A-MSDUs communication.  (see 11.18).

Insert the following text at the end of 7.3.2.25.3:

Table n61-A defines three different types of A-MSDUs: Unprotected  A-MSDUs, Protected A-MSDUs, and Bolstered A-MSDUs. An Unprotected A-MSDU is not encrypted.  Protected A-MSDUs and Bolstered A-MSDUs are both encrypted. A Protected A-MSDU does not include the A-MSDU bit, bit 7, in the QOS control field in the construction of the AAD. A Bolstered A-MSDU does include the A-MSDU bit, bit 7, in the QOS control field in the construction of the AAD.

| Terminology | Frame Control Protected Frame Bit | QOS control A-MSDU bit treatment |
| --- | --- | --- |
| Unprotected A-MSDU | 0 | not applicable |
| Protected A-MSDU | 1 | not included in AAD construction |
| Bolstered A-MSDU | 1 | included in AAD construction |

Table n61-A   A-MSDU Type Definitions

### 8.3.3.3.2 Construct AAD

Change item " g)"  of the last paragraph in 8.3.3.3.2 Construct AAD as follows:

g) QC – QoS Control field, if present, a 2-octet field that includes the MSDU priority. The QC TID and, when dot11AMSDUBolstered is TRUE, bit 7 (A-MSDU presence) are is used in the construction of the AAD, and the .  The remaining QC fields are set to 0 for the AAD calculation (bits 4 to 6 and bits 8 to 15 are set to 0).

Add new subclause “11.18 A-MSDU Bolster” after “11.17 STA-STA HT Information Exchange”

### 11.18 A-MSDU procedures

When dot11RSNAEnabled is TRUE, a STA asserts support for Protected A-MSDU or Bolstered A-MSDU during (re)association.  At (re)association, both STA and peer STA maintain an indication as to whether the A-MSDU is to be Protected A-MSDU or Bolstered A-MSDU based on the negotiated A-MSDU Bolster and A-MSDU Authentication Required RSNIE settings.

Note: If the peer STA should transmit the incorrect type of A-MSDU, the decryption process will fail, and the A-MSDU shall be discarded.

A STA sets the A-MSDU Bolster field in the RSNIE to 1 when dot11AMSDUBolstered is TRUE;  the A-MSDU Authentication Required field in the RSNIE shall be set to 1 when dot11AMSDUAuthenticationRequired is TRUE.

An RSNA and HT-capable STA may choose to associate with RSNA STAs with or without the capability for A-MSDU Bolster protection set in the RSNIE and with or without the A-MSDU Authentication Required set in the RSNIE.  A STA may choose not to associate with a STA that does not advertise A-MSDU Bolster protection or A-MSDU Authentication Required in the RSN capabilities.  The STA behavior for transmitting and receiving A-MSDUs in both an ESS and IBSS is defined in Figure n61-B and Figure n61-C.

Unprotected A-MSDUs may be transmitted across the link regardless of either of the two capabilities bits, A-MSDU Bolster and A-MSDU Authentication Required.   Two HT and RSNA capable STAs may form an unencrypted HT link by the exclusion of the RSNIE during (re)association.  If the link is unprotected, then unencrypted A-MSDU may only be used for packets that would normally be transmitted unencrypted.

Figure n61-B defines the transmit and receive behaviour of a STA that has successfully negotiated an HT and RSNA (re)association.

| <<STA> <State>> | STA Action |
| --- | --- |
| A-MSDU Bolster = 0, A-MSDU Authentication Required = 0 | If the link is not using encryption, then unprotected (i.e. unencrypted) A-MSDU may be transmitted, and shall be received. Protected A-MSDU may be transmitted and shall be received. Bolstered A-MSDU shall not be transmitted and will not be received because decryption will fail. |
| A-MSDU Bolster = 0, A-MSDU Authentication Required = 1 | Protected A-MSDU shall not be transmtited Bolstered A-MSDU shall not be transmitted. An A-MSDU with the Protected bit set to 1 in the Frame Control field shall be discarded. |
| A-MSDU Bolster = 1, A-MSDU Authentication Required = 0 | If the link is using encryption, e.g. RSN is negotiated, and the peer STA is A-MSDU Bolster capable, then: - Protected A-MSDU shall not be transmitted, and will not be received because decryption wil fail. - Bolstered A-MSDU may be transmitted, and shall be received. If the link is using encryption, e.g. RSN is negotiated, and the peer STA is not A-MSDU Bolster capable, then: - Protected A-MSDU may be transmitted, and shall be received. - Bolstered A-MSDU shall not be transmitted, and will not be received because decryption wil fail. |
| A-MSDU Bolster = 1, A-MSDU Authentication Required = 1 | If the link is using encryption, and the peer STA is A-MSDU Bolster capable, then: - Protected A-MSDU shall not be transmtited, and will not be received because decryption wil fail. - Bolstered A-MSDU may be transmitted, and shall be received. If the link is using encryption, and the peer STA is not A-MSDU Bolster capable, then: - Protected A-MSDU shall not be transmtited, and shall not be received. - Bolstered A-MSDU shall not be transmitted, and shall not be received |

Table n61-B   A-MSDU STA Behavior

The behavior defined in Table n61-B determine the link behavior for the three different types of A-MSDUs.  That link behavior is summarized in Table n61-C.  Unprotected A-MSDUs may be transmitted across the link regardless of either of the two capabilities bits, A-MSDU Bolster and A-MSDU Authentication Required.  Protected A-MSDUs may only be transmitted if A-MSDU Authentication Required is FALSE for both STAs on the link.  Bolstered A-MSDUs may be transmitted only if both STAs have A-MSDU Bolstered set to TRUE.

| A-MSDU Authentication Required OR Peer A-MSDU Authentication Required | A-MSDU Bolster AND Peer A-MSDU Bolster | Allow Unprotected A-MSDU | Allow Protected A-MSDU | Allow Bolstered A-MSDU |
| --- | --- | --- | --- | --- |
| FALSE | FALSE | Y | Y | N |
| FALSE | TRUE | Y | N | Y |
| TRUE | FALSE | Y | N | N |
| TRUE | TRUE | Y | N | Y |

Table n61-C   A-MSDU Link Behavior

### Annex D

Add the following text in Annex D at the end of the "dot11HTStationConfigEntry" SEQUENCE, as new entries:

dot11AMSDUBolstered		TruthValue,

dot11AMSDUAuthenticationRequired	TruthValue

Add the following text in Annex D after the last "dot11RDResponderOptionImplemented" OBJECT-TYPE:

dot11AMSDUBolstered	OBJECT-TYPE

MAX-ACCESS	read-write

Status			current

DESCRIPTION

"This attribute, when TRUE, indicates that the STA implementation is capable of protecting the A-MSDU bit (Bit 7) in the QOS Control Field when dot11RSNAEnabled is TRUE..  The default value of this attribute is FALSE."

::= { dot11HTStationConfigEntry 14 }

dot11AMSDUAuthenticationRequired	OBJECT-TYPE

MAX-ACCESS	read-write

Status			current

DESCRIPTION

"This attribute, when TRUE, indicates that the STA is configured to disallow the reception of unbolstered A-MSDUs when dot11RSNAEnabled is TRUE.   The default value of this attribute is FALSE."

::= { dot11HTStationConfigEntry 15 }

References:

IEEE P802.11n/ D2.0