| A-MSDU Protection |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-04-19 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | Email |
| Nancy Cam-Winget | Cisco Systems | 190 W Tasman Dr, San Jose, CA 95134 | 1 408 853 0532 | ncamwing@cisco.com |
| Doug Smith | Cisco Systems | 500 Alden Road, Ste 207 Markham, Ontario L3R 5H5 Canada | 1 905 470 4803 | dsmit@cisco.com |
| Brett Douglas | Cisco Systems | 301 S. Prospect Rd, Ste. 4 Bloomington, Il 61704-4908 | 1 309 661 7656 | brettld@cisco.com |
| Matthew Fischer | Broadcom | 190 Mathilda Place Sunnyvale, CA 94086 | +1 408 543 3370 | mfischer@broadcom.com |
| Henry Ptasinski | Broadcom | 190 Mathilda Place Sunnyvale, CA 94086 | +1 408 543 3316 | henryp@broadcom.com |
| Solomon Trainin | Intel Corporation | POB 1659, Haifa 31015, Israel | +972547885738 | solomon.trainin@intel.com |
| Adrian Stephens | Intel Corporation |  | +1 (503) 616-3800 | adrian.p.stephens@intel.com |

This submission defines a means to allow the protection of A-MSDUs and coexistence with STAs that do not support A-MSDU protection.

TGn Editor: Include into the TGn draft with the following text:

### 3. Definitions

Insert the following definitions in their appropriate alphabetical order:

Editorital note:  Based on the 802.11-2007 specification, Payload Protected A-MSDU should follow 3.114 and Signaling and Payload Protected A-MSDU should follow 3.135

### 3.114n Payload Protected A-MSDU (PP A-MSDU): An A-MSDU that is CCMP protected but does not include the A-MSDU Present field (bit 7 of the QoS control field) in the construction of the AAD.

3.135n Signaling and Payload Protected A-MSDU (SPP A-MSDU): An A-MSDU that is CCMP protected and does include the A-MSDU Present field (bit 7 of the Qos control field) in the construction of the AAD.

### 4.Abbreviations and acronyms

Insert  the following acronyms in their alphabetical order:

PP A-MSDU	Payload Protected A-MSDU

SPP A-MSDU	Signaling and Payload Protected A-MSDU

### 7.3.2.25.3 RSN capabilities

Replace Figure 91 with the following figure that assigns 2 bits to signal the SPP A-MSDU capability and SPP A-MSDU required bits, changing "Reserved" as appropriate:

Editorial note: the bits are currently positioned in B10 and B11 so as not to collide with other current drafts, but the fields are left as <ANA1> and <ANA2>  as they are to be assigned by ANA.  The actual bit positions will be inserted once the 802.11 ANA has assigned them.

| B0 | B1 | B2-3 | B4-5 | B6 | B7 | B8 | B9 | <ANA1> | <ANA2> | B12-15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pre-Auth | No Pairwise | PTKSA Replay Counter | GTKSA Replay Counter | AES-128-CMAC | Robust Management Frame protection | Reserved | Peerkey Enabled | SPP A-MSDU Capable | SPP A-MSDU Required | Reserved |

Figure 91  RSN Capabilities field format

Insert the following text in the appropriate Bit assignment order in 7.3.2.25.3:

-- bit <ANA1> : SPP A-MSDU Capable.  A STA sets the SPP A-MSDU Capable subfield of the RSN Capabilities field to 1 to signal that it supports SPP A-MSDU (see 11.18).   Otherwise, it shall be set to 0.

-- bit <ANA2>: SPP A-MSDU Required.  A STA sets the A-MSDU Authentication Required subfield of the RSN Capabilities field to 1 when it disallows (i.e. will not send or receive) PP A-MSDUs .  (see 11.18).  Otherwise, it shall be set to 0.

### 8.3.3.3.2 Construct AAD

Change item " g)"  of the last paragraph in 8.3.3.3.2 Construct AAD as follows:

g) QC – QoS Control field, if present, a 2-octet field that includes the MSDU priority. The QC TID and, only when dot11SPPAMSDUCapable is TRUE, bit 7 (the A-MSDU Present field) are is used in the construction of the AAD, and the .  The remaining QC fields are set to 0 for the AAD calculation (bits 4 to 6, bits 8 to 15, and bit 7 when dot11SPPAMSDUCapable is FALSE are set to 0 ).

Add new subclause “11.18 RSNA A-MSDU procedures” after “11.17 STA-STA HT Information Exchange”

### 11.18 RSNA A-MSDU procedures

When dot11RSNAEnabled is TRUE, a STA indicates support for PP A-MSDU or SPP A-MSDU during (re)association.  On (re)association, both STA and its peer STA determine and maintain a record of whether an encrypted A-MSDU sent to its peer is to be PP A-MSDU or SPP A-MSDU based on both STAs SPP A-MSDU Capable and SPP A-MSDU Required RSNIE settings.

Note: If the peer STA transmits the wrong type of A-MSDU, the decryption process fails, and the A-MSDU is discarded.

The SME of an RSNA and HT-capable STA may choose to associate with RSNA STAs with or without the SPP A-MSDU Capable field set in the RSNIE and with or without the SPP A-MSDU Required field set in the RSNIE.  A STA may choose not to associate with a STA that does not advertise SPP A-MSDU Capable or SPP A-MSDU Required in the RSN capabilities.  .

Figure n61-B defines both transmit and receive behaviour of a STA (STA1) that has successfully negotiated an HT and RSNA (re)association with another STA (STA2).

| STA1 State |  | STA2 State |  | STA1 Action with respect to STA2. |
| --- | --- | --- | --- | --- |
| SPP A-MSDU Capable | SPP A-MSDU Required | SPP A-MSDU Capable | SPP A-MSDU Required |  |
| 0 | 0 | X | 0 | May transmit PP A-MSDU Shall not transmit SPP A-MSDU Shall receive PP A-MSDU Received SPP A-MSDU decryption will fail |
| 0 | 0 | X | 1 | Shall not transmit PP A-MSDU Shall not transmit SPP A-MSDU Shall discard received PP A-MSDU Received SPP A-MSDU decryption will fail |
| 0 | 1 | X | X | Shall not transmit PP A-MSDU Shall not transmit SPP A-MSDU Shall discard received (PP and SPP) A-MSDU |
| 1 | 0 | 0 | 0 | May transmit PP A-MSDU Shall not transmit SPP A-MSDU Shall receive PP A-MSDU Received SPP A-MSDU decryption will fail |
| 1 | 0 | 0 | 1 | Shall not transmit PP A-MSDU Shall not transmit SPP A-MSDU Shall discard received (PP and SPP) A-MSDU |
| 1 | X | 1 | X | Shall not transmit PP A-MSDU May transmit SPP A-MSDU Received PP A-MSDU decryption will fail Shall receive SPP A-MSDU |
| 1 | 1 | 0 | X | Shall not transmit PP A-MSDU Shall not transmit SPP A-MSDU Shall discard received (PP and SPP) A-MSDU |
| NOTE: X = Don’t Care |  |  |  |  |

Table n61-B   A-MSDU STA Behavior for RSNA associations

### Annex D

Add the following text in Annex D after existing entries of "dot11HTStationConfigEntry" SEQUENCE, as new entries:

dot11SPPAMSDUCapable		TruthValue,

dot11SPPAMSDURequired	TruthValue

Add the following text in Annex D after the last "dot11RDResponderOptionImplemented" OBJECT-TYPE and update the object numbering as appropriate:

dot11SPPAMSDUCapable	OBJECT-TYPE

MAX-ACCESS	read-write

Status			current

DESCRIPTION

"This attribute, when TRUE, indicates that the STA implementation is capable of protecting the A-MSDU bit (Bit 7) in the QOS Control Field when dot11RSNAEnabled is TRUE.  The default value of this attribute is FALSE."

::= { dot11HTStationConfigEntry 14 }

dot11SPPAMSDURequired	OBJECT-TYPE

MAX-ACCESS	read-write

Status			current

DESCRIPTION

"This attribute, when TRUE, indicates that the STA is configured to disallow the reception of PP A-MSDUs when dot11RSNAEnabled is TRUE.   The default value of this attribute is FALSE."

::= { dot11HTStationConfigEntry 15 }

References:

IEEE P802.11n/ D2.0