| A-MSDU Protection |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-04-19 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | Email |
| Nancy Cam-Winget | Cisco Systems | 190 W Tasman Dr, San Jose, CA 95134 | 1 408 853 0532 | ncamwing@cisco.com |
| Doug Smith | Cisco Systems | 500 Alden Road, Ste 207 Markham, Ontario L3R 5H5 Canada | 1 905 470 4803 | dsmit@cisco.com |
| Brett Douglas | Cisco Systems | 301 S. Prospect Rd, Ste. 4 Bloomington, Il 61704-4908 | 1 309 661 7656 | brettld@cisco.com |
| Matthew Fischer | Broadcom | 190 Mathilda Place Sunnyvale, CA 94086 | +1 408 543 3370 | mfischer@broadcom.com |
| Henry Ptasinski | Broadcom | 190 Mathilda Place Sunnyvale, CA 94086 | +1 408 543 3316 | henryp@broadcom.com |
| Solomon Trainin | Intel Corporation | POB 1659, Haifa 31015, Israel | +972547885738 | solomon.trainin@intel.com |
| Adrian Stephens | Intel Corporation |  | +1 (503) 616-3800 | adrian.p.stephens@intel.com |
| Menzo Wentink | Conexant | Oudegracht 3, Utrecht, Netherlands | +31 65 183 6231 | menzo.wentink@conexant.com |

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart@ok-brit.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Abstract

This document contains proposed changes to the IEEE P802.11n Draft to address the following LB97 comments assigned to the author: (arranged in page.line order)

- 289, 302, 643, 1117, 1822, 2144, 2835, 3000, 3286

The changes marked in this document are based on TGn Draft version 2.02.

# Introduction

This submission defines a means to allow the protection of A-MSDUs and coexistence with STAs that do not support A-MSDU protection

## Interpretation of a Motion to Adopt

A motion to approve this submission means that the editing instructions and any changed or added material are actioned in the TGn Draft.  This introduction, is not part of the adopted material.

TGn Editor:  Editing instructions preceded by “TGn Editor” are instructions to the TGn editor to modify existing material in the TGn draft.   As a result of adopting the changes, the TGn editor will execute the instructions rather than copy them to the TGn Draft.

Summission Note: Notes to the reader of this submission are not part of the motion to adopt.  These notes are there to clarify or provide context

# CID 289

| 289 |  |  | General | A-MSDU is a different encapsulation than MSDU, it's designation as an A-MSDU, e.g. QoS bit 7 should be protected to guard against attack that at minimum leads to a flood of traffic even on the wired side. | The QoS bit 7 should be protected when dot11HighThroughputOptionImplemented is TRUE and the frame is an A-MSDU. |
| --- | --- | --- | --- | --- | --- |

### Proposed Resolution: Accept

This document defines a mechanism for protecting QoS bit 7 by including it in the AAD.

# CID 302

| 302 | 91 | 19 | 8.3.3.3.2 | To protect A-MSDU, the only addition needed is the inclusion of QoS bit 7 in the AAD construction. So, when using HT Qos bit 7 should not be muted. | Suggestion is to define new MIB, dot11AMSDUProtected as a TruthValue in Annex D and change the text in this clause to read " g) QC – QoS Control field, if present, a 2-octet field that includes the MSDU priority. The QC TID and, when dot11AMSDUProtected is TRUE, bit 7 (A-MSDU presence) are is used in the construction of the AAD, and the remaining QC fields are set to 0 for the AAD calculation (bits 4 to 6 and bits 8 to 15 are set to 0). |
| --- | --- | --- | --- | --- | --- |

### Proposed Resolution: Counter

This document defines two new MIB variables:

dot11SPPAMSDUCapable which signifies a capability to protect QoS bit 7

dot11SPPAMSDURequired which signifies a policy for whether protect QoS bit 7 must be protected.

# CID 643

| 643 | 91 | 19 | 8.3.3 | Bit 7 is masked out, which may become a problem inviting man-in-middle attacks. It may be flipped by an attacker which causes the receiver to mistake if A-MSDU is present. | Protect QoS control field bit 7. |
| --- | --- | --- | --- | --- | --- |

### Proposed Resolution: Accept

This document defines a mechanism for protecting QoS bit 7 by including it in the AAD.

# CID 1117

| 1117 | 91 | 19 | 8.3.3.3.2 | masking the A-MSDU indication in the QoS Control field allows a MITM attack. This bit shold be protected | change g) to keep bit 7 intact for this calculation |
| --- | --- | --- | --- | --- | --- |

### Proposed Resolution: Accept

This document defines a mechanism for protecting QoS bit 7 by including it in the AAD.

# CID 1822

| 1822 | 91 | 20 | 8.3.3.3.2 | The QoS bit 7 should nto be set to zero in makign this calculation, as it leads to a security flaw where rogue devices can recover a pcaket, flip this bit and resend it. | Indicate that the A-MSDU Present indicator (QoS bit 7) is not set to zero in making this computation. |
| --- | --- | --- | --- | --- | --- |

### Proposed Resolution: Accept

This document defines a mechanism for protecting QoS bit 7 by including it in the AAD.

# CID 2144

| 2144 | 91 | 21 | 8.3.3.3.2 | The A-MSDU indicator bit (QoS Control field bit 7) should be unmasked in the AAD calculation. As it is currently masked, an attacker can modify the value of this bit without detection, resulting in injection of garbage MSDUs up the stack (i.e. by turning a replayed A-MSDU into an MSDU and vice versa). While it is hard to see how this can be exploited, it is clearly a flaw that is capable of being fixed. | Unmask QoS control field bit 7 from the AAD. Note, we have to face up to the reality of the final TGn draft coexisting with large numbers of "pre-N" devices designed to support D2.0 and earlier drafts in circulation. The unmasking needs to be done in a backwards compatible fashion, i.e. using a capability bit set to 1 to indicate the new behaviour. |
| --- | --- | --- | --- | --- | --- |

### Proposed Resolution: Accept

This document defines a mechanism for protecting QoS bit 7 by including it in the AAD.  This submission also provides a means for stations capable of protection QoS bit 7 to interoperate with Draft 2.0 devices.

# CID 2835

| 2835 | 91 | 20 | 8.3.3.3.2 | Unmasking of the QoS bit 7 may improve A-MSDU protection but will lead to incompatibility with devices that does not unmask this bit | Solve the incompatibility problem |
| --- | --- | --- | --- | --- | --- |

### Proposed Resolution: Accept

This document defines a mechanism for protecting QoS bit 7 by including it in the AAD.  This submission also provides a means for stations capable of protection QoS bit 7 to interoperate with Draft 2.0 devices.

# CID 3000

| 3000 | 182 | 1 | 8.3.3.3.2 | The A-MSDU present bit should be unmasked in the AAD, to avoid that the bit can be flipped on a replay. A replay with the bit flipped would cause junk MSDUs to be generated at the receiving MAC. | Unmask bit 7 of the QC field (A-MSDU present) inside the AAD. |
| --- | --- | --- | --- | --- | --- |

### Proposed Resolution: Accept

# This document defines a mechanism for protecting QoS bit 7 by including it in the AAD

# CID 3286

| 3286 | 90 |  | 8.3.3.3.2 | A-MSDU signaling bit (QOS control field bit 7) should be protected by the AAD. | Make A-MSDU reception optional. |
| --- | --- | --- | --- | --- | --- |

### Proposed Resolution: Reject

Very large numbers of Draft 2.0 devices have been shipped.  Those Draft 2.0 devices believe A-MSDU reception is mandatory.  Making A-MSDU reception optional would lead to interoperability problems between

# Proposed Edits

TGn Editor: Include into the TGn draft with the following text:

### 3. Definitions

Insert the following definitions in their appropriate alphabetical order:

Editorital note:  Based on the 802.11-2007 specification, Payload Protected A-MSDU should follow 3.114 and Signaling and Payload Protected A-MSDU should follow 3.135

### 3.114n Payload Protected A-MSDU (PP A-MSDU): An A-MSDU that is CCMP protected but does not include the A-MSDU Present field (bit 7 of the QoS control field) in the construction of the AAD.

3.135n Signaling and Payload Protected A-MSDU (SPP A-MSDU): An A-MSDU that is CCMP protected and does include the A-MSDU Present field (bit 7 of the Qos control field) in the construction of the AAD.

### 4.Abbreviations and acronyms

Insert  the following acronyms in their alphabetical order:

PP A-MSDU	Payload Protected A-MSDU

SPP A-MSDU	Signaling and Payload Protected A-MSDU

### 7.3.2.25.3 RSN capabilities

Replace Figure 91 with the following figure that assigns 2 bits to signal the SPP A-MSDU capability and SPP A-MSDU required bits, changing "Reserved" as appropriate:

Editorial note: the bits are currently positioned in B10 and B11 so as not to collide with other current drafts, but the fields are left as <ANA1> and <ANA2>  as they are to be assigned by ANA.  The actual bit positions will be inserted once the 802.11 ANA has assigned them.

| B0 | B1 | B2-3 | B4-5 | B6 | B7 | B8 | B9 | <ANA1> | <ANA2> | B12-15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pre-Auth | No Pairwise | PTKSA Replay Counter | GTKSA Replay Counter | AES-128-CMAC | Robust Management Frame protection | Reserved | Peerkey Enabled | SPP A-MSDU Capable | SPP A-MSDU Required | Reserved |

Figure 91  RSN Capabilities field format

Insert the following text in the appropriate Bit assignment order in 7.3.2.25.3:

-- bit <ANA1> : SPP A-MSDU Capable.  A STA sets the SPP A-MSDU Capable subfield of the RSN Capabilities field to 1 to signal that it supports SPP A-MSDU (see 11.18).   Otherwise, it shall be set to 0.

-- bit <ANA2>: SPP A-MSDU Required.  A STA sets the  SPP A-MSDU Required subfield of the RSN Capabilities field to 1 when it disallows (i.e. will not send or receive) PP A-MSDUs .  (see 11.18).  Otherwise, it shall be set to 0.

### 8.3.3.3.2 Construct AAD

Change item " g)"  of the last paragraph in 8.3.3.3.2 Construct AAD as follows:

g) QC – QoS Control field, if present, a 2-octet field that includes the MSDU priority. The QC TID field is always included in the AAD construction. When both the station and its peer have SPP A-MSDU Capable bits set to TRUE, bit  7 (the A-MSDU Present field) will be used in the construction of the AAD, .  The remaining QC fields are set to 0 for the AAD calculation (bits 4 to 6, bits 8 to 15, and bit 7 when either the station or its peer has the SPP A-MSDU bit set to FALSE ).

Add new subclause “11.18 RSNA A-MSDU procedures” after “11.17 STA-STA HT Information Exchange”

### 11.18 RSNA A-MSDU procedures

When dot11RSNAEnabled is TRUE, a STA indicates support for PP A-MSDU or SPP A-MSDU during (re)association.  On (re)association, both STA and its peer STA determine and maintain a record of whether an encrypted A-MSDU sent to its peer is to be PP A-MSDU or SPP A-MSDU based on both STAs SPP A-MSDU Capable and SPP A-MSDU Required RSNIE settings.

Note: If the peer STA transmits the wrong type of A-MSDU, the decryption process fails, and the A-MSDU is discarded.

The SME of an RSNA and HT-capable STA may choose to associate with RSNA STAs with or without the SPP A-MSDU Capable field set in the RSNIE and with or without the SPP A-MSDU Required field set in the RSNIE.  A STA may choose not to associate with a STA that does not advertise SPP A-MSDU Capable or SPP A-MSDU Required in the RSN capabilities.  .

Figure n61-B defines both transmit and receive behaviour of a STA (STA1) that has successfully negotiated an HT and RSNA (re)association with another STA (STA2).

| STA1 State |  | STA2 State |  | STA1 Action with respect to STA2. |
| --- | --- | --- | --- | --- |
| SPP A-MSDU Capable | SPP A-MSDU Required | SPP A-MSDU Capable | SPP A-MSDU Required |  |
| 0 | 0 | X | 0 | May transmit PP A-MSDU Shall not transmit SPP A-MSDU Shall receive PP A-MSDU Received SPP A-MSDU decryption will fail |
| 0 | 0 | X | 1 | Shall not transmit PP A-MSDU Shall not transmit SPP A-MSDU Shall discard received PP A-MSDU Received SPP A-MSDU decryption will fail |
| 0 | 1 | X | X | Shall not transmit PP A-MSDU Shall not transmit SPP A-MSDU Shall discard received (PP and SPP) A-MSDU |
| 1 | 0 | 0 | 0 | May transmit PP A-MSDU Shall not transmit SPP A-MSDU Shall receive PP A-MSDU Received SPP A-MSDU decryption will fail |
| 1 | 0 | 0 | 1 | Shall not transmit PP A-MSDU Shall not transmit SPP A-MSDU Shall discard received (PP and SPP) A-MSDU |
| 1 | X | 1 | X | Shall not transmit PP A-MSDU May transmit SPP A-MSDU Received PP A-MSDU decryption will fail Shall receive SPP A-MSDU |
| 1 | 1 | 0 | X | Shall not transmit PP A-MSDU Shall not transmit SPP A-MSDU Shall discard received (PP and SPP) A-MSDU |
| NOTE: X = Don’t Care |  |  |  |  |

Table n61-B   A-MSDU STA Behavior for RSNA associations

### Annex D

Add the following text in Annex D after existing entries of "dot11HTStationConfigEntry" SEQUENCE, as new entries:

dot11SPPAMSDUCapable		TruthValue,

dot11SPPAMSDURequired	TruthValue

Add the following text in Annex D after the last "dot11RDResponderOptionImplemented" OBJECT-TYPE and update the object numbering as appropriate:

dot11SPPAMSDUCapable	OBJECT-TYPE

MAX-ACCESS	read-write

Status			current

DESCRIPTION

"This attribute, when TRUE, indicates that the STA implementation is capable of protecting the A-MSDU bit (Bit 7) in the QOS Control Field when dot11RSNAEnabled is TRUE.  The default value of this attribute is FALSE."

::= { dot11HTStationConfigEntry 14 }

dot11SPPAMSDURequired	OBJECT-TYPE

MAX-ACCESS	read-write

Status			current

DESCRIPTION

"This attribute, when TRUE, indicates that the STA is configured to disallow the reception of PP A-MSDUs when dot11RSNAEnabled is TRUE.   The default value of this attribute is FALSE."

::= { dot11HTStationConfigEntry 15 }

References:

IEEE P802.11n/ D2.0