IEEE P802.11
Wireless LANs

| Draft Text to address CID 147 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-03-13 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Mark Kobayashi | Broadcom Corporation | 190 Mathilda Place Santa Clara, CA 94086 | 408-543-3318 | mmk@broadcom.com |
| Dalton Victor | Broadcom Corporation | 190 Mathilda Place Santa Clara, CA 94086 | 408-922-5824 | dvictor@broadcom.com |

Abstract

This draft text addresses CID 147.

-CID 147: In summary form, explain which test environment is applicable to what metric and measurement

It also begins to address CID 159, 160, 162.

-CID 159:  Measured Test system loss should be noted in addition to measurements called out

-CID 160:  Decide on definition of permissible error margins and reliability of test and implement this language throughout document

-CID 162:  Include a section describing test measurement uncertainty

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart@ok-brit.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

# 4.1 Categorization of Metrics and Environments

Table : Table of Metrics and Environments

The metrics and environments within this document can be divided into 2 sets.  One category, absolute, describes those that are repeatable in different locations at any time.  The other category, relative, describes metrics and environments which are repeatable in the same location within a short period of time.

## 4.1.1 Absolute

Absolute metrics and environments directly give an indication of a device’s wireless performance.  They are desirable for their high repeatability and correlation.  They are calibrated and defined such that completely independent parties can reproduce the results with high precision and accuracy.  The results from 2 sets of absolute measurements can always be compared.  Absolute measurements require calibration to determine insertion loss for each possible RF path.  Non-WLAN effects like codecs, buffering, memory, cpu power, etc should not have a significant impact on measurements for Absolute metrics and environments.  Absolute measurements will yield smaller error margins than relative measurements.

## 4.1.2 Relative

Relative metrics and environments allow comparison of performance for a specific configuration.  They are desirable for their ease of setup and adaptability to situations in which calibration (or characterization of the environment) is not possible or convenient.  The measurements ascertained in such a scenario can only be compared with other measurements done in the same location and configuration within a short period of time.  Relative measurements will yield larger error margins than absolute measurements.

| Absolute Test Environments | Relative Test Environments |
| --- | --- |
| Calibrated Over the Air Test (COAT) | Over The Air (OTA) Outdoor Line Of Sight (LOS) |
| Conducted | Over The Air (OTA) Indoor Non-Line Of Sight (NLOS) |
|  | Over The Air (OTA) shielded enclosure |

Table 1: Categorized list of Absolute and Relative Environments

| Absolute Metrics | Relative Metrics |
| --- | --- |
| Throughput versus attenuation | Throughput versus range |
| Throughput versus received power |  |
| Transmit Rate Adaptation |  |
| Antenna Diversity |  |
| Adjacent Channel Interference (ACI) in conducted environment |  |
| BSS transition time |  |
| Fast BSS transition time |  |
| Receiver sensitivity in a conducted environment |  |
| Receiver sensitivity in a COAT environment |  |
| Unicast intra-BSS throughput |  |
| Unicast ESS throughput |  |
| Multicast forwarding rate |  |
| Endstation association rate | Coexistence of overlapping BSSes in an OTA environment (using OTA LOS, OTA indoor NLOS or OTA shielded enclosure) |
| Endstation database capacity |  |
| Power consumption |  |
| Packet Loss |  |
| Latency |  |
| Jitter |  |
| Video performance (MDI) | Video Performance (VDER, VQM) |

Table 2: Categorized list of Absolute and Relative Metrics