IEEE P802.11
Wireless LANs

| Normative Text Changes to provide location capability of a STA – Layer Management |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-MAR-12 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | Email |
| Donghee Shim | LG Electronics, Inc | 16, Woomyeon, Secho, Seoul, Korea | 82-10-7997-6678 | dhshim@lge.com |

Abstract

This document contains normative text for the use of location capability by a STA in Layer Management. This document is based on the normative texts already agreed upon during last sessions.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart@ok-brit.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

[NOTE TO EDITOR:  Added or modified texts are underlined from draft 802.11vD0.09]

Modify Clause 10.3.6 and 10.3.7 as follows

- Layer management

  - MLME SAP Interface

Change 10.3.6 as follows:

- 10.3.6 Associate

- 10.3.6.1 MLME-ASSOCIATE.request

- 10.3.6.1.2 Semantics of the service primitive

Change the primitive parameter list as shown:

MLME-ASSOCIATE.request	(
PeerSTAAddress,
AssociateFailureTimeout,
CapabilityInformation,
ListenInterval,
Supported Channels,
RSN,
QoSCapability,
WirelessManagementCapabilities,

PresenceParameters
FBMSRequest,
TrafficGeneration,
SupportedRegulatoryClasses
)

Insert the following row at the end of the parameter table:

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| WirelessManagementCapabilities | As defined in the Wireless Network Management Capabilities element | As defined in the Wireless Network Management Capabilities element | Specifies the Wireless Network Management capabilities of the non-AP STA. This parameter shall only be present if the MIB attribute dot11WirelessManagementImplemented is true. |
| PresenceParameters | Set of presence elements | Set of presence elements | Specifies a set of presence elements describing the STA presence information. See 7.3.2.37. This parameter shall only be present if the MIB attribute dot11WirelessManagementImplemented is true. |
| FBMSRequest | As defined in FBMS Request element | As defined in FBMS Request element | Specifies the proposed service parameters for the FBMS. This parameter shall only be present if the MIB attribute dot11WirelessManagementImplemented is true and FBMS bit in the Wireless Network Management Capability is set to 1. |
| Traffic Generation | As defined in the Traffic Generation element | As defined in the Traffic Generation element | Specifies the Traffic Generation flags of the non-AP STA. This parameter shall only be present if the MIB attribute dot11WirelessManagementImplemented is true. |
| SupportedRegulatoryClasses | As defined in the Supported Regulatory Classes element | As defined in the Supported Regulatory Classes element | Specifies the supported Regulatory Classes capabilities of the non-AP STA. This parameter shall only be present if the MIB attribute dot11WirelessManagementImplemented is true. |

- 10.3.6.3 MLME-ASSOCIATE.indication

- 10.3.6.3.2 Semantics of the service primitive

Change the primitive parameter list as shown:

MLME-ASSOCIATE.indication	(
PeerSTAAddress,
CapabilityInformation,
ListenInterval,
SSID,
Supported Rates,
RSN,
QoSCapability,
WirelessManagementCapabilities,

PresenceParameters,
FBMSRequest,
TrafficGeneration,
SupportedRegulatoryClasses
)

Insert the following row at the end of the parameter table:

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| WirelessManagementCapabilities | As defined in the Wireless Network Management Capabilities element | As defined in the Wireless Network Management Capabilities element | Indicates the Wireless Network Management capabilities of the non-AP STA. This parameter shall only be present if the MIB attribute dot11WirelessManagementImplemented is true. |
| PresenceParameters | Set of presence elements | Set of presence elements | Specifies a set of presence elements describing the STA presence information. See 7.3.2.54. This parameter shall only be present if the MIB attribute dot11WirelessManagementImplemented is true. |
| FBMSRequest | As defined in FBMS Request element | As defined in FBMS Request element | Specifies the proposed service parameters for the FBMS. This parameter shall only be present if the MIB attribute dot11WirelessManagementImplemented is true and FBMS bit in the Wireless Network Management Capability is set to 1. |
| Traffic Generation | As defined in the Traffic Generation element | As defined in the Traffic Generation element | Specifies the Traffic Generation Flags of the non-AP STA. This parameter shall only be present if the MIB attribute dot11WirelessManagementImplemented is true. |
| SupportedRegulatoryClasses | As defined in the Supported Regulatory Classes element | As defined in the Supported Regulatory Classes element | Specifies the supported Regulatory Classes capabilities of the non-AP STA. This parameter shall only be present if the MIB attribute dot11WirelessManagementImplemented is true. |

Change 10.3.7 as follows:

- 10.3.7 Reassociate

- 10.3.7.11 MLME-REASSOCIATE.request

- 10.3.7.11.2 Semantics of the service primitive

Change the primitive parameter list as shown:

MLME-REASSOCIATE.request	(
NewAPAddress,
ReassociateFailureTimeout,
CapabilityInformation,
ListenInterval,
Supported Channels,
RSN,
QoSCapability,
Content of FT Authentication Information Elements,
WirelessManagementCapabilities,

PresenceParameters,
FBMSRequest,
TrafficGeneration,
SupportedRegulatoryClasses
)

Insert the following row at the end of the parameter table:

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| WirelessManagementCapabilities | As defined in the Wireless Network Management Capabilities element | As defined in the Wireless Network Management Capabilities element | Specifies the Wireless Network Management capabilities of the non-AP STA. This parameter shall only be present if the MIB attribute dot11WirelessManagementImplemented is true. |
| PresenceParameters | Set of presence elements | Set of presence elements | Specifies a set of presence elements describing the STA presence information. See 7.3.2.54. This parameter shall only be present if the MIB attribute dot11WirelessManagementImplemented is true. |
| FBMSRequest | As defined in the FBMS Request element | As defined in the FBMS Request element | Specifies the proposed service parameters for the FBMS. This parameter shall only be present if the MIB attribute dot11WirelessManagementImplemented is true and FBMS bit in the Wireless Network Management Capability is set to 1. |
| Traffic Generation | As defined in the Traffic Generation element | As defined in the Traffic Generation element | Specifies the Traffic Generation flags of the non-AP STA. This parameter shall only be present if the MIB attribute dot11WirelessManagementImplemented is true. |
| SupportedRegulatoryClasses | As defined in the Supported Regulatory Classes element | As defined in the Supported Regulatory Classes element | Specifies the supported Regulatory Classes of the non-AP STA. This parameter shall only be present if the MIB attribute dot11WirelessManagementImplemented is true. |

- 10.3.7.13 MLME-REASSOCIATE.indication

- 10.3.7.13.2 Semantics of the service primitive

Change the primitive parameter list as shown:

MLME-REASSOCIATE.indication	(
PeerSTAAddress,
CurrentAPAddress,
CapabilityInformation,
ListenInterval,
SSID,
SupportedRates,
RSN,
QoSCapability,
Content of FT Authentication Information Elements,
WirelessManagementCapabilities,

PresenceParameters,
FBMSRequest,
TrafficGeneration,
SupportedRegulatoryClasses)

Insert the following row at the end of the parameter table:

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| WirelessManagementCapabilities | As defined in the Wireless Network Management Capabilities element | As defined in the Wireless Network Management Capabilities element | Indicates the Wireless Network Management capabilities of the non-AP STA. This parameter shall only be present if the MIB attribute dot11WirelessManagementImplemented is true. |
| PresenceParameters | Set of presence elements | Set of presence elements | Specifies a set of presence elements describing the STA presence information. See 7.3.2.54. This parameter shall only be present if the MIB attribute dot11WirelessManagementImplemented is true. |
| FBMSRequest | As defined in the FBMS Request element | As defined in the FBMS Request element | Specifies the proposed service parameters for the FBMS. This parameter shall only be present if the MIB attribute dot11WirelessManagementImplemented is true and FBMS bit in the Wireless Network Management Capability is set to 1. |
| Traffic Generation | As defined in the Traffic Generation element | As defined in the Traffic Generation element | Specifies the Traffic Generation flags of the non-AP STA. This parameter shall only be present if the MIB attribute dot11WirelessManagementImplemented is true. |
| SupportedRegulatoryClasses | As defined in the Supported Regulatory Classes element | As defined in the Supported Regulatory Classes element | Specifies the supported Regulatory Classes of the non-AP STA. This parameter shall only be present if the MIB attribute dot11WirelessManagementImplemented is true. |