IEEE P802.11
Wireless LANs

| Normative Text Addition to 07/0327r0 to Make it Consistent with the comment IDs 22, 192, 241, 244 and 284 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-03-09 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Lei Du, Yong Bai, Lan Chen, Daqing Gu | DoCoMo Beijing Labs Inc. | 7/F Raycom Infotech Park Tower A, No.2 Kexueyuan South Road, Haidian District, Beijing, P.R.China | +86-10-82861501ex.336 | {du, bai, chen, gu}@docomolabs-beijing.com.cn |
| Sunghyun Choi | Seoul National University | Kwanak P.O.Box 34, Seoul 151-600, Korea | +82-2-880-1753 | schoi@snu.ac.kr |
| Sunggeun Jin | ETRI | 161 Gajeong-Dong, Yuseong-gu, Daejeon, 305-700, Korea | +82-2-880-1755 | sgjin@etri.re.kr |
| Fujio Watanabe, Moo Ryong Jeong | DoCoMo USA Labs Inc. | 3240 Hillview Ave, Palo Alto, CA 94304-1201, USA | +1-650-496-4726 | {watanabe, jeong}@docomolabs-usa.com |
| Akira Yamada | NTT DoCoMo Inc. | 3-5, Hikarinooka,Yokosuka-shi, Kanagawa 239-8536, Japan | +81-46-840-6527 | yamadaakir@nttdocomo.co.jp |
| Emily Qi | Intel Corp. | 2111 NE 25th Ave Hillsboro OR | 503-264-7799 | Emily.H.Qi@intel.com |

Abstract

This document contains a normative text addition to 11-07-0327-00-000v-normative-text-proposal-qos-aware-load-balancing to make it consistent with the comment IDs 22, 192, 241, 244 and 284 from the Internal Review of TGv Draft specification.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart@ok-brit.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

- 7.3.2.37 Neighbor Report element

- Change the following subclause title below table 43B as indicated below:

AC Station Count is optionally present in the Neighbor Report element included in BSS Transition Management Request frame. The format of AC Station Count sub-element is defined in 7.3.2.37.6.

- Insert the following clause after 7.3.2.37.5:

- 7.3.2.37.6 AC Station Count sub-element

The AC Station Count sub-element contains a list of Station Count fields at different Access Categories as shown in Figure VV.

|  | Sub-element ID (06) | Length | Station Count Bitmask | Station Count List |
| --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 2 x (total number of non-zero bits in Station Count Bitmask) |

Figure VV--- AC Station Count sub-element format

The Length field in octets shall be set to 2Nnz , where Nnz equals the total number of non-zero bits in Station Count Bitmask.

The Station Count Bitmask field indicates the AC values that have Station Count specified in the following Station Count List. The format of the Station Count Bitmask is defined in Table VVV.

Table VVV－ Station Count Bitmask Definition

| Bit (s) | Station Count Reported |
| --- | --- |
| 0 | AC 0 (VO) |
| 1 | AC 1 (VI) |
| 2 | AC 2 (BE) |
| 3 | AC 3 (BK) |
| 4-7 | Reserved |

The bit is set to 1 to indicate that the Station Count for the corresponding access category is present in the Station Count List field. The bit is set to 0 to indicate that the Station Count for the corresponding access category is not present in the Station Count List field.

The Station Count List comprises a sequence of Station Count fields corresponding respectively to the non-zero bits in the Station Count Bitmask field.

The Station Count field is 2 octets long and contains an unsigned integer that specifies the number of STAs that are currently associated with this QBSS for the corresponding AC traffic. The field is helpful for roaming non-AP QSTAs to select a QAP that is likely to result in higher total throughput and QoS effectiveness.