IEEE P802.11
Wireless LANs

| EMSA MIC Update |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-03-13 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Tony Braskich | Motorola Inc. | 1301 E Algonquin Rd, Schaumburg, IL 60196 | +18475380760 | Tony.Braskich@motorola.com |
| Steve Emeott | Motorola Inc. | 1301 E Algonquin Rd, Schaumburg, IL 60196 | +18475768268 | Steve.Emeott@motorola.com |
| Meiyuan Zhao | Intel Corporation | RNB-6-61, 2200 Mission College Blvd, Santa Clara, CA 95052 | +1-408-653-5517 | meiyuan.zhao@intel.com |
| Jesse Walker | Intel Corporation | JF3-206, 2111 N.E. 25th Avenue, Hillsboro, OR 97124 | +1-503-712-1849 | jesse.walker@intel.com |

Abstract

This submission updates the MIC algorithm used by the 4-way handshake and GTK handshake within a mesh that employs EMSA, to resolve comments such as 592 and 4732.  See also 11-07/0286 and 11-07/0287 for updates to the MIC algorithm in other clauses.

Further, the EMSAIE is updated to remove all fields that were not in use (e.g., see comment 1073).

This submission resolves the following comments: 1073, 1408, 2800, 4038.

This submission partially resolves the following comments: 592, 1488, 4732, 5111, 5112, 5113, 5114, 5118, 5119, 5120, 5121.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart@ok-brit.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Replace 7.3.2.64 with the following:

7.3.2.64 EMSA Handshake element [EMSAIE]

The EMSA handshake information element includes information needed to perform the authentication sequence during an EMSA handshake.  This information element is depicted in Figure A.

|  | Element ID | Length | MA-ID | Optional Parameters |
| --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 6 | variable |

Figure A - EMSA Handshake information element

The Element ID of this element shall be TBD.  The Length field for this information element indicates the number of octets in the information field (fields following the Element ID and Length fields).

The MA-ID field contains the MA’s identity, which is used by the supplicant MP for deriving the PMK-MA.  It is encoded following the conventions from 7.1.1.

The format of the optional parameters is shown in Figure B.

|  | Sub-element ID | Length | Data |
| --- | --- | --- | --- |
| Octets: | 1 | 1 | variable |

Figure B - Optional parameters field

The Sub-element ID is one of the values from Table a.

Table a - Sub-element IDs

| Value | Contents of data field | Length |
| --- | --- | --- |
| 0 | Reserved |  |
| 1 | MKD-ID | 6 |
| 2 | EAP Transport List | variable |
| 3-255 | Reserved |  |

MKD-ID indicates the MKD that the supplicant MP may contact to initiate the mesh key holder security handshake.

EAP Transport List contains a series of transport selectors that indicate the EAP transport mechanism.  A transport selector has the format shown in Figure C.

|  | OUI | Transport Type |
| --- | --- | --- |
| Octets: | 3 | 1 |

Figure C - Transport selector format

The order of the organizationally unique identifier (OUI) field shall follow the ordering convention for MAC addresses from 7.1.1.  The transport selectors defined by this amendment are provided in Table b.

Table b - Transport selectors

| OUI | Transport Type | Meaning |
| --- | --- | --- |
| 00-0F-AC | 0 | EAP Transport mechanism as defined in 8.8.6. |
| 00-0F-AC | 1-255 | Reserved |
| Vendor OUI | Any | Vendor specific |
| Other | Any | Reserved |

The transport selector 00-0F-AC:0 shall be the default transport selector value.

- Security

8.5 Keys and key distribution

8.5.2 EAPOL-Key frames

Change List Item 1 of “Key Information” (list entry b) in 8.5.2 as shown:

1) Key Descriptor Version (bits 0-2) specifies the key descriptor version type.

i) The value 1 shall be used for all EAPOL-Key frames to and from a STA when neither the group nor pairwise ciphers are CCMP for Key Descriptor 1. This value indicates the following:

—HMAC-MD5 is the EAPOL-Key MIC.

—ARC4 is the EAPOL-Key encryption algorithm used to protect the Key Data field.

ii) The value 2 shall be used for all EAPOL-Key frames to and from a STA when either the pairwise or the group cipher is AES-CCMP for Key Descriptor 2. This value indicates the following:

—HMAC-SHA1-128 is the EAPOL-Key MIC. HMAC is defined in IETF RFC 2104; and SHA1, by FIPS PUB 180-1-1995. The output of the HMAC-SHA1 shall be truncated to its 128 MSBs (octets 0-15 of the digest output by HMAC-SHA1), i.e., the last four octets generated shall be discarded.

—The NIST AES key wrap is the EAPOL-Key encryption algorithm used to protect the Key Data field. IETF RFC 3394 defines the NIST AES key wrap algorithm.

iii) The value 3 shall be used for all EAPOL-Key frames between MPs when dot11RSNAAuthenticationSuiteSelected is 5 or 6. This value indicates the following:

—AES-128-CMAC is the EAPOL-Key MIC. AES-128-CMAC is defined by FIPS SP800-38B. The output of the AES-128-CMAC shall be 128 bits.

—The NIST AES key wrap is the EAPOL-Key encryption algorithm used to protect the Key Data field. IETF RFC 3394 defines the NIST AES key wrap algorithm.

Change list entry h, “Key MIC” in 8.5.2 as shown:

h)  Key MIC. This field is 16 octets in length when the Key Descriptor Version subfield is 1, 2 or 23. The EAPOL-Key MIC is a MIC of the EAPOL-Key frames, from and including the EAPOL protocol version field to and including the Key Data field, calculated with the Key MIC field set to 0. If the Encrypted Key Data subfield (of the Key Information field) is set, the Key Data field is encrypted prior to computing the MIC.

1) Key Descriptor Version 1: HMAC-MD5; IETF RFC 2104 and IETF RFC 1321 together define this function.

2) Key Descriptor Version 2: HMAC-SHA1-128.

3) Key Descriptor Version 3: AES-128-CMAC.

8.5.3 4-Way Handshake

8.5.3.1 4-Way Handshake Message 1

Change 8.5.3.1 as follows:

Key Information:

Key Descriptor Version = 1 (ARC4 encryption with HMAC-MD5) or 2 (NIST AES key wrap with HMAC-SHA1-128) or 3 (NIST AES key wrap with AES-128-CMAC)

8.5.3.2 4-Way Handshake Message 2

Change 8.5.3.2 as follows:

Key Information:

Key Descriptor Version = 1 (ARC4 encryption with HMAC-MD5) or 2 (NIST AES key wrap with HMAC-SHA1-128) or 3 (NIST AES key wrap with AES-128-CMAC) - same as Message 1.

8.5.3.3 4-Way Handshake Message 3

Change 8.5.3.3 as follows:

Key Information:

Key Descriptor Version = 1 (ARC4 encryption with HMAC-MD5) or 2 (NIST AES key wrap with HMAC-SHA1-128) or 3 (NIST AES key wrap with AES-128-CMAC) - same as Message 1.

8.5.3.4 4-Way Handshake Message 4

Change 8.5.3.4 as follows:

Key Information:

Key Descriptor Version = 1 (ARC4 encryption with HMAC-MD5) or 2 (NIST AES key wrap with HMAC-SHA1-128) or 3 (NIST AES key wrap with AES-128-CMAC) - same as Message 1.

8.5.4 Group Key Handshake

8.5.4.1 Group Key Handshake Message 1

Change 8.5.4.1 as follows:

Key Information:

Key Descriptor Version = 1 (ARC4 encryption with HMAC-MD5) or 2 (NIST AES key wrap with HMAC-SHA1-128) or 3 (NIST AES key wrap with AES-128-CMAC)

8.5.4.2 Group Key Handshake Message 2

Change 8.5.4.2 as follows:

Key Information:

Key Descriptor Version = 1 (ARC4 encryption with HMAC-MD5) or 2 (NIST AES key wrapwith HMAC-SHA1-128) or 3 (NIST AES key wrap with AES-128-CMAC) - same as Message 1.