IEEE P802.11
Wireless LANs

| Resolution to CID 483 – MAC Address Spoofing in Mesh |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-03-14 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Michael Bahr | Siemens AG, Corporate Technology | Otto-Hahn-Ring 6 81730 München, Germany | +49-89-636-49926 | bahr@siemens.com |

Abstract

This document provides text to be included in the TGs draft as resolution of comment CID 483 (MAC address spoofing). A motivation for this is given in submission 11-06/1837.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart@ok-brit.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

# Comment CID 483

CID 483

MAC address spoofing by non-MP associating with MAP

Resolution by Commenter

informal note that it may be checked that the MAC address of the associating non-MP STA does not exist within mesh as MP, motivation for this is given in submission 11-06/1837

# Proposed Resolution

A presentation on this topic is available as document 11-06/1837r2.

Note to the editor: All clause numbers are based on IEEE 802.11s Draft D1.01

Insert new clause 11.3.3 at the end of clause 11.3 “STA Authentication and Association”:

11.3.3 Additional Mechanisms for APs with Mesh Functionality

After the initial state for a non-AP STA with no mesh functionality has been established for authentication / association, the AP with mesh functionality may verify in a timely fashion that the MAC address of the non-AP STA does not belong to another STA with mesh functionality in the mesh network.

If the MAC address of the non-AP STA does already exist in the mesh network, the AP with mesh functionality will reject the station. Depending on the progress of the authentication / association, the rejected station will be disassociated and/or deauthenticated with Status Code “unspecified reason”.

The mechanism for verfying disjunct MAC addresses depends on the active path selection protocol and might be vendor specific. See clause 11A.6.8 for HWMP and clause 11A.7.13.7 for RA-OLSR.

Insert new clause 11A.6.8

11A.6.8 Considerations for support of STAs without mesh functionality

The verification of disjunct MAC addresses between a non-AP STA without mesh functionality and mesh points during authentication/association of the non-AP STA (cf. clause 11.3.3) may be done by issuing a RREQ for the MAC address of the non-AP STA by the AP with mesh functionality. The destination only flag of the RREQ shall be set to 1.

The MAC address of the non-AP STA does already exist in the mesh network if the AP with mesh functionality receives a RREP for the MAC address of the non-AP STA and it can be derived from the RREP that the requested MAC address is originated from a mesh point. (The AE flag of the RREP is set to 0, see clause 7.3.2.71).

Insert new clause 11A.7.13.7

11A.7.13.7 Considerations for support of STAs without mesh functionality

The verification of disjunct MAC addresses between a non-AP STA without mesh functionality and mesh points during authentication/association of the non-AP STA (cf. clause 11.3.3) may be done by a lookup of the MAC address of the non-AP STA in the forwarding information of the AP with mesh functionality.

The MAC address of the non-AP STA does already exist in the mesh network if the AP with mesh functionality already contains the MAC address in its forwarding information.

References:

[1]	IEEE 802.11s Draft D1.01

[2]	11-06/1837r2 “MAC Address Spoofing in Mesh”