IEEE P802.11
Wireless LANs

| Suggested text for resolution of Intra-Mesh Congestion Control comments |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-03-14 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | Email |
| Bahareh Sadeghi | Intel Corporation | 2111 NE 25th Ave, JF3-206 Hillsboro, OR 97124 | 503-803-2471 | Bahareh.sadeghi@ intel.com |

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Replace clause 7.3.2.53 with the following:

7.3.2.53 Congestion Duration Element

Congestion

Duration

(AC

-

BK)

Congestion

Duration

(AC

-

BE)

Congestion

Duration

(AC

-

VI)

Congestion

Duration

(AC

-

VO)

length

ID

1 byte

1 byte

2 bytes

2 bytes

2 bytes

2 bytes

Congestion

Control

Mode

1 byte

Figure s23: Congestion Duration Element Formant

“Congestion Duration” element, Figure s23, is used in the “Congestion Control Request” frame by a Mesh Point to indicate to its neighbors its congestion status per AC and the duration it expects for the congestion to last. It contains four Congestion Duration fields for the four EDCA access categories and a Congestion Control Mode byte. Congestion duration is indicated in 0.1 TU.

Replace clause 7.3.2.54 with the following:

7.3.2.54 Congestion Control Response Element

Congestion

Control

Mode

length

ID

1 byte

1 byte

1 byte

Figure s24: Congestion Control Response Element Formant

“Congestion Control Response” element, Figure s24, is used in “Congestion Control Response” frame by a Mesh Point in response to a “Congestion Control Request” message. It contains a Congestion Control Mode byte.

Remove clause 7.3.2.55.

Replace clause 7.4.9.6 with the following:

7.4.9.6 Congestion Control Request frame format

The congestion Control Request frame format uses the Action frame body format and is sent by an MP to its neighboring MP to indicate its congestion status. This frame can be transmitted in a unicast or broadcast manner. The frame format is shown in figure s73.

Congestion duration element

action

Category

1 byte

1 byte

11 bytes

Figure s73: Congestion Control Request frame format

The category field shall be set to 5 (representing mesh management).

The Action field shall be set to 6 (representing Congestion Control Request).

The Congestion Duration element field shall be set following the guidelines described in Clause 11A.7.

Replace clause 7.4.9.7 with the following:

7.4.9.7 Congestion Control Response frame format

The congestin Control Response frame format uses the Action frame body format and is sent by an MP as a response to a unicast “Congestion Control Request” message. This frame is transmitted in a unicast manner. The frame format is shown in Figure s74.

Figure s74: Congestion Control Response frame format

The category field shall be set to 5 (representing mesh management).

The Action field shall be set to 7 (representing Congestion Control Response).

The Congestion Control Response element field shall be set following the guidelines described in Clause 11A.7.

Remove clause 7.4.9.8.

Replace clause 11A.8 with the following:

11A.8 Intra-Mesh Congestion Control

Intra-Mesh congestion control is based on three main mechanisms: Local congestion monitoring and congestion detection, congestion control signaling, and local rate control.

The local congestion monitoring and congestion detection is considered implementation matter and is outside the scope of this specification. This specification defines the congestion control signaling. What the local rate control is and how it is implemented is not defined in this specification.

11A.8.1 Congestion Control Signaling

This specification does not define the exact conditions that would trigger the congestion control signaling; congestion control signaling is triggered after congestion is detected at a node through local congestion monitoring (out of scope of this specification).

Congestion control signaling consists of two messages:

“Congestion Control Request” message is transmitted by a congested node. “Congestion Control Request” message can be transmitted as a unicast or a broadcast message. The frame contains “Congestion Duration Element” which specifies the duration of the congestion per AC as estimated by the congested node. The “Congestion Control Mode” field in the Congestion Duration Element allows for implementation of different congestion control schemes and should be set accordingly. Congestion Control Mode set to zero indicates the default congestion control scheme which is limited to signaling the existence of congestion and an estimation of its duration.

The nodes that receive a “Congestion Control Request” message sent as a unicast message, shall reply to the congestion control request with a “Congestion Control Response” element. If the Congestion Control Mode in the received “Congestion Duration Element” is set to zero, or the Congestion Control Mode is unknown to the receiver of the “Congestion Control Request” message, the node shall set the Congestion Control Mode in “Congestion Control Response” element to zero; otherwise, it shall set the Congestion Control Mode equal to the value received in “Congestion Duration Element”.