IEEE P802.11
Wireless LANs

| LB97 Submission to resolve PHY CCA comments |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-03-14 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Assaf Kasher | Intel Corporation | Matam Industrial Park Haifa Israel 31015 | +972-4-8651547 | Asssaf.kasher@intel.comment |

Abstract

This document suggests resolution for PHY CCA comments with the following CIDs: 45, 1263, 2746, 2747, 2748, 346, 406, 1678, 1804, 1805

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart@ok-brit.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

This documents suggest resolution for lb97 comments on subclause 20.3.21.5 – Clear Channel Assesment

| 1263 | CCA shouldn't need to check the validity of a signal that it senses. It should be sufficient to detect the energy, and not to actually decode the signal checking coding, CRCs, frame formats, etc etc etc. | delete "valid", here and on line 51 of same page |
| --- | --- | --- |

Suggestion: reject

Reason:  The notion of valid transmission refers to the sensitivity required to detect valid transmissions.  The difference between CCA levels on a valid transmission and a non-valid one is due to the need to respect valid transmissions better than stray (microwave oven) energy bursts.  The detection of the transmission may be based on energy and on preamble structure.

| 2746 | "On the start of a valid non-HT transmission on the primary channel refer to Clause 17 and 19" Which of the non-HT formats does this include? non-HT duplicate? Or is it any transmission by a non-HT STA? | Use more precise terminology than "non-HT". |
| --- | --- | --- |

Suggestion: counter

TGn Editor replace the following sentence in page 299 line 43 of subclause 20.3.21.5

On the start of a valid non-HT transmission on the primary channel refer to Clause 17 and 19.

With

CCA sensitivity requirements for non-HT frame signals in the primary channel are described in clauses 17 and 19.

| 45 | "The receiver shall hold both the 20 MHz primary channel CCA and the 20 MHz secondary channel CCA busy for any signal present in both the primary and secondary channel that is 20 dB or more above the minimum modulation and coding rate sensitivity (-59 dBm)." This seems to be covered in the previous cases and add nothing. | Delete this cited sentence. |
| --- | --- | --- |

Suggestion: reject

Reason: The previous sentences don’t refer to the case in which transmission is present in both the secondary and the primary.  Sensitivity for transmissions in the secondary are defined in previous sentences only when the primary is idle.

| 2747 | "The receiver shall hold the CCA signal busy for any signal 20 dB or more above the minimum modulation and coding rate sensitivity (-62 dBm)" This is ambiguous. It is not clear whether the -62dBm includes the adjustment of 20dB or not. | Reword so it's unambiguous. |
| --- | --- | --- |

Suggestion: Counter

Replace -62dBm with -82+20=-62dBm in the first time this is mentioned

TGn editor: change the following on page 299 line 55 of  subclause 20.3.21.5.2:

above the minimum modulation and coding rate sensitivity (-82+20=-62 dBm) in the 20 MHz channel.

| 346 | The minimum modulation and coding rate sensitivity of -82 dBm is insufficient for determining whether the secondary channel is idle. | Adapt a sensitivity of -62 dBm or at least -72 dBm. |
| --- | --- | --- |
| 406 | The minimum modulation and coding rate sensitivity of -82 dBm is too high for determining whether the secondary channel is idle. | Lower it to -62 dBm or at least -72 dBm. |

Suggestion: Defer

The group considers that -82dBm is a reasonable sensitivity for secondary channel “valid” transmissions.

| 1804 | This subclause does not cover the sensitivity level for non-GF device receiving a GF packet | add accordingly |
| --- | --- | --- |

Suggestion: Counter:

TGn Editor: Add the following paragraph after line 55 in page 299 in draft 2.0

A receiver that does not support the reception of HT-GF format PPDUs shall hold the CCA signal busy (PHY_CCA.indicate(BUSY)) for any valid HT-GF signal in the 20MHz channel at a receive level equal to or greater than -72dBm.

| 1805 | This subclause does not cover the sensitivity level for non-GF device receiving a GF packet | add accordingly |
| --- | --- | --- |

Suggestion: Counter

TGn Editor: Add the following paragraph after line 9 in page 300 in draft 2.0

In a receiver that does not support the reception of HT-GF format PPDUs, when the secondary channel is idle, the start of a valid 20 MHz HT-GF PPDU signal in the primary channel at a receive level equal to or greater than -72dBm shall cause the PHY to set PHY-CCA.indicate(BUSY) with a probability > 90% within 4 μs. In a receiver that does not support the reception of HT-GF format PPDUs, the start of a valid 40 MHz HT-GF PPDU signal that occupies both the primary channel and the secondary channel at a receive level equal to or greater than -69dBm shall cause the PHY to set PHY-CCA.indicate(BUSY) for both the primary channel and the secondary channel with a probability per channel > 90% within 4 μs.

A receiver that does not support the reception of HT-GF format PPDUs shall hold the CCA signal busy (PHY_CCA.indicate(BUSY, {primary})) for any valid HT-GF signal in the primary channel at a receive level equal to or greater than -72dBm when the secondary channel is idle.   A receiver that does not support the reception of HT-GF format PPDUs shall hold both the 20MHz primary channel CCA and the 20MHz secondary channel CCA busy (PHY_CCA.indicate(BUSY, {primary, secondary})) for any valid 40MHz HT-GF signal in both the primary and the secondary channel at a receive level equal to or greater than -69dBm.