IEEE P802.11
Wireless LANs

| Misc Comment Resolution to 802.11u-D0.03 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-03-08 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Hong Cheng | Panasonic Singapore Laboratories | BLK1022 TaiSeng Ave #06-3530 TaiSeng Ind. Est. Singapore 534415 | +65 65505477 | hong.cheng@sg.panasonic.com |

Abstract

This document describes proposed text changes to resolve comments # 13, 219 (244), 235, 246 of the TGu internal comment resolution summary 11-06-1857r8.

This document is based on P802.11u-d0.03 and P802.11ma-d9.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@ok-brit.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Change the definition of Destination Network as indicated

3. Definitions

Destination Network: The destination/source network for the user plane traffic traveling to and from the non-AP STA accessing the Interworking Serv ices. It may or may not coincide with the SSPN.

Change the following paragraph as indicated.

P.2 QoS Mapping Guidelines for Interworking with External Networks

The EDCA and HCCA mechanism defined in clause 9.9 provides QoS control at MAC layer. However, the QoS control parameters used by the EDCA and HCCA may not match directly with other QoS control parameters of the interworked external networks, e.g. SSPN. For example, the SSPN could use a different QoS scheme, and thus have different metrics for defining the QoS levels. Therefore, mapping from these external QoS control parameters to the  QoS parameters of this standard is necessary.

The QoS parameters mapping may happen for both uplink and downlink data transmission:.

-

For uplink:

At the non-AP STA, external QoS parameters are mapped to IEEE802.11 QoS parameters, e.g. DSCP to IEEE802 User Priority (in turn to EDCA ACs), or  a bandwidth QoS parameters to HCCA parameters. These mappings help the non-AP STA to construct correct QoS request to the QoS AP, e.g. ADDTS Request.

- For downlink:

At the AP, 802.1p User Priority needs to be mapped to EDCA ACs.

[Note: Optionally, the non-AP STAcan use  TSPEC to setup the stream at AP with filtering information (TCLAS).]

As listed above, some of these mapping could be static, e.g. the 802.1p User Priority to EDCA AC mappings. And some mapping could be more flexible, e.g. the DSCP to EDCA AC mapping. Different external network may use different DSCP sets for the same services. For example, a 3GPP network would use different code points from that of a corporate network. The QoS Map distribution mechanism defined in clause 11.10.3 provides means to synchronize the STA’s mapping information with that available from the network.

Insert the following paragraph as indicated under clause 11.10.3

P.2.2 11.10.3.1 QoS Map Distribution Operations

The QoS Map Distribution operation described below provides a method for synchronize the QoS mapping at the STA and AP.

After the non-AP QSTA finished the RSNA security procedures, its SME shall generate a MLME-QoSMap.request primitive to the MLME. The MLME of the non-AP QSTA shall send a QoS Map Request action frame to the AP.

When the MAC entity at the AP receives a QoS Map Request action frame from a non-AP QSTA, the MLME shall generate a MLME-QoSMap.indication primitive to its SME.

When SME of the AP receives a MLME-QoSMap.indication, it shall generate a MLEM-QoSMap.response primitive. The SME of the AP can also decide when to generate an unsolicited MLME-QoSMap.response primitive to the MLME based on its configuration status.

When the MLME of the AP receives a MLME-QoSMap.response, it shall send a QoS Map Configure action frame to the corresponding non-AP QSTA MAC entity.

P.2.311.10.3.2 Transport of QoS Map Set

As described in clause 11.10.3.1, the QoS Map Set IE is carried by the QoS Map Configure frame from AP to a non-AP STA.