IEEE P802.11
Wireless LANs

| Resolution of CID 159 for Some Subclauses |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-03-12 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Charles R. Wright | Azimuth Systems, Inc. | 31 Nagog Park Acton MA 01720 | 978-263-6610 | charles_wright @azimuthsystems.com |

Abstract

This document proposes resolutions for CID 159, as listed in 11-06-872r24.  CID 159 applies to many subclauses; this proposal suggests remedies for subclauses 6.2, 6.7, 6.8, 6.9, 6.17, 6.18, and 6.19.  This proposal differs from that in 11-06/414r1 in that much redundant text in 6.17, 6.18 and 6.19 is removed through reorganization.  See below for additional discussion of the differences.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart@ok-brit.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

This proposal partially resolves CID 159:

Comment: Measured Test system loss should be noted in addition to measurements called out

Suggested Resolution: Add instructions calling out the measurement of test system loss and then recording this loss in the test results

# Explanation of Changes

While working on the proposal for 11-07/0414r1, the author noticed a great deal of redundancy in clauses 6.17, 6.18 and 6.19 in the text in the “Permissible Error Margins” clauses.  The proposal below for these clauses condenses these redundant sections to remove irrelevant differences between the clauses and to reduce the amount of verbage.  This was carried out for both the “Path loss accuracy” subclauses and the “Packet loss counting accuracy” and “Packet latency accuracy” subclauses.  The additional changes proposed in 11-07/0414r1 are also provided here.

The purpose of both documents is to provide two choices for the task group.  In 11-07/0414r1, the changes strictly attempt to address CID 159; in this document, CID 159 is addressed, but it goes on to propose text reorganization believed to cause no normative changes.

# 5.3.3.2 Calibration and Monitoring

Note: this addition intends to address CID 159 as it applies to 6.2.3.4

Add the following sentence to this clause:

The setup path loss should be measured and reported.  Setup path loss is the loss of the components in the RF path connecting the WLCP and the DUT, with any variable attenuators in that path set to their minimum value.

# 6.7 BSS Transition Time

## 6.7.3.5 Permissible Error Margins and Reliability of Test

Add the following text to this subclause:

The actual path loss between an endstation and AP is not relevant to the measurement outcome of the test.  However, the minimum loss (attenuators set to minimum) between the connection to the endstation and connections to each of the APs should be verified by measurement to be within +3 dB of sum of the rated loss of the individual components.

## 6.7.4.4 Reported Results

Add the following text to this subclause:

The minimum and maximum attenuation values used for the attenuation function defined in 6.7.3.4 should be reported, along with the attenuator step size, Tincrease, sweep time and inter-roam period.  The setup path loss (path loss between APs and client with attenuators set to minimum) should also be reported.

# 6.8 Fast BSS Transition Time

## 6.8.3.4 Permissible Error Margins and Reliability of Test

Add the following text to this subclause:

The actual path loss between an endstation and AP is not relevant to the measurement outcome of the test.  However, the minimum loss (attenuators set to minimum) between the connection to the endstation and connections to each of the APs should be verified by measurement to be within +3 dB of sum of the rated loss of the individual components to ensure proper operation of the individual components.

## 6.8.4.4 Reported Results

Add the following text to this subclause:

The minimum and maximum attenuation values used for the attenuation function defined in 6.7.3.4 should be reported, along with the attenuator step size, Tincrease, sweep time and inter-roam period.  The setup path loss (path loss between APs and client with attenuators set to minimum) should also be reported.

Note: from this point on, this document diverges from 11-07/0414r1.

# 6.17 Packet Loss

## 6.17.2.3.1 Path loss accuracy

Insert the following text (indicated by red):

For an accurate measurement of packet loss as a function of path loss, the RF path between the RF connection ports of the AP (or, optionally, the AETE) and the endstation under test (DUT) should be characterized to the desired level of accuracy. There are two components to the accuracy of the path loss. The first is the fixed path loss when the variable attenuator is set to a minimum, and the second is the additional loss introduced as the variable attenuator setting is increased.

## 6.17.2.7 Reported Results

Make the following changes to the first sentence in the clause:

The packet loss should be reported in tabular form for each combination of frame size, frame rate and 1 dB attenuator setting step tested.

## 6.17.3.3.1 Path loss accuracy

Delete the contents of this clause and replace with the following text:

Path loss accuracy should be ensured in the manner described in 6.17.2.3.1, in this case with the loss measured between the endstation (or optionally, the SETE) and the AP under test.

## 6.17.3.3.2 Packet loss counting accuracy

Delete the contents of this clause and replace with the following text:

Packet loss counting accuracy should be ensured using the manner described in 6.17.2.3.2.

## 6.17.3.7 Reported Results

Make the following changes to the first sentence in the clause:

The packet delay should be reported in tabular form for each combination of frame size, frame rate and 1 dB attenuator setting step tested.

# 6.18 Latency

## 6.18.2.3 Permissible error margins and reliability of test

Add the following new subclauses and text after the existing paragraph:

## 6.18.2.3.1 Path loss accuracy

Path loss accuracy should be ensured in the manner described in 6.17.2.3.1.

## 6.18.2.3.2 Packet latency accuracy

The measured packet latency depends on the IEEE Std 802.11 traffic analyzer's ability to receive frames without error, and to accurately record the time at which each frame was transmitted (i.e., timestamp). The RF path loss should be configured to ensure a signal level at the analyzer that is well above its receiver sensitivity and well below its rated maximum input level. Under these conditions, packet reception is limited by the residual error rate of the analyzer's receiver.  The timestamp recorded for each packet should be accurate enough to allow delay measurements to be made at the desired level of accuracy. For instance, in voice applications, an accuracy of 1 ms is sufficient to produce errors smaller than 0.1 in the R-factor computed by the E-model.

## 6.18.2.7 Reported Results

Make the following changes to the first sentence in the clause:

The packet delay should be reported in tabular form for each combination of frame size, frame rate and 1 dB attenuator setting step tested.

## 6.18.3.3 Permissible error margins and reliability of test

## 6.18.3.3.1 Path loss accuracy

Delete the existing contents of this subclause and replace with the following text:

Path loss accuracy should be ensured in the manner described in 6.17.2.3.1, in this case with the loss measured between the endstation (or optionally, the SETE) and the AP under test.

## 6.18.3.3.2 Packet latency accuracy

Delete the contents of this subclause and replace with the following text:

The packet latency accuracy should be ensured in the manner described in 6.17.2.3.2.

## 6.18.3.7 Reported Results

Make the following changes to the first sentence in the clause:

The packet loss should be reported in tabular form for each combination of frame size, frame rate and 1 dB attenuator setting step tested.

Note: the remainder of the proposal matches the same suggested changes in 11-04/0414r1:

# 6.19 Jitter

## 6.19.2.3 Permissible error margins and reliability of test

Add the following sentence to the end of the paragraph:

Path loss accuracy should be ensured using in the manner described in 6.17.2.3.1.

## 6.19.2.7 Reported Results

Make the following changes to the first sentence in the clause:

The packet jitter should be reported in tabular form for each combination of frame size, frame rate and 1 dB attenuator setting step tested.

## 6.19.3.3 Permissible error margins and reliability of test

Add the following sentence to the end of the paragraph:

Path loss accuracy should be characterized using the methodology of 6.18.2.3.1.

## 6.19.3.7 Reported Results

Make the following changes to the first sentence in the clause:

The packet jitter should be reported in tabular form for each combination of frame size, frame rate and 1 dB attenuator setting step tested.

References:

IEEE-802-11-07/0414r1, “Resolution of CID 159 for Some Subclauses,” C. Wright