IEEE P802.11
Wireless LANs

| Dynamic Multi level Power Control |  |  |  |
| --- | --- | --- | --- |
| Date: 2007-May-15 |  |  |  |
| Author(s): |  |  |  |
| Name | Company | Phone | email |
| Floyd Backes | AutoCell Labs | 978-264-4884 | fbackes@autocell.com |
| Joe Kwak | Interdigital | 630-739-4159 | joekwak@sbcglobal.net |
| Roger Durand | RIM | 603-275-4101 | rdurand@rim.com |
| Emily H. Qi | Intel Corp. | 503-264-7799 | Emily.h.qi@intel.com |
| Alex Ashley | NDS Ltd | +44-1784-848770 | aashley@nds.com |

Abstract

This document contains a normative text proposal in support of a mechanism for dynamic multi level transmitter power control in accordance with REQ2040 and REQ2041.  This document is based largely on P802.11v-D0.09, P802.11k-D6.0, and P802.11-REVma-D9.0.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart@ok-brit.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

7. Frame formats

7.1 MAC frame formats

7.2 Format of individual frame types

7.2.3 Management frames

#### 7.2.3.1 Beacon frame format

Insert new row into Table 8 as shown below:

Table 8—Beacon frame body

| Order | Information | Notes |
| --- | --- | --- |
| 36 | Relative Power Limit Request | The Relative Power Limit Request element may be present if dot11WirelessManagementImplemented is true. |

7.2.3.5 Association Response frame format

Insert new row into table 11 as shown below:

Table 11—Association Response frame body

| Order | Information | Notes |
| --- | --- | --- |
| 15 | Relative Power Limit Request | The Relative Power Limit Request element may be present if dot11WirelessManagementImplemented is true. |

#### 7.2.3.7 Reassociation Response frame format

Insert new row into table 13 as follows:

Table 13—Reassociation Response frame body

| Order | Information | Notes |
| --- | --- | --- |
| 16 | Relative Power Limit Request | The Relative Power Limit Request element may be present if dot11WirelessManagementImplemented is true. |

#### 7.2.3.9 Probe Response frame format

Insert new row into Table 15 as follows:

Table 15—Probe Response frame body

| Order | Information | Notes |
| --- | --- | --- |
| 34 | Relative Power Limit Request | The Relative Power Limit Request element may be present if dot11WirelessManagementImplemented is true. |

7.3 Management frame body components

7.3.2 Information Elements

Insert Element IDs x and x+1 into Table 26 and change the Reserved row accordingly:

Table 26—Element IDs

| Information Element | Element ID |
| --- | --- |
| Relative Power Limit Request | x |
| Relative Power Limit Response | X+1 |
| Reserved | X+2-220 |

NOTE—x will be defined.

Insert the following new clauses after clause 7.3.2.63:

7.3.2.64 Relative Power Limit Request element

The Relative Power Limit Request element defines information about the maximum transmit power to be used by the non-AP STA. The format of the Relative Power Limit Request element is shown in Figure v75a.

|  | Element ID | Length (1) | Relative Power Limit |
| --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 |

Figure v86—Relative Power Limit Request element format

The Element ID field is equal to the Relative Power Limit Request value in Table 26.

The value of Length field is set to 1.

The Relative Power Limit field specifies the relative power level and is coded as an unsigned integer in units of decibels. The use of the Relative Power Limit Request element is described in 11.15.10.

7.3.2.65 Relative Power Limit Response element

The Relative Power Limit Response element contains information that is coveyed to a STA to indicate the value of the relative power limit in use by a non-AP for transmitting MPDUs. The format of the Relative Power Limit Response element is shown in Figure v75b.

|  | Element ID | Length (3) | Status | Relative Power Limit Used | Maximum Relative Power Limit |
| --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 1 | 1 |

Figure v75b—Relative Power Limit Response element format

The Element ID field is equal to the Relative Power Limit Used value in Table 26.

The value of Length field is set to 3.

The Status field is set to a value in Table v-3, indicating the STA’s response to the Relative Power Limit requested in Association Responses, (Re) Association Responses or Wireless Management Action frames.

Table v-3 — Relative Power Limit Status values

| Status | Description |
| --- | --- |
| 0 | Accept. |
| 1 | Reject - Unspecified reject reason |
| 2 | Reject - The relative power limit exceeds the maximum value supported by the STA. |
| 3 | Reject - The transmit power of MPDUs cannot be modified by the Relative Power Limit Request element. |
| 4 | Reject - Exceeds Local Power Constraint. The maximum transmit power level specified for the channel in the Country element minus the local power constraint specified for the channel (from the MIB) in the Power Constraint element is less than the maximum transmit power level specified for the channel in the Country element minus the relative power management specified for the current channel (from the MIB) in the Relative Power Limit Request element. |
| 5-255 | Reserved |

The Relative Power Limit Used field is coded as an unsigned integer in units of decibels.  The use of the Relative Power Limit Response element is described in 11.15.11.

The Maximum Relative Power Limit field indicates the maximum relative power limit that can be supported by the STA.  The Maximum Relative Power Limit field is coded an unsigned integer in units of decibles.

7.4 Action frame format details

7.4.8 Wireless Network Management action details

Insert the following rows into table v49 and update the reserved field appropriately:

Table v49 — Wireless Network Management Action field values

| Action field value | Description |
| --- | --- |
| x | Relative Power Limit Request |
| x+1 | Relative Power Limit Response |
| x+2-255 | Reserved |

NOTE—x will be defined.

#### Insert the following clauses after clause 7.4.6.15

#### 7.4.8.16 Relative Power Limit Request frame format

The Relative Power Limit Request frame uses the Action frame body format. It is transmitted by a STA requesting a non-AP STA to limit its maximum transmit power. The format of the frame is shown in Figure v87a.

|  | Category | Action | Dialog Token | Relative Power Limit Request Element |
| --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 3 |

Figure v87a—Relative Power Limit Request frame body format

The Category field is set to the value indicating the Wireless Network Management category, as specified in Table 24 in 7.3.1.11.

The Action field is set to the value indicating a Relative Power Limit Request as specified in Table v48 in clause 7.4.8.

The Dialog Token field is a nonzero value which identifies the Relative Power Limit Request/Response transaction. The dialog token is unique for each Relative Power Limit Request frame sent to a given destination MAC address.

The Relative Power Limit Request Element field contains the Relative Power Limit Request element as described 7.3.2.64.

#### 7.4.6.17 Relative Power Limit Response frame format

The Relative Power Limit Response frame uses the Action frame body format. It is transmitted by a non-AP STA in response to a Relative Power Limit Request frame. The format of the frame is shown in Figure v87b.

|  | Category | Action | Dialog Token | Relative Power Limit Response Element |
| --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 5 |

Figure v87b—Relative Power Limit Response frame body format

The Category field is set to the value indicating the Wireless Network Management category, as specified in Table 24 in 7.3.1.11.

The Action field is set to the value indicating a Relative Power Limit Response as specified in Table v49 in clause 7.4.8.

The Dialog Token field is set to the value in any corresponding Relative Power Limit Request frame. If the Relative Power Limit Response frame is being transmitted, but not in response to a Relative Power Limit Request frame then the Dialog token is set to zero.

The Relative Power Limit Response Element field contains the Relative Power Limit Response element as described 7.3.2.65.

10. Layer management

10.3 MLME SAP Interface

Change 10.3.2 as follows:

10.3.2.2 MLME-SCAN.confirm

10.3.2.2.2 Semantics of the service primitive

Insert the following row at the end of the BSSDescription table:

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Relative Power Limit Request | As defined in the Relative Power Limit Request element | As defined in the Relative Power Limit Request element | Specifies the maximum transmit power to be used by the receiving STA for transmitting MPDUs. This parameter shall only be present if dot11WirelessManagementImplemented is true. |

Change 10.3.6 as follows:

10.3.6.2 MLME-ASSOCIATE.confirm

10.3.6.2.2 Semantics of the service primitive

Change the primitive parameter list as shown:

MLME-ASSOCIATE.confirm		(

ResultCode,

CapabilityInformation,

AssociationID,

SupportedRates,

EDCAParameterSet,

WirelessManagementCapabilities,

FBMSRequest,
TrafficGeneration,
SupportedRegulatoryClasses

Relative Power Limit Request,

)

Insert the following row at the end of the parameter table:

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Relative Power Limit Request | As defined in the Relative Power Limit Request element | As defined in the Relative Power Limit Request element | Specifies the maximum transmit power to be used by the receiving STA for transmitting MPDUs. This parameter shall only be present if dot11WirelessManagementImplemented is true. |

10.3.6.4 MLME-ASSOCIATE.response

10.3.6.4.2 Semantics of the service primitive

Change the primitive parameter list as shown:

MLME-ASSOCIATE.response		(

PeerSTAAddress,

ResultCode,

CapabilityInformation,

AssociationID,

WirelessManagementCapabilities,

FBMSRequest,
TrafficGeneration,
SupportedRegulatoryClasses,

Relative Power Limit Request

)

Insert the following row at the end of the parameter table:

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Relative Power Limit Request | As defined in the Relative Power Limit Request element | As defined in the Relative Power Limit Request element | Specifies the maximum transmit power to be used by the receiving STA for transmitting MPDU. This parameter shall only be present if dot11WirelessManagementImplemented is true. |

Change 10.3.7 as follows:

10.3.7.12 MLME-REASSOCIATE.confirm

10.3.2.12.2 Semantics of the service primitive

Change the primitive parameter list as shown:

MLME-REASSOCIATE.confirm		(

ResultCode,

CapabilityInformation,

AssociationID,

SupportedRates,

EDCAParameterSet,

WirelessManagementCapabilities,

FBMSRequest,
TrafficGeneration,
SupportedRegulatoryClasses

Relative Power Limit Request,

)

Insert the following row at the end of the parameter table:

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Relative Power Limit Request | As defined in the Relative Power Limit Request element | As defined in the Relative Power Limit Request element | Specifies the maximum transmit power to be used by the receiving STA for transmitting MPDUs. This parameter shall only be present if dot11WirelessManagementImplemented is true. |

10.3.7.14 MLME-REASSOCIATE.response

10.3.7.14.2 Semantics of the service primitive

Change the primitive parameter list as shown:

MLME-REASSOCIATE.response		(

PeerSTAAddress,

ResultCode,

CapabilityInformation,

AssociationID,

WirelessManagementCapabilities,

FBMSRequest,
TrafficGeneration,
SupportedRegulatoryClasses

Relative Power Limit Request,

)

Insert the following row at the end of the parameter table:

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Relative Power Limit Request | As defined in the Relative Power Limit Request element | As defined in the Relative Power Limit Request element | Specifies the maximum transmit power to be used by the receiving STA for transimiting MPDUs. This parameter shall only be present if dot11WirelessManagementImplemented is true. |

Insert the follow clauses after 10.3.52:

    - Relative Power Limit request

This set of primitives supports the exchange of relative power limit parameter between peer SMEs. Figure v-3 depicts the relative power limit request process.  The figure illustrates the basic protocol and is only an example and therefore is not meant to be exhaustive of all possible protocol uses.

- Figure v-3—Relative Power Limit Request Protocol Exchange

      - MLME-POWERLTREQUEST.request

        - Function

This primitive requests the transmission of Relative Power Limit request action frame to a peer entity.

        - Semantics of the service primitive

The primitive parameters are as follows:

MLME-POWERLT.request	(
Peer MAC Address,
Dialog Token,
Relative Power Limit Request
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Peer MAC Address | MACAddress | Any valid individual MAC Address, or the Broadcast MAC address | The address of the peer MAC entity to which the Relative Power Limit Request shall be sent. |
| Dialog Token | Integer | 0 – 255 | The dialog token to identify the Relative Power Limit Request transaction. |
| Relative Power Limit Request | Set of Relative Power Limit Request elements | Set of Relative Power Limit Request elements | A set of Relative Power Limit Request elements describing the STA Relative Power Limit information. See 7.3.2.64. |

        - When generated

This primitive is generated by the SME to request that a Relative Power Limit Request frame be sent to a peer entity to convey Relative Power Limit Request information.

        - Effect of receipt

On receipt of this primitive, the MLME constructs a Relative Power Limit Request frame containing the set of Relative Power Limit Request elements specified. This frame is then scheduled for transmission.

      - MLME-POWERLTREQUEST.confirm

        - Function

This primitive reports the result of a Relative Power Limit request.

        - Semantics of the service primitive

The primitive parameters are as follows:

MLME-POWERLTREQUEST.confirm	(
Dialog Token,
ResultCode
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Dialog Token | Integer | 0 – 255 | The dialog token to identify the Relative Power Limit Request transaction. |
| ResultCode | Enumeration | SCCESS, INVALID PARAMETERS, or UNSPECIFIED FAILURE | Reports the outcome of a request to send a Relative Power Limit Request frame. |

        - When generated

This primitive is generated by the MLME when the request to transmit a Relative Power Limit Request frame completes.

        - Effect of receipt

On receipt of this primitive, the SME evaluates the result code.

      - MLME-POWERLTREQUEST.indication

        - Function

This primitive indicates that a Relative Power Limit Request frame has been received requesting a Relative Power Limit Request transaction.

        - Semantics of the service primitive

The primitive parameters are as follows:

MLME-POWERLTREQUEST.indication	(
Peer MAC Address,
Dialog Token,
Relative Power Limit Request
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Peer MAC Address | MACAddress | Any valid individual MAC Address or the broadcast MAC address | The address of the peer MAC entity to which the Relative Power Limit request shall be sent. |
| Dialog Token | Integer | 0 – 255 | The dialog token to identify the Relative Power Limit request transaction. |
| Relative Power Limit Request | Set of Relative Power Limit Request elements | Set of Relative Power Limit Request elements | A set of Relative Power Limit Request elements describing the requested Relative Power Limit Request information. See 7.3.2.64. |

        - When generated

This primitive is generated by the MLME when a valid Relative Power Limit Request frame is received.

        - Effect of receipt

On receipt of this primitive, the SME either rejects the request or commences the Relative Power Limit Request transaction.

    - Relative Power Limit response

This set of primitives supports the exchange of Relative Power Limit parameters between peer SMEs.

      - MLME-POWERLTRESPONSE.request

        - Function

This primitive requests the transmission of Relative Power Limi Response information to a peer entity, in response to a received Relative Power Limit Request frame.

        - Semantics of the service primitive

The primitive parameters are as follows:

MLME-POWERLTRESPONSE.request	(
Peer MAC Address,
Dialog Token,
Relative Power Limit Response
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Peer MAC Address | MACAddress | Any valid individual MAC Address | The address of the peer MAC entity to which the Relative Power Limit request shall be sent. |
| Dialog Token | Integer | 0 – 255 | The dialog token to identify the Relative Power Limit transaction. |
| Relative Power Limit Response | Set of Relative Power Limit Response elements | Set of Relative Power Limit Response elements | A set of Relative Power Limit Response elements describing the STA Relative Power Limit Response information. See 7.3.2.65. |

        - When generated

This primitive is generated by the SME to request that a Relative Power Limit Response frame be sent to a peer entity to convey Relative Power Limit Response information.

        - Effect of receipt

On receipt of this primitive, the MLME constructs a Relative Power Limit Response frame containing the set of Relative Power Limit Response elements specified. This frame is then scheduled for transmission.

      - MLME-POWERLTRESPONSE.confirm

        - Function

This primitive reports the result of a request to send a wireless network management Relative Power Limit response action frame.

        - Semantics of the service primitive

The primitive parameters are as follows:

MLME-POWERLTRESPONSE.confirm	(
Dialog Token,
ResultCode
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Dialog Token | Integer | 0 – 255 | The dialog token to identify the Relative Power Limit transaction. |
| ResultCode | Enumeration | SCCESS, INVALID PARAMETERS, or UNSPECIFIED FAILURE | Reports the outcome of a request to send a Relative Power Limit Response frame. |

        - When generated

This primitive is generated by the MLME when the request to transmit a Relative Power Limit Response frame completes.

        - Effect of receipt

On receipt of this primitive, the SME evaluates the result code.

      - MLME-POWERLTRESPONSE.indication

        - Function

This primitive indicates that a Relative Power Limit Response frame has been received.

        - Semantics of the service primitive

The primitive parameters are as follows:

MLME-POWERLTRESPONSE.indication	(
Peer MAC Address,
Dialog Token,
Relative Power Limit Response
)

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| Peer MAC Address | MACAddress | Any valid individual MAC Address | The address of the peer MAC entity to which the Relative Power Limit response shall be sent. |
| Dialog Token | Integer | 0 – 255 | The dialog token to identify the Relative Power Limit transaction. |
| Relative Power Limit Response | Set of Relative Power Limit Response elements | Set of Relative Power Limit Response elements | A set of Relative Power Limit Response elements describing the Relative Power Limit Response information. See 7.3.2.65. |

        - When generated

This primitive is generated by the MLME when a valid Relative Power Limit Response frame is received.

11. MAC sublayer management entity

11.15 Wireless Network Management procedures

Insert the following clauses after clause 11.15.9:

11.15.10 Relative Power Limit Request and Response Procedure

The Relative Power Limit Request element is used by a STA to communicate to a non-AP STA the maximum transmit power to be used by the receiving STA for transmitting MPDUs.  The maximum transmit power to be used is defined as the lesser of:

1) The maximum transmit power level specified for the channel in the Country element minus the relative power management specified for the current channel (from the MIB) in the Relative Power Limit Request element.

2) The maximum transmit power level specified for the channel in the Country element minus the local power constraint specified for the channel (from the MIB) in the Power Constraint element.

Upon receipt of a Relative Power Limit Request frame or (Re) Association Response frame which includes the Relative Power Limit Request element, the non-AP STA may respond with a Relative Power Limit Response frame including the Relative Power Limit Response element. The Relative Power Limit Response element is used by a non-AP STA to communicate to a STA the actual maximum transmit power to be used by the STA to transmit MPDUs. If the STA supports the requested Relative Power Limit, the STA shall send the Relative Power Limit Response frame with the Status field set to “0” (accept) and the Relative Power Limit Used field set to the requested Relative Power Limit level communicated in the corresponding Relative Power Limit Request frame. If the STA cannot support the requested Relative Power Limit, the STA shall send the Relative Power Limit Response frame with the Status field set to a non-zero value and the Relative Power Limit Used field set to the value that the STA currently uses.

When the Relative Power Limit Request element is received in a Relative Power Limit Request frame the value of relative power limit shall override the value received in a Beacon frame, Probe Response frame, or an (Re) Association Response frame.

Annex A

A.4 PICS proforma-IEEE Std. 802.11, 1999 Edition

A.4.16 Wireless Network Management extensions

Add the following row to the bottom of the table:

| RME11 | Relative Power Limit | 11.15.10 | CFv:M | Yes, No, N/A |
| --- | --- | --- | --- | --- |
| RME11.1 | Relative Power Limit Request | 7.4.8.16 | CFv:M | Yes, No, N/A |
| RME11.2 | Relative Power Limit Response | 7.4.8.17 | CFv:M | Yes, No, N/A |