IEEE P802.11
Wireless LANs

| TXOP comments (Frame Format Adhoc Group) LB97 Comments |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-3-30 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Naveen K. Kakani | Nokia, Inc | 6021 Connection Drive, Irving TX 75039 | +1-214-763-1041 | Naveen.kakani@nokia.com |

Abstract

The submission also addresses 4 comments received on P802.11n Draft 2.0. The CIDs are: 9, 1878, 1879, 2023.

The text is aligned with P802.11-REVma/D9.0 and P802.11n-D2.0.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart@ok-brit.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Submission note: The following 4 CIDs are concerned with su clause 7. The proposed text contained in this document address these comments.

CIDs and their proposed resolution:

| 9 | 52 |  | 7.3.2 | There is a sentence in 7.3.2.29 (baseline) about TXOPLimit=0 case. Following the change made in 9.9.1.2, this sentence should be also changed. | Add clause 7.3.2.29 in the draft and modify the sentence in the baseline "A TXOP Limit field value of 0 indicates that a single MSDU or MMPDU, in addition to a possible RTS/CTS exchange or CTS to itself, may be transmitted at any rate for each TXOP." to "A TXOP Limit field value of 0 indicates that a single MSDU, or MMPDU, or A-MPDU along with any ACK or BlockAck response frame as well as any NDP that are announced within the TXOP, in addition to a possible RTS/CTS exchange or CTS to itself, may be transmitted at any rate for each TXOP." | Counter: Text as indicated in doc: 11-07-0520/r1 |
| --- | --- | --- | --- | --- | --- | --- |
| 1878 | 22 |  | 7.1.4 | The text allows a TXOP holder to force third party stations to set the NAV up to the TXOP Limit duration even in the case the protected TXOP is not entirely used. Since sending a CF-End is not mandatory at the TXOP holder even when it runs out of frames and the remaining protected TXOP is significant (e.g. a couple of milliseconds), this could cause a serious capture effect (i.e. the TXOP holder can re-start decreasing the Backoff window for all its pending ACs, while other stations have to wait until NAV expires). The results is that it is highly probable that the old TXOP holder re-gain access to the channel and other QoS/Legacy transmissions could be either suppressed or seriously delayed. | Add a sentence like this in either this section or section 9.9.1.3 (Obtaining an EDCA TXOP): "the TXOP holder cannot restart decrementing its backoff window, for any of its pending ACs, until its last protected TXOP is elapsed (i.e. NAV is clear at third party stations)" | Counter : Text as indicated in doc: 11-07-0520/r1 |
| 1879 | 22 | 50 | 7.1.4 | The sentence leads to several interpretations. Is it the TXOP Limit (as communicated by the AP in the beacon) or just any value between zero and the TXOP Limit? | Clarify | Counter : Text as indicated in doc: 11-07-0520/r1 |
| 2023 | 22 | 13 | 7.1.4 | "The time, not exceeding the value given by 2) below if TXOP Limit is non-zero, required for the transmission the pending MPDUs of the AC and the associated ACKs, if any, and applicable SIFS durations," What is a "pending MPDU of the AC"? There is no definition of this term, yet it is part of a normative requirement. | Add the definition: "where pending MPDU is an MPDU that will be transmitted in current TXOP, provided that any required acknowledgement (Ack or BlockAck) is received." | Counter : Text as indicated in doc: 11-07-0520/r1 |

Updated with  discussion from Frame Format Adhoc call on April 11th 2007.

TGn Editor: Change the following text in TGn Draft 2.0 to as indicated along with  instructions to the 802.11 base standard editor.

7.1.4 Duration/ID field in data and management frames

------------

1) The time, not exceeding the value given by 2) below if TXOP Limit is non-zero, required for

the transmission of MPDUs of the same AC that are available to be transmitted and the associated responses, if any, and applicable SIFS durations, and

TGn Editor: Change the following text in TGn Draft 2.0 to as indicated along with  instructions to the 802.11 base standard editor.

- 7.2.1.1 RTS frame format

-Otherwise, the duration value is set to any value between:

1) The time, not exceeding the value given by 2) below if TXOP Limit is non-zero, required for

the transmission the pending MPDUs of the AC and the associated ACKs, if any, and applicable

SIFS durations, and

2) The time limit imposed by the MIB attribute dot11EDCATableTXOPLimit

(dot11EDCAQAPTableTXOPLimit for the AP) for that AC minus the already used time within

the TXOP.

TGn Editor: Change the following text in TGn Draft 2.0 to as indicated along with  instructions to the 802.11 base standard editor.

- 7.3.2.29 EDCA Parameter Set element

…. A TXOP Limit field value of 0 indicates that a single MSDU or MMPDU, or A-MSDU or A-MPDU along with any frames required for protection, beamforming or link adaptation and acknowledgements, in addition to a possible RTS/CTS exchange or CTS to itself, may be transmitted at any rate for each TXOP.

TGn Editor: Change the following text in TGn Draft 2.0 to as indicated along with  instructions to the 802.11 base standard editor.

- 9.9.1.2 EDCA TXOPs

The TXOP limit duration values are advertised by the AP in the EDCA Parameter Set information element

in Beacon and Probe Response frames transmitted by the AP. A TXOP limit value of 0 indicates that a single

MSDU or MMPDU or A-MSDU or A-MPDU along with any frames required for protection, beamforming or link adaptation and acknowledgements,

in addition to a possible RTS/CTS exchange or CTS to itself

(or a dual CTS sequence as defined in 9.2.5.5a (Dual CTS protection)), may be transmitted at any rate for

each TXOP. A CF-End frame may be transmitted to cancel remaining NAV time if the prediction of the expected

frame exchange duration was not accurate. An HT-TXOP holder shall set its NAV to the Duration/ID of the last frame on the medium.