IEEE P802.11
Wireless LANs

| Resolution proposal to CIDs 1653, 840, 1949, 1670 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-04-04 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Jeremy Gosteau | Motorola | Parc les Algorithmes de Saint Aubin 91193 Gif sur Yvette, France | +33 169 35 2564 | jeremy.gosteau@motorola.com |
| Marc de Courville | Motorola | Parc les Algorithmes de Saint Aubin 91193 Gif sur Yvette, France | +33 169 35 2518 | marc.de.courville@motorola.com |

Abstract

This document describes resolution proposal to a number of CIDs of LB97, related to the TGn draft 2.0, and regarding STBC features with respect to the Secondary Beacon functionality at the MAC layer.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart@ok-brit.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

# Introduction

## Interpretation of a Motion to Adopt

A motion to approve this submission means that the editing instructions and any changed or added material are actioned in the TGn Draft.  This introduction, is not part of the adopted material.

Editing instructions formatted like this are intended to be copied into the TGn Draft (i.e. they are instructions to the 802.11 editor on how to merge the TGn amendment with the baseline documents).

TGn Editor:  Editing instructions preceded by “TGn Editor” are instructions to the TGn editor to modify existing material in the TGn draft.   As a result of adopting the changes, the TGn editor will execute the instructions rather than copy them to the TGn Draft.

Summission Note: Notes to the reader of this submission are not part of the motion to adopt.  These notes are there to clarify or provide context.

# REVISION NOTES :

r0 – 07/04/04 –

# CID :

Complete list: 1653, 840, 1949, 1670

Suggested resolutions:

| 1653 | Dual Beacons are completely unnecessary at 2.4 GHz because the DSSS PHY modes are more robust than the 6.5 Mbps STBC mode. They only provide 1 dB of downstream link gain at 5 GHz. We are already suffering from "beacon pollution" as the density of access points and the amount of information conveyed in beacons increases. | Remove Dual Beacons from the spec. | Reject - See 07/523r0 The Secondary Beacon is necessary for several reasons exposed and technically justified in submission 06/1402r2 related to Dual CTS and Dual Beacon feature. Main reasons for maintaining this feature are as follows: - Improve perf balance between preamble and data - Do not throw away 0.9dB! (increase of 6% in range, 13% in surface) - It does not show a large penalty in throughput (see 06/1402r2) - Besides, it is OPTIONAL |
| --- | --- | --- | --- |
| 840 | The current definition of STBC does not extend the range so substantially that justify the secondary beacon to be sent by STBC. All the related duplication of management and multicast increases complexity. | Remove the secondary beacon feature from the spec | Reject - See 07/523r0 The Secondary Beacon is necessary for several reasons exposed and technically justified in submission 06/1402r2 related to Dual CTS and Dual Beacon feature. Main reasons for maintaining this feature are as follows: - Improve perf balance between preamble and data - Do not throw away 0.9dB! (increase of 6% in range, 13% in surface) - It does not show a large penalty in throughput (see 06/1402r2) - Besides, it is OPTIONAL |
| 1949 | Secondary beacon transmission is only needed in the case where STBC-capable STAs associated to the HT BSS are unable to receive non-STBC beacons and multicast frames. Given that STBC provides no increase in total range, but rather improvement only in what rates are possible at any given range, and given that all STBC-capable STAs are required to receive non-STBC frames, this mechanism is not needed and the resulting overhead should be eliminated. | Remove all language which allows for secondary beacon transmission as well as secondary broadcast and multicast traffic, as the mechanism and the additional overhead are unwarranted. Be sure to also remove the signalling for these mechanisms in the HT information element definition. | Reject - See 07/523r0 The Secondary Beacon is necessary for several reasons exposed and technically justified in submission 06/1402r2 related to Dual CTS and Dual Beacon feature. Main reasons for maintaining this feature are as follows: - Improve perf balance between preamble and data - Do not throw away 0.9dB! (increase of 6% in range, 13% in surface) - It does not show a large penalty in throughput (see 06/1402r2) - Besides, it is OPTIONAL |
| 1670 | This is completely unnecessary at 2.4 GHz, where DSSS provides better range than STBC. At 5 GHz, STBC provides only 7% range improvement at 6.5 Mbps. Duplicating all the multicast traffic for an extra 7% range makes no sense | Remove the text added to that section. | Reject - See 07/523r0 The Secondary Beacon is necessary for several reasons exposed and technically justified in submission 06/1402r2 related to Dual CTS and Dual Beacon feature. Main reasons for maintaining this feature are as follows: - Improve perf balance between preamble and data - Do not throw away 0.9dB! (increase of 6% in range, 13% in surface) - It does not show a large penalty in throughput (see 06/1402r2) - Besides, it is OPTIONAL Finally, strictly speaking, CID 2544 removed the text added to this section |

References:

IEEE TGn Draft 2.0, February 2007