IEEE P802.11
Wireless LANs

| Changes resulting from the apaption of 11-07/0527 to Section 3 of 802.11.2-D1.0 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-04-05 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Emmelmann, Marc | Technical University Berlin | Einsteinufer 25 10587 Berlin Germany | +49–30–31424580 | emmelmann@ieee.org |

Abstract

This document highlits the changes resulting in Section 3 of 802.11.2-D1.0 if all the motions presented in 11-07/527r0 were approved by the task group.

The following document was created as follows:

- original text of IEEE 802.11.2-D1.0 Section 3 was copied into the document (left unmodified)

- “Track changes” functionality was turned on to highlight the modifications

- Changes according to the motions in 11-07/527r0 were incorporated in the document.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart@ok-brit.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

3. Definitions, acronyms, and abbreviations

3.1 Conformance levels

Several key words are used to differentiate between different levels of requirements and options, as

described in this subclause.

3.1.1 may: Indicates a course of action permissible within the limits of the standard with no implied prefer-

ence ("may" means "is permitted to").

3.1.2 shall: Indicates mandatory requirements to be strictly followed in order to conform to the standard and

from which no deviation is permitted ("shall" means "is required to").

3.1.3 should: An indication that among several possibilities, one is recommended as particularly suitable,

without mentioning or excluding others; or that a certain course of action is preferred but not necessarily

required; or that (in the negative form) a certain course of action is deprecated but not prohibited ("should"

means "is recommended to").

3.2 Definitions

3.2.1 Application layer metric: A metric that measures performance at the application layer of a wireless

local area network.

3.2.2 Calibrated over the air test: A test in which RF signals propagate between the transmitter and

receiver through the air in a calibrated and well characterized environment, with path loss and channel con-

ditions being measured and noted.

3.2.3 Communication test signal: An RF test signal carrying the communication traffic to or from the DUT

or SUT.

3.2.4 Conducted test environment: A test environment where RF test signals are directly injected into and

received from the radio of the DUT(s) using a cable.

3.2.5 Controlled test environment: Any test environment where the circumstances and conditions sur-

rounding or influencing the DUT/SUT are controlled to provide a known level of uncertainty.

3.2.6 Data traffic: Traffic representing the exchange of throughput and loss sensitive data, but not delay or

jitter sensitive data, between two endstations in a wireless local area network.

3.2.7 Data usage case: The usage case representing the flow of data traffic that is not time-critical.

3.2.XXX Device under test (DUT): a single device being tested; e.g. a single client, AP, mesh point, etc. Usually, the DUT is tested in combination with a WLCP.

3.2.8 Endstation: An originator or terminator of traffic, such as an IEEE Std 802.11 STA that is not con-

tained within an AP. (Frequently referred to as a “client” or a “non-AP STA.”)

3.2.9 Extraneous signal: Any signal whose source is external to the test setup, and whose introduction into

the test setup is considered undesirable.

3.2.10 Fresnel zone: One of a (theoretically infinite) number of concentric ellipsoidal regions surrounding

the LOS path between a transmitter and a receiver; electromagnetic waves propagating within the nth Fresnel

zone have a path length that is nλ/2 greater than the LOS path length.

3.2.11 Indoor LOS test environment: An indoor test environment with a LOS channel between the

DUT / SUT and the device emitting the RF test signal.

3.2.12 Indoor NLOS test environment: An indoor test environment with a NLOS channel between the

DUT / SUT and the device emitting the RF test signal (i.e., an unobstructed path does not exist between

the two devices).

3.2.13 Indoor test environment: An over-the-air test environment representing indoor usage of the WLAN.

3.2.14 Internal interference signals: Platform noise or other similar signals that are internal to the DUT/

SUT rather than externally applied as part of the test. This does not include thermal noise.

3.2.15 Jitter: A secondary metric, which is an estimate of the statistical variance of data packet interarrival

time.

3.2.16 Latency: A secondary metric that measures the delay between packet transmission and packet recep-

tion over a data network between two endstations, a transmitter and a receiver.

3.2.17 Latency-sensitive traffic: Traffic that represents the exchange of latency-sensitive data between two

endstations in a wireless local area network.

3.2.18 Latency-sensitive usage case: The usage case representing the flow of latency-sensitive traffic such

as voice conversations.

3.2.19 Line of sight (LOS) channel: A channel between two devices in which there is an unobstructed 1st Fresnel zone

between the two devices. For example this channel is usually modelled using Ricean statistics.

3.2.20 Measurement path: An RF signal path whose attenuation or gain affects the resulting measured

quantity.

3.2.21 Metric: A particular characteristic that is quantified by measurements directly and/or derived from

measurements.

3.2.22 Model: A description of an observed behavior, refined by using specific assumptions that can be used

to establish the correlation between two or more metrics.

3.2.23 Outdoor LOS test environment: An outdoor OTA test environment

with a LOS channel between the DUT / SUT and the device emitting the RF test

signal.

3.2.XXX Outdor OTA test environment: An OTA test environment representing outdoor usage of the WLAN.

3.2.24 Open OTA environment: An unshielded indoor or outdoor OTA environment that resembles an

actual usage situation.

3.2.25 Over-the-air (OTA) test environment: A test environment in which the RF test signals propagate

through the air to and from the antenna of the DUT(s).

3.2.26 Packet loss: A secondary metric that measures packets lost in transmission over a data network

between two endstations, a transmitter and a receiver.

3.2.27 Primary metric: A metric that directly affects the user experience.

3.2.28 Quiet zone: A test volume wherein the desired RF field uniformity is met.

3.2.29 Near-field: The region of space bounded on the outside by a distance r from an antenna or radiating

object with a largest dimension of D such that r < 2D2/λ. This is divided into the reactive near-field, which is

the region of space immediately surrounding the antenna, bounded by a distance r from an antenna such that

0 < r < 2π/λ; and the radiating near-field, which extends from the boundary of the reactive near-field to the

extent of the radiating far field.

3.2.30 Radiating far field: The region of space bounded on the inside by a distance r from an antenna or

radiating object with a largest dimension of D such that r > 2D2/λ; also known as the Fraunhofer region.

NOTE—The distance r above can be approximated as 10λ.

3.2.31 Repeatable in location: The property of test that such that the test can be performed in different loca-

tions and the results from the tests performed at different locations agree, to within the specified precision of

the test.

3.2.32 Repeatable in time: The property of a test such that the test can be performed at different times and

the results from the tests performed at these different times agree, to within the specified precision of the

test.

3.2.33 RF test signal: An RF signal introduced to the DUT/SUT for the purpose of testing its performance

under a given set of conditions.

3.2.34 Secondary metric: A metric that directly affects a primary metric.

3.2.35 Shielded enclosure: A shielded box, screen room, or chamber used to isolate a test environment from

outside RF signals.

3.2.36 Streaming media usage case: The usage case representing the flow of streaming traffic (which in

turn represents the delivery of streaming media such as audio and video) from one endstation to another endstation

in the wireless local area network.

3.2.XXX Sytem under test (SUT): a system of devices, i.e. a specific combination of DUTs, being tested at the same time; e.g. client-AP system or two-AP-one-client system.

3.2.37 Traffic analyzer: An entity that gathers delivered data payload over time via an interface above the

Layer 2 level of the ISO protocol hierarchy; this entity can be a device connected by a cable, embedded soft-

ware, or other means that fulfils the same purpose.

3.2.38 Traffic generator: An entity that generates data traffic from the DUT or WLCP to the WLCP or the

DUT, via an interface above the Layer 2 level of the ISO protocol hierarchy; this entity can be a device con-

nected by a cable, embedded software, or other means that fulfils the same purpose.

3.2.39 Test environment: The set of external physical circumstances and conditions surrounding or influ-

encing the wireless performance of the 802.11 DUT/SUT.

3.2.40 Traffic: A representation of the information flowing over the wireless local area network.

3.2.41 Usage cases: The scenarios (e.g. data oriented, streaming media, latency-sensitive) used to represent

the end-user experience when using WLAN devices.

3.2.42 Wireless counterpart (WLCP): A reference AP or reference endstation, depending on the test

objectives.

3.3 Abbreviations and acronyms

This recommended practice contains the following abbreviations and acronyms:

ACI Adjacent channel interference

ACRMAdjacent channel rejection margin

AETE AP emulation test equipment

AWG Arbitrary waveform generator

CDF Cumulative distribution function

COAT Calibrated over the air test

DF Delay factor

DUT Device under test

DVI Digital visual interface

FER Frame error rate

IBSS Independent basic service set

LOS Line of sight

MAC Medium access control layer

MDI Media delivery index

MLR Media loss rate

MOS Mean opinion score

NIC Network interface card

NLOS Non line of sight

NTSC National television system committee

OTA Over the air

PHY Physical layer

PLCP PHY layer convergence procedure

PPDU PLCP protocol data unit

QoS Quality of service

SETE Station emulation test equipment

SUT System under test

S-VideoSeparate video

TTL Theoretical throughput limits

VB Virtual buffer

VDER Video delivery error rate

VGA Video graphics array

VoIP Voice over Internet Protocol

VPAA Video playback analysis application

VQAAVideo quality analysis application

WLCP Wireless counterpart