IEEE P802.11
Wireless LANs

| TGn LB97 Submission related to FRAME: ASEL, LINK and RD groups |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-05-08 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Adrian Stephens | Intel Corporation |  |  | adrian.p.stephens@intel.com |

Abstract

This document contains proposed changes to the IEEE P802.11n Draft to address the following LB84 comments:

- 134, 136, 137, 1072

These comments relate to the following comment groups:

- LINK ADAPT

- RD

The changes marked in this document are based on TGn Draft version D2.0

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart@ok-brit.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

# Introduction

## Interpretation of a Motion to Adopt

A motion to approve this submission means that the editing instructions and any changed or added material are actioned in the TGn Draft.  This introduction, is not part of the adopted material.

Editing instructions formatted like this are intended to be copied into the TGn Draft (i.e. they are instructions to the 802.11 editor on how to merge the TGn amendment with the baseline documents).

TGn Editor:  Editing instructions preceded by “TGn Editor” are instructions to the TGn editor to modify existing material in the TGn draft.   As a result of adopting the changes, the TGn editor will execute the instructions rather than copy them to the TGn Draft.

Summission Note: Notes to the reader of this submission are not part of the motion to adopt.  These notes are there to clarify or provide context.

# CID 136

| 136 | 19.04 | 7.1.3.5a | "A responder may choose to provide feedback for a previous request at any time, in which case, the MFB and MFSI values correspond to the previous request, even though they may be provided in a frame which is an immediate response to a more current request." Whew! Is that a complicated sentence or what? | Change to: "A responder may provide feedback for an outstanding request at a later time with the MFB and MFSI values of the previous request. Feedback for any outstanding MCS request may be provided as a response to a later MCS request." | LINK ADAPT |
| --- | --- | --- | --- | --- | --- |

## Background information:  CID 2015

| CID | Page | Clause | Resn Status | Comment | Proposed Change | Resolution | Owning Ad-hoc |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2015 | 19.01 | 7.1.3.5a | A | The whole paragraph starting at line 1 needs work: 1. The first two sentences are duplicated in the list that follows 2. The last sentence duplicates the statement at line 19. | Delete the first paragraph (lines 1-6). | Accept | EDITOR |

## Discussion

The paragraph starting “When a responder…” on page 19, line 1 existed before the list  starting on line 8 was added.  My guess is that the intent of the submission was to replace the imprecise text with the more precise list of conditions.  Certainly it’s an imprecise “duplicate” of the text that follows.

## Proposed Resolution: Counter

See resolution to CID 2015, which deletes the referenced text because it is duplicated at line 19 of the same page.

# CID 137

| 137 | 19.12 | 7.1.3.5a | This contradicts the previous statement at line 2 "When a responder responds immediately to an MRQ and sets MFB to all-ones it also sets MFSI to 7." | Perhaps this sentence means that this combination is allowed when NOT responding immediately, in which case such a condition should be added to the text. | LINK ADAPT |
| --- | --- | --- | --- | --- | --- |

## Proposed Resolution: Counter

See resolution to CID 2015, which deletes the paragraph containing the referenced text.

# CID 134

| 134 | 16.25 | 7.1.3.5.3 | There should be reference to RD, 9.14.3, since this ACK policy is valid together with a RD grant. | Add reference to 9.14.3 or a more correct section of 9.14. | RD |
| --- | --- | --- | --- | --- | --- |

## Proposed Resolution: Counter

Add a reference to 9.14 (Reverse Direction Protocol) to the list.

# CID 1072

| 1072 | 67.41 | 7.3.2.49.5 | Support for RD responder should be moved to the Extended Capabilities ID in 7.3.2.27 | as in comment | RD |
| --- | --- | --- | --- | --- | --- |

## Proposed Resolution: Reject

The commenter provides no technical rationale for the proposed change.

The implication of what’s being considered is to decouple support for the RD protocol from support for HT, enabling a non-HT STA to support RD.   However RD is there specifically to support higher throughput through reduction in the number of channel access attempts and aggregation of control (ack) and data in both directions.  While RD may be of some value with a non-HT PHY and without the ability to aggregate control and data, supporting RD in isolation would require additional substantial changes to decouple RD and HT (i.e. to move the RD signalling out of the HT control field) that would add complexity to the standard and implementations.  On balance these undoubted costs outweigh the unproven benefits.