IEEE P802.11
Wireless LANs

| Resolution proposal to CIDs 1599, 1600 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-04-17 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Jeremy Gosteau | Motorola | Parc les Algorithmes de Saint Aubin 91193 Gif sur Yvette, France | +33 169 35 2564 | jeremy.gosteau@motorola.com |
| Marc de Courville | Motorola | Parc les Algorithmes de Saint Aubin 91193 Gif sur Yvette, France | +33 169 35 2518 | marc.de.courville@motorola.com |
| Stéphanie Rouquette Léveil | Motorola | Parc les Algorithmes de Saint Aubin 91193 Gif sur Yvette, France | +33 169 35 4824 | stephanie.rouquette@motorola.com |

Abstract

This document describes corrections to be inserted to the TGn draft 2.0 that will address comments raised in various CIDs of LB97 regarding STBC features at the PHY layer. Those comments mainly address improvements aiming at clarifying the draft text.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart@ok-brit.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

# Introduction

## Interpretation of a Motion to Adopt

A motion to approve this submission means that the editing instructions and any changed or added material are actioned in the TGn Draft.  This introduction, is not part of the adopted material.

Editing instructions formatted like this are intended to be copied into the TGn Draft (i.e. they are instructions to the 802.11 editor on how to merge the TGn amendment with the baseline documents).

TGn Editor:  Editing instructions preceded by “TGn Editor” are instructions to the TGn editor to modify existing material in the TGn draft.   As a result of adopting the changes, the TGn editor will execute the instructions rather than copy them to the TGn Draft.

Summission Note: Notes to the reader of this submission are not part of the motion to adopt.  These notes are there to clarify or provide context.

# REVISION NOTES :

r0 – 07/04/17 – Initial Doc

r1 – 07/04/17 – Updated Version following a comment by Tomoya Yamaura for CID 1600

# CID :

Complete list:

1599, 1600

Suggested resolutions:

| 1599 | There is no explicit statement which symbols STBC can be applied to. | Add statement such as; "STBC or hybrid STBC/SM can be applied only for Data OFDM symbols, i.e., the SERVICE field and PSDU." | Counter: 07/0544r1 Section 20.3.4 (overview of PPDU encoding) should provide along with figures n62 and n64 enough information on the fact that STBC should be applied to DATA symbols. Nevertheless, the commenter requests a clarification and recalls the use of “hybrid STBC/SM” which is defined nowhere and should therefore be eliminated from the draft text As a result, we propose the following changes: - Editor, please remove all (three) occurrences of “hybrid STBC/SM” in the text - Editor, please add a clarifying sentence after the paragraph ending line 21 in 20.3.10.8.1 “STBC can be applied only to Data OFDM symbols” |
| --- | --- | --- | --- |
| 1600 | There is no explicit statement which STA can use STBC. | Add paragraph here such as; " The STA supporting transmitting PSDU with SERVICE field using STBC shall announce its capability with setting Tx STBC field to 1 in the HT Capabilities Info field in all HT Capabilities element that it transmits. The STA supporting receiving PSDU with SERVICE field using STBC shall announce its capability with setting Rx STBC field to 1 in the HT Capabilities Info field in all HT Capabilities element that it transmits." (This seems that it should be described in MAC. If this would be better for us, it would be described in clause 9, rather than in clause 20.) | Accept: 07/0544r1 This comment is more MAC related as it proposes to use the HT Capabilities info field to announce RX and TX STBC capabilities Proposed resolution = Transfer to MAC Informational Note = The usage of the HT Capabilities info field is defined in the MAC section (see Table n25 in section 7.3.2.49.2) |

References:

IEEE TGn Draft 2.0, February 2007