IEEE P802.11
Wireless LANs

| Suggested comment resolution on Power save clause |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2007-05-16 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | Email |
| Kazuyuki Sakoda | Sony Corporation | 5-1-12 Kita-Shinagawa, Shinagawa-ku, Tokyo, 141-0001 Japan | +81-3-5448-4017 | sako@wcs.sony.co.jp |
| Juan Carlos Zuniga | InterDigital | 1000 Sherbrooke W, 10th floor Montreal QC, H3A 3G4 | +1-514-904-6251 | j.c.zuniga@ieee.org |
| Jarkko Kneckt | Nokia | Itämerenkatu 11-13 00180 Helsinki, Finland | +358504821550 | Jarkko.Kneckt@nokia.com |
| Dee Denteneer | Philips | Philips Research, HTC 27 (WL 1.132), 5656 AE Eindhoven, The Netherlands | +31-402-746-937 | dee.denteneer@philips.com |
| Guido R. Hiertz | Philips | ComNets, RWTH Aachen University, Kopernikusstr. 16, 52074 Aachen, Germany | +49-241-802-5829 | hiertz@ieee.org |

Abstract

This document provides the suggested updates to the current draft spec D1.03, which resolve CID92, 93, 109, 110, 774, 775, 780, 1091, 1092, 1465, 1466, 1471, 1472, 1473, 1474, 1480, 1924, 1996, 2000, 2003, 2094, 3437, 3537, 3609, 3767, 3926, 3931, 3934, 3935, 3938, 3941, 3945, 3948, 4453, 4596, 5592, 5597, 5667, 5674, and 5680.

Notice: This document has been prepared to assist IEEE 802.11. It is offered as a basis for discussion and is not binding on the contributing individual(s) or organization(s).  The material in this document is subject to change in form and content after further study. The contributor(s) reserve(s) the right to add, amend or withdraw material contained herein.

Release: The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication.  The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.11.

Patent Policy and Procedures: The contributor is familiar with the IEEE 802 Patent Policy and Procedures <http:// ieee802.org/guides/bylaws/sb-bylaws.pdf>, including the statement "IEEE standards may include the known use of patent(s), including patent applications, provided the IEEE receives assurance from the patent holder or applicant with respect to patents essential for compliance with both mandatory and optional portions of the standard."  Early disclosure to the Working Group of patent information that might be relevant to the standard is essential to reduce the possibility for delays in the development process and increase the likelihood that the draft publication will be approved for publication.  Please notify the Chair <stuart.kerry@philips.com> as early as possible, in written or electronic form, if patented technology (or technology under patent application) might be incorporated into a draft standard being developed within the IEEE 802.11 Working Group. If you have questions, contact the IEEE Patent Committee Administrator at <patcom@ieee.org>.

Notes for the editor:

1)  Change the draft spec D1.03 according to the updates indicated by the following text.

2)  If the 11-07/0268 (Mesh Configuration Element) is adopted, replace all occurrences of “Mesh Capability” with “Mesh Configuration” in the new clause 11A.11, in order to maintain the entire consistency.
There are 2 refernces to “Mesh Capability” in the new clause 11A.11.
Regarding to “Mesh Capability element”, note that this contribution suggests reducing the power management related field to 1 bit, and is renamed to “Power Save Support Enabled”. This bit was formerly named “Supporting Power Save Mode” in D1.03 or “Power Save Supported” in 11-07/0268.

Following text is captured from draft spec D1.03, and modified with “Track Changes” on.

7. Frame formats

7.1.3 Frame fields

7.1.3.1 Frame control field

7.1.3.1.6 Power Management field

Change the text in 7.1.3.1.6 as shown:

The Power Management field is 1 bit in length and is used to indicate the power management mode of a

STA. The value of this field remains constant in each frame from a particular STA within a frame exchange

sequence defined in 9.12.

A value of 1 indicates that the STA will be in PS mode. A value of 0 indicates that the STA will be in active

mode. This field is always set to 0 in frames transmitted by an AP.

In the case of STA in a BSS, the value indicates the mode in which the station will be after the successful completion of the frame exchange sequence.

In the case of MP in a Mesh, the value indicates the mode in which the MP will be after the successfull completion of the frame exchange sequence with all of its peer MPs.

7.2.1.4 PS-Poll frame format

Change the text in 7.2.1.4 as shown:

The frame format for the PS-Poll frame is as defined in Figure 26.

When the frame is transmitted by a STA in a BSS, tThe BSSID is the address of the STA contained in the AP.  When the frame is transmitted by an MP in a mesh, is the BSSID field is simply interpreted as RA. The TA field is the address of the STA transmitting the frame.

When the frame is transmitted by a STA in a BSS, the AID is the value assigned to the STA transmitting the frame by the AP in the association response frame that established that STA’s current association.

When the frame is transmitted by an MP in a mesh, the AID is the value assigned to the power saving MP (MP which operates in power save mode) transmitting the frame by the power save supporting MP (MP which supports power save service) in the association response frame that established that MP’s current peer link.

7.3 Management frame body components

7.3.1.17 QoS Info field

Insert the following text to the end of 7.3.1.17

The format of the QoS Info field, when sent by the MP, is as same as when sent by a non-AP QSTA.

7.3.2 Information elements

7.3.2.49 Mesh Capability element

The “Mesh Capability” element shown in Figure s13 is used to advertise Mesh services. It is contained in

Beacon frames transmitted by MPs, and is also contained in probe request/response messages and (re)association

request/response messages.

| Octets: 1 | 1 | 1 | 4 | 4 | 2 | 1 | 1 | 2 | 4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ID | Length | Version | Active Protocol ID | Active Metric ID | Peer Capacity | Power Save capability | Synch-ronization Capability | MDA Capability | Channel Precedence |

Figure s13: WLAN Mesh Capability Element

The Element ID is set to the value given in Table 26 for this information element. The Length field is set to

18. The version is set to 1.

MPs support one or more path selection protocols and one or more path metrics. However, only one path

selection protocol and one path metric may be active in a particular mesh network at a time. The Active Protocol

ID field indicates the path selection protocol in use and is described in 7.3.2.49.1. The Active Metric

ID field indicates the path selection metric in use and is described in 7.3.2.49.2.

The Peer capacity field indicates the MP’s capacity for establishing additional mesh links and is described in

7.3.2.49.3.

The Power Save capability field indicates support for Power Save mode and is

described in 7.3.2.49.4.

The Synchronization Capability field indicates support for synchronization services and current synchronization

status and is described in 7.3.2.49.5.

The MDA Capability field indicates support for MDA services and current status and is described in

7.3.2.49.6.

The channel precedence field is set to the value of the channel precedence of the unified channel graph to

which the MP interface belongs. It is described in 7.3.2.49.7

7.3.2.49.4 Power Save Capability field

The Power Save capability field is shown in Figure s17.

| B0 | B1 B7 |
| --- | --- |
| Power Save Support Enabled | Reserved |

Figure s17—Power Save Capability field

The “Power Save Support Enabled” bit is set to 1 if the MP is a power save supporing MP and capable of maintaining peer link with MPs in Power Save mode, and set to

0 otherwise.

- Mesh networking

- Mesh discovery and peer link management

- Processing Peer Link Management Messages

- Overview

The MP shall classify the incoming peer link management messages to decide either to accept, reject, or silently ignore the message. If the message contains a broadcast/multicast address in TA, it shall be silently ignored. The result of message process shall trigger an event accordingly (see Error: Reference source not found). The mechanism that so classifies messages is beyond the scope of this standard.

The MP shall verify the link instance identifier in a peer link management message determining whether the identifier identifies a known link instance, fails to match any instance, or is incomplete. The rules for verifying instance identifier are message specific; see Error: Reference source not found, Error: Reference source not found, and Error: Reference source not found.

The MP shall also verify the configuration parameters, if present, conveyed in the Open and Confirm messages. The Mesh Capability and Active Profile Announcement information elements and Frame Control field supply the configuration parameters. If either is present in the Confirm, the MP shall verify that the parameters reported by the Candidate peer MP match those the MP has agreed to use for this link instance. In particular, the MP shall verify the following fields or subfields. This verification is needed to satisfy the consistency property, i.e., to guarantee that MPs agree on the configuration before establishing a mesh link.

- Fields in Mesh Capability element

- Active Protocol ID field

- Active Metric ID field

- Peer Capacity field, including the following subfields

- “Operating as MP”

- Power Save capability, including the following subfields

- Power Save Support Enabled

- Synchronization Capability element

- Supporting Synchronization

- MDA Capability

- MDA Active Request in Mesh

- Active Profile Announcement

- Active Protocol ID

- Active Metric ID

- Frame Control field

- Power Management field

MPs shall verify that the same Path Selection Protocol (identified by Active Protocol ID) and the same Path Selection Metric (identified by Active Metric ID) are used.

MPs shall verify that the same parameters defined in peer capability subfield are used.

The MP shall verify that it supports power save support services when the candidate peer MP sets the Power Management field  to 1.

The MP shall verify that the candidate peer MP supports power save support serves when the the MP intends to operate in Power Save mode.

The MP shall verify that it supports synchronization services when the candidate peer MP sets “Require Synchronization from Peer” subfield to 1.

The MP shall verify that it supports MDA services when the candidate peer MP sets “MDA Active Request in Mesh”.

The MP shall ignore all security related parameters if the RSN information element is not present.

- 11A.11 Power Management in a Mesh

The need for power save in a mesh environment depends on specific scenarios of operation. In certain

scenarios where the MPs are all MAPs or only carry backbone traffic, the devices may not be expected to be

power constrained. Specifically MAPs are expected to be awake all the time. However, in scenarios with

battery powered MPs power save can be useful, since this class of devices are expected to be power

constrained. Power saving in MPs is specified as an optional feature in this standard. The expectation is that

devices manufactured to operate in specific scenarios choose to implement power save mechanism, while

other devices may be spared the additional overhead of supporting it.

- 11A.11.1 Overview

Mesh power management is enabled between an MP which supports power save service and an MP which operates in power save mode.

An MP which supports power save service is, for convenience, referred to as Power Save Supporting MP, and is capable of signal processing and frame delivery scheme to communicate with MP in power save mode. An MP which operates in power save mode or intends to operate in power save mode is, for convenience, referred to as Power Saving MP, and can establish and maintain peer link only with Power Save Supporting MPs.

A Power Save supporting MP shall utilize the frame transmission rule defined in this clause, in order to deliver frames to power saving MPs. The Power Save supporting MP shall initialize the power save service as described in 11A.11.3, and follow the frame transmission rule described in 11A.11.5.  A Power Saving MP shall utlize the frame reception rule defined in this clause, in order to receive frames from Power Save supporting MPs. The Power Saving MP shall initialize the power save service as described in 11A.11.3, change its Power Management Mode as described in 11A.11.2, and follow the frame reception rule described in 11A.11.4.

MPs shall advertise their capability to support power save in the Power Save Support Enabled bit in the Mesh Capability element, included in Beacon

and Probe Response frames.

An MP which is in Power Save mode or is transitting to Power Save mode shall set the Power Management field of the Frame Control field to 1. Such an MP is also called a power saving MP.

In case a neighbor of an MP does not support power save, the MP intending to be in Power Save mode may

choose not to open a peer link with that particular neighbor MP

, or it may choose to operate in active mode and open peer link with that

neighbor MP.

An MP which intends to operate in Power Save mode may reject a peer link establishment attempt from another MP if

this MP does not support power save. Similarly, an MP which does not support power save may reject a peer link establishment attempt from MP whose Power Management field in the Frame Control field is set to 1.

The decision of whether to enter Power Save mode or not should be made

considering the power versus communication constraints. Such a decision can be changed dynamically. An MP may close one or more of the established peer links with neighboring MPs, prior to changing its power management mode from active mode to Power Save mode, in order to conserve power consumption.

An MP that has power save capability may or may not have power save supporting capability and an MP with power save supporting capability may or may not have power save capability.  It is possible for an MP to have both power save capability and power save supporting capability..

- A

- power saving MP shall periodically listen for Mesh DTIM beacons of peer MPs, which is determined by the Mesh DTIM Period field in the Mesh TIM element in Beacon and Probe Response frames transmitted by these peers. A power saving MP waking to transmit or receive a beacon frame shall stay in the awake state for a minimum period of ATIM window as indicated in their Beacon frames, before returning to the doze state.

A power save supporting MP which has a peer link with an MP in power save mode shall buffer MSDUs destined for the MP and only transmit them at designated times. MSDUs that are to be transmitted to an MP in power save mode are first announced via the Mesh TIM element in the beacon frame, or by an ATIM frame transmission during the ATIM window following the Mesh DTIM beacon when neighboring MPs are awake. A power saving MP shall listen for these announcements to determine if it needs to remain in the awake state.

A power save supporting MP which sets up TSPEC with a power saving MP may send traffic to power saving MPs on agreed schedules as negotiated as part of APSD (Automatic Power Save Delivery) TSPEC setup. An MP in Power Save mode which sets up TSPEC with a power save supporting MP shall wakeup according to any negotiated schedule as part of TSPEC setup

with the power save supporting MP. The MP remains in the awake state until the end of the service period.

- 11A.11.2 MP Power Management modes

An MP is  in one of two different power states:,

— Awake: the MP is able to transmit or receive frames and is fully powered.

— Doze:the  MP is not able to transmit or receive and consumes very low power.

The manner in which an MP transitions between these two power states shall be determined by the MP’s

Power Management mode.

— Active mode: the MP shall be in the Awake state all the time

— Power Save mode: the MP alternates between Awake and Doze states, as determined by the frame transmission and reception rules.

An MP shall remain in its current Power Management mode until it informs all of its peer MPs of a Power Management mode change via successful frame exchanges.

An MP may change its Power Management mode from Active mode to Power Save mode only when the following conditions

are fulfilled:

- All of its peer MPs are power save supporting MPs, and set the Power Save Support Enabled bit in the Mesh Capability element in beacon or probe response frame to 1.

- The MP has informed all of its peer MPs that it is changing Power Management mode to Power Save mode. The Power Management mode change is informed through successful frame exchanges initiated by the MP. To notify that it is changing to Power Save mode, the MP shall set Power Management bit in the Frame Control field of the frame to 1.

When an MP intends to operate in Power Save mode, it shall send a Null-Data frame with the Power Management bit in its Frame Control field set to 1, to all of its peer MPs. If a peer MP is in Power Save mode, this frame shall be transmitted during the ATIM window following the mesh DTIM beacon of the peer MP.

If the MP transmits the Null-Data frame in broadcast manner, the MP shall stay in awake state until it receives beacon  frames with the Neighbor Power Management mode bit of the specific MP in the Neighbor List element set to 1, from all the peer MPs to assure that the Null-Data frame was successfully received by all the peer MPs.

If the MP transmits the Null-Data frame in unicast manner, the MP shall stay in awake state until it completes successful frame exchange with all of its peer MPs. Once the MP starts sending this frame, it shall complete the frame exchanges with all of its peer MPs.

Once the MP assures that all the peer MPs recognize its Power Management mode as Power Save mode, it may transmit to Sower Save mode.

When the MP is in Power Save mode, the MP shall set the Power Management bit of the Frame Control field in all transmitted frames to 1 in order to notify its Power Management mode.

An MP changing its Power Management mode from Power Save mode to Active mode shall inform all of its peer MPs of the change by sending a Null-Data frame with the Power Management bit of its Frame Control field set to 0. Once the MP starts sending this frame, it shall stay in awake state all the time, and shall complete the frame exchanges with all of its peer MPs.

If the peer MP is a power saving MP, this frame shall be transmitted during the ATIM window following the mesh DTIM beacon of the peer MP.

The algorithm to trigger the change of power management mode is beyond the scope of the standard.

- 11A.11.3 Initialization of Power Management within a mesh

All the Power saving MP or MP which intends to operate in Power Save mode shall include Mesh ATIM parameter element in beacon and probe response frames, in order to announce the presence of the ATIM window and its ATIM window parameter to its neighbor MPs. The ATIM Window is defined as a specific period of time, defined by the value of the ATIM Window parameter in the Mesh ATIM parameter element.

The start of the ATIM window is measured from TBTT.

Power save supporting MP shall include Mesh TIM element in beacon and probe response frames in order to announce its DTIM count to its neighbors, even if it does not maintain peer link with power saving MP currently.

Two different TIM types are distinguished: TIM and DTIM. After a DTIM, the power save supporting MP shall send out the buffered broadcast/multicast MSDUs using normal frame transmission rules, before transmitting any unicast

frames.

Every DTIMPeriod, a TIM of type DTIM is transmitted within a beacon, rather than an ordinary TIM. The power save supporting MP expects that its peer MPs in Power Save mode wake up per TBTT when the beacon with DTIM is transmitted. The beacon frame with DTIM is referred to as mesh DTIM beacon.

Synchronizing MP may not transmit beacon frame regularly, based on the rules described in the beaconing subclause.

The power management initialization procedure depends on which synchronization mode the MP is operating.

- 11A.11.3.1 Initialization of Power Management of unsync MP

An unsynchronizing MP that creates or joins a mesh shall set the ATIM window, beacon interval, Mesh DTIM interval, and power management mode. These parameters shall be advertised in its Beacon frames regularly.

Unsynchronizing MPs may utilize the independent parameters such as ATIM window, Mesh DTIM interval, regardless of these parameters utilized by peer MPs.

A power save supporting unsynchronizing MP assigns AID to every peer MP through the peer link establishment procedure. AID 0 is reserved to indicate the presence of buffered MSDUs with a groupaddresse such as broadcast and multicast. This AID is used so that the MP identifies those peer MPs for which it is prepared to deliver buffered MSDUs by setting bits in the TIM’s partial virtual bitmap that correspond to the appropriate AIDs. MP shall assign AID value uniquely to each of the peer MPs. In case both MPs are power save supporting MP in establishing a peer relationship, both MPs assign AID each other, unidirectionally.

- 11A.11.3.2 Initialization of Power Management of sync MP

Synchronizing MPs shall utilize the same parameters such as ATIM window, Mesh DTIM interval, among all the synchronizing peer MPs.

A synchronizing MP that creates a mesh or joins a mesh where all the neighbor MPs are unsynchronizing MPs shall set the  ATIM window, beacon interval, Mesh DTIM interval, and power management mode. These parameters are advertised in its Beacon frames.

A synchronizing MP that joins a mesh where one or more neighbor MPs are synchronizing MPs shall update its ATIM window and Mesh DTIM interval according to the values of received Beacon frames from the synchronizing MP and set beacon interval and power management mode. These values are advertised in its Beacon frames.

- 11A.11.4 Receive operation for MPs in Power Save mode

A power saving MP shall enter awake state prior to every TBTT that matches the mesh DTIM beacon timing of its own and of its neighbor MPs with which it maintains a peer relationship. A Power saving MP shall also enter awake state at the TBTT when the MP is scheduled to transmit beacon frame.

After the Mesh DTIM beacon reception or the beacon transmission, the MP in PS mode may return to the doze state if any of the neighbor MPs did not make any announcement that the neighbor MP has a frame to transmit to the MP in PS mode, as described later in this subclause.

If an MP enters awake state in order to receive Mesh DTIM beacon and receives broadcast/multicast MSDUs, it shall remain awake until the More Data field of the broadcast/multicast MSDUs indicates there are no further buffered broadcast/multicast MSDUs or until a TIM is received indicating there are no more buffered broadcast/multicast MSDUs.

A power saving MP may transit to awake state if it has traffic to transmit at any given point of time. If an MP transmits beacon frame containing Mesh TIM element indicatig the buffered traffic existence, it shall stay awake at least until the next TBTT. If an MP transmits ATIM frame or other frames during the ATIM window in order to waken its peer MP, it shall stay awake at least until the next TBTT.

The designated beacon broadcaster can enter the doze state where it sends only mesh DTIM Beacon frames and

stay awake for the next beacon period. MPs shall not send any frames to the designated beacon broadcaster during

normal beacon periods.

If the designated beacon broadcaster is in Power Save mode it sets the Power management mode of reporting MP bit of

the MP control field in the Neighbor List element to 1.

- 11A.11.4.1 Receiving frames from unsync MP

Power saving MPs shall operate as follows to receive an MSDU or management frame from the power save supporting unsynchronizing MP.

- If a power saving MP receives a Beacon frame with the bit corresponding to its AID is set in the Mesh TIM element, the power saving MP shall stay awake after the beacon reception and issue a PS-Poll to retrieve the buffered MSDU or management frame. If the More Data field in the received MSDU or management frame indicates that more traffic for the Power Saving MP is buffered, the Power Saving MP shall remain in awake state, and,  at its convenience, shall Poll until no more MSDUs or management frames are buffered for that STA.

- If a power saving MP transmits beacon frame at a certain TBTT, it shall stay in awake state at least during the ATIM window.
If the power saving MP receives an ATIM frame or other frames with the more data bit in the frame control field set to 1 during the ATIM Window following the beacon transmission, it shall stay awake after the ATIM window expiration, waiting for the announced MSDU(s) or management frames to be received.

- If a power saving MP does not receive beacon frame indicating buffered traffic directed to it, and does not receive any frame during the ATIM window following the beacon transmission, it may return to doze state after the ATIM window expiration.

- If an MP operating in Power Save mode received a frame with the more data bit in the frame control field cleared from all the peer MPs that announce the buffered traffic, it may return to doze state.

- 11A.11.4.2 Receiving frames from sync MP

Power savin MPs shall operate as follows to receive an MSDU or management frame from the power save supporting synchronizing MP.

- An MP that entered the awake state due to the Mesh DTIM TBTT event and had not sent an ATIM, a broadcast frame, or a multicast frame, and did not receive a unicast or multicast ATIM may return to the Doze state following the end of the ATIM window

- If an MP received an ATIM frame, broadcast frame, or multicast frame, it may return to doze state after receiving a frame with the more bit in the control field cleared from all the sources that sent an ATIM, broadcast, or multicast frame during the ATIM window.

- An MP receiving a broadcast or multicast frame during the ATIM window with the more data bit of its control field cleared may return to the doze state either following the end of the ATIM window or after receiving a frame with the more bit in the control field cleared from all other active sources, whichever comes later.

- 11A.11.4.3 Receive operation using APSD

A Power Saving MP using APSD shall operate as follows to receive an MSDU or management frame from the Power Save Supporting MP which is capable of supporting APSD.

- If a periodic SP has been set up, the Power Saving MP wakes up at its scheduled start time. (The Power Saving MP shall wake up early enough to receive transmissions at the scheduled SP.)

- If the Power Saving MP is initiating an aperiodic SP, the Power Saving MP wakes up and transmits a trigger frame to the Power Save Supporting MP. When one or more ACs are not delivery-enabled, the Power Saving MP may retrieve MSDUs and MMPDUs belonging to those ACs by sending PS-Poll frames to the Power Save Supporting MP.

- The Power Saving MP shall remain awake until it receives a QoS data frame addressed to it, with the EOSP subfield in the QoS Control field set to 1.

- The Power Saving MP may send additional PS-Poll frames if the More Data subfield is set to 1 in receiving unicast data or management frames that do not belong to any deliver-enabled ACs. The Power Saving MP may send additional trigger frames if the More Data subfield is set to 1 in receiving unicast data or management frames that belong to delivery-enabled ACs.

- 11A.11.5 Transmit operation for MPs transmitting to MPs in Power Save mode

The Power Save supporting MP which has a  peer link with Power Saving MP shall not arbitrarily transmit MSDUs to Power Saving MPs, but shall buffer MSDUs and only transmit them at designated times so that the frames are received by Power Saving MPs.

All broadcast and multicast traffic shall be buffered by Power Save supporting MPs that maintains peer relationship with Power Saving MP. These frames are transmitted immediately after the Mesh DTIM beacon transmission.

All unicast frames targeted to Power Saving MPs shall be buffered.

Prior to the transmission of unicast frames that are to be received by a Power Saving MP, Power Save supporting MP shall announce the buffered traffic existence. This announcement scheme depends on which synchronization mode the Power Save supporting MP is operating.

- 11A.11.5.1 Operation of power save supporting unsync MP

Unsynchronizing MPs announce the traffic by transmitting a beacon frame with Mesh TIM element indicating the existence of the traffic buffered to the Power Saving MP.

At every beacon interval, the Power Save supporting unsync MP shall assemble the partial virtual bitmap containing the buffer status per destination for peer MPs in the Power Save mode and shall send this out in the Mesh TIM field of the beacon.

Its peer MP in Power Save mode shall issue the PS-Poll frame upon the reception of such a beacon frame to retrive the frame, and buffered frames at the power save supporting MP are transmitted upon the reception of PS-poll frames.

The More Data field of the transmitting Data frame shall be set to indicate the presence of further buffered MSDUs or management frames for the polling power saving MP.

Power Save supporting unsync MPs may transmit frame to a power saving MP during the ATIM window in order to signal them to remain awake and wait for further traffic, if it does not establish peer link with the power saving MP. After the positive acknowledgement upon the frame transmission during the ATIM window, MP may further transmit buffered frame to the Power Saving MP.

- 11A.11.5.2 Operation of power save supporting sync MP

Synchronizing MPs announce the traffic by transmitting ATIM frames or null frame to an MP in Power Save mode during ATIM window, in order to signal them to remain awake and wait for further traffic. Synchronizing MP shall follow the

Frame transmission rules as following.

- The only types of frames MPs may transmit during the ATIM window of synchronizing MPs are ACK, CTS, RTS, ATIM, Beacon, broadcast or multicast MPDU and Null Data frames. An MP may transmit any type of frames during the ATIM window of unsynchronizing MPs.

- An MP may transmit one short broadcast or multicast MPDU during the ATIM window if the MAC frame length of the MPDU is less than dot11shortMulticastFrameLengthLimit. If the MP has more than one broadcast or multicast frame to transmit, it should set the more data bit of the broadcast or multicast frame transmitted during the ATIM window and contend for the channel following the end of the ATIM window to transmit the additional frames.

- MPs that transmit to MPs in Power Save mode (including broadcast and multicast) shall set the More data bit in frame control headers to indicate if more frames are to be transmitted for the specific destination.

- All other aspects of ATIM based transmission are as defined in 11.2.2.4

- 11A.11.5.3 Operation of APSD supporting MP

Power Save Supporting MPs that signal their support of APSD shall maintain an APSD and an access policy status for each ACs of peer MPs which utilize APSD.

If the Power Saving MP has set up to use aperiodic SPs, the Power Save supporting MP shall buffer frames belonging to delivery-enabled ACs until it has received a trigger frame associated with a trigger-enabled AC from the Power Saving MP, which indicates the start of an aperiodic SP. The Power Save supporting MP transmits frames destinated for the Power Saveing MP and associated with delivery-enabled ACs during aperiodic SP.

For traffic that belongs to a flow for which an APSD TSPEC and schedule was setup with another MP, the transmission is performed according to the agreed schedule.

When the Power Save Supporting MP has transmitted a directed frame to the Powe Saving MP with the ESOP subfield set to 1 during the SP and receives acknowledgement, the Power Save Supporting MP shall not transmit any more frames using this mechanism until the next SP. The Power Save Supporting MP shall set the ESOP subfield to 1 to indicate the end of the SP in APSD.

- 11A.11.6 Power management with APSD

MPs capable of supporting automatic power save delivery (APSD) shall signal this capability through

the use of the APSD subfield in the Capability Information field in Beacon, Probe Response, and (Re)Association

Response management frames. APSD is enabled only at the peer link which both MPs are capable of APSD.

APSD is an additional frame delivery scheme to a basic Power Save mode, and is enabled only at the power save supporting MP and power saving MP.

In mesh, APSD defines two delivery mechanisms,

periodic and aperiodic APSD. The periodic APSD mode is similar to

Scheduled APSD. The aperiodic APSD is similar to Unscheduled APSD. The periodic and aperiodic APSD

modes use the same signaling as scheduled and unscheduled APSD

.

- 11A.11.6.1 Aperiodic APSD

The power saving  MP

transmits trigger frame to the Power Save supporting MP in order to trigger aperiodic SP to receive  frames from the Power Save Supporting MPtraffic back.

If there is no unscheduled SP in progreseduled SP begins when the MP receives a trigger frame from a peer MP in power save modePower Save mode, which is a QoS data or QoS Null frame associated with an AC the STA has configured to be trigger-enabled. An aperiodic SP ends after the Power Save supporting MP has attempted to transmit at least one MSDU or MMPDU associated with a delivery-enabled AC and destined for the Power Saving MP, but no more than the number indicated in the Max SP Length field if the field has a nonzero value.

In order to configure an Power Save Supporting MP to deliver frames during an aperiodic SP, the Power Saving MP designates ots ACs to be delivery-enabled and one or more of its AC to be trigger-enabled. A Power Saving MP may configure an Power Save Supporting MP to use U-APSD using two methods. First, a non-AP STA may set individual U-APSD Flag bits in the QoS Info subfield of the QoS Capability element carried in (Re)Association Request frames.

When a U-APSD Flag bit is set, it indicates that the corresponding AC is both delivery- and trigger-enabled. When all four U-APSD Flag subfields are set to 1 in (Re)Association Request frames, all the ACs associated with the Power Saving MP are trigger- and delivery-enabled during (re)association. When all four U-APSD Flag subfields are set to 0 in (Re)Association Request frames, none of the ACs associated with the non-AP STA is trigger- or delivery-enabled during (re)association.

- 11A.11.6.2 Periodic APSD

Periodic APSD can be used for all cases (i.e., both MPs in power save or only one, EDCA or HCCA access).

The ADDTS request is modified to include a Schedule element that describes the requested schedule from

the MP. The ADDTS response includes the Schedule that can be supported by the other MP and that should

be used for this flow. If this schedule is not acceptable to the originating MP it may reattempt the ADDTS

request with modified schedule or tear down the flow. An unsynchronizing MP shall set the service start

time field of the TSPEC element to the four lower octets of the TSF timer value of the MP that initiates the

ADDTS request.

For periodic APSD both sides can initiate transactions as long as they are sent within the service interval

defined. The service interval lasts up to the maximal service duration as defined in the schedule information

element or if EOSP is declared in frames sent/received for that flow. For a unidirectional flow the

originating MP sets the EOSP when it wants to end the service interval. The interval is considered

terminated once the ACK is received for that frame (if ACK is required). For a bidirectional flow the

service period ends only after both ends of the flow send a frame with EOSP bit set and the matching ACK

frames are received.

An MP wishing to reinstate a TS with another MP that is operating in Power Save mode sends a QoS-Null

frame to the MP in Power Save mode during its ATIM window.

- T.9 Power Save parameters selection

The power save operation among synchronizing MPs is controlled by a set of global parameters. The following are the global

mesh parameters with their default recommended values:

Beacon Period: 100TU

Mesh DTIM period: 10

ATIM Window: 10TU

MPs may wish to use other parameters but doing so may effect the power save efficiency and also delay the

service initiation in the mesh.

Unsynchronizing MPs may utilize individual parameters regardless of these parameters of neighbor MPs or peer MPs.

MP which tries to establish a new peer link with MPs in Power Save mode shall perform passive scan.

Since MPs in Power Save mode may transmit beacon at low rate, MP which tries to establish a new peer

link with MPs in Power Save mode should perform passive scan relatively longer time compared to passive

scan in BSS infrastructrure mode operation. MPs in Power Save mode which sets longer DTIM interval

may not be found by the neighbor MPs due to the shorter scan duration. MPs in Power Save mode should

set shorter DTIM interval, if it intends to establish a new peer link with MPs with higher probability.

References:

[1] Draft Amendment: ESS Mesh Networking.  doc.: IEEE P802.11s/D1.03, April 2007.

[2] 11-07-0023-28-000s-lb93-tgs-draft-1-0-comment-spreadsheet.xls

[3] 11-07-0548-01-000s-suggested-comment-resolution-on-power-save-clause.ppt